| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F260 |
| --- | --- |

| Title: | CE3: Cross-check report for test2.1 JCTVC-F247 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81-3-5435-3308 Kenji.Kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification result of CE3 Subtest 1 described in JCTVC-F247. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

# Software analysis

The following modifications are confirmed through the analysis of software provided by Samsung:

- The number of filter tap is changed from 8 tap to 6 tap:

  - The filter coefficients are {2, -9, 57, 19, -7, 2} and {1, -7, 38, 38, -7, 1} for quarter pel and half pel.

# Results

Software provided by Samsung was successfully compiled by Gnu Compiler Collection (GCC) version 4.1.2-50

Simulation was conducted according to the CE3 common test coding conditions based on JCTVC-E700. Attached Excel sheets, JCTVC-F260.xls, contain the detailed results. It was confirmed that BD-PSNR/Rate values obtained are the same as those of provided by proponent.

And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.4GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output. However, we were not given simulation time by Samsung, So we don’t verify about simulation time.

# Conclusion

Except with simulation time, it is confirmed that the CE3 test 2.1 results reported by proponent are accurate.

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).