| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F268 |
| --- | --- |

| Title: | Unified PIPE-Based Entropy Coding for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Detlev Marpe, Heiner Kirchhoffer, Benjamin Bross, Valeri George, Tung Nguyen, Matthias Preiß, Mischa Siekmann, Jan Stegemann, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-619 detlev.marpe@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents a unified entropy coding architecture for HEVC. The proposed method is based on probability interval partitioning entropy (PIPE) coding and supports two operation modes: a low-complexity (LC) mode as a substitute for CAVLC and a high-efficiency (HE) mode as a substitute for CABAC. Both entropy coding modes of this unified architecture share the same syntax and semantics, the same binarization schemes as well as the usage of the same set of PIPE/V2V codes. In addition, a common process for initialization of probability models based on 8-bit initialization values is included. The HE operation mode is conceptually similar to CABAC in the sense that it provides the same degree of adaptivity, both by exploiting higher order statistical dependencies through the use of the same set of conditional probabilities and by adapting the model probabilities to the actual source statistics using the same probability estimation process. In the LC operation mode, all conditional probabilities based on prior encoded/decoded symbols as well as all adaptive probability models are replaced by appropriately initialized, fixed model probabilities. In terms of coding efficiency, both operation modes of the proposed unified entropy coding method provide approximately the same R-D performance as their currently specified counterparts. The measured decoder run times in HE and LC configuration indicate some advantages in terms of decoding complexity relative to CABAC and CAVLC, respectively.

# Problem statement and proposed solution

The current Working Draft (WD3) of HEVC includes with CAVLC and CABAC the support of two conceptually different entropy coding schemes. Although both schemes are currently used in different application-oriented test configurations, certain risks and conflicts are inherently involved, especially when considering the experience gained during standardization of H.264/AVC and subsequent efforts of standard compliant product development. Some of these potential problem areas are given as follows.

- Burden of higher developing effort during standardization phase by maintaining support for two entropy coding schemes

- Lack of common subset of tools (i.e., lack of complexity scalability) when defining profiles and levels

- Burden of increased implementation and verification efforts for product development

- Burden of increased area and power cost when the need for simultaneous support of two entropy coding schemes (like in H.264/AVC Main and High Profiles) is required

- Higher risk of conformance defects

As a proposed solution to these problems, this contribution presents a unified entropy coding scheme. Unification is achieved by defining a common subset of tools for use in a low-complexity (LC) operation mode (as a substitute for CAVLC) and a high-efficiency (HE) operation mode (as a substitute for CABAC). Both operation modes are designed under the constraint of achieving (at least) the same R-D performance and throughput as their currently specified counterparts. Note however that our generic approach is complexity scalable in the sense of being capable of maintaining a wide spectrum of operation points in terms of trade-off between coding efficiency and implementation complexity.

# Overview of the unified PIPE-based entropy coding framework

The proposed unified entropy coding approach is based on probability interval partitioning entropy (PIPE) coding [1][2]. It supports two operation modes:

- Low complexity (LC) PIPE

- High efficiency (HE) PIPE

Both entropy coding modes are sharing

- the same syntax and semantics

- the same binarization schemes for all syntax elements (as currently specified for CABAC)

- the usage of the same PIPE/V2V codes

- the usage of 8 bit probability model initialization values (instead of 16 bit initialization values as currently specified for CABAC)

LC-PIPE is operating under the following constraints:

- All probability models are non-adaptive, i.e., all models will be initialized at the beginning of each slice with appropriate model probabilities (depending on the choice of slice type and slice QP) and will be kept fixed throughout processing of the slice.

- All context-based selection processes for deriving conditional probabilities based on prior encoded/decoded symbols are completely removed.

- Only 8 different model probabilities corresponding to 8 different PIPE/V2V codes are supported.

- Some specific syntax elements for residual coding, like significant_coeff_flag and coeff_abs_level_greaterX_flag (with X=1,2), are using multiple (fixed) probability models such that the selection simply depends on the number of prior encoded/decoded bins.

HE-PIPE is conceptually similar to CABAC with the following differences:

- Binary arithmetic coding (BAC) is replaced by PIPE coding (same as in the LC-PIPE case).

- Each probability model, i.e., each ctxIdx is represented by a valMps, pipeIdx and a refineIdx, where pipeIdx with values in the range from 0...7 represents the model probability of the 8 different PIPE/V2V codes. This change affects only the internal representation of states, not the behavior of the state machine (i.e., probability estimation) itself.

- Initialization of probability models use 8 bit initialization values as stated above.

- Backward scanning of syntax elements coeff_abs_level_greaterX_flag (with X = 1, 2), coeff_abs_level_minus3, and coeff_sign_flag is performed along the same scanning path as the forward scan. Context derivation for coding of coeff_abs_level_greaterX_flag (with X = 1, 2) is also simplified.

Figure 1: Block diagram of a CABAC encoder (neglecting the CABAC initialization process).

# Brief analysis of CABAC data dependencies in view of a unified entropy coding framework

CABAC is a highly sequential process, both for encoding and decoding. For sake of clarity, we restrict our following brief analysis of the corresponding data dependencies on the CABAC encoding process. Figure 1 shows a high-level block diagram of a CABAC encoder. Basically, it consists of three processing stages: the binarization stage, the stage of probability estimation and assignment of conditional probabilities, also referred to as the context modeling stage, and finally the binary arithmetic coding stage. Apart from the binarization stage, the remaining two stages require sequential data processing. The most critical part in that regard is given by the binary arithmetic encoder/decoder (BAC). In the regular BAC mode, the range of the code interval (codeIRange) gets updated depending on every bin value to encode/decode, while for processing the next bin value the updated interval range is used to look-up the estimated range codeIRangeLPS of the corresponding LPS-related code interval. Even in the bypass BAC mode, where the range of the code interval is kept fixed, the lower bound of the code interval (or the interval offset in the decoder) gets updated for every bin such that this update propagates into subsequent BAC cycles. Given this severe restrictions, it is obvious that even by removing all further data dependencies (as discussed below), the throughput of any unified entropy coding design cannot be increased substantially without substituting the BAC stage by a more SIMD friendly and/or parallel processing friendly design.

For the second CABAC stage, there are dependencies resulting from a feedback loop that has its source at the regular BAC module, as shown in Figure 1. The corresponding feedback of already encoded/decoded bins serves two purposes. First, it is used in the context derivation process for the encoding/decoding of specific bins only. For this purpose, the values of prior decoded/encoded bins need to be stored in a so-called context memory, as illustrated in Figure 1. Typically, for most syntax elements in the current draft, context derivation involves the values of the two neighboring syntax elements to the left (L) and above (A) the current syntax element to encode/decode. As a second purpose, the feedback loop feeds the probability estimator with already encoded/decoded bins in order to update the model probability pLPS(i,t) for a given bin index i at each time instant t. This dependency is critical especially for those cases, where multiple consecutive bins are sharing the same probability model. The latter aspect is also important in view of a PIPE-based coding approach, where we want to process multiple consecutive bins by a fixed PIPE encoder/decoder in one cycle, which in turn requires the model probabilities associated to those bins to stay in the same assigned probability interval.

Figure 2: Block diagram of the proposed unified PIPE-based entropy coding scheme in HE operation mode. Highlighted areas of the block diagram correspond to parts of the common subset of tools (neglecting the PIPE initialization process).

# The HE operating mode of unified PIPE-based entropy coding

The proposed unified entropy coding framework is designed in a way that it serves different needs in terms of trading off coding efficiency against implementation complexity. Given the coding efficiency/complexity trade-off of the current CABAC method in WD3 as the high efficiency (HE) target, we started with a design that borrows as much from the current WD as possible. Figure 2 illustrates the functional elements of the corresponding HE operation mode of the proposed framework. Comparing this block diagram to that of CABAC in Figure 1 it can be seen that the most relevant change is given by the replacement of BAC by PIPE.

As already noted in the previous section, this algorithmic change is necessary in order to serve the higher throughput needs of an adequately designed low complexity (LC) operation point, where we want to make use of the multiple-bin processing capability of PIPE. The specific instance of V2V codeset that is used for the unified design of PIPE is a stripped-down version of the systematic V2V codes as presented in [2]. This initial set of 10 V2V codes was further condensed in order to finally arrive at a minimal codeset of 8 V2V codes. Thus, for statistical coding, only 8 different representative probability values corresponding to 8 disjoint probability intervals are supported.

By referring to the naming conventions introduced in [2], the proposed set of V2V codes is given as {EP, BP3, BP2, TB, UR2, UR3, UR4, UR5} with the four code families EP = equal probability (uncoded), BP = bin-pipe code, TB = three-bin code, and UR = unary-to-rice code. Since the set of 63 representative probability states (pStateIdx) remains the same as in CABAC, a unique assignment of CABAC states to PIPE identifiers (ID) with the PIPE ID being identical to the order of V2V codes in the set as given above is required. This assignment is given in Table 1 and is used in the module “probability interval selector” of the block diagram in Figure 2 in order to select the proper PIPE encoder/decoder for the given bin to encode/decode.

Adaptive probability estimation in our HE operating mode is identical to what is currently specified for CABAC by means of the state transition table (9-53) in the current WD. Apart from a few exceptions for the syntax elements coeff_abs_level_greaterX_flag (with X = 1, 2), as further described in section 6 below, context derivation in the HE operating mode is also identical to the corresponding CABAC specifications. As already stated above, all binarization schemes of CABAC have been reused without any change for both the HE and LC operation mode of our proposed unified entropy coding framework.

Regarding initialization of probability models, i.e., context variables, we have introduced a modification relative to the current WD which is described in section 10.

Table 1: Assignment of CABAC states to PIPE IDs

| CABAC state | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIPE ID | 0 |  |  | 1 |  |  |  |  |  |  | 2 |  |  |  |  |  |  |
| CABAC state | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| PIPE ID | 3 |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |
| CABAC state | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |  |  |  |
| PIPE ID | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CABAC state | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 |
| PIPE ID | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |

Figure 3: Block diagram of the proposed unified PIPE-based entropy coding scheme in LC operation mode. Highlighted areas of the block diagram correspond to the common subset of tools (including the PIPE initialization process).

# The LC operating mode of unified PIPE-based entropy coding

As already noted above and illustrated by highlighted areas in the block diagram of Figure 3, the low complexity operation mode of the unified entropy coding framework shares the initialization, the binarization, and the PIPE encoding/decoding process as a common subset of tools with the HE operation mode. Figure 3 also shows that relative to the HE mode all feedback loops have been removed such that a simultaneous processing of multiple bins is supported. In section 7, it will be described in more detail how this capability is used for achieving a significant speedup of transform coefficient parsing in the LC mode.

In general, a fixed tuple (valMPS, PIPE ID) is assigned to each bin with index binIdx. For the syntax elements significant_coeff_flag and coeff_abs_level_greaterX_flag (with X=1,2), we allow the usage of multiple (fixed) probability models, i.e., multiple tuples (valMPS, PIPE ID), where the selection of each tuple, however, depends only on a grouping of the position information within the corresponding scan. More details about this procedure are given in section 6.2.2.

Furthermore, all context derivation processes depending on prior encoded/decoded symbols are disabled.

# Residual coding

## Residual coding syntax

The HE and LC operating mode use the same binarization for blocks of transform coefficients. The syntax (relative to [3]) for coding last_significant_coeff_x, last_significant_coeff_y, and significant_coeff_flag is unchanged. For the remaining syntax elements (coeff_abs_level_greaterX_flag (with X = 1, 2), coeff_abs_level_minus3, coeff_sign_flag), two alternative interleaving modes are available as depicted in Table 2 and Table 3. These syntax elements use the same scan as the significant_coeff_flags, but in reverse order. This concept is similar to [4]. The two scans differ in the way, the syntax elements are interleaved. In the interleaved scan, after coding the significance map, all syntax element associated with one scan position are processed together. In the non-interleaved scan, all syntax elements of one type are processed together. Each of the two scans can be used for both, the HE and the LC operating mode. In the HM 3.2-based implementation of the proposed unified PIPE-based entropy coding framework, we used the interleaved scanning for the HE operating mode and the non-interleaved scanning for the LC operating mode. Further details will be discussed in the experimental results section.

| for( n = 0; n < NumCoeff − 1; n+ + ) |  |
| --- | --- |
| significant_coeff_flag[ n ] | ae(v) |
| for( n = NumCoeff − 1; n >= 0; n− − ) { |  |
| if( significant_coeff_flag[ n ] ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| coeff_sign_flag | ae(v) |
| } |  |
| } |  |

Table 2: Interleaved scanning of residual coding syntax elements

| NumSig = 0 |  |
| --- | --- |
| NumGreater1 = 0 |  |
| for( n = 0; n < NumCoeff − 1; n+ + ) { |  |
| significant_coeff_flag[ n ] | ae(v) |
| NumSig += significant_coeff_flag[ n ] |  |
| } |  |
| for( n = NumCoeff − 1; n >= 0; n− − ) { |  |
| if( significant_coeff_flag[ n ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| NumGreater1 += coeff_abs_level_greater1_flag[ n ] |  |
| } |  |
| for( n = NumCoeff − 1; n >= 0; n− − ) { |  |
| if(coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = NumCoeff − 1; n >= 0; n− − ) { |  |
| if(coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| for( n = NumCoeff − 1; n >= 0; n− − ) { |  |
| if( significant_coeff_flag[ n ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| } |  |

Table 3: Non-interleaved scanning of residual coding syntax elements

## Aspects of probability modeling of residual coding syntax elements

### Context modeling for residual coding syntax elements in high-efficiency operation mode

In the high-efficiency operation mode, context modeling for syntax elements last_significant_coeff_x, last_significant_coeff_y, and significant_coeff_flag is not changed relative to CABAC. For coeff_abs_level_greaterX_flag (with X = 1, 2), the derivation of ctxIdxInc is described by the following pseudo-code (with coeff_abs_level_greaterX_flag_ctxIdxInc[ n ] being the ctxIdxInc for coeff_abs_level_greaterX_flag[ n ] ):

uiCtxSet = scanPosLast > 15 ? 15 : 0
c1 = 1
c2 = 0
for( n = scanPosLast; n >= 0; --n )
{
    if( significant_coeff_flag[ n ] )
    {
        coeff_abs_level_greater1_flag_ctxIdxInc[ n ] = uiCtxSet + min( c1, 4 )
        coeff_abs_level_greater2_flag_ctxIdxInc[ n ] = uiCtxSet + min( c2, 4 )
        coeff_abs_level_greater1_flag[n]
        if( coeff_abs_level_greater1_flag[n] )
        {
            coeff_abs_level_greater2_flag[n]
            c1 = 0
            c2++
            uiNumGrOneInSet++
        }
        else if( c1 )
        {
          c1++
        }
    }
    if( (n & 15) == 0 )
    {
        uiCtxSet = (n > 16 ? 3 : 0 ) + min( 2, uiNumGrOneInSet )
        uiCtxSet *= 5
        uiNumGrOneInSet = 0
        c1 = 1
        c2 = 0
    }
}

Syntax element coeff_abs_level_minus3 is coded in the same way as specified for CABAC in the current WD.

### Assignment of multiple probability models for residual coding syntax elements in low-complexity operation mode

For the syntax elements significant_coeff_flag and coeff_abs_level_greaterX_flag (with X=1,2), we allow the usage of multiple (fixed) probability models, i.e., multiple tuples (valMPS, PIPE ID), where the selection of each tuple depends on the scan position of the syntax element only. For each transform block size a separate set of probability models is defined for luma and chroma, respectively.

#### Assignment of multiple probability models for significant_coeff_flags

Successive scan positions are grouped into subsets of elements sharing the same probability model. The significant_coeff_flag for the DC position is coded with its own probability model. Then, the number of successive scan positions which share one probability model is doubled within the scanning path. An example of a mapping from scan position to differently indexed probability models for a 4x4 transform block using zig-zag scan is shown in Table 4.

| 0 | 1 | 2 | 2 |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 3 |
| 2 | 3 | 3 | 3 |
| 3 | 3 | 3 | NA |

Table 4: Example of probability model assignment for coeff_significant_flags for 4x4 transform blocks and zig-zag scan.

#### Assignment of multiple probability models for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

Successive significant scan positions are grouped into subsets, and for each subset the coeff_abs_level_greater1_flags are encoded/decoded using the same probability model. The subset size is equal to the transform block width. An example of the probability model assignment for coeff_abs_greater1 flags is given for a 4x4 transform block with 10 significant scan positions and shown in Table 5.

| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 5: Example of probability model assignment for coeff_abs_greater1 flags for a 4 x 4 transformation block with 10 significant flags equal to 1.

Successive scan positions with coefficients greater than 1 are grouped into subsets, and for each set the coeff_abs_level_greater2_flags are coded using the same probability model. The subset size is equal to the transform block width. An example of the probability model assignment for coeff_abs_greater2 flags is given for a 4x4 transform block and shown in Table 6.

| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |

Table 6: Example of probability model assignment for coeff_abs_greater2 flags for a 4 x 4 transform block with 7 coeff_abs_greater_1 flags equal to 1.

#### Alternative assignment of multiple probability models for coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag

The method for multiple probability models assignment used for significant_coeff_flags can be adopted for coeff_abs_level_greater1_flags and coeff_abs_level_greater2_flags. Thus, the subset size of coeff_abs_level_greater_(1/2)_flags sharing the same probability model, depends on the number of already coded coeff_abs_level_greater_(1/2)_flags within the same block. The size of each subset is twice the size of the previous subset. This results in a reduced number of necessary probability models and a small decrease of the average BD-rate.

# Processing of multiple bins

The PIPE coding engine allows the coding of multiple bins when the same probability model is used for a given number of successive bins. This is achieved for the syntax elements related to the coding of transform coefficient levels in the LC operation mode, where a specific number of successive bins are coded with a fixed probability model. In case of coeff_significant_flag, the number of bins that can be coded at the same time is variable and depending on the scan position. For coeff_abs_level_greaterX_flag (with X=1,2), the number of successive bins using the same model probability is exactly the width of the transform block. Due to the high portion of bins for these 3 syntax elements, the concept of multiple bins results in a significant speedup relative to a sequential bin-by-bin processing.

In addition, processing of multiple bins in both HE and LC operation mode has also been used, when a certain number of bins in bypass mode (with fixed probability of 0.5) is given such that this particular number is known by the encoder/decoder. Following parts of syntax elements have been handled in such a way:

- Remainder/suffix of the Exp-Golomb binarization

- Remainder/suffix of the Rice binarization for coeff_abs_minus3

- The coeff_sign_flag syntax element

# Exemplary application of complexity-scalability

A key feature of the unified PIPE-based entropy coding stage is the ability to adapt its operation to a variety of applications on different levels of complexity by simply enabling or disabling certain features. In this paragraph, a third operating mode – the medium complexity operating mode (MC operating mode) – is introduced. It uses a subset of tools which are employed in the LC and in the HE operating mode. The MC operating mode is aimed to achieve a coding efficiency that is between the coding efficiency of the LC and the coding efficiency of the HE operating mode. Correspondingly, the MC operating mode is designed to operate on a level of complexity that is between the complexity of the LC and the complexity of the HE operating mode.

## The MC operating mode of unified PIPE-based entropy coding

The MC operating mode is identically configured as the LC operating mode as specified in chapter 5 apart from 2 exceptions:

- Context modeling template for encoding/decoding the coeff_significant_flag

- Activation of adaptive probability estimation for all bins except those related to the residual data

The context modeling of coeff_significant_flag from the LC operating mode as described in section 6.2.2.1 is replaced by the default context modeling template of the HM 3.2 HE anchor. Note, that here just the derivation of the context index is changed while the probability models of the residual data still operate at fixed probabilities. The second modification consists of reactivating the adaptivity of the probability models for all syntax elements except those probability models used for coding the residual data. Equal to the default HM 3.2 HE setting, the internal state of the probability models is updated with the last decoded bin. Hence, the MC operating mode exploits the advantages of the probability estimator for all probability models but the ones that are used for coding the residual data.

## Demonstration of the complexity-scalability with the MC operating mode

For demonstrating the complexity-scalability, the proposed unified PIPE-based entropy coding scheme is simulated with the LC-, MC- and the HE operating mode. In all three cases the default low complexity encoder configuration as specified in [7] is used with the exception of setting the SymbolMode equal to 1. Figure 4 illustrates the results of the three PIPE-based operating modes in terms of luma BD-rate saving and decoding time relative to HM3.2 LC anchor for the Intra, Random Access, Low Delay B and Low Delay P coding constraints. Each graph consists of three data points:

- The data points at the bottom of the figure which are indicated by a circle marker are the results of the LC operating mode.

- The data points that are in the middle of each graph and displayed with a square marker show the performance of the MC operating mode.

- The data points that are indicated by a diamond marker at the top of each graph show the performance of the HE operating mode.

Figure 4: Luma BD-rate compared to the decoding time relative for the LC-, MC- and HE operating mode of the proposed unified PIPE-based entropy coding relative to the HM3.2 LC anchor.

Given the same encoder configuration it can be seen that the LC operating mode of the proposed unified PIPE-based coding is between 94% and 95 %  of the decoding time of the HM3.2 LC anchor while the BD-rate savings are between -1% and 0.25%. The MC operating mode achieves for all coding constraints a BD-Rate saving of 3-4 % while the decoding time is between 96% and 100% compared to the HM3.2 LC anchor. For the HE operating mode the BD-rate saving is between 5.75% and 6.75% while the decoding time is between 98% and 103 % relative to the HM 3.2 LC anchor. Note, the detailed results of the MC-operating mode for every sequence can be found in the enclosed excel sheet.

# Representation of the internal state of probability models

The internal state of a probability model for the usage with PIPE is designed in a two level manner and can be stored in a single 8 bit memory.

Table 7: Representation of the internal state of probability models

| First Level |  |  |  | Second Level |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| valMps | pipeIdx (0-7) |  |  | refineIdx (0-15) |  |  |  |

The first level requires 4 bits and consists of the values of PIPE ID (pipeIdx) and the value of the MPS (valMps). Another 4 bits are used to store the second level which is denoted as the refinement index (refineIdx). The refineIdx supplements the first level and represents the internal state of the original CABAC probability models (also denoted as pStateIdx in the WD). Hence the number of refineIdx values related to each value of pipeIdx is equal to the number of CABAC states that are mapped to the corresponding probability interval of a given pipeIdx (as given by Table 1)and is depicted in Table 8.

Table 8: Number of CABAC states assigned to each value of pipeIdx

| pipeIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of CABAC states | 3 | 7 | 5 | 7 | 10 | 14 | 16 | 1 |

Given this setup, the behaviour of the original CABAC probability estimator can be exactly emulated and the update of the internal state index is performed after a bin is encoded or decoded with PIPE in the HE-mode. For the decoding process in the LC-mode, the refineIdx is not required.

# Probability model initialization using 8 bit initialization values

This section describes the process of initializing the internal state of probability models using an 8 bit initialization value. The 8 bit value consists of two parts and is comparable to the 16 bit values (m, n) used for the initialization of the probability models in CABAC. The two parts represent the two parameters of a linear equation to compute a pipeIdx for a given QP:

- The first part describes the slope and it exploits the dependency of the internal state in respect to the slice_QP.

- The second part defines a pipeIdx at a given slice_QP including the valMps.

Table 9: Setup of the 8 bits of an initialization value for a probability model.

| First Part |  |  |  | Second Part |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| b7 | b6 | b5 | b4 | b3 | b2 | b1 | b0 |
| Slope Index |  |  |  | PIPE Probability Index |  |  |  |

The slope index is used to determine the current slope of the linear equation. It represents one out of 16 predefined slopes that are shown in Table 10. Note the slope values are scaled by a factor of 256 for accurate integer arithmetic precision.

Table 10: Slope indices and their assigned slope values.

| Slope Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Slope Value | -239 | -143 | -85 | -51 | -31 | -19 | -11 | 0 |
| Slope Index | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| Slope Value | 11 | 19 | 31 | 51 | 85 | 143 | 239 | 399 |

The second part denotes the intercept point of the linear slope shifted along the abscissa to QP = 26 and covers the probability range from the least probable internal state to the most probable internal state. Table 11 displays the assignment of the PIPE probability index to the corresponding pipeIdx along with the valMps.

Table 11: Mapping of the second part of the initialization value to PIPE coders and valMps: UR = unary-to-rice-code, TB = three-bin-code, BP = bin-pipe-code, EP = equal probability (uncoded).

| PIPE probability index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pipeIdx | UR5 | UR4 | UR3 | UR2 | TB | BP2 | BP3 | EP |
| valMps | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| PIPE probability index | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| pipeIdx | EP | BP3 | BP2 | TB | UR2 | UR3 | UR4 | UR5 |
| valMps | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Both parts are required to retrieve the internal state index of the probability model. The method of retrieving is identical to solving a linear equation where the first part of the integer index denotes the slope and the second part denotes the intercept point shifted along the abscissa. Depending on these two parts and the slice_QP the initial internal state index is calculated for any probability model. The result of the calculation is mapped to the closest integer state index, which is the pipeIdx whose probability interval contains the probability of the derived internal state index. The decimal part of the result is used to determine the refineIdx by employing linear interpolation among the probability interval of one pipeIdx. In the HE-mode the refineIdx is exploited for exact probability estimation of the probability models and for accurate encoder decisions during bitrate estimation in LC- and HE-mode. Though, for a decoding application of PIPE in the LC-mode it is sufficient to solely compute the pipeIdx.

# Number of used probability models

Table 12 lists the number of used probability models for the three different complexity modes and slice types. It can be seen that the amount of memory, necessary for storing the internal states of the probability models, is also scalable.

Table 12 Number of used probability models for each complexity mode. Values marked with an asterisk represent the coeff_abs_level_greater(1/2)_flag  probability model assignment as described in 6.2.2.2.

# Hardware specific implementation aspects

As explained in [5] there are several throughput challenges that exist for CABAC as well as for other entropy coding methods. These challenges mainly arise because of serial dependencies and feedback loops which hinder the utilization of parallelism and pipelining and thus make the entropy coding stage a bottleneck in video codec implementations. In order to increase the throughput and enable the parallel processing of multiple bins, speculative computations are required but these are very expensive in terms of area and power consumption (e.g., the number of operations increases exponentially with the number of bins that are decoded in parallel).

The proposed PIPE concept enables gains in the throughput of the decoder given a high coding efficiency, a low area consumption and the flexibility to adapt it to different requirements (high efficiency (HE) and low complexity (LC) operating mode). The gains in throughput are achieved by 1.) removing feedback loops and serial dependencies, 2.) reducing the length of the critical path and 3.) exploiting parallelism.  Furthermore, even the coding efficiency can be varied by adapting the frequency of adaptive probability estimation to the requirements of the application (no adaptivity for LC, full adaptivity for HE).

To measure the impact of the improvements, a PIPE decoder was implemented in VHDL and synthesized in different configurations for a Xilinx FPGA. Synthesis results have shown that the throughput of the decoder design can be scaled by introducing multiple instances of the Probability Interval Selector and thus allowing the decoder to process multiple bins in parallel.

## Removing feedback loops and serial dependencies on the decoder-side

As described in [5],CABAC is “a well known throughput bottleneck in the video codec implementations” which is especially true for the decoder-side. The low throughput is a result of feedback loops and serial dependencies that prevent the parallel processing of multiple bins. In particular there are feedback loops between the Context Modeler and the Bin Decoder and after a bin has been decoded the corresponding context and range values must be updated before the next bin can be decoded. If implemented in hardware these steps consume several clock cycles and high clock frequencies are necessary to meet the throughput requirements of the application.

Obviously the need to update the range values is not applicable to PIPE since it uses eight V2V look-up-tables instead of arithmetic coding for the binary decoding step. In addition the contexts are substantial simplified (they can only have 16 different meanings [eight possible PIPE-ID values combined with two possible MPS values]) which reduces the complexity and area of the hardware implementation. Furthermore the updated context is used later in the processing chain. The reason for this is PIPE's Binary Decoder which -in contrast to the one in CABAC- does not rely on the context information. Thus the complex Binary Decoding step is moved out of the critical path and higher throughput rates can be achieved.

The Probability Interval Selector still needs the information of the preceding bin in order to select the next bin from the correct Bin Buffer and this serial dependency limits the achievable gain in throughput. To address this problem fixed instead of adaptive context models are used in the low complexity mode of the PIPE concept. Thus the Bin Decoder used for a given syntax element is always the same and independent of the preceding bins. With this customization the feedback loops and serial dependencies between the bins are resolved and multi-bin requests can be decoded in parallel.

## Reducing the length of the critical path

A further improvement of the throughput can be achieved by dividing the PIPE decoding process in several pipeline stages by the introduction of pipeline registers. As already described before PIPE's Binary Decoder doesn't rely on the context of the preceding bins or any other information produced in the following processing steps of the entropy decoder. To take advantage of this, pipeline registers should be inserted after the Binary Decoders (and after the Bin Buffers as well). This separates complex processing steps from the remaining decoding steps and thus shortens the critical path for the low complexity as well as the high efficiency case. In contrast to CABAC, the introduction of these pipeline registers will not raise the number of clock cycles that are necessary to decode a bin. The FPGA implementation shows that the pipeline registers enable higher clock frequencies and throughputs.

Also, other enhancements contribute to a shortened critical path, for example the simplified context models and the removal of adaptive probability estimation in the LC case.

## Exploiting parallelism

As described in [5], state-of-the-art CABAC implementations use speculative computation to raise the number of bins that can be processed in parallel. It is also explained that this approach is very expensive in terms of area-consumption. To give a rough estimate the area of a CABAC decoder that processes N-bins in parallel is 2**N-1 times the area of a single-bin decoder.

In contrast to CABAC, the PIPE low complexity concept is well suited for parallel decoding of multiple bins, since all serial dependencies that exist in CABAC have been resolved. In particular only the Probability Interval Selectors and the Bin Derivers need to be replicated for each bin. These two components account for approximately 20% of the area of a single-bin PIPE decoder. Accordingly the area of an N-bin PIPE decoder will be approximately 0.8+0.2*N times the area of a single-bin decoder.

For the high efficiency case, the PIPE concept offers several solutions to achieve high throughputs. The most convenient way would be to reduce the amount of probability estimates (e.g. the estimate could be constant for all single bins within a multi-bin request; a new estimate could be performed for the whole group after all bins have been decoded). Another option would be to use speculative computation. In that case, the area for an N-bin PIPE decoder would approximately be 0.8+0.2*(2**N-1) times the area of a single-bin decoder which still should be considerably less than for an N-bin CABAC decoder.

## Synthesis results

As a proof-of-concept, a low-complexity PIPE decoder has been implemented in hardware. The simplified schematic of the design (the control flow is not fully illustrated; repeated structures are only indicated) is depicted in Figure 5.

Figure 5: Schematic representation of a hardware implementation of the low complexity PIPE decoder.

The FPGA synthesis results for different configurations of the VHDL design are listed in Table 13.

| Configuration | 1-bin | 2-bin | 4-bin | 8-bin | 12-bin |
| --- | --- | --- | --- | --- | --- |
| Slices | 649 | 788 | 987 | 1424 | 1823 |
| Slice Flip Flops | 367 | 405 | 465 | 549 | 612 |
| 4 Input LUTs | 1238 | 1508 | 1900 | 2741 | 3517 |
| Clock Frequency | 159.132MHz | 143.69MHz | 148.836MHz | 128.632MHz | 125.572MHz |

Table 13: Synthesis results for different low-complexity PIPE decoder configurations on Xilinx XC4VSX35 FPGA.

# Experimental results

  - Software

The proposed unified PIPE-based entropy coding method has been implemented on top of HM 3.2 [6]. For the simulations, the binaries are compiled on 64 bit Ubuntu 10.10 Linux using the included makefiles and gcc version 4.4.5. The test conditions and software configurations of the simulations are specified in [7]. The default settings are applied during compile time and for producing the reference points as anchor.

  - Hardware

Two types of computers have been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system. Their specification is listed in Table 14.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Decoding on 1 core without yuv output for time measurement | 1x4 | 4 x Intel(R) Core(TM)2 Extreme CPU Q9300 @ 2.53GHz | 4 GB |
| 2 | Encoding and decoding for BD-rate retrieval and encoding time measurement | 2x4 | 2 x 4 x Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB |

Table 14: Computers used to run the simulations

  - Simulation results

The results in terms of coding efficiency are shown in BD rates. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets. Please note that the reported decoding times for the HE setting are not yet accurate.

Figure 6: Simulation results of the unified PIPE-based entropy coding approach in HE- and LC-mode.

## Revised results with updated init values

The results shown in the last preceding paragraph are based on the usage of context initialization values that are derived from the statistics of the simulation with sequences of 50 frames length only. This section presents results with an updated set of initialization values that are based on the statistics of the sequences with full length. Excluding the set of initialization values the software is identical to the results of section 13.3 and the hardware specified in Table 14 was used. Figure 7 displays the revised simulation results.

Figure 7: Revised simulation results of the unified PIPE-based entropy coding approach in HE- and LC-mode with update probability model initialization values

## Results for LC-mode with the probability model assignment for coeff_abs_level_greater(1/2)_flags as described in 6.2.2.3

Figure 8 Results for LC-Mode with the alternative assignement of probability models for coeff_abs_level_greater(1/2)_flags

# References

- D. Marpe, H. Schwarz, T. Wiegand, “Novel entropy coding concept”, JCTVC-A032, Dresden, DE, April 2010.

- H. Kirchhoffer, D. Marpe, H. Schwarz, C. Bartnik, A. Henkel, M Siekmann, J. Stegemann, T. Wiegand, “Reduced complexity PIPE coding using systematic V2V codes”, JCTVC-D380, Daegu, KR, Jan. 2011.

- T. Wiegand, W.-J. Han, B. Bross, J.-R. Ohm, G. Sullivan, “WD3: Working Draft 3 of High-efficiency Video Coding”, JCTVC-E603_d8, Geneva, CH, March 2011.

- J. Sole, R. Joshi, M. Karczewicz, “Unified scans for the significance map and coefficient level coding in high coding efficiency”, JCTVC-E335, Geneva, CH, March 2011.

- Vivienne Sze, Madhukar Budagavi, “Further study on entropy coding”, JCTVC-E354, Geneva, CH, March 2011.

- JCT-VC, “HEVC Test Model Software 3.2”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.2

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).