| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F269 |
| --- | --- |

| Title: | Modified Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Stephane PAUTET Christophe GISQUET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the Intra Mode Coding process. The current HM design does not take (in CABAC configuration) or partially takes (in CAVLC configuration) into account the intra mode statistics. In this proposal, intra mode statistics are updated on the fly and further used to decode the intra mode when the mode is not equal to one of the neighboring Most Probable Modes (MPMs).

In the CABAC case, counters and related mode ranking tables are used and updated each time an intra mode is decoded. This updating is achieved when the intra mode is signaled as an MPM or not. Ranking tables are further used to adaptively binarize the mode value before arithmetic coding.

In the CAVLC case, the number of modes-codewords mapping tables is reduced from 4 (2 for 4x4 CUs, 2 for 8x8 to 32x32 CUs) to 2 (1 for 4x4 CUs, 1 for 8x8 to 32x32 CUs). The swapping process is achieved based on the real mode value (not the decoded codeword value) when the intra mode is signaled as an MPM or not.

The tool has been tested in HM3.1. The overall average luma BD-rate improvement is of 0.3% for the High Efficiency All Intra configuration, 0.1% for the Low Complexity All Intra and High Efficiency Random Access configurations. No impact is observed for the other configurations. Higher gains are observed on high resolution than on low resolution sequences. The proposal has also been evaluated on the HM3.0_SDIP branch. Consistent results are obtained, showing that SDIP and the proposal give additive gains.

# Introduction

In the current HEVC design, intra mode decoding makes use of the two neighboring Top and Left CUs to derive one or two Most Probable Modes (MPMs) values. The mode of the current CU is then compared to the MPMs.

- If it is equal to one of the MPMs, a first coding process (Process 1) is applied, consisting of coding a flag (prev_intra_luma_pred_flag) signaling that the mode is one of the MPM, and eventually a second flag to signal which MPM is used.

- If the intra mode is not equal to one of the MPMs, a second coding process (Process 2) is applied, consisting of coding the mode value. The proposal relates to this second process.

Currently there is no link between Process 1 and Process 2. Mode statistics are therefore not fully exploited to code the mode value. This may generate a loss of coding efficiency, in particular when dominant intra modes are present in the picture. For instance, in Figure 1, ‘floor mode’ is dominant over the picture. After coding foreground objects PUs, when ‘floor mode’ PUs reappear, they are however coded without any priority on the modes. When there is a frequent switching between a dominant mode and other modes, this may result in lesser coding efficiency.

To solve this issue, it is proposed to make use of intra mode statistics updated on the fly. They are further used to decode the intra mode in Process 2.

Figure 1: illustration of possible coding efficiency loss in presence of a dominant intra direction.

# Proposal description

The proposal consists of establishing an explicit link between Process 1 and Process 2 as depicted in Figure 2. Modes probabilities (counters for CABAC, mapping tables for CAVLC) are used and updated once Process 1 or Process 2 has been applied. This enables to better take into account the intra mode statistics.

- Compared to the current CABAC implementation, new modes counters and mode ranking tables are used.

- Compared to the current CAVLC implementation, 2 mapping tables are used instead of 4 in the current design and these tables are updated once Process 1 or Process 2 has been applied.

Figure 2: proposed modified intra mode coding.

In the remainder of this document, N corresponds to the number of intra modes. When the CU is of 4x4 pixels, N is equal to 17; when the CU is of 8x8 to 32x32 pixels, N is equal to 34. The proposal does not apply for other PU sizes.

## CABAC case

Figure 3 gives the block diagram of the respectively current (left figure) and proposed (right figure) HM design of the CABAC intra mode decoding process. The following differences can be noticed:

- A mode counter table count[m] of size N is defined (one table of 17 elements for 4x4 CUs, one table of 34 elements for 8x8 to 32x32 CUs). This table is updated once the intra mode has been coded either by Process 1 or by Process 2 as follows:

  - Let’s m be the mode just having been coded

  - If (count[m] = = 255), all the elements of the table are divided by 2

  - count[m] = count[m]+1

- In addition, a mode rank table of size N is defined (one table of 17 elements for 4x4 CUs, one table of 34 elements for 8x8 to 32x32 CUs). When a mode m has been coded, once the mode counter table has been updated, mode m is swapped with its preceding mode m_prec in the rank table if count[m] > count[m_prec]. This process is similar to the CAVLC table swapping when counters are used in CAVLC.

- The decoded value no more relates to the mode value but to the rank of the mode. The incrementing step is applied based on the MPMs rank value, not on the MPMs value.

- The binarization is modified so that it better takes into account the intra mode statistics. A shorter codeword is used for the first symbol 0, and possibly a longer codeword is used for the latest symbol values. Details are given in attached document JCTVC-F269_WDModifications describing the WD modifications. In particular new binarization is described in section ‘9.3.2.9   Binarization process for rem_intra_luma_pred_mode’.

Figure 3: current (left) and proposed (right) intra mode decoding design for CABAC.

## CAVLC case

Figure 4 gives the block diagram of the respectively current (left figure) and proposed (right figure) HM design of the CAVLC intra mode decoding process. The following differences can be noticed:

- One index table of size N is used instead of 2 tables of size N-1 and N-2.

- The swapping process is applied once the intra mode has been decoded, either by Process 1 or by Process 2. The swapping applies on the unique table of N elements, whilst in the current HM design, it applies either on the table of N-2 or N-1 elements, depending on the number of distinct MPMs values.

- The decoded value relates no more to the index value of the VLC codeword in the VLC table, but to the rank of the mode in the mode index table.

Figure 4: current (left) and proposed (right) intra mode decoding design for CAVLC.

# Results

## Results with HM3.1

The experiments were conducted in accordance with the 6 common test conditions described in [1]. The proposed modifications were implemented in version HM3.1 of the common software. The results are shown in Table 1. The excel sheet corresponding to this result is attached as part of the contribution (file JCTVC_F269_HM31_vs_CanonIMC.xls).

Table 1: results of the proposal in HM3.1.

The proposal achieved overall average luma BD-rate gains of 0.3% for Intra All High Efficiency and of 0.1% Intra All Low Complexity coding conditions. A gain of 0.1% is also observed for Random Access High Efficiency. There was no impact on the coding performance for the other cases. The encoding and decoding run time are similar to those of HM3.1.

The contribution has been cross-checked by MediaTek in JCTVC-F430.

## Additional results with separate codewords for DC and Planar modes

An additional modification has been tested. In current HM3, DC and Planar modes are signaled using an identical VLC codeword (in CAVLC) or an identically binarized value (in CABAC).

In JCTVC-F123, it is proposed to separate in the VLC tables the DC and Planar modes for CAVLC. This modification has been combined with this proposal.

In addition, for CABAC case, DC and Planar modes are binarized using different values.

Results, depicted in Table 2, show an average gain of 0.3% for All Intra High Efficiency and of 0.3% for All Intra Low Complexity.

Table 2: proposal combined with separate DC and planar VLC codewords.

## Results with HM3.0 SDIP branch

The proposed modifications were also implemented in the SDIP branch, delivered by Huawei on June 22, 2011. The experiments were conducted for the Intra Only test conditions. The results are shown in Table 3. The excel sheet corresponding to this result is attached as part of the contribution (file JCTVC_F269_HM30_SDIP_vs_CanonIMC.xls).

Table 3: results of the proposal in HM3.0 SDIP branch.

The results are consistent with those observed for the HM3.1 implementation: 0.3% gain for Intra All High Efficiency and 0.1% gain Intra All Low Complexity. SDIP and the proposal give additive gains.

The contribution has been cross-checked by Huawei in JCTVC-Fxxx.

# Changes in the WD specification

The HM specification changes made in JCTVC-E603_d7 version, are detailed in the attached document JCTVC-F269_WDChanges_JCTVC-E603_d7.doc.

The following sections were modified:

- 8.3.1      Derivation process for luma intra prediction mode

- 9.2.2.5   Initialisation process for intraModeTable and intraModeCounterTable

- 9.2.3      Codeword index mapping update process

- 9.2.4.6   Parsing process for rem_intra_luma_pred_mode

- 9.3.2.9   Binarization process for rem_intra_luma_pred_mode

# Conclusion

In this contribution, it is proposed to use and update on the fly intra mode statistics in the intra mode coding. These statistics are further used to decode the intra mode when the mode is not equal to one of the neighboring Most Probable Modes (MPMs). The proposal leads to a gain mainly in Intra Only test conditions in HM3.1, with higher gain observed for high resolution sequences. Similar gains are also observed in the SDIP HM3.0 software. Encoding and decoding runtime are not impacted.

It is proposed that this contribution be adopted into the HM or further studied in a CE.

# Reference

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, Switzerland, March, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).