| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F270 |
| --- | --- |

| Title: | On BD-rate calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Wang Xiang Yu Dake He | Tel: Email: | wangj xiayu @rim.com dhe |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

An issue about BD-rate calculation was observed and studied for Class-A sequences. This contribution summarizes the BD-rate issue and presents some recent studies that follow some suggestions from the reflector. As Nebuta and BQTerrace are the most problematic sequences, it is recommended that BD-rate results excluding Nebuta and BQTerrace are also reported for evaluation purpose and additional information on these sequences such as the PNSR-bitrate curves are to be checked when inconsistency is observed. The BD-rate calculation based on piece-wise cubic interpolation also appears to be a practical fix to this problem. It is recommended to further investigate the use of the current BD-rate tool in evaluation of proposals.

# Review of BD-rate calculation

In [1][2], the BD-rate(BD-PSNR) metric is introduced to compute the average rate(PSNR) difference between two PSNR-bitrate curves. By using a logarithmic representation of the bitrate, the BD-rate metric computes the area between the two interpolated PSNR-log(bitrate) curves. The integration is performed over the overlap of the two PSNR (bitrate) intervals. In [1][2], a 3rd order polynomial interpolation is used to fit the PSNR-log(bitrate) curves.

In VCEG-AI11 [2], it is stated that:

“When the fitted curves are close to straight lines the accuracy of the calculated differences are good even if the bitrate for the end points may vary with as much as a factor of 20. Be careful to interpret the result in cases where the fitted curves deviate much from a straight line.”

To ensure that BD measurements are accurate, in VCEG-AL22 [3], a reliability measure is introduced based on the overlapping ratio of the two PSNR (bitrate) intervals. It is suggested that when the reliability is close to 1 the test is considered valid, and if the reliability is significantly less than 1, more test points may be needed. However there were also some concerns raised on this reliability metric [4].

# BD-rate results for Class-A sequences

Figure 1 - Figure 4 show the BD-rate results with PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves using degree-3 polynomials for the four Class-A sequences. Data are obtained from HM3.0, RDOQ-off vs. HM3.0, RDOQ-on with the RA-HE setting [5].

Figure 1 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Traffic, HM3.0, RA-HE, RDOQ-off vs. RDOQ-on. BD-rate shows 4.7% loss while BD-PSNR shows 0.16dB loss.

Figure 2 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for PeopleOnStreet, HM3.0, RA-HE, RDOQ-off vs. RDOQ-on. BD-rate shows 4.6% loss while BD-PSNR shows 0.20dB loss.

Figure 3 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Nebuta, HM3.0, RA-HE, RDOQ-off vs. RDOQ-on. BD-rate shows 13.6% loss while BD-PSNR shows 0.23dB loss.

Figure 4 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for SteamLocomotive, HM3.0, RA-HE, RDOQ-off vs. RDOQ-on. BD-rate shows 5.8% loss while BD-PSNR shows 0.08dB loss.

The following table summarizes the BD-rate and reliability results of the four Class A sequences. The BD rates at high and low bitrates are calculated as in VCEG-AI11 [2]. The corresponding reliability metrics are calculated as in VCEG-AL22 [3]. While all reliability metrics are close to 1, it can be seen that the three BD-rates of Nebuta are not consistent.

|  | BD-rate | Reliability | BD-rate low | Reliability for low bitrates | BD-rate high | Reliability for high bitrates |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | 4.7 % | 0.965281 | 4.9 % | 0.909709 | 4.7 % | 0.894131 |
| People | 4.6 % | 0.963501 | 4.0 % | 0.947757 | 5.5 % | 0.893906 |
| Nebuta | 13.6 % | 0.911514 | 2.7 % | 0.883516 | 8.3 % | 0.815997 |
| SteamLoco | 5.8 % | 0.955294 | 5.4 % | 0.928807 | 7.4 % | 0.860413 |

# Further investigation on BD-rate calculation for Nebuta

One case as shown below in Figure 5 is from [5] for Nebuta, RA-HE, HM3.0 RDOQ-off vs. RDOQ-on, T3 (the third temporal level), where a 0.49dB PSNR loss by BD-PSNR calculation is translated to a 1747% rate loss by BD-rate calculation, with a reliability of 0.968456.

| BD-rate | Reliability | BD-rate low | Reliability for low bitrates | BD-rate high | Reliability for high bitrates |
| --- | --- | --- | --- | --- | --- |
| 1747.2 % | 0.968456 | -1.1 % | 0.865402 | -38.6 % | 0.765528 |

Figure 5 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Nebuta, RA-HE, HM3.0 RDOQ-off vs. RDOQ-on. BD-rate shows 1747% loss while BD-PSNR shows 0.49dB loss.

The example in Figure 6 is from the results of CE4 subtest 3.3.2.a [6] for Nebuta, RA-HE, where a 0.07dB PSNR gain by BD-PSNR calculation is translated to a 3.5% rate loss by BD-rate calculation.

| BD rate | Reliability | BD rate low | Reliability for low bitrates | BD rate high | Reliability for high bitrates |
| --- | --- | --- | --- | --- | --- |
| 3.5 % | 0.936920 | 0.6 % | 0.934585 | -5.2 % | 0.829770 |

Figure 6 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Nebuta, RA-HE, HM3.0+ARL. BD-PSNR shows 0.07dB gain, while BD-rate calculation shows 3.5% loss.

The example in Figure 7 shows the result of Nebuta, RA-LC, HM3.0, from one JCT-VC proposal, where a 0.04dB PSNR gain by BD-PSNR calculation is translated to an 11.1% rate saving by BD-rate calculation.

| BD rate | Reliability | BD rate low | Reliability for low bitrates | BD rate high | Reliability for high bitrates |
| --- | --- | --- | --- | --- | --- |
| -11.1 % | 0.987365 | 0.2 % | 0.850867 | 3.5 % | 0.880414 |

Figure 7 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for one JCT-VC proposal, Nebuta, RA-LC, HM3.0. BD-rate shows 11.1% rate saving while BD-PSNR shows 0.04dB PSNR gain.

One more example shown in Figure 8 is from another JCT-VC proposal for Nebuta, RA-HE, HM2.0, where a 0.01dB PSNR gain by BD-PSNR calculation, which also agrees with the observation of the PSNR-Bitrate curves, is translated to a 5.1% rate saving by BD-rate calculation.

| BD rate | Reliability | BD rate low | Reliability for low bitrates | BD rate high | Reliability for high bitrates |
| --- | --- | --- | --- | --- | --- |
| -5.1 % | 0.987409 | 0.9 % | 0.828161 | 2.4 % | 0.930461 |

Figure 8 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for one JCT-VC proposal, Nebuta, RA-HE, HM2.0. BD-rate shows 5.1% gain while BD-PSNR shows 0.01dB gain.

During the previous discussions on the reflector, similar issues were also raised for BQTerrace. An example is given in Figure 9 for RA-HE, HM3.0 RDOQ-on vs. RDOQ-off, which shows the S-shaped fitted curves for BQTerrace, which potentially may result in unreliable BD-rate calculation.

Figure 9 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for BQTerrace, RA-HE, HM3.0 RDOQ-off vs. RDOQ-on. BD-rate shows 3.9% loss while BD-PSNR shows 0.05dB loss.

In addition to Nebuta and BQTerrace, it was also suggested that more sequences need to be examined. In the attachment, sample PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves are provided for all test sequences and settings.

# Studies on some suggestions from the JCT-VC reflector

## Reliability

The reliability metric proposed in [3] is computed for all flag cases discussed in the above.

## Average BD-rates of HM3.0 with/without Nebuta and BQTerrace

It was suggested by a few experts on the reflector to report averages excluding Nebuta and BQTerrace to see how consistent they are with the overall averages. These results may help to identify cases where the BD-rate metric is unreliable. Additional information may need to be checked when significant inconsistency is observed.

The following tables summarize the BD-rate averages with and without Nebuta and BQTerrace for comparison between the HM3.0 RDOQ-off case and the HM3.0 RDOQ-on case.

|  | Random Access HE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nebuta and BQTerrace included |  |  |  | Nebuta and BQTerrace NOT included |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| Class A | 7.2 | -2.6 | -2.3 |  | 5.0 | -3.1 | -5.5 |  |
| Class B | 6.4 | 0.4 | -0.1 |  | 7.1 | 1.9 | 1.7 |  |
| Class C | 5.9 | 3.2 | 2.9 |  | 5.9 | 3.2 | 2.9 |  |
| Class D | 5.5 | 1.4 | 1.8 |  | 5.5 | 1.4 | 1.8 |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 6.3 | 0.6 | 0.5 |  | 5.9 | 1.1 | 1.0 |  |

|  | Random Access LC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nebuta and BQTerrace included |  |  |  | Nebuta and BQTerrace NOT included |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| Class A | 3.2 | 4.1 | 3.8 |  | 3.6 | 6.6 | 6.0 |  |
| Class B | 5.1 | 8.8 | 9.4 |  | 5.4 | 8.0 | 7.5 |  |
| Class C | 4.7 | 7.8 | 8.1 |  | 4.7 | 7.8 | 8.1 |  |
| Class D | 4.7 | 7.6 | 7.9 |  | 4.7 | 7.6 | 7.9 |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 4.5 | 7.2 | 7.4 |  | 4.7 | 7.6 | 7.6 |  |

|  | Lowdelay HE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BQTerrace included |  |  |  | BQTerrace NOT included |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 5.4 | 2.4 | 1.8 |  | 5.9 | 3.5 | 3.1 |  |
| Class C | 4.6 | 3.0 | 3.0 |  | 4.6 | 3.0 | 3.0 |  |
| Class D | 4.4 | 1.6 | 2.2 |  | 4.4 | 1.6 | 2.2 |  |
| Class E | 2.6 | -1.8 | -2.9 |  | 2.6 | -1.8 | -2.9 |  |
| Overall | 4.4 | 1.6 | 1.3 |  | 4.5 | 1.8 | 1.7 |  |

|  | Lowdelay LC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BQTerrace included |  |  |  | BQTerrace NOT included |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 4.2 | 8.5 | 9.8 |  | 4.3 | 8.0 | 8.4 |  |
| Class C | 3.8 | 7.2 | 8.0 |  | 3.8 | 7.2 | 8.0 |  |
| Class D | 3.6 | 9.6 | 9.5 |  | 3.6 | 9.6 | 9.5 |  |
| Class E | 2.4 | 9.5 | 6.7 |  | 2.4 | 9.5 | 6.7 |  |
| Overall | 3.6 | 8.7 | 8.7 |  | 3.6 | 8.5 | 8.2 |  |

It is observed that the Luma BD-rate averages with and without Nebuta and BQTerrace are most inconsistent for RA setting. A summary of the difference between the two BD-rate averages is as follows:

RA-HE: 2.2% difference for Class A, 0.7% difference for Class B

RA-LC: 0.4% difference for Class A, 0.3% difference for Class B

LD-HE: 0.5 % difference for Class B

LD-HE: 0.1 % difference for Class B

Another example is from the results in [6], shown in the table below, where the two BD-rate averages differ by 0.9% for class A sequences, which suggests a possible unreliable BD-rate calculation for Nebuta.

|  | Random Access HE |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nebuta and BQTerrace included |  |  |  | Nebuta and BQTerrace NOT included |  |  |  |
|  | Y | U | V |  | Y | U | V |  |
| Class A | 0.6 | -0.4 | -0.4 |  | -0.3 | 0.0 | 0.6 |  |
| Class B | -0.5 | -0.5 | 0.2 |  | -0.7 | -1.0 | -0.5 |  |
| Class C | -0.8 | -0.8 | -0.7 |  | -0.8 | -0.8 | -0.7 |  |
| Class D | -0.4 | 0.3 | 0.2 |  | -0.4 | 0.3 | 0.2 |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.3 | -0.4 | -0.2 |  | -0.6 | -0.4 | -0.2 |  |

By looking at the PSNR-bitrate curves as additional information, which are given in Figure 6, it is evident that the inconsistency is caused by the ill-fitted curves for Nebuta, which results in a 3.5% rate loss by BD-rate calculation that is against the observation from the PSNR-Bitrate curves as well as the 0.07dB PSNR gain by BD-PSNR calculation.

## Increasing number of coded points for Nebuta

Another suggestion from the JCT-VC reflector is to increase the number of coded points, and to change the degree of the polynomial accordingly.

In the following, we re-examine one problematic case of Nebuta by increasing the number of coded points from four (QP = 22, 27, 32, 37) to eight (QP = 21, 23, 25, 27, 29, 32, 35, 38).

  - Four points (QP = 22, 27, 32, 37)

| QP | HM3.0 |  |  |  | HM3.0 + ARL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bitrate | PSNR Y | PSNR U | PSNR V | Bitrate | PSNR Y | PSNR U | PSNR V |
| 22 | 220617.76 | 39.59 | 39.71 | 38.06 | 203948.96 | 38.98 | 39.79 | 38.16 |
| 27 | 96189.96 | 33.71 | 38.31 | 36.67 | 99555.72 | 34.09 | 38.39 | 36.76 |
| 32 | 30063.71 | 29.78 | 37.20 | 35.47 | 29754.64 | 29.78 | 37.22 | 35.50 |
| 37 | 7064.54 | 28.09 | 35.94 | 34.07 | 8148.83 | 28.20 | 35.91 | 34.05 |

Figure 9 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Nebuta, RA-HE, HM3.0+ARL, four points (QP = 22, 27, 32, 37), degree-3 polynomial fitting.

  - Eight points (QP = 21, 23, 25, 27, 29, 32, 35, 38)

| QP | HM3.0 |  |  |  | HM3.0 + ARL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bitrate | PSNR Y | PSNR U | PSNR V | Bitrate | PSNR Y | PSNR U | PSNR V |
| 21 | 248248.3 | 40.5135 | 40.1277 | 38.5812 | 232046.5584 | 39.9274 | 40.2106 | 38.6947 |
| 23 | 195511.8 | 38.6006 | 39.3766 | 37.6794 | 179128.7888 | 38.015 | 39.4582 | 37.7744 |
| 25 | 149615.4 | 36.4783 | 38.8316 | 37.1478 | 136719.672 | 36.0793 | 38.9221 | 37.2477 |
| 27 | 96189.96 | 33.706 | 38.3098 | 36.6722 | 99555.7168 | 34.0932 | 38.3916 | 36.7624 |
| 29 | 65795.33 | 31.9805 | 37.8783 | 36.2375 | 65881.704 | 32.0751 | 37.9151 | 36.2754 |
| 32 | 30063.71 | 29.7791 | 37.2018 | 35.4717 | 29754.6352 | 29.7811 | 37.2245 | 35.4977 |
| 35 | 10890.74 | 28.4613 | 36.4411 | 34.6204 | 12196.0736 | 28.5635 | 36.4293 | 34.6146 |
| 38 | 5576.102 | 27.9136 | 35.5933 | 33.7522 | 6520.7568 | 28.0245 | 35.5905 | 33.7424 |

Figure 10 PSNR-Bitrate curves and fitted PSNR-log(Bitrate) curves for Nebuta, RA-HE, HM3.0+ARL, eight points (QP = 21, 23, 25, 27, 29, 32, 35, 38), degree-6 polynomial fitting.

For the tested case, the four-point fitting result shows a 3.5% BD-rate loss while the eight-point fitting result shows a 1.93% BD-rate gain. It is observed that increasing the number of testing points better preserves the shape of the PSNR-bitrate curve, and the ill-fitted S-shaped curves are avoided by using high order polynomials. However, there are still a few issues need to be addressed:

- Number of points to be tested.

- Degree of the polynomial to be used.

- The fitted curve may not be close to a straight line, which still does not satisfy the requirement in VCEG-AI11 [2].

One possible improvement to this solution is to calculate the BD-rates for the 4 high rate points and the 4 low rate points separately to get better fitting for each part, where the current degree-3 polynomial fitting can be used. This is similar to the “BD rate low” and “BD rate high” as suggested in VCEGAI-11 [2]. For the two cases discussed above, the figures below show the corresponding fitted PSNR-log(bitrate) curves for the 4 high-rate points and the 4 low-rate points, respectively.

QP = 21, 23, 25, 27                                                 QP = 29, 32, 35, 38

BD-rate high = -2.29%, BD-rate low = -1.47%

## Piece-wise cubic interpolation

A similar problem of BD-rate calculation with polynomial interpolation was also reported for some sequences in SVT test sequence set [7]. It was reported in [7] that Piecewise Cubic Hermite Polynomial Interpolation seems to provide a better fit for the given data points.

As a result of the recent discussion regarding BD-rate calculation on the JCT-VC reflector, an excel template for BD-rate calculation based on Piece-wise Cubic Interpolation is provided in [8]. Table 1 and Table 2 below summarize the BD-rate results for CE4 subtest 3.3.2.a obtained by piece-wise cubic interpolation and polynomial interpolation, respectively. The excel template in [8] is used for obtaining the piece-wise cubic interpolation results in Table 1. The polynomial interpolation result does not include sequences Nebuta and BQTerrace.

Table 1 BD-rate results for CE4 subtest 3.3.2.a with Piece-wise Cubic Interpolation

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.4% | -0.3% | -1.4% | -3.7% | -3.8% |
| Class B | -0.5% | -0.5% | 0.2% | -0.8% | -2.3% | -2.4% |
| Class C | -0.8% | -0.8% | -0.7% | -1.2% | -2.4% | -2.3% |
| Class D | -0.4% | 0.3% | 0.2% | -0.8% | -0.7% | -0.7% |
| Class E |  |  |  |  |  |  |
| Overall | -0.7% | -0.4% | -0.1% | -1.0% | -2.3% | -2.3% |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -0.5% | -1.0% | -1.4% | -2.4% | -3.0% |
| Class C | -1.1% | -1.2% | -1.1% | -1.7% | -2.2% | -2.4% |
| Class D | -0.7% | -0.1% | 0.5% | -1.4% | -1.2% | -1.3% |
| Class E | 0.0% | 0.7% | 0.6% | -0.5% | 0.0% | -0.8% |
| Overall | -0.7% | -0.3% | -0.3% | -1.3% | -1.6% | -2.0% |

Table 2 BD-rate results for CE4 subtest 3.3.2.a with degree-3 polynomial interpolation

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |
| Class A excluding Nebuta | -0.3 | 0.0 | 0.6 |  | -0.6 | -2.7 | -2.7 |  |
| Class B excluding Terrace | -0.7 | -1.0 | -0.5 |  | -1.1 | -2.1 | -2.1 |  |
| Class C | -0.8 | -0.8 | -0.7 |  | -1.2 | -2.4 | -2.3 |  |
| Class D | -0.4 | 0.3 | 0.2 |  | -0.8 | -0.8 | -0.7 |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall excluding Nebuta & Terrace | -0.6 | -0.4 | -0.2 |  | -0.9 | -1.9 | -1.9 |  |

|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B excluding Terrace | -1.0 | -1.1 | -1.2 |  | -1.8 | -2.3 | -2.5 |  |
| Class C | -1.1 | -1.3 | -1.1 |  | -1.7 | -2.2 | -2.3 |  |
| Class D | -0.7 | -0.1 | 0.5 |  | -1.4 | -1.2 | -1.3 |  |
| Class E | 0.0 | 0.6 | 0.6 |  | -0.5 | 0.0 | -0.8 |  |
| Overall excluding Nebuta & Terrace | -0.8 | -0.5 | -0.4 |  | -1.4 | -1.5 | -1.8 |  |

It is observed from Table 1 and Table 2 that the piece-wise cubic interpolation results are mostly consistent with that of the polynomial interpolation (excluding Nebuta and BQTerrace).

For the problematic case for Nebuta discussed in Section 4.3, the BD-rate calculated by degree-3 polynomial fitting is 3.5% (as shown in Figure 9). By using piece-wise cubic interpolation, the calculated BD-rate is now -3.1% (see the attached HM30_CE4SubTest3_RDOQon_ARL_RIM_piecewisecubic.xls). This result is more consistent with the observation from the PSNR-bitrate curves in Figure 9.

In the attachment, more sample BD-rate results of degree-3 polynomial interpolation and piece-wise cubic interpolation are provided for all test sequences and settings.

# Conclusions

As evidenced in this proposal, the current BD-rate calculation for Nebuta is problematic, and may provide misleading conclusions. Similar issues in BD-rate calculation are also observed for Class-B sequence BQTerrace. It is thus recommended that BD-rate results excluding Nebuta and BQTerrace are also reported for evaluation purpose and additional information such as the PNSR-bitrate curves are to be checked when inconsistency is observed. The BD-rate calculation based on piece-wise cubic interpolation also appears to be a practical fix to this problem.

It is also recommended to further investigate the use of the current BD-rate tool in evaluation of proposals.

# References

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-Curves,” ITU-T SG16 Q.6 Document, VCEG-M33, Austin, US, April 2001.

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document, VCEG-AI11, Berlin, Germany, July 2008.

- K. Andersson, R. Sjoberg, A. Norkin, "Reliability metric for BD measurements", ITU-T SG16 Q.6 Document, VCEG-AL22, Geneva, CH, July 2009.

- Stéphane Pateux, Joel Jung, "Improvements of Excel macro for BD-gain computation", COM 16 - C.358, October 2009.

- Results of CE4, HM3.0 RDOQ-off vs. RDOQ-on, CE4SubTest3_RDOQ_on_off_RIM.xls.

- Results of CE4 subtest 3.3.2.a, CE4SubTest3_RDOQ_on_ARL_RIM.xls.

- Jie Zhao, Yeping Su, Andrew Segall, "On the calculation of PSNR and bit-rate differences for the SVT test data", COM 16 - C.404, April 2008.

- Frank Bossen, “Excel template for BD-rate calculation based on Piece-wise Cubic Interpolation”, hm32piecewisecubic2.xls.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).