| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F271 |
| --- | --- |

| Title: | Grid displacements for in-loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the complexity of the LCU- (Largest Coding Unit) based processing of Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF) operations via displacements in the filter grids. With the contribution the “Filter Design Window” in the encoder and “Filter Application Window” in decoder are matched in order to achieve uniformity in filtering within a “Filter Application Window”. Moreover the new placements of the ALF and SAO grids reduce the line memory requirement for in-loop filtering by 3 lines, which corresponds to 25% reduction.

# Introduction

# In HM3.0 three in-loop filters are utilized in sequential order, namely Deblocking Filter (DF), Sample Adaptive Offset (SAO) and Adaptive Loop Filter (ALF). The SAO and ALF filters make use of spatial adaptation which means that the filter patters and coefficients can be changed in different regions of the picture frame.

The SAO process which is defined in [1] achieves local adaptation by selecting a pixel classifier and computing the offset values independently for each filtering region. The filtering regions comprise one or more LCUs and region boundaries are aligned with the LCU boundaries. The “Region-based filter selection” for ALF, which is described in [2] also makes use of filtering regions that are aligned with the LCU boundaries. As a consequence theoretically a different SAO and ALF filter can be used to process each LCU.

There are mainly two types of philosophies in the hardware implementation of decoder, namely frame-based decoding and LCU-based decoding. In the frame-based decoding each filtering stage is performed on the whole video frame and the intermediate output is written out to the external memory. Although no line memory implementation is required in the frame based decoding operation, external memory access is formidable, since the whole reconstructed image has to be retrieved from and written back to the external memory at each filtering stage. In LCU-based decoding, the three filtering operations are carried out on an LCU one after the other. Therefore all deblocking filtering, SAO and ALF can be applied on a current LCU before the output is written to the reference frame memory. Since the frame based filtering requires formidable amount of external memory access, LCU-based filtering is considered preferable in hardware implementations.

# Problems in LCU-based Filtering

## Unnecessary filter switches in LCU-based decoding

According to LCU-based filtering DF, SAO and ALF are applied one after the other on a current LCU and the output is written to the external frame buffer. The processing of the LCU boundaries require samples from neighboring LCUs since the filter taps of DF and ALF extend beyond the LCU boundaries. Therefore a special type of on-chip memory which is called “line memory” has to be implemented for the processing of LCU boundaries.

To explain the filter/offset application in an LCU-based decoding process two terms are introduced which are “filter design window in encoder” and “filter application window in decoder”. The filter design window stands for one of the plurality of regions on a frame that are used for spatial filter adaptation. The “filter application window in decoder” stands for the region which can be filtered by the decoder during the decoding of an LCU.

In the following Figure 1, application of SAO by the decoder on the current LCU is shown. The SAO parameters are designed in an LCU aligned fashion in the encoder. Therefore the “SAO design window” in the encoder is simply the LCU (an SAO parameter set is designed for each LCU). However due to the preceding deblocking operation, “SAO application window” in the decoder is shifted by 3 pixels to the left and top. In other words the whole LCU cannot be filtered in the decoder until the right and bottom LCU neighbors are available, causing misalignment of SAO application and design windows.

Figure 1: SAO design and application windows.

In Figure 1 the 3 lines at the right and bottom LCU border are not yet processed by the deblocking filter (diagonally shaded region), hence inaccessible to the SAO operation. As a result the SAO application window, which describes the region where SAO operation is going to be applied at the current instant, is shifted by 3 lines accordingly.

In the figure below SAO application window in the decoder is shown superimposed on 4 LCUs. In the encoder an SAO parameter set is designed for each LCU (SAO parameter set comprising a pixel classification rule and offset values corresponding to each pixel class). As a result the SAO application window is divided into 4 sub-regions and different SAO parameter sets need to be applied in each sub-region.

Figure 2: SAO application window and 4 sub-regions where different SAO sets are used.

In the figure below the same problem is depicted for region-based adaptation of ALF. The filter adaptation regions are aligned with LCU boundaries. For region-based ALF, the filter application window in the decoder is off by 7 pixels to the left and 6 pixels to the top with respect to the filter design window in the encoder. As a result the same problem of sub-division of filter application window exists for region based ALF. As it is depicted in Figure 2 for the case of SAO, ALF application window is divided into 4 sub-regions and a different ALF filters need to be applied in each sub-region

Figure 3: ALF design and application windows.

In SAO and region-based adaptive ALF the filtering/offsetting operations are position dependent hence not uniform over the filter application window. As a result during the offsetting and filtering of an LCU frequent filter and SAO parameter set switches occur (although not intended) according to the coordinates of the filtered sample. Although for each LCU a single SAO parameter set and ALF filter is designed (for region-based adaptation), 4 sets of SAO parameters and 4 different ALF filters are used during the filtering of an LCU in the decoder.

## Line memory increase of 3 lines

The Figure 4 shows the situation for the block-based adaptation of ALF which is described in [3]. In the current HM the 4x4 filter selection blocks are aligned with the LCU boundaries instead of ALF application window. As a result the filter selection blocks intersect the non-deblocked region, unnecessarily delaying the ALF operation. The line memory requirement arising from the misalignment of the SAO and ALF filter grids is 3 lines in total.

Figure 4: Filter selection block in ALF which intersects non-deblocked region at the LCU boundary.

In Figure 4 the solid red square represents the 4x4 filter selection block that is utilized by block-adaptive ALF. The surrounding dashed square represents the 8x8 filter computation block. The filter decision for the 4x4 block is a function of all of the pixels inside the surrounding filter computation block. In the figure an example block is shown close to the LCU border where filter decision operation is delayed due to unavailability of 3 lines at the LCU borders. Due to the current filter grid placement in HM3.0, the ALF filtering operation is delayed unnecessarily increasing the required line memory.

In the current HM increase in the line memory caused by SAO is 1 line and block-based filter adaptation for ALF is 2 lines. Moreover the line memory increase of 3 lines in total is caused merely by the placement of the ALF and SAO grids.

# Proposed Method

In the current HM3.0 the smallest filter design window for region-based ALF and SAO is an LCU. In other words the filter switching is performed at the LCU boundaries. In the 4x4 block-based filter adaptation of ALF, the block boundaries are also aligned with the LCU boundaries.

On the other hand the filtering processes in the decoder are applied in a delayed fashion on a window which is not aligned with the LCU. The delay experienced by SAO is 3 pixels at the bottom LCU border and 3 pixels at the right border (see Figure 1). In other words the filter application window in the decoder is shifted 3 pixels up and 3 pixels to the left with respect to the LCU borders. In the case of ALF the delay is 6 pixels from the bottom LCU border and 7 pixels from the right LCU border (see Figure 3).

The proposed method is based on the principle of aligning the filter design window in the encoder and filter application window in the decoder. According to this principle the filter grids for ALF and SAO are no longer aligned with LCU borders, but rather with filter application windows of the decoder. The following figures (Figure 5 and Figure 6) show the actual displacements for the filtering grids for SAO and ALF respectively.

Figure 5: Proposed modification for the SAO filter grid.

Figure 6: Proposed modification for the ALF filter grid.

The proposed filter grid displacements offer the following benefits:

- Uniform filtering within a filter/offset application window in the decoder. For region-based ALF and SAO no switching is required depending on the pixel position.

- The line memory requirement is reduced by 1 line for SAO and 2 lines for ALF, 3 lines in total.

According to the proposal no change in the details of SAO and ALF is required.

## Processing at frame/slice boundaries

At the frame boundaries the filter application window in the decoder shrinks and expands according to the position of the LCU that is currently processed. Same situation occurs at the slice boundaries when the in-loop filtering is switched off across the slice boundaries. According to the proposed method filter design window in the encoder is also shrank or expanded to match the filter application window in the decoder.

The figure below shows an example at the top/left frame boundary for the SAO process. During the decoding of the top/left LCU, the decoder cannot process 3 lines at the bottom and right LCU borders since the preceding deblocking is not performed yet. As a result SAO application window is 3 pixels smaller than an LCU in the horizontal and vertical direction. According to the proposal the SAO design window in the encoder is also shrank to match the application window.

Figure 7: Proposed modification at the frame borders.

# Experimental Results

As a reference, the official tagged 3.0 version of the HM software [4][5] is used. The simulations are performed according to [6]. Table 1 shows the experimental results obtained for the proposal.

Table 1: Experimental results.

The obtained results show that the change in the coding gain is less than 0.01%. Moreover there is only a very slight increase in the decoding runtimes which can be considered within the limits of noise.

# Conclusion

With this contribution we propose a method for reducing the complexity of the LCU-based filtering operation, as well as reducing the line required line memory. The offsetting/filtering grids of SAO and ALF are displaced in order to match the “filter design window” in the encoder and “filter application window” in the decoder. As a result uniformity is achieved in the LCU based filtering process, which means filtering and offsetting operations are no longer position dependant. Moreover the line memory requirement for in-loop filtering is reduced from 12 lines in HM3.0 to 9 lines, a reduction corresponding to 25%. The contribution is proposed to be adopted into the next HM.

# References

- Chih-Ming Fu et. al., CE13: Sample Adaptive Offset with LCU-Independent Decoding, JCTVC-E049, Geneva, CH, March 2011.

- C.-Y. Chen et. al., CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection, JCTVC-E046, Geneva, CH, March 2011.

- I. S. Chong et. al., CE8 Subtest 2: Block based adaptive loop filter, JCTVC-E323, Geneva, CH, March 2011.

- HM 3.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.0

- T. Wiegand, W. J. Han, B. Bross, J. R. Ohm, G. J. Sullivan, WD3: Working Draft 3 of High-Efficiency Video Coding, JCTVC-E603, Geneva, Switzerland, March 2010.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).