| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F274 m20694 |
| --- | --- |

| Title: | Wavefront Parallel Processing for HEVC Encoding and Decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) Stéphane Pateux (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution describes a method to perform parallel encoding and decoding of video using HEVC. LCU lines that are processed in parallel by encoding/decoding threads. In order to limit performance degradation, a wavefront pattern of processing ensures that spatial and motion vector dependencies are fully preserved, as recommended in JCTVC-D073. In addition, a single additional probability buffer is used to synchronize CABAC probabilities down the second LCU column. The average BD-rate degradation is +0.7% (intra +0.2%, random access +0.7%, low delay +1.2%). This contribution also describes a parallel software implementation of the HM3.0 decoder using wavefront parallel processing. The average decoding time compared to anchors (sequential HM3.0) on larger sequences  (classes A,B,E) is 55% with 2 decoding threads and 33% with 4 decoding threads. This contribution also describes a combination of wavefront parallel processing with Tiles from JCTVC-E408.

# Introduction / Problem Statement

Multi-core architectures are becoming more widely available on a variety of devices including mobile terminals, and these terminals are no longer only used for content display, but also for content production. Consequently, efficient parallelism of both the HEVC video encoder and decoder is of paramount importance.

The concept of wavefront processing of the LCU has already been proposed for the decoding process of the buffered symbols when using Entropy Slices (ES) such as defined in JCTVC-D070 “Lightweight slicing for entropy coding” [1]. Using wavefront processing allows the maintenance of spatial, motion vector, and in-loop filter dependencies between LCUs. These dependencies are mandatory if  decent rate-distortion performance is to be maintained. We propose to extend the wavefront processing pattern into entropy coding in order to achieve full Wavefront Parallel Processing (WPP) of the LCUs, without requiring the step of buffering decoded symbols after entropy decoding.

# Sequential LCU processing in HEVC

In HEVC encoding and decoding LCUs are processed in raster scan order, with the CABAC entropy codec updating its probabilities on the fly. This is represented on Fig. 1. In order to encode the current LCU, the left, top-left, top, and top-right LCUs need to be available (already decoded) in order to use spatial and motion information for Intra and Inter prediction, for example. This represents a pixel and MV dependency, represented in Fig. 1 by the light grey arrows. On the other hand, when starting the entropy coding of the symbols, the CABAC probabilities used are simply the ones that are available after the previous LCU (left LCU) has been processed. In particular, due to the raster scan order, the first LCU of a line uses the CABAC probabilities available after the processing of the last LCU from the previous line.

Fig. 1 - Illustration of sequential raster scan processing of the LCUs in a frame.

# Wavefront processing of the LCUs (ideally)

Wavefront processing of LCUs is a straightforward approach to try and achieve encoding and decoding parallelism without breaking dependencies. Fig. 2 shows an example of wavefront processing of LCUs with 4 threads. Due to the spatial and MV dependency of HEVC, there must be a shift of 2 LCUs between processed LCUs of adjacent lines.

However, due to the CABAC probabilities dependency of the first LCU of each line (dependent on the processing of the last LCU of the previous line), it is not possible to start threads 2, 3, 4 before the thread processing the upper line has finished, thus preventing parallelism. This is illustrated in Fig. 2.

To solve this problem, it is possible, for example, to re-initialize the CABAC probabilities at the beginning of each line of LCUs. This is was happens in HM3.0 when using entropy slices with an LCU limit (configured so that each line of LCUs is an entropy slice). However, a large performance loss is incurred (+0.7 % in Intra,  +5.9% in Random Access, +7.8% in Low Delay).  This loss is due to the fact that the CABAC probabilities adapted to the specificities of the current image need to be learned again after each re-initialization, and this takes some time (typically, a few LCUs).

Fig. 2 - Illustration of desired wavefront processing of LCUs, unavailable probabilites at the start of each line prevent the threads from starting the encoding/decoding.

# Wavefront Parallel Processing (proposal)

In order to reduce the performance penalty, one needs to avoid losing the probabilities that have already been acquired. To achieve this in a simple way, we propose to initialize the CABAC probabilities of the first LCU of each line with the probabilities obtained after the second LCU of the upper line has been processed. This proposal is illustrated on Fig. 4. This allows for rapid learning of the probabilities down the first column of LCUs. Moreover, this can be performed without modifying the wavefront structure, as the second LCU of the upper line is always available as a consequence of spatial and MV dependencies. To achieve this, a single additional probability buffer is required. When a thread starts to process the first LCU of the line, it reads the probabilities from the buffer. When a thread has finished processing the second LCU of a line, it writes its probabilities into the buffer. Due to the wavefront approach, this guarantees that the probabilities are read and written in the proper line order.

Fig. 3 - Illustration of the proposed method. At the start of each line, the CABAC probabilities are synchronized with the probabilities of the upper right LCU.

# Advantages

It is asserted that the proposed method has the following advantages.

## Fixed encoding and decoding design

The size, shape, and number of parallel processed areas do not change between each frame. We assert that this is useful for H/W design. Also, it allows a guarantee that if N processing units are available, N threads are used on each frame, regardless of whether the sequence is class D or class A. We assert that the increasing availability of multi-core mobile devices makes parallel encoding and decoding of even moderate resolutions a desired feature.

## Works both for encoder and decoder

Since the design is fixed, it is not only possible to use WPP when decoding, but also when encoding. We assert that parallel encoding will also become useful, for instance on a multi-core mobile device with high resolution video capture capabilities (smartphones, tablets) and/or for mobile videoconference.

## Parallelizes entropy codec AND pixel codec

In the proposed approach, the complete LCU processing path is parallelized (including both entropy processing and pixel processing). In particular, during decoding, there is no need to buffer the decoded symbols of the whole frame before starting the decoding process.

## Processes interleaved lines of LCUs

As described by a previous study by Texas Instrument (JCTVC-D243 “Analysis of entropy slice approaches” [3]), we assert that processing interleaved lines of LCUs is good for:

- throughput (because interleaved lines of LCUs process similar areas and « cross » the same type of content)

- memory bandwidth (entropy encoder/decoder and pixel encoder/decoder both run in parallel)

## Transcoding is only an entropy-level operation

In many use cases, it may be useful to convert a sequential bitstream into a parallel-friendly bitstream, or to convert a bitstream between different levels of parallelism. Using the WPP approach, these conversions are only entropy-level operations. In fact, entropy decoding and encoding can be done on the fly, without buffering the symbols. Moreover, converting a compressed bitstream into a parallel friendly bitstream is a lossless operation with limited compression degradation.

# Experiment1: R/D performance using WPP

We integrated Wavefront Parallel processing (WPP) in HM3.0 and tested with the following conditions:

- High efficiency (for CABAC use)

- Intra, Low delay, Random access

- JCTVC-E700 [4] conditions

Moreover, since each thread generates its own substream, it is necessary to provide substream pointers in the bitstream, so that the decoder can find the beginning of each bitstream and start decoding immediately. The performance figures shown here have been produced for 2 substreams per frame.

As shown on table 1, the average loss is 0.7% (0.2% in Intra, 0.7% in Random Access, and 1.2% in Low Delay, mostly due to class E sequences).

Table 1. Summary RD results of WPP versus HM3.0 anchors

# Experiment 2: multithreaded software implementation of HM3.0 decoder

A real multithreaded version of the HM3.0 decoder has been implemented. As the HM software is based on sequential coding of LCUs, the addition of parallel processing involves only high-level software modifications enabling wavefront parallel processing of the main decode loop, and parallelization of several post-processing steps that are performed in a “full-frame” fashion.

A brief discussion of these high-level modifications follows.

## WaveFront Parallel Processing of the Main Decode Loop

A small “work management” object is created that allows a thread to determine which LCU it should decode.  This object guarantees that the “wavefront” synchronization required for correct LCU decoding is honored.

## Deblocking Filter

For operating the deblocking filter, the frame is split into horizontal bands of LCUs, the number of bands corresponding to the number of threads.  Each thread contains an instance of the original TComLoopFilter object, but only performs filtering in its assigned band.  The loop filtering itself occurs in two passes (the first performing vertical filtering of horizontal edges, the second performing horizontal filtering of vertical edges).  These passes have been separated out into two methods, loopFilterPicH and loopFilterPicV.  Each thread performs these two passes in its band.  A given thread waits for the 1st pass of the thread above to complete before starting its second pass – this requires a minimal amount of synchronization between threads.

## Adaptive Loop Filter

As with the deblocking filter above, the frame is split into horizontal bands of LCUs, each thread operating in a particular band.  The current ALF implementation required a few extra synchronization points between threads during initialization operations in order to preserve the current implementation as much as possible.  It is the first thread that traverses the quadtree structure initializing the ALF control flags of each LCU, for example.

The later processing that occurs in the routine TComAdaptiveLoopFilter::calcVar occurs in two passes.  For a given thread, the 2nd pass requires some information from the 1st pass of the thread “below” – the presence of this information is signaled with a semaphore.

The implementation of the second pass occurring within calcVar was also modified in a minor way.  The original second pass was re-storing some temporary values in a global store even though those values were never re-used later.  These temporary values are now no longer re-stored.  This modification was not simply due to an efficiency concern – the re-storing of these temporary values overwrites original data that is potentially required by another thread at LCU band boundaries.

## Sample Adaptive Offset

Due to the nature of SAO, the frame is divided into columns of LCUS rather than bands, the number of columns corresponding to the number of threads.

SAO processing at the decoder operates through a quad-tree structure layered over the frame.  Each thread traverses this structure but only processes LCUs that intersect with the thread’s column of LCUs.

## Motion Data Compression

In order to simulate compression of motion vector information, the frame is divided into bands of LCUs.  Each thread runs TComDataCU::compresMV on CUs in its band.

## Start-Code Searching

An implementation issue was noticed on windows systems, concerning the way in which NALU start-codes were searched for.  A std::fstream::read() call was used on a byte-by-byte basis to read the compressed stream data when searching for the “0001” NALU header marker.  Although functionally correct, this resulted in significant locking overhead on windows systems operating with multi-threading libraries, even though there was no locking conflict (only the main thread scans the compressed input stream in this way).  This single-threaded overhead reduced the amount of achievable parallelism when using multiple-threads.  A small rewrite of TVideoIOBitsStartCode::xFindNextStartCode was performed to simply read the input stream in units of 1000 bytes rather than byte by byte, in order to reduce this overhead. It should be noted that the unthreaded “reference times” reported here, against which other decodes are compared, include this rewrite. This is in order to not unduly improve the parallelized timing results.  (Reading the stream in 1000 byte chunks is slightly faster than reading it byte by byte.)

## Results

We have tested the parallel software implementation of HM3.0 decoder  with the following conditions:

- High efficiency (for CABAC use)

- Intra, Low delay, Random access

- JCTVC-E700 [4] conditions

Test sequences have been encoded once with HM3.0 encoder using WPP and 16 substreams or less (less when the sequence has less than 16 lines of LCUs per frame).

The current code base assumed that LFCrossSliceBoundaryFlag: 1 is set in the configuration file (set as part of the common test conditions).  Parallelization is not fully present when LFCrossSliceBoundaryFlag: 0 is set.  Only one-slice-per-frame has been tested.

The compressed files are then decoded with the parallel software implementation of the HM3.0 decoder using 2 threads, and using 4 threads. The following tables shows the decoding time compared to the reference HM3.0 decoding times.

Table 2. Summary of decoding time using the parallel software implementation, compared to HM3.0 reference decoding time. Green indicates the most relevant scenarios.

| DECODING with 2 THREADS | Intra | Random Access | Low Delay |
| --- | --- | --- | --- |
| class A (2560x1600) 4Mp/frame | 54 % | 56 % | - |
| classB (1920x1080) 2Mp/frame | 55 % | 57 % | 55 % |
| classE (1280x720) 1Mp/frame | 55 % | - | 57 % |
| class C (832x480) 0.4Mp/frame) | 58 % | 59 % | 59 % |
| class D (416x240) 0.1Mp/frame | 67 % | 68 % | 69 % |
| DECODING with 4 THREADS | Intra | Random Access | Low Delay |
| class A (2560x1600) 4Mp/frame | 32 % | 32 % | X |
| classB (1920x1080) 2Mp/frame | 33 % | 34 % | 33 % |
| classE (1280x720) 1Mp/frame | 33 % | - | 37 % |
| class C (832x480) 0.4Mp/frame) | 41 % | 41 % | 41 % |
| class D (416x240) 0.1Mp/frame | 60 % | 66 % | 66 % |

Table 3 shows the rate-distortion performance achieved when encoding the sequences with 16 substreams or less (less when the sequence has less than 16 lines of LCUs per frame). The difference of performance compared to experiment 1 is due to the additional substream entry points that need to be signaled (this is independent from the parallelization method).

Table 3. Summary RD results of WPP versus HM3.0 anchors

# Experiment 3: Compatibility with Tiles

We have implemented a combination of WPP and Tiles [5] using HM3.0+Tiles software provided by eBrisk. In this combination, wavefront processing is simply performed within a tile. We have then tested the following combinations in a side-by-side scenario:

- Scenario A: 2 independent  tiles per frame (side-by-side), where each tile is processed by 2 threads (see figure 4)

- Scenario B: 2 dependent tiles per frame (side-by-side), where the whole frame is processed by 2 threads (see figure 5)

Figure 4: Two independent tiles per frame are used, and each tile is processed by 2 threads. The green arrows indicate the CABAC synchronization of WPP and the red squares indicate the termination of a substream.

Figure 4: Two dependent tiles per frame are used, and the whole frame is processed by 2 threads. The green arrows indicate the CABAC synchronization of WPP. After processing the first tile, each thread simply continues to the next tiles, therefore a9 follows a8 and b9 follows b8.

Table 4 shows the performance achieved by the combination.

Table 4. Summary RD results of HM3.0+WPP+Tiles versus HM3.0+Tiles reference

|  | Scenario A | Scenario B |
| --- | --- | --- |
| Intra | 0.2 | 0.2 |
| Random Access | 0.9 | 0.7 |
| Low Delay | 1.4 | 1.1 |
| Low Delay P | 1.2 | 0.9 |

# Syntax

We propose to support WPP with the following syntax and semantics:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| if(entropy_coding_mode_flag ) { |  |
| entropy_coding_synchro | u(v) |
| if(entropy_coding_synchro!=0) |  |
| num_substreams_minus1 | ue(v) |
| } |  |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if(entropy_coding_synchro != 0) { |  |
| for(i=0;i< num_substreams_minus1;i++){ |  |
| substream_length_mode; | u(2) |
| substream_length[i]; | u(v) |
| } |  |
| } |  |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

entropy_coding_synchro specifies whether a specific synchronization process for context variables is invoked before decoding the first macroblock of a line of macroblocks or the first macroblock of a slice. If entropy_coding_synchro is different from zero, the specific synchronization process for context variables is invoked. entropy_coding_synchro also specifies whether a specific memorization process for context variables is invoked prior to the decoding decoding of a CUwhere x0 is equal to (1<<log2MaxCUSize)*(entropy_coding_synchro) and y0%(1<<log2MaxCUSize) is equal to 0.

num_substreams_minus1 specifies the number of substream_length_mode and substream_length syntax elements to be decoded from the slice header.

substream_length_mode specifies the number of bits to be used to decode syntax element substream_length., as defined by the following table.

substream_length_mode

| - substream_length_mode | - Bits used to decode substream_length |
| --- | --- |
| - 0 | - 8 |
| - 1 | - 16 |
| - 2 | - 24 |
| - 3 | - 32 |

substream_length[i] specifies the number of bits used to set the bistream table BitStreamTable[i].

# Conclusion

We propose Wavefront Parallel Processing (WPP) for HEVC video encoding and decoding. The proposed approach combines moderate R/D loss with a number of advantages:

- fixed parallel design,

- applicable to both encoding and decoding,

- parallelization of the full LCU processing path (memory bandwidth efficiency)

- bitstream size is independent from number of threads

- interleaved lines of LCUs (throughput efficiency)

- conversion between sequential and parallel-friendly bitstreams is only an entropy level operation

We have shown that this design can achieve substantial acceleration of HEVC decoder in our parallel software implementation.

We have shown that this design is compatible with other LCU-scan orders such as Tiles.

Given the significance of parallelization for HEVC standard, we submit WPP to JCT-VC for adoption into the HM.

# Patent rights declaration(s)

Orange Labs may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-D070, “Lightweight slicing for entropy coding”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-D073, “Periodic inits for wavefront coding functionality”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-D243, “Analysis of entropy slice approaches”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-E700, “Common test conditions and software reference configurations”, 5th JCT-VC Meeting, Geneva, March 2011.

- JCTVC-E408 , “Tiles”, 5th JCT-VC Meeting, Geneva, March 2011.

# Appendix: Modification of HEVC Working Draft 3 to include WPP

## 5.9 Variables, syntax elements, and tables

Syntax elements in the bitstream are represented in bold type. Each syntax element is described by its name (all lower case letters with underscore characters), its one or two syntax categories, and one or two descriptors for its method of coded representation. The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements. When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e., not bold) type.

In some cases the syntax tables may use the values of other variables derived from syntax elements values. Such variables appear in the syntax tables, or text, named by a mixture of lower case and upper case letter and without any underscore characters. Variables starting with an upper case letter are derived for the decoding of the current syntax structure and all depending syntax structures. Variables starting with an upper case letter may be used in the decoding process for later syntax structures without mentioning the originating syntax structure of the variable. Variables starting with a lower case letter are only used within the subclause in which they are derived.

In some cases, "mnemonic" names for syntax element values or variable values are used interchangeably with their numerical values. Sometimes "mnemonic" names are used without any associated numerical values. The association of values and names is specified in the text. The names are constructed from one or more groups of letters separated by an underscore character. Each group starts with an upper case letter and may contain more upper case letters.

NOTE – The syntax is described in a manner that closely follows the C-language syntactic constructs.

Functions that specify properties of the current position in the bitstream are referred to as syntax functions. These functions are specified in subclause 0 and assume the existence of a current bitstream pointer with an indication of the position of the next bit to be read by the decoding process from the bitstream. The derivation of the current bitstream pointer is specified in subclause 8.2.4.2.

Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round parentheses including zero or more variable names (for definition) or values (for usage), separated by commas (if more than one variable).

Functions that are not syntax functions (including mathematical functions specified in subclause 5.7) are described by their names, which start with an upper case letter, contain a mixture of lower and upper case letters without any underscore character, and end with left and right parentheses including zero or more variable names (for definition) or values (for usage) separated by commas (if more than one variable).

A one-dimensional array is referred to as a list. A two-dimensional array is referred to as a matrix. Arrays can either be syntax elements or variables. Subscripts or square parentheses are used for the indexing of arrays. In reference to a visual depiction of a matrix, the first subscript is used as a row (vertical) index and the second subscript is used as a column (horizontal) index. The indexing order is reversed when using square parentheses rather than subscripts for indexing. Thus, an element of a matrix s at horizontal position x and vertical position y may be denoted either as s[ x, y ] or as syx.

Binary notation is indicated by enclosing the string of bit values by single quote marks. For example, '01000001' represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Hexadecimal notation, indicated by prefixing the hexadecimal number by "0x", may be used instead of binary notation when the number of bits is an integer multiple of 4. For example, 0x41 represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Numerical values not enclosed in single quotes and not prefixed by "0x" are decimal values.

A value equal to 0 represents a FALSE condition in a test statement. The value TRUE is represented by any value different from zero.

[…]

### Bitstream pointer process

### 8.2.4.1 Initialisation process for current bitstream pointer

This process is invoked before the slice decoding process.

Input to this process is:

–	a bitstream pointer

Outputs of this process are:

–	a current bitstream pointer

–	a variable BitStreamTable specifying a bitstream pointer table to use for later current bitstream pointer derivation

A mapping table BitStreamTable containing num_substreams_minus1+1 entries is derived.

BitStreamTable[0] is initialized to the input bitstream pointer.

When i>1 and i< num_substreams_minus1+1, BitStreamTable[i] contains a bitstream pointer pointing  substream_length[i] bits after BitStreamTable[i-1].

The current bitstream pointer is set to indicate BitStreamTable[0].

### 8.2.4.2 Derivation process for current bitstream pointer

Input to this process is:

–	a variable BitStreamTable specifying a bitstream pointer table to use for current bitstream pointer derivation

–	y0, the y-coordinate of the top-left luma sample of the current LCU being processed, relative to the y-coordinate of the top-left sample of the picture.

Output to this process is:

–	a current bitstream pointer

The current bitstream pointer is set to indicate element (( (y0/(1<< log2MaxCUSize))- (InverseRasterScan( sliceAddr, 1 ) /(1<< log2MaxCUSize)) ) %  (num_substreams_minus1+1) of BitStreamTable.

[…]

## 9.3. CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found when entropy_coding_mode_flag is equal to 1.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialisation process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found, and the initilisation process for current bitstream pointer described in subclause 8.2.4.1 is invoked.

Figure X-X – Spatial neighbour T that is used to invoke neighbourhood availability subclause relative to the current largest coding unit (informative)

Variable availableFlagT is the output of CU availability subclause 6.4.2, where the input is

cuAddress(x0+ entropy_coding_synchro *(1<< log2MaxCUSize) -1, y0-1)

In case syntax element entropy_coding_synchro is different from zero, when  starting the parsing of  a coding tree as specified in subclause 7.3.5,  if  x0 is equal to zero and y0 % log2MaxCUSize is equal to 0 and availableFlagT is equal to 1 the synchronization process of the CABAC parsing process is invoked as specified in subclause  9.3.y, otherwise if x0 is equal to 1<<log2MaxCUSize* entropy_coding_synchro and y0 % log2MaxCUSize is equal to 0 the memorization  process of the CABAC parsing process is invoked as specified in subclause  9.3.x .

In case syntax element entropy_coding_synchro is different from zero, when  starting the parsing of  a coding tree as specified in subclause 7.3.5,  if  x0 is equal to zero and y0 % log2MaxCUSize is equal to 0 , the derivation process for current bitstream pointer described in subclause 8.2.4.2 is invoked.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element mb_type and the decoded value of mb_type is equal to I_PCM, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause Error: Reference source not found.

9.3.x Memorization process of context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables TableStateSync and TableMPSSync containing the values of the variables n  and m used in the initialisation process of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag and those related to CAVLC.

For each context variable, the corresponding entries n and m of tables TableStateSync and TableMPSSync are initialized to the corresponding pStateIdx and valMPS.

9.3.y Synchronization process of context variables

Inputs of this process are variables TableStateSync and TableMPSSync containing the values of the variables n  and m used in the memorization process of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag and those related to CAVLC.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries n and m of tables TableStateSync and TableMPSSync.