| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 |  |  | Document: JCTVC-F275 m20695 |
| --- | --- | --- | --- |
| Title: | Wavefront and Cabac Flush: Different Degrees of Parallelism Without Transcoding |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) Stéphane Pateux (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

In JCTVC-F274, wavefront parallel processing (WPP) is proposed in order to enable parallel encoding and decoding. WPP consists of synchronizing the CABAC probabilities of the first LCU in each line from the second LCU of the line above, while maintaining inter-block dependencies. Thus, parallel encoding and decoding is possible with average BD-rate degradation of +0.7% (intra +0.2%, random access +0.7%, low delay +1.2%). In WPP, converting a compressed video from a given level of parallelism to another is an entropy transcoding operation. In the present contribution, it is proposed to combine WPP with a flush and re-initialization of the internal state variables of CABAC at the end of each line of LCUs. Thus, each line of LCUs is compressed into a “chunk” of bits that is independent from the desired level of parallelism. Consequently, it is asserted that converting a bitstream from a given level of parallelism to another can be achieved either by re-ordering the chunks in the bitstream or providing chunk entry points as SEI messages. Using this approach, the average BD-rate degradation is +0.9% (intra +0.2%, random access +0.9%, low delay +1.6%).

# Introduction / Problem Statement / …

Multi-core architectures are becoming more widely available on a variety of devices including mobile terminals, and these terminals are no longer only used for content display, but also for content production. Consequently, efficient parallelism of the HEVC video encoder and decoder is of paramount importance.

Furthermore, it is clear that there will be devices with very different multi-core capabilities, in particular in terms of the number of cores. Therefore it will be important to be able to provide compressed video content that matches the parallel processing capabilities of such devices without sacrificing compression efficiency.

The concept of Wavefront Parallel Processing has been introduced in JCTVC-E196 and JCTVC-F274. This proposal allows performing parallel encoding and decoding of a video with moderate BD-rate loss.

# Cabac internal state initialization at the end of each LCU line

In the current proposal, the CABAC probabilities synchronization down the first two columns of LCUs (WPP) is combined with a initialization of CABAC internal state at the end of each LCU line. This initialization consist of:

- Writing all bits that may be pending in CABAC internal state variables

- Writing a stop bit

- Resetting CABAC internal variables L and R (representing respectively the lower bound of the interval and the range of the interval) to their default values.

As a consequence, at the beginning of a LCU line,  CABAC internal state no longer depends on what has been encoded previously. We call “chunk” the part of the bitstream created by the encoding of a line of LCUs.  Figure 1 shows the chunks associated with the encoding of each line of LCUs.

Figure 1: When the encoding of the last symbol of the last LCU of every line is finished, CABAC internal state is flushed (all remaining bits are written in the bitstream) and the internal state variables L and R are initialized.

The final parallel-friendly bitstream is created by concatenating the chunks together. Two different methods are possible, depending on the syntax element lcu_lines_mode.

If lcu_lines_mode is equal to zero, the chunks are sequentially ordered (see figure 2). In this case, it is preferable that chunk entry points are provided in an SEI message (although this is not strictly necessary, as chunk entry points can otherwise be found by parsing the bitstream).

Figure 2: Bitstream organization when lcu_lines_mode is zero

If lcu_lines_mode is equal to one, the chunks are ordered so that each parallel WPP thread (see figure 3) can read the chunks sequentially. In this case, only the entry points required to identify the beginning of each substream is necessary.

Figure 3: Bitstream organization when lcu_lines_mode is one

# Discussion on transcodability

## lcu_lines_mode is zero

In the case where lcu_lines_mode is zero, the bitstream matches any level of parallelism (up to the number of LCU lines in a frame), which may be a useful feature. On the other hand, the large number of entry points to be signaled may create a substantial reduction of compression efficiency. This may be especially critical for smaller sequences and highly compressed sequences, because the entry points represent a fixed cost that may substantially increase the bitrate.

## lcu_lines_mode is one

In the case where lcu_lines_mode is one, a higher compression efficiency is achieved (measured by the current experiment, see section 4), because only the entry points that are useful to achieve the desired level of parallelism are written in the bitstream. Of course, achieving a  higher level of parallelism requires to convert the bitstream, however, this conversion is  only a parsing operation: a parsing is performed to identify the chunks entry points, and then the chunks are simply reordered to achieve the desired level of parallelism as required by WPP. Therefore no entropy re-encoding is necessary.

## Mixed scenarios

As seen previously, the two modes provide different advantages. However, it is possible to combine those advantages for certain use cases. For instance, let us imagine a video shot by a user in sequential mode. This video is uploaded to a content distribution server delivering content to mobile devices. There, it is converted into a parallel-friendly bitstream with lcu_lines_mode=0 (this being an entropy-level operation as explained in JCTVC-F274 [2]). Then a mobile device requests to access this content, informing the server about its parallel processing capabilities. The server is able to immediately provide a compressed video with lcu_lines_mode=1 (optimal compression efficiency) optimized for the parallel processing capabilities of the device. This operation simply consists in changing the syntax of the compressed file and re-ordering the chunks. Therefore, it requires no parsing, no entropy coding, while achieving the highest rate-distortion performance for the target device parallel capabilities.

# Results

We integrated the proposed method on top of Wavefront Parallel processing (WPP) in HM3.0 and tested with the following conditions:

- High efficiency (for CABAC use)

- Intra, Low delay, Random access

- JCTVC-E700 [4] conditions

The performance figures shown here have been produced for 2 substreams (in order to allow fair comparison with the performance of WPP).

As shown on table 1, the average loss is 0.9% (0.2% in Intra, 0.9% in Random Access, and 1.6% in Low Delay, mostly due to class E sequence).

Note also that compared to regular WPP coding in the same conditions, the additional CABAC flush  at the end of each line produces an average additional loss of performance of 0.2% (0% in Intra, 0.2% in Random Access, 0.4% in Low Delay).

Table 1. Summary RD results of WPP versus HM3.0 anchors

# Syntax

We propose to support WPP with end-of-line Cabac Flush with the following syntax and semantics (Note: modifications highlighted in grey correspond to Wavefront Parallel Processing from contribution JCTVC-F274 [2]. Modifications highlighted in yellow correspond to the present proposal):

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| if(entropy_coding_mode_flag ) { |  |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset | u(1) |
| if(cabac_istate_reset!=0) |  |
| lcu_lines_mode | (u1) |
| if(entropy_coding_synchro!=0) |  |
| num_substreams_minus1 | ue(v) |
| } |  |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if(entropy_coding_synchro != 0 && lcu_lines_mode==1) { |  |
| for(i=0;i< num_substreams_minus1;i++){ |  |
| substream_length_mode; | u(2) |
| substream_length[i]; | u(v) |
| } |  |
| } |  |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

entropy_coding_synchro specifies whether a specific synchronization process for context variables is invoked before decoding the first macroblock of a line of macroblocks or the first macroblock of a slice. If entropy_coding_synchro is different from zero, the specific synchronization process for context variables is invoked. entropy_coding_synchro also specifies whether a specific memorization process for context variables is invoked after decoding of a CU where x0 is equal to (1<<log2MaxCUSize)*entropy_coding_synchro and y0%(1<<log2MaxCUSize) is equal to 0.

num_substreams_minus1 specifies the number of substream_length_mode and substream_length syntax element to be decoded from the slice header.

substream_length_mode specifies that the number of bits to be used to decode syntax element substream_length, as defined in the following table.

substream_length_mode

| - substream_length_mode | - Bits used to decode substream_length |
| --- | --- |
| - 0 | - 8 |
| - 1 | - 16 |
| - 2 | - 24 |
| - 3 | - 32 |

substream_length specifies the number of bits to be read in the substream

cabac_istate_reset specifies that a specific Cabac state re-initilization process  for internal variables is invoked at the end of each LCU line.

lcu_lines_mode specifies whether chunks are provided sequentially for a whole frame or sequentially within a bitstream.

# Conclusion

We propose to combine Wavefront Parallel Processing (WPP) with an end-of-LCU-line CABAC flush  for HEVC video encoding and decoding. The proposed approach combines moderate R/D loss with the possibility to serve any level of parallelism without requiring transcoding.

# Patent rights declaration(s)

Orange Labs may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E196, “Wavefront Parallel Processing ”, 5th JCT-VC Meeting, Geneva, March 2011.

- JCTVC-F274, “Wavefront Parallel Processing for HEVC Encoding and Decoding”, 6th JCT-VC Meeting, Torino, July 2001

# Appendix: Modification of HEVC Working Draft 3 to include WPP

Note: modifications highlighted in grey correspond to Wavefront Parallel Processing. Modifications highlighted in yellow correspond to the present proposal.

## 5.9 Variables, syntax elements, and tables

Syntax elements in the bitstream are represented in bold type. Each syntax element is described by its name (all lower case letters with underscore characters), its one or two syntax categories, and one or two descriptors for its method of coded representation. The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements. When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e., not bold) type.

In some cases the syntax tables may use the values of other variables derived from syntax elements values. Such variables appear in the syntax tables, or text, named by a mixture of lower case and upper case letter and without any underscore characters. Variables starting with an upper case letter are derived for the decoding of the current syntax structure and all depending syntax structures. Variables starting with an upper case letter may be used in the decoding process for later syntax structures without mentioning the originating syntax structure of the variable. Variables starting with a lower case letter are only used within the subclause in which they are derived.

In some cases, "mnemonic" names for syntax element values or variable values are used interchangeably with their numerical values. Sometimes "mnemonic" names are used without any associated numerical values. The association of values and names is specified in the text. The names are constructed from one or more groups of letters separated by an underscore character. Each group starts with an upper case letter and may contain more upper case letters.

NOTE – The syntax is described in a manner that closely follows the C-language syntactic constructs.

Functions that specify properties of the current position in the bitstream are referred to as syntax functions. These functions are specified in subclause 0 and assume the existence of a current bitstream pointer with an indication of the position of the next bit to be read by the decoding process from the bitstream. The derivation of the current bitstream pointer is specified in subclause 8.2.4.2.

Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round parentheses including zero or more variable names (for definition) or values (for usage), separated by commas (if more than one variable).

Functions that are not syntax functions (including mathematical functions specified in subclause 5.7) are described by their names, which start with an upper case letter, contain a mixture of lower and upper case letters without any underscore character, and end with left and right parentheses including zero or more variable names (for definition) or values (for usage) separated by commas (if more than one variable).

A one-dimensional array is referred to as a list. A two-dimensional array is referred to as a matrix. Arrays can either be syntax elements or variables. Subscripts or square parentheses are used for the indexing of arrays. In reference to a visual depiction of a matrix, the first subscript is used as a row (vertical) index and the second subscript is used as a column (horizontal) index. The indexing order is reversed when using square parentheses rather than subscripts for indexing. Thus, an element of a matrix s at horizontal position x and vertical position y may be denoted either as s[ x, y ] or as syx.

Binary notation is indicated by enclosing the string of bit values by single quote marks. For example, '01000001' represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Hexadecimal notation, indicated by prefixing the hexadecimal number by "0x", may be used instead of binary notation when the number of bits is an integer multiple of 4. For example, 0x41 represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Numerical values not enclosed in single quotes and not prefixed by "0x" are decimal values.

A value equal to 0 represents a FALSE condition in a test statement. The value TRUE is represented by any value different from zero.

[…]

### Bitstream pointer process

### 8.2.4.1 Initialisation process for current bitstream pointer

This process is invoked before the slice decoding process.

Input to this process is:

–	a bitstream pointer

Outputs of this process are:

–	a current bitstream pointer

–	a variable BitStreamTable specifying a bitstream pointer table to use for later current bitstream pointer derivation

A mapping table BitStreamTable containing num_substreams_minus1+1 entries is derived.

BitStreamTable[0] is initialized to the input bitstream pointer.

When i>1 and i< num_substreams_minus1+1, BitStreamTable[i] contains a bitstream pointer pointing  substream_length[i] bits after BitStreamTable[i-1].

The current bitstream pointer is set to indicate BitStreamTable[0].

### 8.2.4.2 Derivation process for current bitstream pointer

Input to this process is:

–	a variable BitStreamTable specifying a bitstream pointer table to use for current bitstream pointer derivation

–	y0, the y-coordinate of the top-left luma sample of the current LCU being processed, relative to the y-coordinate of the top-left sample of the picture.

Output to this process is:

–	a current bitstream pointer

The current bitstream pointer is set to indicate element (( (y0/(1<< log2MaxCUSize))- (InverseRasterScan( sliceAddr, 1 ) /(1<< log2MaxCUSize)) ) %  (num_substreams_minus1+1) of BitStreamTable.

[…]

## 9.3. CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found when entropy_coding_mode_flag is equal to 1.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialisation process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found, and the initilisation process for current bitstream pointer described in subclause 8.2.4.1 is invoked.

Figure X-X – Spatial neighbour T that is used to invoke neighbourhood availability subclause relative to the current largest coding unit (informative)

Variable availableFlagT is the output of CU availability subclause 6.4.2, where the input is

cuAddress(x0+ entropy_coding_synchro *(1<< log2MaxCUSize) -1, y0-1)

In case syntax element entropy_coding_synchro is different from zero, when  starting the parsing of  a coding tree as specified in subclause 7.3.5,  if  x0 is equal to zero and y0 % log2MaxCUSize is equal to 0 and availableFlagT is equal to 1 the synchronization process of the CABAC parsing process is invoked as specified in subclause  9.3.y, otherwise if x0 is equal to 1<<log2MaxCUSize* entropy_coding_synchro and y0 % log2MaxCUSize is equal to 0 the memorization  process of the CABAC parsing process is invoked as specified in subclause  9.3.x .

In case syntax element entropy_coding_synchro is different from zero, when  starting the parsing of  a coding tree as specified in subclause 7.3.5,  if  x0 is equal to zero and y0 % log2MaxCUSize is equal to 0 , the derivation process for current bitstream pointer described in subclause 8.2.4.2 is invoked.

If x0 is equal to zero and y0 % log2MaxCUSize is equal to 0 and cabac_istate_reset is equal to 1,  then decoding process for binary decisions before termination specified in sunclause 9.3.3.2.4	is invoked, followed by	the Initialisation process for the arithmetic decoding engine specified in subclause 9.3.1.2.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element mb_type and the decoded value of mb_type is equal to I_PCM, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause Error: Reference source not found.

9.3.x Memorization process of context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables TableStateSync and TableMPSSync containing the values of the variables n  and m used in the initialisation process of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag and those related to CAVLC.

For each context variable, the corresponding entries n and m of tables TableStateSync and TableMPSSync are initialized to the corresponding pStateIdx and valMPS.

9.3.y Synchronization process of context variables

Inputs of this process are variables TableStateSync and TableMPSSync containing the values of the variables n  and m used in the memorization process of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag and those related to CAVLC.

Outputs of this process are the CABAC context variables indexed by ctxIdx..

For each context variable, the corresponding context variables pStateIdx and valMPS are initialized to the corresponding entries n and m of tables TableStateSync and TableMPSSync.