| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F276 |
| --- | --- |

| Title: | CE4-subtest3.3.2: Quantization with Adaptive Reconstruction Levels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Da-ke He En-hui Yang | Tel: Email: | xiayu wangj @rim.com dhe ehyang |
| Source: | Research in Motion Ltd., Canada |  |  |

_____________________________

# Abstract

This document reports results of CE4 subtest 3.3.2 for evaluating RIM’s proposal D384 about adaptive reconstruction levels (ARL) for four inter settings with RDOQ on or off.  For the RDOQ-on case, the overall  BD-rate averages show 0.3%, 1.0%, 0.7% and 1.3% rate saving for RAhe, RAloco, LDhe, and LDloco, respectively; The average rate saving without Nebuta and BQTerrace become 0.6%, 0.9%, 0.8%, and 1.4%; The average rate savings based on the piece-wise cubic interpolation are 0.7%, 1.0%, 0.7%, and 1.3%. For the RDOQ-off case, the overall BD-rate averages show 2.0%, 2.1%, 1.7%, and 1.8% rate saving for RAhe, RAloco, LDhe, and LDloco, respectively; The average rate saving without Nebuta and BQTerrace become 2.0%, 2.2%, 1.7% and 1.8%; The average rate saving based on the piece-wise cubic interpolation are 2.2%, 2.2%, 1.7%, and 1.8%.

# Introduction

At Daegu meeting, a data-based design of Adaptive reconstruction levels (ARL) was proposed  [2], where the reconstruction levels were computed as the centroid of empirical data fell into corresponding partitions. At Geneva meeting, a model-based solution called Adaptive deQuantization Offset (AQO) was proposed in [3], where reconstruction levels were estimated based on a Laplacian model.

The major differences between ARL in [2] and AQO in [3] are as follows:

- Data-based solution vs. model-based solution. ARL collects empirical averages as the reconstruction levels; AQO estimates the reconstruction levels by assuming the Laplacian distribution of the data.

- Multiple offsets vs. one offset. For each quantizer, ARL uses an adaptive number of offsets to determine the whole set of reconstruction levels; AQO uses one offset to represent the whole set of reconstruction levels.

- Lambda adjustment. ARL does not change lambda, while AQO is proposed with lambda change.

The core experiment 4, subtest 3 was established at Geneva meeting to evaluate the performance of the two quantization designs. This document provides results for ARL.

# Algorithm Description

ARL design shows three major features. The first is to compute the empirical centroid for reconstruction levels.  For a coded frame, the optimal reconstruction levels to minimize the quantization distortion can be computed as follows:

δi=1Nin=1Nic(i)n−i⋅q.   --- (1)

where c(i)ndenotes the n-th coefficient that is quantized to i, and Ni is the total number of coefficients that are quantized to i.

The second feature is to adaptively determine the number of offsets for representing the reconstruction set. In this contribution, a threshold Ti is set to test Ni. Only offsets corresponding to Ni >Ti  will be transmitted, while for all partitions corresponding to Ni <=Ti , an average is calculated and used for reconstruction, as

δ=Ni≤Tiδi⋅NiNi≤TiNi.    --- (2)

The third feature is to utilize the cross-frame correlation. The empirical centroids for one frame minimize the distortion for the current frame, yet it involves in an iterative/multi-pass process. This issue is addressed in ARL by utilizing the cross-frame correlation. Specifically, empirical centroids collected from one frame are transmitted and used for another frame.

# Experimental results

As a temporary resolution to the BD-rate calculation issue discussed in [4], two sets of BDrate averages are provided, i.e., BD-rate averages with Nebuta and BQTerrace and BD-rate averages without Nebuta and BQTerrace. BD-rate averages using the piece-wise cubic interpolation [5] [6] is also provided.

## Results for CE4-Subtest 3.3.2.a (ARL with RDOQ-on)

|  | Common BD-rate over all sequences |  |  |  |  |  | Common BD-rate excl. Nebuta and BQTerrace |  |  |  |  |  | PCHIP-based BD-rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V. | EncT. | DecT. | RVM | Y | U | V. | EncT. | DecT. | RVM | Y | U | V | EncT. | DecT. | RVM |
| RAHE | -0.3 | -0.4 | -0.2 | 100 | 99 | - | -0.6 | -0.4 | -0.2 | 100 | 99 | - | -0.7 | -0.4 | -0.1 | 100 | 99 | - |
| RALC | -1.0 | -2.3 | -2.3 | 101 | 98 | - | -0.9 | -1.9 | -1.9 | 101 | 98 | - | -1.0 | -2.3 | -2.3 | 101 | 98 | - |
| LBHE | -0.7 | -0.4 | -0.4 | 100 | 98 | 104 | -0.8 | -0.5 | -0.4 | 100 | 98 | 104 | -0.7 | -0.3 | -0.3 | 100 | 98 | 104 |
| LBLC | -1.3 | -1.6 | -2.0 | 100 | 97 | 104 | -1.4 | -1.5 | -1.8 | 100 | 97 | 104 | -1.3 | -1.6 | -2.0 | 100 | 97 | 104 |
| Aver | -0.8 | -1.2 | -1.2 | 100 | 98 | 104 | -0.9 | -1.1 | -1.1 | 100 | 98 | 104 | -0.9 | -1.2 | -1.2 | 100 | 98 | 104 |

## Results for CE4-Subtest 3.3.2.a (ARL with RDOQ-off)

|  | Common BD-rate over all sequences |  |  |  |  |  | Common BD-rate excl. Nebuta and BQTerrace |  |  |  |  |  | PCHIP-based BD-rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V. | EncT. | DecT. | RVM | Y | U | V. | EncT. | DecT. | RVM | Y | U | V | EncT | DecT. | RVM |
| RAHE | -2.0 | -1.4 | -1.3 | 101 | 100 | - | -2.0 | -1.4 | -1.6 | 101 | 100 | - | -2.2 | -1.1 | -1.3 | 101 | 100 | - |
| RALC | -2.1 | -1.2 | -1.3 | 99 | 99 | - | -2.2 | -1.4 | -1.6 | 99 | 99 | - | -2.2 | -1.2 | -1.3 | 99 | 99 | - |
| LBHE | -1.7 | -0.7 | -1.1 | 101 | 101 | 99 | -1.7 | -0.7 | -1.0 | 101 | 101 | 99 | -1.7 | -0.7 | -1.0 | 101 | 101 | 99 |
| LBLC | -1.8 | -0.5 | -0.7 | 100 | 101 | 99 | -1.8 | -0.6 | -0.8 | 100 | 101 | 99 | -1.8 | -0.5 | -0.8 | 100 | 101 | 99 |
| Aver | -1.9 | -1.0 | -1.1 | 100 | 100 | 99 | -1.9 | -1.0 | -1.0 | 100 | 100 | 99 | -2.0 | -0.9 | -1.1 | 100 | 100 | 99 |

## Results for CE4-Subtest 3.3.2.c (varying QP with MaxDeltaQP=2)

|  | Common BD-rate over all sequences |  |  |  |  |  | Common BD-rate excl. Nebuta and BQTerrace |  |  |  |  |  | PCHIP-based BD-rate |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V. | EncT. | DecT. | RVM | Y | U | V. | EncT. | DecT. | RVM | Y | U | V | EncT | DecT. | RVM |
| RAHE | -1.2 | -3.5 | -3.5 | 432 | 101 | - | -1.6 | -3.5 | -3.5 | 432 | 101 | - | -1.6 | -3.6 | -3.5 | 432 | 101 | - |
| RALC | -1.6 | -4.1 | -4.2 | 479 | 99 | - | -1.7 | -3.9 | -3.9 | 479 | 99 | - | -1.7 | -4.1 | -4.2 | 479 | 99 | - |
| LBHE | -0.7 | -0.9 | -1.1 | 474 | 99 | 104 | -0.8 | -1.0 | -1.1 | 474 | 99 | 104 | -0.7 | -0.9 | -1.1 | 474 | 99 | 104 |
| LBLC | -1.0 | -1.7 | -2.1 | 591 | 99 | 103 | -1.1 | -1.8 | -2.0 | 591 | 99 | 103 | -1.0 | -1.7 | -2.1 | 591 | 99 | 103 |
| Aver | -1.1 | -2.6 | -2.7 | 494 | 100 | 104 | -1.3 | -2.6 | -2.6 | 494 | 100 | 104 | -1.3 | -2.6 | -2.7 | 494 | 100 | 104 |

## PSNR-rate curves

PSNR-rate curves for all four settings for both RDOQon and RDOQoff are enclosed in the pdf files.

Fitted PSNR-log(rate) curves are enclosed as well.

## Bit-allocation

### As bit-allocation may affect the RD coding gain, the average bit-rate difference and average psnr difference for frames at different temporal levels and with different types are reported below.

### RDOQ-on, ARL (CE4-subtest 3.3.2.a)

Average bit-rate difference between ARL and anchor (kbps) for RDOQ-on cases:

|  | -rate-all | -rate-I | -rate-B | -rate-T0 | -rate-T1 | -rate-T2 | -rate-T3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA_HE | -88.43 | -1.56 | -89.59 | 1514.10 | -423.69 | -414.90 | -246.61 |
| RA_LC | -297.22 | 0.05 | -302.64 | 890.68 | -534.29 | -392.21 | -490.49 |
| LD_HE | 52.88 | 12.16 | 52.95 | 459.39 | -111.82 | -68.01 |  |
| LD_LC | -78.36 | 0.08 | -78.53 | 92.56 | -189.19 | -108.40 |  |

Average PSNR-difference (dB) between ARL and anchor for RDOQ-on cases:

|  | -psnr_I | -psnr_B | -psnr-T0 | -psnr-T1 | -psnr-T2 | -psnr-T3 |
| --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 0.00 | 0.05 | 0.16 | 0.01 | 0.02 | 0.05 |
| RA_LC | 0.00 | 0.02 | 0.09 | -0.03 | -0.02 | 0.02 |
| LD_HE | 0.01 | 0.04 | 0.09 | 0.02 | 0.03 |  |
| LD_LC | 0.00 | -0.02 | 0.00 | -0.03 | -0.02 |  |

### RDOQ-off, ARL (CE4-subtest 3.3.2.b)

Average bit-rate difference (kbps) between ARL and anchor for RDOQ-off cases:

|  | -rate-all | -rate-I | -rate-B | -rate-T0 | -rate-T1 | -rate-T2 | -rate-T3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 35.84 | 0.04 | 36.17 | -236.25 | -28.01 | 27.28 | 124.74 |
| RA_LC | 127.37 | 0.05 | 129.29 | -126.52 | 57.51 | 115.43 | 214.88 |
| LD_HE | -16.95 | 0.06 | -17.00 | -59.94 | -10.42 | 1.28 |  |
| LD_LC | -11.70 | 0.05 | -11.74 | -53.32 | -9.27 | 7.89 |  |

Average PSNR-difference (dB) between ARL and anchor for RDOQ-off cases:

|  | -psnr_I | -psnr_B | -psnr-T0 | -psnr-T1 | -psnr-T2 | -psnr-T3 |
| --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 0.00 | 0.06 | 0.07 | 0.09 | 0.06 | 0.05 |
| RA_LC | 0.00 | 0.07 | 0.09 | 0.09 | 0.07 | 0.06 |
| LD_HE | 0.00 | 0.03 | 0.02 | 0.04 | 0.03 |  |
| LD_LC | 0.00 | 0.04 | 0.03 | 0.04 | 0.04 |  |

### Varying QP tests (CE4-subtest 3.3.2.c)

Average bit-rate difference (kbps) between single QP and the varying-QP:

|  | -rate-all | -rate-I | -rate-B | -rate-T0 | -rate-T1 | -rate-T2 | -rate-T3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 344.28 | 5223.77 | 235.34 | 3393.47 | -327.58 | -352.01 | 88.72 |
| RA_LC | -46.07 | 4874.52 | -154.71 | 2574.09 | -560.02 | -427.25 | -389.72 |
| LD_HE | 73.41 | 1643.38 | 70.01 | 532.36 | -111.23 | -63.74 |  |
| LD_LC | -12.69 | 1291.10 | -15.50 | 178.53 | -115.45 | -56.91 |  |

Average PSNR-difference (dB) between single QP and the one with varying-QP:

|  | -psnr_I | -psnr_B | -psnr-T0 | -psnr-T1 | -psnr-T2 | -psnr-T3 |
| --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 0.67 | 0.17 | 0.49 | 0.12 | 0.12 | 0.16 |
| RA_LC | 0.61 | 0.12 | 0.38 | 0.08 | 0.09 | 0.11 |
| LD_HE | 0.42 | 0.07 | 0.14 | 0.04 | 0.05 |  |
| LD_LC | 0.33 | 0.03 | 0.06 | 0.02 | 0.02 |  |

Note that the varying-QP tests in CE4-Subtest 3.3.2.c uses delta-QP for all I frames. Thus, the coding gain partially comes from the bit allocation effect, as the average I-frame psnr increases by 0.33dB~0.67dB. To better understand the coding gain of varying qp for non-I-slices, a new test has been conducted for varying qp based on HM3.0 anchor with one line change to disable the search of QP for I-slices. Results of this test is summarized in the next sub-section.

## A new test of varying-QP for B-slices only

CE4 subtest3.3.2.c tested the RD performance of varying QP. However, the bit allocation is significantly changed in that test, as the average psnr for I-slices increases by 0.33dB-0.67dB. To understand the coding gain of varying qp for non-I-slices, a new test is conducted, where the varying-QP technique is only enabled for B-slices so that the average I-slice psnr is maintained unchanged in the new test.

### Test Description

- Anchor data for this subtest is HM3.0 RDOQ-on data, i.e., to run HM3.0 anchor with RDOQ on for RA-he, RA-loco, LD-he and LD-loco.

- The test data is obtained by:

  - HM3.0

  - with fix179.patch to fix a dQP transmission bug,  http://hevc.kw.bbc.co.uk/trac/attachment/ticket/179/fix179.patch

  - plus one line of code modification to disable varying-QP for I-slices

Change line 219 in TEncCu.cpp, from

=======================HM3.0 code, BEGIN ==============

218  // single-QP coding mode

219  if ( rpcCU->getSlice()->getSPS()->getUseDQP() == false )

220  {

=======================HM3.0 code, END==============

To:

=======================HM3.0 code, BEGIN ==============

218  // single-QP coding mode

219  if ( rpcCU->getSlice()->getSPS()->getUseDQP() == false ||

rpcCU->getSlice()->getSliceType()==I_SLICE )

220  {

=======================HM3.0 code, END==============

  - with “MaxDeltaQP” set to 2, which can be done by adding “ -d 2 ” to the command line argument.

### RD performance measured with three types of BD-rate numbers.

|  | Common BD-rate(%) over all sequences |  |  |  |  |  | Common BD-rate(%) excl. Nebuta and BQTerrace |  |  |  |  |  | PCHIP-based BD-rate(%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V. | EncT. | DecT. | RVM | Y | U | V. | EncT. | DecT. | RVM | Y | U | V | EncT | DecT. | RVM |
| RAHE | -0.2 | -0.6 | -0.3 | / | / | - | -0.5 | -0.8 | -0.6 | / | / | - | -0.6 | -0.6 | -0.3 | / | / | - |
| RALC | -0.6 | -1.5 | -1.5 | / | / | - | -0.7 | -1.4 | -1.4 | / | / | - | -0.8 | -1.5 | -1.4 | / | / | - |
| LBHE | -0.4 | -0.4 | -0.3 | / | / | 105 | -0.4 | -0.6 | -0.4 | / | / | 105 | -0.4 | -0.5 | -0.3 | / | / | 105 |
| LBLC | -0.8 | -0.9 | -1.1 | / | / | 103 | -0.9 | -1.0 | -1.1 | / | / | 103 | -0.8 | -0.9 | -1.1 | / | / | 103 |
| Aver | -0.5 | -0.9 | -0.8 |  |  | 104 | -0.6 | -1.0 | -0.9 |  |  | 104 | -0.7 | -0.9 | -0.8 |  |  | 104 |

Note that the enc/dec time is not collected as tests were run on different platforms. Yet the enc/dec complexity for the varying qp technique has been well understood.

### Bit allocation effect in the new test of 3.6.1.

Average bit-rate difference (kbps) between single QP and the varying-QP:

|  | -rate-all | -rate-I | -rate-B | -rate-T0 | -rate-T1 | -rate-T2 | -rate-T3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 249.89 | 0.87 | 255.00 | 2631.32 | -320.85 | -348.42 | 88.79 |
| RA_LC | -129.13 | 18.30 | -131.46 | 1881.19 | -540.82 | -420.31 | -389.26 |
| LD_HE | 71.13 | 4.11 | 71.26 | 522.23 | -110.39 | -63.67 |  |
| LD_LC | -14.80 | 10.87 | -14.86 | 168.91 | -115.02 | -56.53 |  |

Average PSNR-difference (dB) between single QP and the one with varying-QP:

|  | -psnr_I | -psnr_B | -psnr-T0 | -psnr-T1 | -psnr-T2 | -psnr-T3 |
| --- | --- | --- | --- | --- | --- | --- |
| RA_HE | 0.00 | 0.10 | 0.30 | 0.04 | 0.04 | 0.08 |
| RA_LC | 0.00 | 0.06 | 0.21 | 0.01 | 0.02 | 0.04 |
| LD_HE | 0.00 | 0.05 | 0.12 | 0.03 | 0.03 |  |
| LD_LC | 0.00 | 0.02 | 0.04 | 0.01 | 0.02 |  |

# Conclusions

ARL provides a consistent RD coding gain for both RDOQ-on and RDOQ-off cases, with negligible increase of encoding/decoding complexity.

# Appendix: related syntax

### Clause 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| arl_param() |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| …… |  |
| } |  |

The following table specifies the ARL parameters syntax.

| arl_param() { | Descriptor |
| --- | --- |
| arl_slice_flag | u(1) |
| If(arl_slice_flag) |  |
| { |  |
| arl_interY_flag | u(1) |
| if (arl_interY_flag) |  |
| { |  |
| read_arl_data(ARL_TYPE_INTERY) |  |
| arl_interUV_flag | u(1) |
| if (arl_interUV_flag) |  |
| { |  |
| read_arl _data(ARL_TYPE_INTERUV) |  |
| } | u(1) |
| } |  |
| arl_intraY_flag | u(1) |
| if(arl_intraY_flag) |  |
| { |  |
| read_arl_data (ARL_TYPE_INTRAY); |  |
| arl_intraUV_flag | u(1) |
| if(arl_intraUV_flag) |  |
| { |  |
| read_arl_data (ARL_TYPE_INTRAUV) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| read_arl_data (arl_data_type_id){ |  |
| arl_delta_idx[slice_qp][ arl_data_type_id][1] | vlc(v) |
| for(i=2; i<=4;i++){ |  |
| arl_delta_i_available | u(1) |
| if (arl_delta_i_available = = 1) { |  |
| arl_delta_idx[slice_qp][ arl_data_type_id][i] | vlc(v) |
| } |  |
| else |  |
| break |  |
| } |  |
| } |  |

Name association to arl_data_type_id

| arl_data_type_id | Data type |
| --- | --- |
| 0 | ARL_TYPE_INTRAY |
| 1 | ARL_TYPE_INRAUV |
| 2 | ARL_TYPE_INTERY |
| 3 | ARL_TYPE_INTERUV |

The following semantics is to be added to the Clause 7.4.3 Slice header semantics.

Clause 7.4.3 Slice header semantics

arl_slice_flag specifies whether there are any ARL data transmitted for this slice.

arl_interUV_flag specifies whether there are ARL for InterUV data.

arl_intraY_flag specifies whether there are ARL for IntraY data.

arl_intraUV_flag specifies whether there are ARL for IntraUV data.

arl_delta_i_available is a one-bit flag to tell the decoder whether to read one level from the bitstream, with 1 means to read.

arl_delta_idx[qP][DataType][i] specifies the reconstruction level for a quantization output of value i for a given type of Data with quantization parameter qP. Four data types are defined for transmitting ARL, i.e., InterY, InterUV, IntraY, and IntraUV. The received index arl_delta_idx[qP][DataType][i]  is firstly converted to a delta value within [-15, 15] by the following procedure:

ARLoffsetd[qP][DataType][i] = Idx2Delta [ arl_delta_idx[qP][DataType][i] ] ;

where Idx2Delta is defined as

int Idx2delta[16] = {-15, -13, -11, -9, -7, -5, -3, 0, 1, 3, 5, 7, 9, 11, 13, 15};
Second, the delta value is used to update the corresponding reconstruction level by

ARLoffset[qP][DataType][i] += ARLoffsetd[qP][DataType][i],   1<=i<=4 ;

ARLoffset[qP][DataType][i] += ARLoffsetd[qP][DataType][4],    i>4;

ARL[qP][DataType][i] = ARLoffset[qP][DataType][i] + (i<<7),   i>0 ;

The following semantics change is to be added to Clause 8.5.3 Scaling process for transform coefficients. Specifically, equation (8-158) is changed to be as follows:

dij = sgn (cij) * ( ARL[qP][DataType][ abs(cij )] * LevelScale(nS)x(nS)[ qP%6 ][ i ][ j ] ) << ( qP/6 + trafoPrecisionExt - 7), with i, j = 0..nS-1	(8-1)

where abs(cij) is the index of the quantized transform coefficient cij and ARL[qP][DataType][ abs(cij )] determines the reconstruction level corresponds to this index.

# References

[1] K. Sato, et al., “Description of Core Experiment 4: Quantization”, JCT-VC E704.

[2] X. Yu, J. Wang, D. He, and E.h. Yang, “Quantization with Hard-decision partition and adaptive

reconstruction levels for the low-delay setting,” JCT-VC D384.

[3] X. Li, X. Guo, and S. Lei, “Adaptive De-Quantization Offset”, JCT-VC E091.

[4]. J. Wang, X. Yu, and D. He, “On BD-rate calculation”, JCT-VC F270.

[5]. Jie Zhao, Yeping Su, Andrew Segall, "On the calculation of PSNR and bit-rate differences for the

SVT test data", COM 16 - C.404

[6] Frank Bossen, “Excel template for BD-rate calculation based on Piece-wise Cubic Interpolation”.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).