| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F277 |
| --- | --- |

| Title: | Method for deriving Chroma QP from Luma QP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ximin Zhang Shan Liu 2860 Junction Ave. San Jose, CA95134, USA | Emails: | ximin.zhang@mediatek.com shan.liu@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This contribution reports a couple of methods for deriving Chroma QP from Luma QP. Experimental results reportedly show an average of more than 4.85% and 2.8% BD-rate decrease for Chroma, with an average of 0.3% and 0.15% BD-rate increase for Luma for Random access configurations and Low delay configurations, respectively. Both encoding and decoding time remain the same. No subjective quality degradations have been observed. Besides of the methods, a “chroma_qp_index_offset” is proposed to add in the picture parameter set RBSP syntax.

- Introduction

In HEVC (High Efficiency Video Coding) current version HM3.0, the Chroma quantization parameter (QP) is derived from Luma quantization parameter in the same way as used in H.264/AVC. That is, it only encodes the luma and Chroma planes using the same quantization parameter up to 29; after that, chroma is quantized by a lower QP than luma until the largest QP with luma at QP=51 and chroma at QP=39. Specifically, the above Luma/Chroma QP mapping was implemented in HM3.0 software as below.

if(eTxtType != TEXT_LUMA) //Chroma

{

iQP  = g_aucChromaScale[ iQP ];

}

const UChar g_aucChromaScale[52]=

{

0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,

12,13,14,15,16,17,18,19,20,21,22,23,24,25,26,27,

28,29,29,30,31,32,32,33,34,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39

};

# Proposed Methods

In this contribution, we report a couple of methods for deriving Chroma QP from Luma QP.

## Method 1

For Intra pictures (or slices) all CU levels, the below mapping table is used instead of the mapping table shown in Section 1.

const UChar g_aucChromaScale_1[52]=

{

0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,

12,13,14,15,16,17,18,19,20,21,21,22,23,24,24,25,

26,27,28,29,29,30,31,32,33,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39

};

For the “next layer” i.e. (QP+1) B pictures (or slices), the below mapping table is used.

const UChar g_aucChromaScale_2[52]=

{

0, 1, 2, 3, 4, 5, 6, 7, 8, 9,10,11,

12,13,14,15,16,17,18,19,20,21,21,22,23,24,25,26,

26,27,28,29,30,30,31,32,33,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39

};

For other B pictures (or slices), the Luma to Chroma QP mapping remains unchanged, i.e. using the mapping table shown in Section 1.

Figure 1 Proposed method 1 Luma to Chroma QP mapping.

## Method 2

For Intra pictures (or slices) and the “next layer” i.e. (QP+1) B pictures (or slices), the Luma to Chroma QP mapping is the same as stated in Method 1. For the (QP+2) B pictures (or slices), the Luma to Chroma QP mapping remains unchanged, i.e. using the mapping table shown in Section 1. For other B pictures (or slices), the below mapping table is used.

const UChar g_aucChromaScale_3[52]=

{

0, 2, 3, 4, 5, 6, 7, 8, 9,10,11,12

13,14,15,16,17,18,19,20,21,22,23,24,25,26,27,28,

28,29,30,31,32,32,33,33,34,34,35,35,36,36,37,37,

37,38,38,38,39,39,39,39

};

Figure 2 Proposed method 2 Luma to Chroma QP mapping.

## Proposed syntax modification

In this contribution, a “chroma_qp_index_offset” is proposed to add in the picture parameter set RBSP syntax. It is highlighted in the following table.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| chroma_qp_index_offset | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

chroma_qp_index_offset specifies the offset that shall be added to Luma QP for addressing the table of Chroma QP values for the Cb and Cr chroma component. The value of chroma_qp_index_offset should be in the range of -12 to +12, inclusive.

# Simulation results

Simulations were conducted following common test conditions defined in JCTVC-E700Error: Reference source not found. Results are shown in table 1 and table 2.

Table 1	 Results for method 1 Luma to Chroma QP mapping.

|  |  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0.2 | -5.2 | -5.6 | 0.3 | -4.3 | -5.1 |
|  | Class B | 0.4 | -5.6 | -5.8 | 0.4 | -5.7 | -6.1 |
|  | Class C | 0.3 | -4.3 | -4.2 | 0.4 | -4.2 | -4.0 |
|  | Class D | 0.4 | -4.8 | -4.9 | 0.3 | -4.5 | -4.5 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0.3 | -5.0 | -5.2 | 0.4 | -4.7 | -5.0 |
|  | Enc Time[%] | 99% |  |  | 101% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.2 | -2.4 | -2.7 | 0.3 | -3.3 | -3.9 |
|  | Class C | 0.2 | -1.9 | -2.0 | 0.2 | -2.1 | -2.2 |
|  | Class D | 0.1 | -2.1 | -1.4 | 0.2 | -2.9 | -2.3 |
|  | Class E | 0.0 | -1.7 | -2.5 | 0.2 | -6.8 | -6.4 |
|  | All | 0.1 | -2.1 | -2.1 | 0.2 | -3.6 | -3.5 |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2	 Results for method 2 Luma to Chroma QP mapping.

|  |  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0.2 | -5.0 | -5.5 | 0.3 | -4.1 | -5.0 |
|  | Class B | 0.4 | -5.6 | -5.8 | 0.4 | -5.6 | -6.1 |
|  | Class C | 0.3 | -4.3 | -4.2 | 0.4 | -4.3 | -4.1 |
|  | Class D | 0.4 | -4.7 | -5.0 | 0.3 | -4.6 | -4.6 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0.3 | -5.0 | -5.2 | 0.3 | -4.7 | -5.0 |
|  | Enc Time[%] | 99% |  |  | 101% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.2 | -2.4 | -2.5 | 0.2 | -3.2 | -3.5 |
|  | Class C | 0.2 | -1.9 | -2.0 | 0.2 | -2.2 | -2.2 |
|  | Class D | 0.1 | -2.2 | -1.5 | 0.2 | -2.6 | -2.5 |
|  | Class E | -0.1 | -2.0 | -2.4 | 0.1 | -7.1 | -6.1 |
|  | All | 0.1 | -2.1 | -2.1 | 0.2 | -3.5 | -3.4 |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution reported a couple of methods for deriving Chroma QP from Luma QP. Experimental results reportedly show an average of greater than 4.85% BD-rate decrease for Chroma, with an average of 0.3% BD-rate increase for Random access configurations; an average of greater than 2.8% BD-rate decrease for Chroma, with an average of 0.15% BD-rate increase for Low delay configurations, respectively. There is no additional encoding or decoding complexity increase. No obvious subjective quality degradations have been observed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.