| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F281 WG 11Number :m20701 |
| --- | --- |

| Title: | CE 7: Cross Check Report for Mode Dependent Intra Residual Coding for 8x8 blocks (JCTVC-F138) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for JCTVC-F138 proposal on Mode Dependent Intra Residual Coding for 8x8 blocks for Intra HE, LC, Random Access HE and LC settings. The simulation results as verified by Samsung match closely with the results provided by the proponents in JCTVC-F138.

# Introduction

JCTVC-F138 [1] proposes a combination of tools for coding the intra prediction residuals by using the information in the intra prediction mode. Mode-dependent DCT/DST was first proposed in the literature in [2], and subsequently in [3-5]. The mode-dependent DCT/DST proposal in [6] for 4x4 blocks was adopted by HEVC standardization committee in the Geneva meeting in March 2011.

In JCTVC-F138, the proponents apply mode-dependent DCT/KLT transform for coding 8x8 blocks of intra-prediction residues. Here is the brief summary of the proposed tool in JCTVC-F138:

Table 1 Summary for Tools in JCTVC-E138

| Applied for Transform Unit sizes | 8x8 |
| --- | --- |
| Number of additional transform matrices at Size 8x8 | 1 |
| HM 3.0 Quantization matrix re-use | Yes |
| 8x8 DST Implementation | Full Matrix Multiplication |
| Scan modification with HM 3.0 Anchor | No |

In this cross-check, we have performed the cross-check for Intra HE, LC, Random Access HE and LC settings only.

# Mode Mappings

The same mode-mappings as being used for 4x4 mode-dependent DCT/DST in HM 3.0 are used for the proposed 8x8 mode-dependent DCT/KLT in JCTVC-F138.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [7] were followed for conducting the simulations. Tests were conducted on Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity configurations.

## R-D Results

Table 2 shows the simulation results for JCTVC-F138 proposal as cross-checked by Samsung. Detailed results are available in the attached Excel sheet JCTVC-F281.xls. The test results matched closely with those provided by the proponents.

Table 1: Simulation Results for JCTVC-F138 proposal mode-dependent intra residual coding in Core Experiment, as cross-checked by Samsung

|  |  |  | All Intra HE |  |  |  |  | All Intra LC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y |  | U | V |  | Y |  | U |  | V |  |  |  |
|  | Class A |  | -0.8 |  | 0.2 | -0.2 |  | -0.4 |  | -0.3 |  | -0.5 |  |  |  |
|  | Class B |  | -0.4 |  | 0.0 | 0.1 |  | -0.1 |  | -0.4 |  | -0.4 |  |  |  |
|  | Class C |  | -0.3 |  | -0.1 | 0.0 |  | -0.3 |  | -0.3 |  | -0.3 |  |  |  |
|  | Class D |  | -0.2 |  | -0.1 | -0.1 |  | -0.2 |  | -0.3 |  | -0.3 |  |  |  |
|  | Class E |  | -0.4 |  | 0.4 | 0.1 |  | -0.2 |  | -0.1 |  | -0.3 |  |  |  |
|  | Overall |  | -0.4 |  | 0.1 | 0.0 |  | -0.2 |  | -0.3 |  | -0.4 |  |  |  |
|  | Enc Time[%] |  | 101% |  |  |  |  | 101% |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 101% |  |  |  |  | 102% |  |  |  |  |  |  |  |
|  |  |  | Random Access HE |  |  |  |  | Random Access LC |  |  |  |  |  |  |  |
|  |  |  | Y |  | U | V |  | Y |  | U |  | V |  |  |  |
|  | Class A |  | -0.3 |  | 0.3 | -0.4 |  | -0.2 |  | -0.1 |  | -0.2 |  |  |  |
|  | Class B |  | -0.3 |  | 0.2 | 0.3 |  | -0.1 |  | 0.0 |  | -0.1 |  |  |  |
|  | Class C |  | -0.2 |  | 0.1 | -0.1 |  | -0.1 |  | -0.2 |  | -0.2 |  |  |  |
|  | Class D |  | -0.1 |  | 0.0 | -0.2 |  | -0.1 |  | -0.1 |  | -0.3 |  |  |  |
|  | Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Overall |  | -0.2 |  | 0.2 | -0.1 |  | -0.1 |  | -0.1 |  | -0.2 |  |  |  |
|  | Enc Time[%] |  | 100% |  |  |  |  | 100% |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 99% |  |  |  |  | 100% |  |  |  |  |  |  |  |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Complexity

The 8x8 transform is implemented via full matrix multiplication.

Operation Counts: The number of operations for performing 8x8 KLT is 1024 multipliers, 1024 adds, and 128 shifts.

Bit Shift at Intermediate Levels: In 8x8 KLT implementation, the proponents have shifted the output of 8x8 KLT (used either as vertical or horizontal transform) by 7-bits similar to the 7-bit shift for DCT in HM 3.0.

Scanning: In JCTVC-F138, the proponents have retained conventional HM 3.0 scans.

# Conclusions

We have cross-checked JCTVC-F138 as part of Core Experiment 7, studied their algorithms and verified the simulation results.

# References

1. R. Joshi et al., “CE7: Tool 1 - Mode dependent intra residual coding for 8x8 blocks,” JCTVC-F138, Torino, Italy, July 2011.

2. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

3. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

4. A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

5. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

6. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction," JCTVC-E125, Geneva, Switzerland, March 2011.

7. R. Cohen et al., “Description of Core Experiment 7 (CE7): Additional Transforms,” JCTVC-E707, Geneva, Switzerland, March 2011.