| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F282 WG 11Number :m20702 |
| --- | --- |

| Title: | CE7: Mode-dependent 8x8 DCT/DST for Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

It was previously shown by Han, Saxena & Rose in ICASSP 2010, and in JCTVC-C108 that following intra prediction, the optimal transform is not DCT, but DST Type-7 with performance close to KLT, along the direction of prediction. The 4x4 DST from JCTVC-E125 was adopted in the HEVC Geneva meeting in March 2011. In this contribution, experimental results for applying mode-dependent DCT/DST at block size 8x8 are presented. The proposed DCT/DST transform scheme is applied at block sizes 8x8, while the conventional DCT is retained at larger block sizes. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. The conventional quantization tables for HM 3.0 are retained and no changes have been made to the scanning order. Experimental results are provided with HM 3.0 as anchor for the test conditions as stipulated in Core Experiment 7, and average BD Rate gains of 0.2 %, 0.0 %, 0.2 % and 0.0 % are respectively obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings.

# Introduction

In our previous proposals [1] at Guangzhou, [2] in Daegu, and [3] in Geneva, we proposed using either the conventional DCT or a DST for the intra prediction residue based on the prediction mode. It was originally shown by Han et al. in [4], that following intra prediction, the optimal transform for vertical (respectively horizontal) modes along the vertical (respectively horizontal) direction of prediction is the following Type -7 DST:

[TS]i,j=22N+1sin(2i−1)jπ2N+1

with performance close to KLT (In the above DST equation, the block size is NxN, and i,j are the row and column indices of the 2-d DST matrix). In [1], we showed that the above DST is the optimal transform for a class of oblique modes in unified intra prediction. We also showed how to choose between the transforms: the conventional DCT and the DST per direction based on the intra prediction mode in unified intra prediction [5]. An independent study, where DST was shown to be the optimal transform for intra-prediction residues was also performed in [6,7].

In this contribution, we present our experimental results for the stipulated test conditions in [8].

# Mapping from Unified Intra Directions to 8x8 DCT/DST

We retain the same mapping table from unified intra prediction modes to the horizontal and vertical transform for the 8x8 mode-dependent DCT/DST as was present in HM 3.0 for 4x4 mode-dependent DCT/DST.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [8] were followed for conducting the simulations. The anchor was HM 3.0 with mode-dependent DCT/DST at size 4x4 only and the conventional DCT at block sizes 8x8 and larger, and the performance of the proposed mode-dependent 8x8 DCT/DST transform scheme was evaluated for the following 4 settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

Experiment 1: Proposed 8x8 DCT/DST with HM 3.0 as anchor

Table 1 shows the simulation results when the proposed 8x8 DCT/DST scheme is applied at size 8x8. Detailed results are available in the attached Excel sheet JCTVC-F282_8x8DST.xls.

Table 1: Simulation Results for mode-dependent 8x8 DCT/DST with HM 3.0 as anchor

|  | Intra |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | -0.6 | 0.0 | -0.3 | -0.2 | -0.4 | -0.6 |  |
| Class B | -0.1 | -0.2 | -0.1 | 0.2 | -0.6 | -0.6 |  |
| Class C | 0.0 | -0.2 | -0.2 | -0.1 | -0.4 | -0.4 |  |
| Class D | 0.0 | -0.2 | -0.2 | 0.0 | -0.3 | -0.3 |  |
| Class E | -0.2 | 0.0 | -0.3 | 0.0 | -0.4 | -0.6 |  |
| All | -0.2 | -0.1 | -0.2 | 0.0 | -0.4 | -0.5 |  |
| Enc Time[%] | 102% |  |  | 100% |  |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  |  |
|  |  | Random access |  |  | Random access LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | -0.3 | 0.1 | -0.4 | -0.1 | -0.2 | -0.4 |  |
| Class B | -0.2 | 0.3 | 0.3 | 0.1 | -0.1 | -0.2 |  |
| Class C | -0.1 | 0.1 | -0.1 | -0.1 | -0.2 | -0.3 |  |
| Class D | -0.1 | -0.1 | -0.2 | -0.1 | -0.4 | -0.2 |  |
| Class E |  |  |  |  |  |  |  |
| All | -0.2 | 0.1 | -0.1 | 0.0 | -0.2 | -0.3 |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |  |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed DST was derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 3.0.

Transform Matrices: We only require the storage of one additional matrix corresponding to DST at block size 8x8. The 8x8 transform is implemented via full matrix multiplication.

Operation Counts : The matrix multiplication for 8x8 DST in our proposal requires 1024 multiplications, 1024 additions and 128 shifts.

Bit Shift at Intermediate Levels: In our implementation, we shift the output of 8x8 DST (used either as vertical or horizontal transform) by 7-bits similar to the 7-bit shift for DCT in HM 3.0.

Scanning: In this proposal we have retained the conventional HM 3.0 scans

## 3.4 8x8 DST Transform Matrix

The following matrix for 8x8 DST was used in simulations. Here the norms of the basis vectors of 8x8 DST are matched to be similar to those for 8x8 DCT in HM 3.0.

const short g_as_DST_MAT_8 [8][8]=

{

{    16,    32,    46,    59,    70,    79,    84,    87,},

{    46,    79,    87,    70,    32,   -16,   -59,   -84,},

{    70,    84,    32,   -46,   -87,   -59,    16,    79,},

{    84,    46,   -59,   -79,    16,    87,    32,   -70,},

{    87,   -16,   -84,    32,    79,   -46,   -70,    59,},

{    79,   -70,   -16,    84,   -59,   -32,    87,   -46,},

{    59,   -87,    70,   -16,   -46,    84,   -79,    32,},

{    32,   -59,    79,   -87,    84,   -70,    46,   -16,},

};

# Conclusions

In this contribution, we have presented the results for our earlier proposals JCTVC-C108, JCTVC-D033 and JCTVC-E125 in Core Experiment 7 for using mode-dependent DCT/DST transform [1,2] at size 8x8 for intra-prediction residues instead of the conventional DCT. The proposed mode-dependent DCT/DST scheme requires the storage of only 1 additional DST matrix at a block-size, is based on the intra prediction mode, and does not require any explicit signaling. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show gains in compression performance as compared to HM 3.0 anchors.

# References

1. A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

2. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

3. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction," JCTVC-E125, Geneva, Switzerland, March 2011.

4. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

5. Min et al., “Unification of the Directional Intra Prediction Methods in TMuC,” ITU-T JCTVC-B100_revision02, Geneva, Switzerland, July 2010.

6. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

7. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Results for mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-C037, Guangzhou, China, October 2010.

8. R. Cohen et al., “Description of Core Experiment 7 (CE7): Additional Transforms,” JCTVC-E707, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).