| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F283 WG 11Number :m20703 |
| --- | --- |

| Title: | CE 7: On fast implementation of 4-point DST Type-7 with 5 multiplications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

The 4x4 mode-dependent DCT/DST proposal from JCTVC-E125 was adopted in Geneva meeting in March 2011. The existing 4-point (29,55) DST in HM 3.0 requires 8 multiplications. In this proposal, a low complexity version of the 4x4 DST termed as (28,56) DST which requires only 5 multiplications is presented. Experimental results are provided with HM 3.0 as anchor for the test conditions as stipulated in Core Experiment 7, and average BD Rate change of 0.0 %, is obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings.

# Introduction

Mode-dependent DCT/DST was first published in literature in [1] and later in [2-5]. The 4-point DST Type-7 transform from JCTVC-E125 [5] was adopted in Geneva meeting in March 2011. Core Experiment 7 [6] was established to evaluate the performance of various additional transforms as well as simplification of the 4x4 DST in HM 3.0. In this contribution, we present our experimental results for the low-complexity DST under stipulated test conditions in [6].

# Tool Description

The 4-point DST Type-7 matrix in HM3.0 is given by the following 4x4 matrix:

{29,  55,    74,   84},

{74,  74,    0 ,  -74},

{84,  -29,   -74,   55},

{55,  -84,    74,  -29}.

For simplicity, we term the above matrix as (29,55) DST matrix. The above matrix requires 8 multiplications and 11 additions to implement in HM 3.0. In this proposal, we make the following minor changes to the 4x4 (29,55) DST matrix elements:

1. Change all the elements with values +/-29 to +/-28

2. Change all the elements with values +/-55 to +/-56

The resulting approximated DST matrix, which we term as “(28,56)” DST matrix then is:

{28,  56,    74,   84},

{74,  74,    0 ,  -74},

{84,  -28,   -74,   56},

{56,  -84,    74,  -28}.

We next show the implementation of the forward and inverse (28,56) DST.

## Forward (28,56) DST:

For a 4-point input vector X, and output Y, the forward (28,56) DST can be implemented via the following steps:

// Intermediate Variables

c[0] = X[0] + X[3];

c[1] = X[1] + X[3];

c[2] = X[0] – X[1];

c[3] = 74* X[2];

// Output values

Y[0] =  28 * (c[0] + (c[1] << 1) )     + c[3]

Y[1] =  74 * (X [0]+ X[1]  - X[3])

Y[2] =  28 * (c[2]  + (c[0] << 1) ))  -  c[3]

Y[3] =  28 * ((c[2]<<1)   - c[1] )   + c[3]

The above implementation requires only 5 multiplications, 11 adds and 3 shifts, where as the HM 3.0 forward (29, 55) DST requires 8 multiplications and 11 adds.

## Inverse (28,56) DST:

For a 4-point input vector Y, and output Z, the inverse (28,56) DST can be implemented via the following steps:

// Intermediate Variables

c[0] = Y[0] + Y[2];

c[1] = Y[2] + Y[3];

c[2] = Y[0] - Y[3];

c[3] = 74* Y[1];

// Output values

Z[0] =   28 * (c[0] + (c[1] << 1)   ) + c[3]

Z[1] =   28 * ((c[2]<<1)  - c[1]   ) + c[3]

Z[2] =   74 * (Y[0] - Y[2]  + Y[3])

Z[3] =   28 * ((c[0]<<1)   + c[2] )  - c[3]

Again, the inverse DST implementation for (28,56) DST requires only 5 multiplications, 11 adds and 3 shifts, where as the HM 3.0 inverse (29, 55) DST requires 8 multiplications and 11 adds.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [6] were followed for conducting the simulations. The anchor was HM 3.0 with (29,55) DST, and the performance of the proposed (28,56) DST was evaluated for the following 4 settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

Experiment 1: Proposed (28, 56) DST with HM 3.0 as anchor

Table 1 shows the simulation results when the proposed (28,56) DST was used instead of HM 3.0’s (29,55) DST. Detailed results are available in the attached Excel sheet JCTVC-F283_28_56_DST.xls.

Table 1: Simulation Results for (28,56) DST instead of (29,55) DST in HM 3.0 with HM 3.0 as anchor

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  |  | Random access HE |  |  | Random access LC |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | -0.4 | 0.1 | -0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.2 | 0.0 | -0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

From the above results, we can see that there is 0.0 % BD Rate change when (28,56) DST is used instead of (29,55) DST in HM 3.0. However, the number of multiplications required to implement the 4x4 DST goes down from 8 to 5, implying (8-5)/8 * 100 = 37.5 % savings for the multiplications.

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed DST was derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 3.0.

Bit Shift at Intermediate Levels: No change from HM 3.0.

Scanning: No change from HM 3.0 scans.

# Conclusions

In this contribution, we have presented the results for a low-complexity (28,56) DST to replace the existing (29,55) DST in HM3.0. The proposed (28,56) DST requires only 5 multiplications instead of 8 for the (29,55) DST, implying a saving of 37.5 % for the multiplications. Simulation results show 0.0 % BD Rate change as compared to HM 3.0 anchors. We recommend adopting this proposal in HM.

# References

1. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

2. A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

3. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

4. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

5. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction," JCTVC-E125, Geneva, Switzerland, March 2011.

6. R. Cohen et al., “Description of Core Experiment 7 (CE7): Additional Transforms,” JCTVC-E707, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).