| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F285 |
| --- | --- |

| Title: | CE8 Subset 2: Parametric Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ehsan Maani 1730 N 1st Street San Jose, CA 95112 |  | Wei Liu 1730 N 1st Street San Jose, CA 95112 |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides a brief description and results of Parametric Adaptive Loop Filter which was introduced as a low complexity alternative to conventional ALF techniques [JCTVC-D270]. Furthermore, in this document, a hybrid mode consisting of combination of PALF and online-trained ALF of HM3.0 is introduced. Parametric ALF is composed of a set of fixed filters which are classified based on their direction and strength. The encoder chooses the best (in RD sense) filter for a Coding Unit (CU) and transmits the index of the filter in the bit stream to the decoder. The selection of the best filter at the encoder requires only a single-pass processing for each Large CU (LCU). On average, this technique,  could achieve 1.6%, 3.1%, 3.7%, and 4.5% gain compared to HM 3.0 anchors for Low Complexity (LC) Intra, and random access, low delay B and P, respectively. The hybrid (combination) of PALF and ALF, on average, achieved 0.1%,  0.4%, 0.9%, and 1.0% gain compared to ALF only filtering in HM3.0 Intra, and random access, low delay B and P, respectively.

# Algorithm description

In contrast to most conventional ALF schemes, Parametric ALF (PALF), uses fixed filters and single-pass, LCU-level optimization to find the best filters at the encoder. In other words, once an LCU is reconstructed at the encoder and de-blocking filter is applied, the PALF process can start immediately to find a of filters for that LCU. No further processing for that LCU will be required once the filter indices are obtained. A more detailed description of PALF can be found in [JCTVC-D270]. The same directional filter shapes introduced in [JCTVC-E320] is used here except that the vertical tap of the filters is reduced to 7 or 5 depending on the experiment. The current implementation of PALF has 8 directions and 4 different strengths for each direction plus an additional all-pass filter (no filtering). Thus, the total number of filter for PALF is 33. Nonetheless,  various types of symmetry described in the next section can reduce the number of filters to 13 (12+1) filters.

## Filter symmetry

ALF filter coefficients are generally assumed to be symmetric with respect to the center. This is also the case for all PALF filters. However, due to the shape and specific direction of the filters, other types of symmetry can also be assumed. For example, mirror symmetry with respect to direction of the filter can be assumed for horizontal, vertical and diagonal () filters. Figure 1(a) shows this symmetry for vertical and diagonal filters. In addition, filter of any direction , is assumed to be a rotated ( rotation) version of the filter designed for its perpendicular direction . This reduces the total number of filters that have to be stored by half. However, the mirror symmetry for the diagonal directions and the rotational symmetries are only valid for filters that have symmetric shapes (e.g., 9x9, 7x7, or 5x5), and thus do not apply to the asymmetric filter shapes such as 9x7 or 9x5 which are mainly used for the luma component in order to reduce line memory.

Figure 1. (a) Mirror symmetry with respect to direction of the filter. (b) Filter of one direction (vertical left) is mirror or another direction (vertical right).

Nonetheless, regardless of the filter shape, vertical left and horizontal down filters are assumed to be mirrors (with respect to vertical axis) of the vertical right and horizontal up filters, respectively.  This symmetry reduces the total number of filters by 8 (2 directions, 4 strengths each). Figure 1(b) illustrates this symmetry for vertical left and vertical right filters.

## Signaling of filter indices

At each LCU, first, the maximum depth in the CU partition quadtree () -at which the PALF parameters are signaled- is transmitted to the decoder. Next, if the depth of a CU within the LCU is less than or equal to , both the filter strength level and the direction index (corresponding to the value of ) are transmitted in the bit stream. The explicit transmission of  at each LCU allows the encoder to process LCUs independently in a single pass.

Both the strength and the direction are predicted from the neighboring CUs. If the value matches the prediction only 1 bit is transmitted to signal this event to the decoder, otherwise, the residual value is transmitted. Truncated Unary and fixed length codes are used to signal the residual values of the strength levels and direction indices, respectively.

Chroma filter signaling is exactly the same as luma, with the exception than  is not transmitted and always assumed to be zero.

## Combination with ALF

For the High Efficiency (HE) case, a combination of PALF (fixed filters) and ALF (online-trained variable filters) can be envisioned. Despite the fact that the online-trained Wiener filters are optimal in MSE sense, since the filter coefficients for multiple filters have to be transmitted, they may not be able to reduce the RD cost significantly, especially at low bit-rates that compression artifacts are more visible. Primarily due to this reason, having the option of selecting fixed filters provides compression efficiency gain and visible picture quality improvements especially at lower bit-rates. Furthermore, since fixed filters are trained offline, more complex classification methods can be used to obtain a "good" set of filters.

In order to construct a hybrid of PALF and existing ALF, we assumed that the online trained filters (along with their classifications) provide an additional filter strength which does require explicit signaling of direction. Thus, together with the online filter, a total of 6 filter strengths (with indices ranging from 0 to 5) are possible, where 0 corresponds to no filtering, 1 corresponds to online filter, and 2-5 correspond to the 4 fixed filter strengths. Only for filter strength levels ranging from 2 to 5, direction is also transmitted.  Fixed filters are selected only if they are useful in reducing the RD cost of the CU and ultimately the entire frame/slice. An additional slice level flag is transmitted to indicate to the decoder whether online filter, fixed filters or both are available. More specifically, five different combination modes are considered where the first two are also available in HM3.0. One of these five options is selected at the slice level and signaled to the decoder:

- ALF without the CU-level on/off switching

- ALF with the CU-level on/off switching

- PALF with CU-level filter switching (PALF is not possible without the CU-level switching)

- ALF + PALF + CU-level filter switching (includes the all-pass filter, i.e., no filtering)

- ALF + PALF + CU-level filter switching, excluding the all-pass filter

At the encoder, first a 5x5 online filter is designed for the entire frame. Then, for each CU the best (RD optimized) filter among the available filters is selected. Note that the available filters depends on the slice-level combination mode described above. For example, if combination mode 5 is selected, then all filters (four fixed strengths and one online filter) are available at the CU-level except the all-pass filter (filter strength zero). Combination of ALF and PALF for chroma, only includes modes 1, 3, and 5.

After CU-level parameters (strength and direction) are obtained, for the next round of designing the online filter, only the regions of the image for which the online filter was selected as the best filter are used in the training.  Since, with the existence of the fixed filter, the encoder usually converges faster, the 5x5 online filter is re-designed only once. Next, different filter taps are also tested similarly to the HM3.0 encoder.

# Results

In our experiments, we consider 9x7 luma and 5x5 chroma together in one set of experiment and 9x5 luma and 5x3 chroma filters in another set.  The number of filters to store is 12 for symmetric filter shapes (i.e., 5x5) and 24 for asymmetric filter shapes (i.e., 9x7, 9x5 and 5x3). Nevertheless, in our current software implementation, we duplicated some of the filters for easy implementation and thus always stored 32 filters. However, in an efficient implementation, we do not need to store a filter if it can be derived from another already stored filter using symmetry. Instead, the function that handles the filtering process multiplies the correct coefficient to the correct pixel location. The average number of non-zero coefficients per filter is 20 for 9x7 and 16 for 9x5.

Exhaustive search is used to find the best filter indices. Common Test conditions are used and the results are compared to HM3.0 Anchors. Error: Reference source not found shows the results for the Intra only, low delay, and random access cases. In this case, 9x7 luma and 5x5 chroma filters were used. In case of HE, a combination of ALF and PALF (hybrid ALF) is used. For LC, on the other hand, only fixed filters (PALF) is used to reduce the encoder and decoder complexity due to ALF. The performance of ALF only is also shown for the LC case.

To better understand the combination of ALF and PALF, for the case of low delay B, we plotted the average percentages of CUs (of any size) that used one of the four PALF strengths in Figure 2. The CUs that chose no filtering (all-pass filter) were counted as neither ALF nor PALF, hence, the percentage of ALF CUs and PALF CUs sums up to 100% in all cases. As expected, PALF is used more often at lower bit-rates since transmitting multiple online filters is more costly in such cases. In addition, as it can seen, sequences with strong sharp edges (e.g., Basketball sequences) also use PALF more frequently.

Figure 2. Percentage of CU's that used PALF in a slice where CU-level filter switching is applied.

Table 2 shows the results with the exact same settings except chroma lambda is increased (to 1.8 times the original lambda) to get a more balanced luma and chroma gain.  The maximum number of bits the encoder can spend on chroma in PALF is much more than that of ALF (obviously with a benefit of lower distortion), therefore, the effect of chroma lambda is more visible.

Results of only fixed filters (only PALF with no online filter) is presented in Table 3. 9x5 luma and 5x3 chroma filters are used in this experiment with the modified chroma lambda. Note that in this case, hybrid ALF is not used since 9x7 filters are used as online filters in HM3.0. By comparing LC results of Table 3 and Table 2 it can be seen that 9x5 filters have on average 0.1% loss compared to 9x7 filters for all 3 cases of Intra, low delay, and random access.

Table 1. HE: Hybrid ALF.  LC: Only fixed filter (PALF).

|  | All intra HE (ALF+PALF) |  |  | All intra LC (PALF) |  |  | All intra LC (ALF) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2 | -1.6 | -2.8 | -2.4 | -1.3 | -1.7 | -3.8 | -1.0 | -1.4 |
| Class B | 0.1 | -1.1 | -2.2 | -1.2 | -2.2 | -2.5 | -2.2 | -2.3 | -1.7 |
| Class C | -0.2 | -0.6 | -1.2 | -1.3 | -2.9 | -3.9 | -2.0 | -2.6 | -3.7 |
| Class D | -0.2 | 0.1 | -0.1 | -0.7 | -2.2 | -2.6 | -0.9 | -2.2 | -2.7 |
| Class E | 0.3 | -1.9 | -1.6 | -1.9 | -2.5 | -3.6 | -3.3 | -3.8 | -4.6 |
| Overall | 0.0 | -1.0 | -1.6 | -1.5 | -2.2 | -2.8 | -2.4 | -2.3 | -2.7 |
| Enc Time[%] | 99% |  |  | 115% |  |  | 139% |  |  |
| Dec Time[%] | 104% |  |  | 125% |  |  | 155% |  |  |
|  | Random access HE |  |  | Random access LC |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1 | -3.5 | -4.9 | -4.1 | -4.6 | -4.6 | -6.2 | -3.0 | -3.9 |
| Class B | -0.3 | -3.2 | -3.9 | -2.6 | -3.6 | -2.8 | -4.3 | -1.4 | 0.0 |
| Class C | -0.4 | -2.5 | -2.6 | -2.5 | -2.1 | -3.1 | -3.0 | 0.8 | -0.6 |
| Class D | -0.5 | -0.8 | -1.2 | -2.8 | -0.9 | -0.9 | -2.7 | 0.6 | 0.7 |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.3 | -2.5 | -3.2 | -3.0 | -2.8 | -2.9 | -4.1 | -0.8 | -0.9 |
| Enc Time[%] | 100% |  |  | 103% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 116% |  |  | 129% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -4.8 | -6.1 | -3.5 | -6.4 | -6.3 | -4.8 | -1.1 | -0.4 |
| Class C | -0.8 | -4.3 | -5.3 | -3.5 | -4.6 | -6.5 | -3.8 | 0.7 | -1.0 |
| Class D | -0.8 | -1.6 | -2.1 | -2.7 | -2.0 | -2.0 | -2.5 | 0.4 | 0.6 |
| Class E | -1.1 | -5.1 | -7.7 | -5.2 | -11.8 | -11.5 | -6.0 | -6.6 | -1.0 |
| Overall | -0.8 | -3.9 | -5.2 | -3.6 | -5.9 | -6.2 | -4.2 | -1.3 | -0.4 |
| Enc Time[%] | 100% |  |  | 103% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 116% |  |  | 131% |  |  |
|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  | Low delay (P) LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.7 | -5.4 | -6.5 | -5.1 | -8.4 | -7.7 | -6.9 | -2.5 | -1.5 |
| Class C | -0.9 | -4.4 | -5.6 | -3.3 | -5.0 | -6.7 | -3.6 | 0.7 | -1.0 |
| Class D | -0.8 | -1.9 | -2.7 | -1.9 | -2.7 | -2.7 | -1.5 | 0.0 | 0.5 |
| Class E | -1.3 | -5.6 | -8.6 | -7.9 | -14.3 | -13.4 | -8.7 | -9.0 | -2.4 |
| Overall | -0.9 | -4.3 | -5.7 | -4.4 | -7.2 | -7.3 | -5.1 | -2.3 | -1.1 |
| Enc Time[%] | 100% |  |  | 105% |  |  | 110% |  |  |
| Dec Time[%] | 101% |  |  | 119% |  |  | 135% |  |  |

Table 2. Modified chroma lambda results. HE: Hybrid ALF. LC: Only fixed filters.

|  | All intra HE (ALF+PALF) |  |  | All intra LC (PALF) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.6 | -1.0 | -2.5 | -0.6 | -1.0 |
| Class B | -0.1 | -0.5 | -1.0 | -1.3 | -1.3 | -1.5 |
| Class C | -0.3 | -0.2 | -0.6 | -1.4 | -2.5 | -3.4 |
| Class D | -0.2 | 1.0 | 1.0 | -0.7 | -1.8 | -2.1 |
| Class E | 0.1 | -0.1 | -0.2 | -2.1 | -0.2 | -0.9 |
| Overall | -0.1 | -0.1 | -0.4 | -1.6 | -1.3 | -1.8 |
| Enc Time[%] | 100% |  |  | 116% |  |  |
| Dec Time[%] | 102% |  |  | 124% |  |  |
|  | Random access HE |  |  | Random access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -1.5 | -2.1 | -4.2 | -2.9 | -3.0 |
| Class B | -0.4 | -1.6 | -1.9 | -2.7 | -1.7 | -1.2 |
| Class C | -0.5 | -1.3 | -1.2 | -2.6 | -1.2 | -2.0 |
| Class D | -0.6 | 0.0 | -0.2 | -2.9 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -1.1 | -1.4 | -3.1 | -1.5 | -1.6 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 116% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -2.7 | -3.4 | -3.6 | -3.5 | -3.4 |
| Class C | -1.0 | -3.0 | -3.7 | -3.6 | -3.0 | -4.5 |
| Class D | -0.9 | -0.6 | -0.4 | -2.8 | -1.0 | -0.9 |
| Class E | -1.2 | -2.4 | -4.1 | -5.5 | -6.4 | -6.5 |
| Overall | -0.9 | -2.2 | -2.8 | -3.7 | -3.3 | -3.6 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 115% |  |  |
|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -3.2 | -3.7 | -5.2 | -5.3 | -4.6 |
| Class C | -1.0 | -3.1 | -3.6 | -3.5 | -3.5 | -5.0 |
| Class D | -0.9 | -1.1 | -1.1 | -2.0 | -1.4 | -1.4 |
| Class E | -1.3 | -2.8 | -4.7 | -8.2 | -9.0 | -8.6 |
| Overall | -1.0 | -2.6 | -3.2 | -4.5 | -4.6 | -4.7 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 119% |  |  |

Table 3. PALF only results with 9x5 luma and 5x3 chroma filters.

|  | All Intra HE |  |  |  |  | All Intra LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | Y | U |  | V |  |
| Class A | 1.6 | -0.1 |  | -0.2 |  | -2.4 | -0.8 |  | -1.2 |  |
| Class B | 1.0 | 0.4 |  | -1.0 |  | -1.2 | -0.9 |  | -1.3 |  |
| Class C | 0.6 | 0.5 |  | 0.0 |  | -1.3 | -2.1 |  | -3.0 |  |
| Class D | 0.2 | 0.9 |  | 0.9 |  | -0.7 | -1.5 |  | -1.6 |  |
| Class E | 1.7 | -0.1 |  | 1.1 |  | -1.9 | -0.4 |  | -0.9 |  |
| Overall | 1.0 | 0.3 |  | 0.1 |  | -1.5 | -1.2 |  | -1.6 |  |
| Enc Time[%] | 89% |  |  |  |  | 113% |  |  |  |  |
| Dec Time[%] | 86% |  |  |  |  | 119% |  |  |  |  |
|  | Random Access HE |  |  |  |  | Random Access LC |  |  |  |  |
|  | Y | U |  | V |  | Y | U |  | V |  |
| Class A | 2.2 | 1.4 |  | 0.8 |  | -4.0 | -3.0 |  | -3.0 |  |
| Class B | 1.6 | 0.2 |  | -1.3 |  | -2.7 | -1.6 |  | -1.1 |  |
| Class C | 0.4 | -1.3 |  | -1.0 |  | -2.5 | -0.8 |  | -1.7 |  |
| Class D | 0.1 | -0.2 |  | -0.4 |  | -2.6 | 0.0 |  | 0.1 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 1.1 | 0.0 |  | -0.5 |  | -2.9 | -1.4 |  | -1.4 |  |
| Enc Time[%] | 97% |  |  |  |  | 102% |  |  |  |  |
| Dec Time[%] | 89% |  |  |  |  | 110% |  |  |  |  |
|  | Low delay B HE |  |  |  |  | Low delay B LC |  |  |  |  |
|  | Y | U |  | V |  | Y | U |  | V |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.1 | -0.8 |  | -2.3 |  | -3.6 | -3.0 |  | -2.8 |  |
| Class C | 0.1 | -3.2 |  | -3.4 |  | -3.5 | -2.8 |  | -4.2 |  |
| Class D | -0.2 | -0.7 |  | -0.2 |  | -2.6 | -1.0 |  | -1.0 |  |
| Class E | 0.6 | -1.7 |  | -3.1 |  | -5.2 | -5.3 |  | -5.4 |  |
| Overall | 0.4 | -1.6 |  | -2.2 |  | -3.6 | -2.9 |  | -3.2 |  |
| Enc Time[%] | 97% |  |  |  |  | 102% |  |  |  |  |
| Dec Time[%] | 87% |  |  |  |  | 110% |  |  |  |  |
|  | Low delay P HE |  |  |  |  | Low delay P LC |  |  |  |  |
|  | Y |  | U |  | V | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.9 |  | -1.4 |  | -3.0 | -5.2 |  | -4.9 |  | -4.3 |
| Class C | 0.1 |  | -3.6 |  | -3.7 | -3.5 |  | -3.2 |  | -4.5 |
| Class D | -0.5 |  | -1.0 |  | -1.2 | -2.0 |  | -1.5 |  | -1.3 |
| Class E | 0.8 |  | -1.4 |  | -4.8 | -7.8 |  | -7.5 |  | -7.9 |
| Overall | 0.6 |  | -1.8 |  | -3.1 | -4.5 |  | -4.1 |  | -4.3 |
| Enc Time[%] | 96% |  |  |  |  | 103% |  |  |  |  |
| Dec Time[%] | 87% |  |  |  |  | 111% |  |  |  |  |

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).