| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F286 WG11 Number: m20706 |
| --- | --- |

| Title: | Redundancy removal for Run-mode in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This proposal removed the redundancy in Run-mode coding of CAVLC by replacing unary code with truncated unary code. Experimental results show that the BD-rate saving is 0.1% for Intra only, 0.1% for random access and 0.0% for low delay B under low complexity configuration.

# Introduction

In HEVC test Model (HM) 3.0, Context-based Adaptive Variable Length Coding (CAVLC) for transform coefficients encodes using Variable Length Code (VLC). These coefficients are scanned to 1-D array and encoded by three steps: 1) last nonzero coefficient position, 2) run-mode and 3) level-mode. In each step, an event is mapped to a codenumber. Then the codenumber is mapped to a codeword based on the corresponding VLC table. There are 14 VLC tables available in HM3.0. The selection of VLC tables is based on the context defined as “maxrun”.

The maximal codenumber can be derived based on “maxrun”. Since the maximum is known, there will be redundancy in encoding the last codenumber when using unary codes.

# Proposed solution

In HM3.0, “maxrun=0” case has been improved using truncated unary code (ref to VLC table index=8). However, “maxrun=1”, “maxrun=2” and other cases are still encoded with unary code or unary-like code, i.e. VLC table index = 0.

To remove the redundancy, we proposed to use truncated unary code for all cases where VLC table index = 0 is selected. In HM3.0, the VLC table indexes are stored in the variables shown as follows.

const UInt g_auiVlcTable8x8Inter[29] = {8,0,0,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

const UInt g_auiVlcTable8x8Intra[29] = {8,0,0,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

const UInt g_auiVlcTable16x16Intra[29] = {8,0,0,0,0,5,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,2};

const UInt g_auiVlcTable16x16Inter[29] = {8,0,1,1,1,1,2,2,2,2,2,2,2,6,6,6,6,6,6,3,3,3,3,3,3,3,3,3,3};

VLC tables are extended to 18 including 14 from HM3.0 to support truncated unary codes with various number of symbols. The extra four VLC tables are VLC table index 14 encodes max number of symbols is 5, VLC table index 15 encodes max number of symbols is 7, VLC table index 16 encodes max number of symbols is 9 and VLC table index 17 encodes max number of symbols is 11. The new VLC table indexes are stored in the variables shown as follows.

const UInt g_auiVlcTable8x8Inter[29] = {8,14,15,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

const UInt g_auiVlcTable8x8Intra[29] = {8,14,15,5,5,5,5,5,5,5,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,6,3};

const UInt g_auiVlcTable16x16Intra[29] = {8,14,15,16,17,5,5,5,5,5,5,5,5,5,5,5,5,5,1,1,1,1,1,1,1,1,1,1,2};

const UInt g_auiVlcTable16x16Inter[29] = {8,14,1,1,1,1,2,2,2,2,2,2,2,6,6,6,6,6,6,3,3,3,3,3,3,3,3,3,3};

# Experimental results

## Coding conditions

The conditions are the same as specified in JCTVC-E700[1] for low complexity settings.

## Test set

- The same set as specified in JCTVC-E700 is used.

## Results

A summary of simulation results is shown in Table 1. Complete results are available in “Results-JCTVC-F286.xls”.

Table 1 Simulation results of redundancy removal in Run-mode

|  | All intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | -0.1 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 |
| Class D | -0.1 | -0.1 | -0.1 |
| Class E | -0.1 | 0.0 | -0.1 |
| Overall | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random access LC |  |  |
|  | Y | U | V |
| Class A | 0.0 | -0.3 | 0.2 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | -0.2 | -0.1 |
| Class E |  |  |  |
| Overall | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay (B) LC |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1 | 0.0 | -0.2 |
| Class C | -0.1 | -0.2 | -0.3 |
| Class D | 0.0 | 0.1 | -0.2 |
| Class E | 0.0 | 0.1 | -0.5 |
| Overall | 0.0 | 0.0 | -0.3 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

This proposal removed the redundancy from Run-mode in CAVLC. BD-rate savings were observed while no complexity was introduced. We proposed to adopt this solution to HEVC.

# Reference

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March. 2011

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).