| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F288 |
| --- | --- |

| Title: | CE11: Unified scans for the significance map and coefficient level coding in high efficiency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In JCTVC-E335, a unification of scans for significance map coding and coefficient level coding in high efficiency was proposed. This contribution analyses and shows the results of the technique in HM3.0. It consists of two parts. In the first one, the scan for the coefficient level coding is the same as the scan used for the coding of the significance map, but in reverse order, i.e., from the last significant coefficient to the first (DC). For this part, the BD-rate for AI-HE, RA-HE and LB-HE configuration is 0.05℅, 0.07℅, and 0.01℅, respectively. In the second part, scans for significance map coding and coefficient level coding are both in the reverse order. For this case, the BD-rate for AI-HE, RA-HE, and LB-HE is 0.25℅, 0.19℅, and 0.07℅, respectively.

- Introduction: scanning process in HEVC

Current high efficiency transform coefficient coding is implemented using CABAC. The scanning patterns for significance map coding and coefficient level coding are different. JCTVC-E335 [2] proposed a unification of these scans. This contribution analyses the JCTVC-E335 unification in HM3.0 within the scope of CE11.

The scanning for significance map coding is a forward zig-zag, horizontal or vertical pattern (depending on the prediction mode). In contrast, for coding absolute transform coefficient levels, each block is processed in sets of 4×4 sub-blocks by using a forward zig-zag scan; while the transform coefficient levels inside a sub-block are processed in a backward zig-zag scan [1]. The context model set for each sub-block consists of five context models for the first bin and five models for the second bin of the coefficient level minus 1. The set of contexts for a sub-block depends on the statistics of already coded sub-blocks.

This approach uses 60 contexts: 6 sets of 10 contexts distributed as follows. For a 4×4 block, 10 models are used (5 models for bin1 and 5 models for bin 2). There are 5 different sets of these 10 models for blocks larger than 4×4. The context set selection depends on the number of coefficients larger than 1 in the previous 4×4 sub-block. Table 1 summarizes these context sets and model bins.

## Issues

The coefficient scanning scheme is complex and it would be desirable to simplify the scanning processes as much as possible without impacting coding efficiency [3]. Part of the current complexity arises from the different scan patterns for significance map and coefficient level data and from the forward-backward scan for the coefficient level. The following issues in the current HM are solved in this contribution:

- No need to store an additional set of scans for the coefficient level coding pass

  - Avoid backward-forward scan within the block

- Less processing due to using the same scan for significant and coefficient level

  - More coefficient levels have to be checked when the scans are different

- Harmonization of context sets for coefficient level from all block sizes

- Parsing: currently, the coefficient level parsing depends on the intra mode

  - Proposed solution makes parsing independent of the intra mode

- SDIP: current 4×4 sub-block processing is not well harmonized with transform sizes smaller than 4 (e.g. 1×16, 16×1, 8×2 and 2×8 transforms found in SDIP)

- Parallelization: harmonizing scans in the same direction allows for an earlier availability of the contexts, thus helping parallelization in the context derivation

- Reduction of the number of context derivation methods for significance map: currently, there 4 different methods depending on the location of the coefficient in the block (cf. 4 colors in Fig. 1)

| Context Set (forward 4x4 sub-block scan) |  |  | Model bin 1 |  |  | Model bin 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | For block size 4x4 only |  | 0 | 1 or more larger than 1 |  | 0 | Initial - no larger than one |
| 1 | 0-3 LargerT1 in prev subblock |  | 1 | Initial - no trailing ones |  | 1 | 1 larger than one |
| 2 | 4-7 LargerT1 in prev subblock |  | 2 | 1 trailing one |  | 2 | 2 larger than one |
| 3 | 8-11 LargerT1 in prev subblock |  | 3 | 2 trailing ones |  | 3 | 3 larger than one |
| 4 | 12-15 LargerT1 in prev subblock |  | 4 | 3 or more trailing ones |  | 4 | 4 or more larger than one |
| 5 | First 4x4 subblock (for blocks>4x4) 16 LargerT1 in prev subblock |  |  |  |  |  |  |

Table 1-Contexts Set and Context for bin 1 and bins 2 for the coefficient level in HM3.0

Figure 1- Context derivation of the significance map for blocks larger than 8×8 in HM3.0

- Technical description

Part 1 of this contribution unifies both scans, so that level data coding re-uses the zig-zag, horizontal or vertical scan employed in the significance map. Additionally, the same context sets are used for all block sizes.

Part 2 of the contribution further unifies the scans, so that both significant map and coefficient level follow the same backward order, from the last significant coefficient to the first. In this way, all the scans in a TU follow exactly the same process: same scan (zig-zag, vertical or horizontal) in the same direction.

## Part 1: Unified scan pattern

The proposed method applies the scan used for the significance map (zig-zag, vertical or horizontal) to coefficient level coding. In this way, no additional scans have to be introduced. The scan is not forward-backward like the current one. Instead, it only follows a backward direction: from the last significant coefficient to the first (DC component).

Current method in HM3.0 divides the coefficient level scan in sub-sets, where each sub-set is a 4×4 sub-block. The proposed algorithm keeps the sub-sets concept in a similar way as in [3]: each sub-set consists of 16 consecutive coefficients in the scanning order. Therefore, the coefficient level scan goes from the last significant coefficient to the first coefficient, conceptually partitioned in different sub-sets in order to derive the contexts to apply.

The context derivation is harmonized, so that it doesn’t depend on the block size. The context set depends on whether the sub-set is sub-set 0 (defined as the sub-set with the lowest frequencies, i.e., containing the DC coefficient) or not, as shown in table 2.

| Context Set (inverse 16 scan) |  |  |
| --- | --- | --- |
| 0 | Sub-set 0 | 0 LargerT1 in previous sub-set |
| 1 |  | 1-3 LargerT1 in previous sub-set |
| 2 |  | >3 LargerT1 in previous sub-set |
| 3 | Other sub-sets | 0 LargerT1 in previous sub-set |
| 4 |  | 1-3 LargerT1 in previous sub-set |
| 5 |  | >3 LargerT1 in previous sub-set |

Table 2- Proposed context set table. There is a dependency on the sub-set; whether it is sub-set 0 (lowest frequencies)

### Results for Part 1

The proposed method is implemented in HM3.0. Simulation results are presented for the high efficiency common test conditions. Table 3 shows the results. The BD-rate is 0.05℅, 0. 07℅ and 0.01℅ for AI, RA and LB configurations, respectively. Detailed results including LP configuration are provided in the accompanying excel file.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.11 | 0.10 | 0.15 | 0.27 | 0.06 | 0.41 |  |  |  |
| Class B | -0.10 | 0.05 | 0.03 | 0.02 | -0.18 | 0.04 | 0.05 | -0.58 | -0.97 |
| Class C | -0.14 | -0.04 | -0.01 | 0.02 | -0.03 | -0.20 | -0.02 | -0.72 | -0.79 |
| Class D | -0.18 | -0.10 | -0.10 | -0.03 | -0.27 | -0.33 | -0.11 | -1.52 | -1.34 |
| Class E | 0.08 | 0.11 | 0.23 |  |  |  | 0.04 | -0.99 | -2.88 |
| All | -0.05 | 0.02 | 0.05 | 0.07 | -0.11 | -0.02 | -0.01 | -0.93 | -1.38 |
| Enc T[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec T[%] | 97% |  |  | 98% |  |  | 99% |  |  |

Table 3. BD-rate for part 1 of the proposal

## Part 2: Unified reverse scan pattern

Part 2 proposes that the scan of the coefficient levels is the same as the scan of the significance map and that both are done backwards (reverse scan of the ones in HM3.0).

The current contexts of the significant map (Figure 1) for blocks larger than 8×8 are modified to deal with the reverse scan and to attain causality for the decoder. Figure 2 shows the proposed contexts. All the coefficients have the same context derivation, except for the DC component, which has its own context. The 5 coefficient neighborhood is maintained, but 5 contexts are used for the first 4 diagonals and other 5 for the rest of the block. Note that this approach uses fewer contexts than the current one.

Figure 2- Contexts for the reverse scan of the significance map

### Results for Part 2

This method is also implemented in HM3.0. Simulation results are presented for the high efficiency common test conditions. Table 4 shows the results for this method. The BD-rate is 0.25℅, 0.19℅ and 0.07℅ for AI, RA and LB configurations, respectively. Detailed results including LP configuration are provided in the accompanying excel file.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.47 | -0.49 | -0.44 | -0.66 | -0.81 | -1.43 |  |  |  |
| Class B | -0.20 | -0.60 | -0.60 | -0.11 | -0.78 | -0.58 | -0.01 | -1.38 | -1.72 |
| Class C | -0.20 | -0.43 | -0.45 | 0.02 | -0.62 | -0.65 | -0.04 | -1.41 | -1.52 |
| Class D | -0.17 | -0.48 | -0.45 | -0.03 | -0.82 | -0.85 | -0.14 | -2.12 | -2.07 |
| Class E | -0.20 | -0.83 | -0.84 |  |  |  | -0.14 | -3.15 | -2.37 |
| All | -0.25 | -0.55 | -0.55 | -0.19 | -0.76 | -0.86 | -0.07 | -1.90 | -1.88 |
| Enc T[%] | 99% |  |  | 97% |  |  | 94% |  |  |
| Dec T[%] | 97% |  |  | 98% |  |  | 99% |  |  |

Table 4. BD-rate for part 2 of the proposal

- Conclusions

This proposal unifies the residue scans in HM3.0. A first part unifies the scan (zig-zag, vertical and horizontal) for the significance map and the coefficient level. The BD-rate of this unification for the high efficiency AI, RA, and LD configurations is 0.05℅, 0.07℅, and 0.01℅, respectively. The second method further unifies the direction of the scan: all the scans are backward. The BD-rate of this case is 0.25℅, 0.19℅, and 0.07℅ for the AI, RA, and LB configurations, respectively.

- References

- M. Winken et al, “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April, 2010.

- J. Sole, R. Joshi, M. Karczewicz, “Unified scans for the significance map and coefficient level coding in high coding efficiency”, JCTVC-E335, 5th JCT-VC Meeting, Geneva, CH, March 2011.

- T. Davies, “Unified Scan Processing for high efficiency coefficient coding”, JCTVC-D219, 4th JCT-VC Meeting, Daegu, KR, January 2011.

- Text for Working Draft

### Text for Part 1

The syntax table for the residual coding of Part 1 is Table 5 (cf. with WD3 section 7.4.10). Semantics remain the same as in WD3. In the context derivation method, the computation of the ctxSet is modified as follows:

###### Derivation process of ctxIdxInc for the syntax element coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag (9.3.3.1.1.5 and in 9.3.3.1.1.6 WD3)

Inputs to this processes are the color component index cIdx, the transform block size log2TrafoSize, the 16 coefficient subset index i and the current coefficient scan index n within the current subset.

Output of this process is ctxIdxInc.

The variable ctxSet specifies the current context set and for its derivation the following applies.

- If n is equal to 0 or all previous syntax elements coeff_abs_level_greater1_flag[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, the following applies.

- If the current subset index i is 0, then  ctxSet  =  0

- Otherwise, ctxSet  =  3

- If the subset i is the first one in 9.3.3.1.1.5 , the variable greater2Ctx that has been derived during the last invocation of subclause 9.3.3.1.1.6 for the syntax element coeff_abs_level_greater2_flag for the subset i + 1 is set to greater2Ctx>>1.

- The variable lastGreater2Ctx is set equal to greater2Ctx and ctxSet is modified as follows

  - If greater2Ctx > 0, ctxSet++

  - If greater2Ctx > 3, ctxSet++

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| n++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| } |  |
| numLastSubset = n >> 4 |  |
| for( i = numLastSubset; i >= 0; i−− ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n−− ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n−− ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n−−) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

Table 5. Residual coding syntax elements (part 1)

### Text for Part 2

The syntax table for the residual coding of Part 2 is Table 6 (cf. with WD3 section 7.4.10). Semantics remain the same as in WD3. In the context derivation method, the computation of the ctxSet is modified as follows.

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag (9.3.3.1.1.4 WD3)

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ) and the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the transform block size and previously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is less than or equal to 3, sigCtx is derived as follows.

shift     =  log2TrafoSize  = =  3 ? 1 : 0
   sigCtx  =  ( ( yC  >>  shift )  <<  2 )  +  ( xC  >>  shift )

- Otherwise if xC is equal to 0 and yC is equal to 0, sigCtx is 0.

- Otherwise, the following applies.

- The variable sigCtx is initialized using previously decoded bins of the syntax element significant_coeff_flag as follows.

sigCtx  =  	significant_coeff_flag[ xC+1 ][ yC ]  +  significant_coeff_flag[ xC ][ yC+1 ]  +  
		significant_coeff_flag[ xC+1 ][ yC+1 ]

- When xC is greater than 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC+2 ][ yC ]

- When yC is greater than 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC+2 ]

- The variable sigCtx is derived as follows.

  - If xC + yC < 4

    - sigCtx  =  1  +  min( 4, sigCtx )

  - Otherwise

    - sigCtx  =  6  +  min( 4, sigCtx )

The context index increment ctxIdxInc is derived using the color component index cIdx, the transform block size log2TrafoSize and sigCtx as follows. Blocks of sizes 16×16 and 32×32 share contexts with sigCtx>=6 (instead of sigCtx>=4 as in WD3).

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  ( ( 5  −  log2TrafoSize )  *  16 )  +  sigCtx

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  64  +  ( ( 4  −  log2TrafoSize )  *  16 )  +  sigCtx

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = last_significant_coeff_scan_position |  |
| numLastSubset = n >> 4 |  |
| for( i = n − 1; i >= 0; i−− ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| for( i = numLastSubset; i >= 0; i−− ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n−− ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n−− ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n−−) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

Table 6. Residual coding syntax elements (part 2)

# Additional Results

In this section, the results of combining the fixed diagonal scans of JCTVC-F129 with Part 2 of this contribution and also considering parallalization of the contexts. The design for the contexts are shown in the next figure and results in the table below. These results have not been cross-checked.

Figure 3- Contexts for the reverse scan of the significance map

| BD-rate | Intra HE |  |  | Random access HE |  |  | Low delay HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.51 | -0.55 | -0.57 | -0.80 | -1.53 | -1.85 |  |  |  |
| Class B | -0.28 | -0.72 | -0.66 | -0.11 | -0.91 | -0.59 | 0.06 | -1.37 | -1.63 |
| Class C | -0.26 | -0.51 | -0.47 | 0.02 | -0.63 | -0.53 | -0.03 | -1.72 | -1.43 |
| Class D | -0.21 | -0.48 | -0.47 | 0.02 | -0.74 | -0.77 | -0.09 | -1.99 | -1.39 |
| Class E | -0.28 | -1.02 | -0.99 |  |  |  | -0.09 | -3.23 | -2.87 |
| All | -0.31 | -0.64 | -0.62 | -0.21 | -0.95 | -0.92 | -0.03 | -1.96 | -1.75 |
| Enc T[%] | 95% |  |  | 94% |  |  | 95% |  |  |

Table 7. BD-rate for the combination

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).