| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F293 |
| --- | --- |

| Title: | CE11: Cross-check of TI’s fixed diagonal scan (JCTVC-F129) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The contribution reportedly confirms the results presented in the CE11 contribution JCTVC-F129 on fixed diagonal scans for significance map coding with CABAC.

- Introduction

Software implementing JCTVC-F129 on top of HM3.0 basis was released by TI as part of CE11.

Modifications are limited to two files:

- TypeDef.h: adding a control macro

- TComRom.cpp: replacement of the zig-zag scan definition by a diagonal scan were all diagonal scans are in the same direction (starting from the left-bottom sides)

Overall the code change is very small. However, the diagonal scans are proposed only for the HE configurations but the code change affects all configurations. This is should be addressed for a complete integration of the diagonal scan and so, more code modifications should be expected. Alternatively, the diagonal scan could be considered for the LC case as well. In that case, there is no need of further code changes.

- Results

Coding results using common conditions (High-Efficiency settings) are summarized in the tables below. HM3.0 is the anchor.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.2 | -0.2 |
| Overall | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.2 | -0.5 | -0.1 |
| Class B | -0.1 | -0.1 | -0.1 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.2 | -0.1 |
| Class E |  |  |  |
| Overall | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1 | 0.1 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.5 | 0.8 |
| Class E | 0.1 | 0.7 | -0.8 |
| Overall | 0.0 | 0.3 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |