| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F298 |
| --- | --- |

| Title: | Run-level Table Reduction for CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current HM software, a run-level table of size 434 is used in the coding of Inter block coefficients (as well as Intra Chroma block coefficients). This contribution reduces the run-level related table sizes from 434 to 148. The Luma B-D rate changes for different cases are 0.00% AI, -0.14% RA, 0.03% LD, and 0.06% LDP. There is no loss in Chroma.

# Introduction

According to the current HM [1], a table g_auiLumaRun8x8[maxrun][run] is used in the run-level coding of inter block coefficients (both luma and chroma) and intra block chroma coefficients coding. This table is applied for maxrun = 0-27, and the range of run is 0 to maxrun+1, so the table size is 434. Codeword number cn is calculated as follows:

When maxrun <=27

If lev = 0,

cn = g_auiLumaRun8x8[maxrun][run]

if lev = 1,

cn = maxrun +1+g_auiLumaRun8x8[maxrun][run]

When maxrun > 27,

If lev = 0, cn = run;

If lev = 1, cn =marxun+ 2+ run;

# Proposal

## Run-level coding for Luma

For luma block, codenumber cn calculation for maxrun = 0-9 and maxrun > 27 are not changed. For maxrun = 10-27, we store cn for (run=maxrun+1, lev = 0), and let cn increase with run for other (run, lev). The following are the pseudo codes for cn calculation of maxrun = 10-27:

If lev = 0,

If run = maxrun+1,

cn = g_auiMaxRunPlusOneLuma[maxrun-10];

else

cn = (run < g_auiMaxRunPlusOneLuma[maxrun-10])?run: run+1;

If lev = 1,

cn =  maxrun + 2 + run;

The size of g_auiLumaRun8x8[maxrun][run] is 65, and the size of g_auiMaxRunPlusOneLuma[maxrun-10] (maxrun = 10-27) is 18. So the total storage for Luma is 83.

## Run-level coding for Chroma

For chroma block (both intra and inter), the calculate of of cn for maxrun = 0-9 is not changed but the table (g_auiChromaRun8x8) is different from luma. For maxrun >= 10, cn is 0 is for (run=maxrun+1, lev = 0), and cn increases with run for other (run lev). The following are pseudo codes for maxrun >= 10:

If lev = 0,

cn = (run==maxrun+1)? 0: run+1;

else

cn =  maxrun + 2 + run;

The size of g_auiChromaRun8x8[maxrun][run] is 65. This is the only storage required for Chroma.

The total storage for Lum and Chroma is 83+65=148.

# Experimental results

We implement the presented run-level table reduction on top of HM 3.0. The simulations are conducted under low-complexity common conditions specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is measured on clusters with same CPU, and the decoding time is measured by decoding all sequences on a single PC.

Table I summarizes the results of the proposed run-level table reduction. The Y bitrate reduction for AI, RA, LD and LDP are 0.00%, -0.14%, 0.03% and 0.06%. There is no loss in chroma coding. The encoding and decoding time are basically the same as HM3.0.

# Conclusions

In this contribution, we present a method to reduce the run-level table storage from 434 to 148. We observed on average 0.14% Luma bit rate reduction in random access case, and 0.03% and 0.06% in cares for low delay and low delay P cases, and there is no loss for Chroma. There is no change in encoding time and decoding time. Based on the results, we recommend the proposed run-level table reduction be adopted into HM.

|  | All Intra LC |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00 | 0.05 | 0.09 | -0.13 | -0.13 | 0.04 |
| Class B | 0.00 | 0.01 | 0.03 | -0.18 | 0.05 | -0.10 |
| Class C | 0.00 | 0.02 | -0.02 | -0.13 | 0.04 | -0.07 |
| Class D | 0.00 | 0.01 | 0.02 | -0.10 | -0.11 | -0.06 |
| Class E | 0.00 | 0.03 | 0.02 |  |  |  |
| Overall | 0.00 | 0.02 | 0.03 | -0.14 | -0.03 | -0.05 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay B LC |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.06 | 0.12 | -0.28 | -0.01 | 0.27 | 0.14 |
| Class C | 0.00 | 0.19 | -0.27 | 0.02 | -0.13 | -0.18 |
| Class D | 0.01 | -0.07 | -0.28 | 0.01 | -0.06 | -0.69 |
| Class E | 0.25 | 0.03 | -0.25 | 0.33 | -0.28 | -0.54 |
| Overall | 0.03 | 0.07 | -0.27 | 0.06 | -0.02 | -0.28 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 1 Results of run-level table reduction

# Draft texts

#### Parsing process for run_level_one

Inputs to this process are bits from slice data, the scan position n of the previous non-zero transform coefficient in inverse scan order.

Output of this process is the syntax element run_level_one.

maxRun = n

#### When blockType > 2 and blockType is not equal to 5 (Luma)

  - The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag  = codeNum <= maxRun + 1 ? 0 : 1

  - If maxRun is less than 10, the variable runOfZeros is derived from maxRun and codeNum using Table I  g_acstructLumaRun8x8.

If codeNum <=maxrun+1, runOfZeros = g_acstructLumaRun8x8 [maxRun][run]

else if codeNum> maxRun+1 + codeNum_Lev0_MaxrunPlus1,

runOfZeros = g_acstructLumaRun8x8[maxrun][ codeNum -maxrun-1]

else

runOfZeros = g_acstructLumaRun8x8[maxrun][ codeNum -maxrun-2]

where codeNum_Lev0_MaxrunPlus1 is the codeNum for (maxRun+1,0)

  - If maxRun is between 10-27, the variable runOfZeros is derived from maxRun and codeNum using Table II auiMaxRunPlusOneLuma as

If codeNum ==  g_auiMaxRunPlusOneLuma[maxrun-10], runOfZeros = maxrun+1;

else if codeNum > maxrun+1,

runOfZeros = codeNum – maxrun-2

else

runOfZeros = (codeNum < g_auiMaxRunPlusOneY [maxrun-10])? codeNum: codeNum -1

  - Otherwise (maxRun is not less than 28), the variable runOfZeros is derived as

runOfZeros = codeNum <= maxRun + 1 ? codeNum : codeNum – maxRun – 2

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + n : runOfZeros

#### When blockType <2 (Chroma)

  - The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag  = codeNum <= maxRun + 1 ? 0 : 1

  - If maxRun is less than 10, the variable runOfZeros is derived from maxRun and codeNum as shown in Table III g_acstructLumaRun8x8Chroma.

  - Otherwise (maxRun is not less than 10), the variable runOfZeros is derived as

If codeNum>= maxRun + 1,

runOfZeros = codeNum – maxrun-2

else runOfZeros = codeNum == 0? maxRun +1: codeNum-1;

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag? runOfZeros + n : runOfZeros

Table I   g_acstructLumaRun8x8

| maxRun | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 | 0 | 2 | 1 | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 | 0 | 3 | 1 | 2 | NA | NA | NA | NA | NA | NA | NA |
| 3 | 1 | 0 | 3 | 4 | 2 | NA | NA | NA | NA | NA | NA |
| 4 | 0 | 5 | 3 | 2 | 4 | 1 | NA | NA | NA | NA | NA |
| 5 | 0 | 6 | 1 | 4 | 3 | 2 | 5 | NA | NA | NA | NA |
| 6 | 1 | 0 | 2 | 7 | 4 | 5 | 3 | 6 | NA | NA | NA |
| 7 | 0 | 3 | 2 | 1 | 8 | 4 | 5 | 6 | 7 | NA | NA |
| 8 | 0 | 9 | 1 | 2 | 5 | 4 | 3 | 6 | 8 | 7 | NA |
| 9 | 0 | 1 | 10 | 2 | 3 | 5 | 4 | 6 | 7 | 9 | 8 |

Table II g_auiMaxRunPlusOneY

| Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 6 | 7 | 5 | 5 | 4 | 6 | 6 | 8 |
| Index | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
|  | 3 | 1 | 4 | 7 | 8 | 12 | 5 | 2 | 1 |

Table III g_acstructLumaRun8x8Chroma

|  | codeNum%(maxRun+1) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| maxRun | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 0 | 1 | 0 | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 1 | 2 | 1 | 0 | NA | NA | NA | NA | NA | NA | NA | NA |
| 2 | 3 | 0 | 2 | 1 | NA | NA | NA | NA | NA | NA | NA |
| 3 | 4 | 1 | 0 | 2 | 3 | NA | NA | NA | NA | NA | NA |
| 4 | 5 | 0 | 3 | 2 | 4 | 1 | NA | NA | NA | NA | NA |
| 5 | 6 | 0 | 1 | 4 | 5 | 3 | 2 | NA | NA | NA | NA |
| 6 | 7 | 1 | 0 | 2 | 4 | 3 | 6 | 5 | NA | NA | NA |
| 7 | 8 | 0 | 3 | 4 | 2 | 1 | 5 | 7 | 6 | NA | NA |
| 8 | 9 | 0 | 5 | 1 | 6 | 4 | 8 | 3 | 2 | 7 | NA |
| 9 | 10 | 0 | 1 | 6 | 7 | 2 | 9 | 5 | 4 | 3 | 8 |

# References

[1] M. Karczewitz, X. Wang, W.-J. Chien, “CE5: coefficient coding with LCEC for large blocks”, JCTVC-E383, JCT-VC Meeting, Geneva, CH, Mar. 2011.

[2] F.Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC Meeting, Geneva, CH, Mar. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).