| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F300 |
| --- | --- |

| Title: | CE4 Subtest 2: Delta QP prediction results of test 2.2.b and 2.3.f |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Kobayashi, Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Email: | +81-3-3758-2111 kobayashi.masaaki517@canon.co.jp shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of QP prediction techniques based on CU depth and prediction modes, which are test 2.2.b and test 2.3.f of CE4 Subtest2.  These technologies were originally presented in the contribution of JCTVC-E198 at the 5th JCT-VC meeting, and they introduced depth-dependent QP prediction (2.2.b) and the prediction mode dependent QP prediction (2.3.f) respectively.  The experimental results show that the depth-dependent QP prediction technique (2.2.b) causes around 0.1% BD-rate increase for AI_HE, AI_LC, RA_HE and RA_LC configurations and around 0.2% BD-rate increase for LB_HE and LB_LC configurations on average while the prediction mode dependent QP prediction (2.3.f) achieves about 0.2% BD-rate reduction for AI_HE, RA_HE, RA_LC and LB_HE configurations, 0.1% for AI_LC configuration, and 0.3% for LB_LC configuration under the test condition defined in CE4 Subtest2.

# Introduction

The DeltaQP signaling at sub-LCU level was adopted at the 5th JCT-VC meeting, while there were several contributions presented to introduce alternative QP prediction methods.  The depth-dependent QP prediction (2.2.b) is one of them, and it introduces QP prediction from the previously coded CU with the same CU size.  The prediction mode dependent QP prediction (2.3.f) is another and it introduces QP prediction from the neighbouring CUs with the same prediction mode.

# Algorithm description of tested QP prediction methods

Two tested QP prediction methods are described here.

## Test 2.2.b: Depth-dependent QP prediction

Delta QP is calculated as the difference between the QP of current CU and the QP of previously coded QP with the same size as the current CU.  It means calculating delta QP at each CU size respectively.  Fig shows an example of possible CU partition structure in current HM design.

The detailed description of calculating delta QP is shown as follows:

(1) If the size of current CU is 64x64,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 64x64 CU.

(2) If the size of current CU is 32x32,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 32x32 CU.
Example using Fig. 3-1)
-Delta QP of A8 is the difference between QP of A8 and QP of A7.

(3) If size of current CU is 16x16,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 16x16 CU.
Example using Fig. 3-1)
	-Delta QP of B2 is the difference between QP of B2 and QP of B1.
	-Delta QP of B3 is the difference between QP of B3 and QP of B2.

(4) If size of current CU is 8x8,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 8x8 CU.
Example using Fig. 3-1)
	-Delta QP of C3 is the difference between QP of C3 and QP of C2.
	-Delta QP of C5 is the difference between QP of C5 and QP of C4.

(5) If size of current CU is 4x4,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 4x4 CU.

Figure 1: Possible CU partition and its delta QP calculation

## Test 2.3.f: Prediction mode dependent QP prediction

Delta QP is calculated as the difference between the QP of current CU and the QP calculated from QPs of left and top CUs with the same "PredMode" as the current CU. "PredMode" is defined in JCTVC-C403, and it indicates whether CU is inter, intra, or other mode.  Figure  shows an example of "PredMode" and position of neighboring CU to explain this method.

The detailed description of calculating delta QP is shown as follows:

(1) If "PredMode" of current CU is the same as "PredMode" of left CU but not the same as “PredMode” of top CU,
	delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

(2) If "PredMode" of current CU is the same as "PredMode" of top CU but not the same as “PredMode” of left CU,
	delta QP is calculated as the difference between the QP of current CU and the QP of top CU.

(3) If "PredMode" of left CU and top CU are same and "PredMode" of current CU is same as it,
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of left CU and top CU.

(4) If "PredMode" of current CU is different from "PredMode" of left CU or top CU.
	delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

Figure 2: Delta QP calculation by PredMode

# Experiment

Experimental results on Test 2.2.b and Test 2.3.f defined in the CE4 description document (JCTVC-E704) are shown here.

## Experimental results on Test 2.2.b

Table 1 shows the summary of experimental results of Test 2.2.b vs. 2.2.a. with MinCUDQPSize=8x8.  More detailed results are shown in the attached spreadsheets.

Table 1: Summary results of test 2.2.b vs. 2.2.a with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.1 | 0.1 | 0.1 | 3.1% | 0.1 | 0.1 | 0.1 | 2.8% |
| Class B | 0.1 | 0.0 | 0.1 | 2.0% | 0.1 | 0.1 | 0.1 | 1.6% |
| Class C | 0.0 | 0.0 | 0.0 | 1.0% | 0.0 | 0.0 | 0.0 | 0.9% |
| Class D | 0.0 | 0.0 | 0.1 | 0.9% | 0.0 | 0.0 | 0.0 | 0.8% |
| Class E | 0.1 | 0.1 | 0.1 | 2.6% | 0.1 | 0.1 | 0.1 | 2.0% |
| All | 0.1 | 0.1 | 0.1 | 1.9% | 0.1 | 0.1 | 0.1 | 1.6% |
| Enc Time[%] | 102% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.2 | 0.1 | -0.2 | 3.5% | 0.1 | 0.0 | 0.0 | 3.3% |
| Class B | 0.1 | -0.1 | 0.0 | 3.0% | 0.1 | 0.2 | 0.1 | 2.8% |
| Class C | 0.1 | 0.1 | 0.1 | 2.0% | 0.1 | 0.1 | 0.0 | 2.0% |
| Class D | 0.0 | 0.1 | 0.4 | 1.6% | 0.1 | 0.1 | 0.1 | 1.5% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.1 | 2.6% | 0.1 | 0.1 | 0.1 | 2.4% |
| Enc Time[%] | 98% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | 3.3% | 0.2 | 0.2 | 0.2 | 3.3% |
| Class C | 0.1 | 0.1 | 0.0 | 2.3% | 0.1 | -0.1 | 0.1 | 2.4% |
| Class D | 0.0 | -0.3 | -0.4 | 1.8% | 0.1 | 0.0 | 0.3 | 1.9% |
| Class E | 0.4 | 0.5 | -0.4 | 4.7% | 0.2 | 0.5 | 0.1 | 4.3% |
| All | 0.1 | 0.1 | -0.1 | 2.9% | 0.1 | 0.1 | 0.2 | 2.9% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |

## Experimental results on Test 2.3.f

Table 2 shows the experimental results of Test 2.3.f vs. 2.3.a. with MinCUDQPSize=8x8.  More detailed results are shown in the attached spreadsheets.

Table 2: Summary results of test 2.3.f vs. CE4 anchor with MinCUDQPSize = 8x8.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | -0.1 | 0.0 | -1.7% | 0.1 | 0.1 | 0.1 | 2.2% |
| Class B | -0.2 | -0.1 | -0.1 | -4.6% | 0.0 | 0.0 | 0.0 | -1.3% |
| Class C | -0.3 | -0.3 | -0.3 | -8.4% | -0.2 | -0.2 | -0.2 | -5.5% |
| Class D | -0.3 | -0.3 | -0.3 | -8.7% | -0.3 | -0.2 | -0.3 | -6.8% |
| Class E | -0.2 | -0.2 | -0.3 | -5.4% | -0.1 | -0.1 | -0.1 | -2.0% |
| All | -0.2 | -0.2 | -0.2 | -5.7% | -0.1 | -0.1 | -0.1 | -2.6% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | 0.0 | -0.1 | -1.8% | 0.0 | 0.0 | 0.1 | -0.5% |
| Class B | -0.1 | -0.2 | -0.2 | -4.0% | -0.1 | -0.1 | -0.2 | -3.0% |
| Class C | -0.2 | -0.3 | -0.2 | -7.0% | -0.3 | -0.3 | -0.2 | -6.8% |
| Class D | -0.2 | -0.3 | -0.1 | -6.3% | -0.3 | -0.3 | -0.3 | -6.3% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -4.7% | -0.2 | -0.2 | -0.1 | -4.1% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.2 | -3.5% | -0.1 | 0.0 | -0.2 | -3.2% |
| Class C | -0.3 | -0.3 | -0.2 | -6.3% | -0.4 | -0.2 | -0.4 | -6.6% |
| Class D | -0.2 | -0.3 | 0.3 | -5.7% | -0.3 | -0.5 | -0.7 | -6.0% |
| Class E | -0.4 | 0.2 | 0.1 | -7.7% | -0.4 | 0.0 | -0.3 | -7.3% |
| All | -0.2 | -0.2 | 0.0 | -5.5% | -0.3 | -0.2 | -0.4 | -5.5% |
| Enc Time[%] | 101% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  |

# Conclusion

This contribution reports the experimental results of depth dependent QP prediction (Test 2.2.b) and prediction mode dependent QP prediction (Test 2.3.f).  The experimental results on Test 2.2.b show that the depth dependent QP prediction causes BD-rate increase of about 0.1% with respect to the QP prediction by coding order under the CE4 Subtest2 test conditions.  In contrast, the experimental results on Test 2.3.f show that the prediction mode dependent QP prediction achieves BD-rate reduction of 0.1 to 0.3% under the CE4 Subtest2 test conditions.  Therefore, it is recommended that the prediction mode dependent QP prediction or its combination with other QP prediction method be incorporated into the HM.

# Patent rights declaration(s)

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

The modified or added portions are highlighted with yellow colour.

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode and the availability of the neighbouring quantisation group of coding units, the predicted quantisation parameter QPY, PRED is derived as follows.

–	If all of the following conditions are true, then QPY, PRED is derived as follows

- The above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode.

- The left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode.

QPY,PRED = ( ( ( QPY,ABOVE + QPY,LEFT + 1 ) / 2               	(8-3)

where QPY,ABOVE is luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice, and QPY,LEFT is luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the previous quantisation group of coding units in decoding order in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-42)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-53)