| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F301 WG11 Number: m20722 |
| --- | --- |

| Title: | CE8 Subtest 1: Block-based filter adaptation with features on subset of pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes Il-Koo Kim | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents simplification of block-based filter adaptation (BA) scheme in the current HM3 ALF. For each 4x4 block, features are only computed every 4 pixels instead of at every pixel. Also, the summation over 3x3 window for Laplacian is avoided. For BA, the complexity is reduced from the average of over 11 operations per luma pixel to only 2.2 per luma pixel. The proposed method reports 3% ~ 4% decoding time reductions for BA, with 0.2% BD-rate loss for AI, RA, and LD structures. Further comparison against “no classification” is also provided to better evaluate complexity-efficiency trade-off.

# Introduction

In the 5th JCT-VC meeting held at Geneva, Switzerland, block-based filter adaptation was adopted [1]. In the adopted method, each 4x4 block is classified based on directional and Laplacian features computed at every pixel. To reduce complexity, computing features only at subset of pixels for block-based filter adaption was proposed in another contribution, while using other type of features [2]. In this contribution, block-based features as in [1] are computed only at pixels subsampled by 2 in both vertical and horizontal directions. The summation over 3x3 window for Laplacian used in [1] is also avoided. All the other aspects of ALF remain the same as in HM3.0 (or the corresponding HM3.1-dev-adcs branch).

# Computing features at subset of pixels

In JCTVC-D323 [1], to classify a 4x4 block with pixels { X(i,j) | i=0,1,2,3 ; j=0,1,2,3 }, directional and Laplacian features are computed at every pixel X(i,j) as follows:

H(i,j) = abs ( X(i,j)<<1 - X(i,j-1) - X(i,j+1) )

V(i,j) = abs ( X(i,j)<<1 - X(i-1,j) - X(i+1,j) )

L(i,j) = ∑n=-1,0,1 ∑m=-1,0,1 H(i+m,j+n) + V(i+m,j+n)

Then the block B is classified based on the three values:

HB = ∑i=0,1,2,3 ∑j=0,1,2,3 H(i,j)

VB = ∑i=0,1,2,3 ∑j=0,1,2,3 V(i,j)

LB = ∑i=0,1,2,3 ∑j=0,1,2,3 L(i,j)

All blocks classified into the same class will adapt to one filter to be designed. This classification process has to be performed at both encoder and decoder. To reduce computation complexity in this block-based filter adaptation part, this document proposes two simplifications:

- L’(i,j) = H(i+m,j+n) + V(i+m,j+n), i.e. without summation over 3x3 window.

- Only compute H(i,j), V(i,j), and L(i,j) at i=0,2 ; j=0,2, i.e. subsampled by 2 in both directions

H’B = ∑i=0,2 ∑j=0,2 H(i,j)

V’B = ∑i=0,2 ∑j=0,2 V(i,j)

L’B = H’B + V’B

The simplification of computation can be illustrated at Fig. 1 below.

There is no syntax change for the proposed method.

Figure. 1 Simplification of feature computation for block-based filter adaptation.

The computation of the proposed can be analyzed as the following:

- Compute four H(i,j) and four V(i,j): 4 * [1 shift + 2*(2 adds+1 abs) ]  28 operations

- Compute H’B and V’B : 3 adds each 6 operations

- Compute : 1 add

Total: 28 + 6 + 1 = 35 (add/shift/abs) per 4x4 block, i.e. 2.2 (add/shift/abs) per luma pixel

This is significantly reduced from over 11 operations per luma pixel as in HM 3.0 [3].

# Experimental results

## Comparison against HM3.0 and HM3.1-dev-adcs anchors

Following the conditions in CE8 [4], experiments are conducted using both HM3.0 (with filter shapes diamond-9Hx7V, diamond-7x7, and diamond-5x5) and HM3.1-dev-adcs branch which uses new filter shapes (diamond-7Hx5Vand cross-19Hx5V, both with 12 coefficients). It can seen that for both tests the proposed method provide decoding time reduction with 0.2% BD-rate loss.

| Compared to “HM3.0 RA/BA” |  |  |  |  |  |  | Compared to “HM3.1-dev-adcs RA/BA” |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  |  | Intra |  |  |  |
|  | Y BD | U BD |  | V BD |  |  |  | Y BD | U BD | V BD |  |
| Class A | 0.1 | 0.0 |  | 0.0 |  |  | Class A | 0.2 | 0.0 | 0.0 |  |
| Class B | 0.1 | 0.0 |  | 0.0 |  |  | Class B | 0.1 | 0.0 | 0.0 |  |
| Class C | 0.2 | 0.0 |  | 0.0 |  |  | Class C | 0.2 | 0.0 | 0.0 |  |
| Class D | 0.1 | -0.1 |  | -0.1 |  |  | Class D | 0.1 | -0.1 | -0.1 |  |
| Class E | 0.2 | -0.1 |  | -0.1 |  |  | Class E | 0.2 | -0.1 | -0.1 |  |
| All | 0.1 | 0.0 |  | 0.0 |  |  | All | 0.2 | 0.0 | 0.0 |  |
| Enc [%] | 99% (99%) |  |  |  |  |  | Enc [%] | 99% (99%) |  |  |  |
| Dec [%] | 96% (96%) |  |  |  |  |  | Dec [%] | 97% (96%) |  |  |  |
|  | Random access |  |  |  |  |  |  | Random access |  |  |  |
|  | Y BD | U BD | V BD |  |  |  |  | Y BD | U BD | V BD |  |
| Class A | 0.2 | 0.1 | 0.2 |  |  |  | Class A | 0.1 | 0.0 | 0.1 |  |
| Class B | 0.3 | 0.0 | 0.1 |  |  |  | Class B | 0.3 | 0.0 | 0.0 |  |
| Class C | 0.2 | 0.1 | 0.0 |  |  |  | Class C | 0.2 | -0.1 | 0.0 |  |
| Class D | 0.1 | 0.0 | -0.2 |  |  |  | Class D | 0.1 | 0.0 | 0.0 |  |
| Class E |  |  |  |  |  |  | Class E |  |  |  |  |
| All | 0.2 | 0.0 | 0.0 |  |  |  | All | 0.2 | 0.0 | 0.0 |  |
| Enc [%] | 100% (100%) |  |  |  |  |  | Enc [%] | 100% (100%) |  |  |  |
| Dec [%] | 98% (99%) |  |  |  |  |  | Dec [%] | 99% (99%) |  |  |  |
|  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |
|  | Y BD | U BD | V BD |  |  |  |  | Y BD | U BD |  | V BD |
| Class A |  |  |  |  |  |  | Class A |  |  |  |  |
| Class B | 0.2 | 0.2 | 0.0 |  |  |  | Class B | 0.2 | -0.1 |  | -0.2 |
| Class C | 0.2 | 0.0 | -0.1 |  |  |  | Class C | 0.3 | 0.1 |  | 0.0 |
| Class D | 0.1 | 0.0 | 0.2 |  |  |  | Class D | 0.1 | -0.2 |  | -0.4 |
| Class E | 0.3 | 0.1 | 0.0 |  |  |  | Class E | 0.2 | -0.3 |  | -0.5 |
| All | 0.2 | 0.1 | 0.0 |  |  |  | All | 0.2 | -0.1 |  | -0.2 |
| Enc [%] | 100% (100%) |  |  |  |  |  | Enc [%] | 100% (99%) |  |  |  |
| Dec [%] | 98% (98%) |  |  |  |  |  | Dec [%] | 98% (99%) |  |  |  |

## Only Block-based filter adaptation (worst-case analysis)

The current ALF has two filter adaptation modes: Region-based adaptation (RA) [5] and block-based adaptation (BA) [1]. RA does not require computation of features, thus its decoding time will be faster than BA. Since our simplification only applies to BA, to isolate the actual effect, we also conduct supplementary tests by forcing the encoder to only use BA. This is considered as the “worst-case analysis” to determine the efficiency-complexity trade-off of the proposed method in BA. It can be seen below that the decoding time reductions are 3% ~ 4%, with 0.2% BD-rate loss in coding efficiency.

As supplementary data, we also provide results in HM3.0 with no classification at all (one filter per frame, completely remove classification computations). This demonstrates that the proposed method preserves BA gains well, while brings down the classification complexity significantly.

| HM 3.0 BA-ONLY Proposed simplified BA |  |  |  |  |  |  | HM 3.0 BA-ONLY No Classification (sup. Info) |  |  |  |  |  |  | HM 3.1 dev BA-ONLY Proposed simplified BA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  |  |  | Intra |  |  |  |  |  |  | Intra |  |  |  |
|  | Y |  | U |  | V |  |  | Y | U |  | V |  |  |  | Y | U |  | V |
| Class A | 0.2 |  | 0.0 |  | 0.0 |  | Class A | 0.7 | 0.0 |  | 0.0 |  |  | Class A | 0.2 | 0.0 |  | 0.0 |
| Class B | 0.2 |  | 0.0 |  | 0.0 |  | Class B | 0.6 | -0.2 |  | -0.1 |  |  | Class B | 0.2 | 0.0 |  | 0.0 |
| Class C | 0.3 |  | -0.1 |  | -0.1 |  | Class C | 0.6 | -0.3 |  | -0.3 |  |  | Class C | 0.2 | 0.0 |  | 0.0 |
| Class D | 0.1 |  | -0.1 |  | -0.1 |  | Class D | 0.2 | -0.2 |  | -0.2 |  |  | Class D | 0.1 | -0.1 |  | -0.1 |
| Class E | 0.2 |  | -0.1 |  | -0.1 |  | Class E | 0.6 | -0.4 |  | -0.4 |  |  | Class E | 0.2 | -0.1 |  | -0.1 |
| All | 0.2 |  | -0.1 |  | -0.1 |  | All | 0.5 | -0.2 |  | -0.2 |  |  | All | 0.2 | 0.0 |  | 0.0 |
| Enc[%] | 99% |  |  |  |  |  | Enc[%] | 99% |  |  |  |  |  | Enc[%] | 100% |  |  |  |
| Dec[%] | 97% |  |  |  |  |  | Dec[%] | 95% |  |  |  |  |  | Dec[%] | 97% |  |  |  |
|  | Random access |  |  |  |  |  |  | Random access |  |  |  |  |  |  | Random access |  |  |  |
|  | Y | U |  | V |  |  |  | Y | U |  | V |  |  |  | Y | U |  | V |
| Class A | 0.3 | 0.2 |  | 0.1 |  |  | Class A | 1.0 | 0.2 |  | 0.6 |  |  | Class A | 0.2 | 0.0 |  | 0.1 |
| Class B | 0.4 | 0.0 |  | 0.1 |  |  | Class B | 1.2 | -0.1 |  | 0.0 |  |  | Class B | 0.4 | 0.0 |  | -0.1 |
| Class C | 0.2 | -0.1 |  | -0.2 |  |  | Class C | 0.5 | -0.3 |  | -0.3 |  |  | Class C | 0.2 | -0.1 |  | -0.1 |
| Class D | 0.1 | 0.1 |  | 0.2 |  |  | Class D | 0.2 | -0.2 |  | -0.1 |  |  | Class D | 0.1 | -0.1 |  | -0.1 |
| Class E |  |  |  |  |  |  | Class E |  |  |  |  |  |  | Class E |  |  |  |  |
| All | 0.2 | 0.1 |  | 0.1 |  |  | All | 0.7 | -0.1 |  | 0.1 |  |  | All | 0.2 | 0.0 |  | 0.0 |
| Enc[%] | 100% |  |  |  |  |  | Enc[%] | 100% |  |  |  |  |  | Enc[%] | 100% |  |  |  |
| Dec[%] | 96% |  |  |  |  |  | Dec[%] | 95% |  |  |  |  |  | Dec[%] | 96% |  |  |  |
|  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |  |  |  | Low delay |  |  |  |
|  | Y | U |  | V |  |  |  | Y | U |  | V |  |  |  | Y | U |  | V |
| Class A |  |  |  |  |  |  | Class A |  |  |  |  |  |  | Class A |  |  |  |  |
| Class B | 0.3 | -0.1 |  | -0.1 |  |  | Class B | 0.8 | -0.2 |  | -0.5 |  |  | Class B | 0.2 | 0.0 |  | -0.4 |
| Class C | 0.2 | 0.0 |  | -0.2 |  |  | Class C | 0.5 | -0.4 |  | -0.5 |  |  | Class C | 0.2 | -0.1 |  | -0.1 |
| Class D | 0.0 | 0.0 |  | -0.2 |  |  | Class D | 0.2 | -0.2 |  | -0.8 |  |  | Class D | 0.1 | 0.3 |  | 0.0 |
| Class E | 0.2 | 0.1 |  | -0.3 |  |  | Class E | 0.7 | -0.5 |  | -0.5 |  |  | Class E | 0.3 | 0.3 |  | -0.1 |
| All | 0.2 | 0.0 |  | -0.2 |  |  | All | 0.5 | -0.3 |  | -0.6 |  |  | All | 0.2 | 0.1 |  | -0.1 |
| Enc[%] | 100% |  |  |  |  |  | Enc[%] | 100% |  |  |  |  |  | Enc[%] | 100% |  |  |  |
| Dec[%] | 96% |  |  |  |  |  | Dec[%] | 94% |  |  |  |  |  | Dec[%] | 97% |  |  |  |

# Changes in Working Draft Text

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = -1..(nS)+1 is derived as

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-1)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-2)
	varTemp1[ x ][ y ] = varTempH[ x ][ y ] + varTempV[ x ][ y ]	(8-3)

- The variable varTemp2[ x, y ] with x, y = 0..(nS)-1 is derived as

varTemp2[ x ][ y ] = ij varTemp1[ x + i ][ y + j ] with i, j = -1..1	(8-4)

- The variables varTemp3[ x, y ], varTempH1[ x, y ] and varTempV1[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTemp3[ x ][ y ] = (ij varTemp2[ (x << 2 ) + i ][ (y << 2) + j ]) >> 4 with i, j = 0..3	(8-5)
varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0..3 0,2	(8-6)
varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 0..3 0,2	(8-7)
varTemp3[ x ][ y ] = (varTempH1[ x ][ y ] + varTempV1[ x ][ y ])>>2			(8-8)

... ...

... ...

- The variable avgvar is derived as

varTab[16] = { 0, 1, 2, 2, 2, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4 }					(8-9)
avgVar = Clip3( 0, 15, (varTemp3[ x >> 2 ][ y >> 2 ] * 114 1024) >> (3 + BitDepthY) )		(8-10)

# Conclusions

Simplification of block-based filter adaptation (BA) scheme in the current HM3 ALF, by computing features only at every 4 pixels, is presented in this contribution. It reports 3% ~ 4% decoding time reductions for BA, with 0.1~0.2% BD-rate loss for AI, RA, and LD structures. Comparison against “no classification” demonstrates that the proposed method preserve BA gain while effectively reduce classification complexity. With the tangible computation reduction, no syntax change required, and minor efficiency loss, it is recommended to adopt the proposed method.

# References

- “CE8 Subtest 2: Block based adaptive loop filter”, JCTVC-E323, Geneva, Switzerland, Mar. 2011

- “Loop filtering with directional features”, JCTVC-E288, Geneva, Switzerland, Mar. 2011

- “ALF Complexity Analysis”, JVTVC-F342, Torino, Italy, Jul. 2011

- “Description of Core Experiment 8 (CE8): Non-deblocking loop filtering”, JCTVC-E708, Geneva, Switzerland, Mar. 2011

- “CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection”, JCTVC-E046, Geneva, Switzerland, Mar. 2011

Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).