| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F302 |
| --- | --- |

| Title: | Merge Candidate Selection in 2NxN, Nx2N, and NxN Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yunfei Zheng Xianglin Wang Wei-Jung Chien Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | 1-858-651-9568 zhengy@qualcomm.com xianglin@qualcomm.com wchien@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a change to the rules currently used in determining merge candidates for a PU under 2NxN, Nx2N or NxN mode. Based on the proposal, when a current PU under these partition modes is not the first PU in a CU, the previous PU’s merge index is checked to determine if motion information of the previous PU can be used as a merge candidate for the current PU. It is asserted that the proposed changes can help solve the parsing robustness issue while preserving coding efficiency.

# Introduction

In Merge mode, neighboring motion information is borrowed and directly used for a current block. In 2NxN, Nx2N, or NxN partition mode as shown in Figure 1, potential candidate locations for deriving motion information for the second or fourth PU (marked in orange) are indicated with green blocks. According to the current HM3.0, motion information of the candidate that’s located inside the left or upper neighboring PU is always not to be used. Any other candidate that shares the same motion information with that candidate is also not to be used.

More exactly, such checking is performed as follows:

For Nx2N and 2NxN as shown in Figure 1, when coding the second PU (PU_1), motion information of each merge candidate (L, A, T, RA, and BL) will be compared with that of the first PU (PU_0). If some candidate has the same motion information as PU_0, the candidate will be set invalid and to be avoided.

For NxN mode as shown in Figure 1, when coding the last PU (PU_3), motion information of each candidate (L, A, T, RA, and BL) will be compared with those of PU_1 and PU_2, to avoid forming the 2Nx2N, Nx2N or 2NxN modes which are redundant.

The reason of doing so is to avoid obtaining a merge mode which is already available under a larger size partition, such as 2Nx2N.

Motion information comparison between different candidate locations is needed during the checking process described above. However, such comparison operation may cause issues to parsing robustness. One way of solving the issue is to use all neighboring candidates without considering the possibility of the mode being redundant relative to a merge mode under a larger partition. However, such a scheme can also cause coding performance degradation.

Figure 1. Merge candidate checking positions PUs

# Proposed Method

In this contribution, for a second PU shown in Figure 1, we propose to check the merge index of the first PU to determine whether the motion information of the candidate that’s located inside that first PU should be used. The checking of the first PU’s merge index can be used to effectively avoid obtaining a merge mode which is already available under a larger size partition.

Figure 2. Example of Nx2N

Figure 2 shows an example of an Nx2N CU for illustration of the proposed method. Figure 2-a shows the Nx2N CU and the merge candidates (green or red colored blocks) of the first partition (Nx2N_0, orange colored block). Figure 2-b shows the same CU and the merge candidates (green colored blocks) of the second partition (Nx2N_1, orange colored block). According to HM3.0, candidate L of the second partition is not to be used based on the assumption that such a case should have been covered under 2Nx2N merge mode. However, when the selected merge index of the first partition is RA or T (red colored blocks), it can be seen that even if candidate L of the second partition is used, the resulted merge mode is actually different from those covered under 2Nx2N merge mode shown in Figure 2-c. Instead, the resulted merge mode derives its motion information from RA or T, as shown in Figure 2-d. Since such cases are not covered under 2Nx2N merge mode, enabling it may improve coding efficiency. The same analysis is also true for a CU with 2NxN partition.

According to this proposal, for an Nx2N CU, if the merge index of the first PU indicates candidate RA or T, the candidate L of the second PU is considered as a valid merge candidate. More exactly, the scheme can be described as follows:

For Nx2N mode, candidate L of PU_1 is checked. If merge index of PU_0 equals to T or RA, candidate L of PU_1 is valid. Otherwise, it is invalid.

For 2NxN mode, candidate A of PU_1 is checked. If merge index of PU_0 equals to T or BL, candidate A of PU_1 is valid. Otherwise, it is invalid.

Since the case under NxN mode rarely happens, for simplicity there is no change for this case.

It is mentioned above that motion comparison and pruning currently used in merge mode may incur parsing issue. One way of avoiding parsing issue is proposed in [2]. It can address the issue efficiently with coding performance well preserved. It is worth mentioning that the proposed scheme works with this solution in addressing the parsing issue.

# Experimental Results

The proposed method is implemented in HM3.0 with bug 146 fixed (CE9 anchor) and simulated under the common test conditions [1]. Error: Reference source not found summarizes the results of the proposed scheme. There is around 0.1% coding gain comparing to the anchor.

If the scheme in [2] is used in addressing the parsing issue, Error: Reference source not foundTable 2 summarizes results relative to anchor with and without the proposed method respectively. It can be seen that relatively there is also about 0.1~0.2% improvement with the proposed method.

There is no obvious complexity increasing at both encoder and decoder. The detailed simulation results can be found in the attached excel sheet.

Table 1. Proposed method compared to CE9 anchor

|  | HE |  |  | LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RA | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| LB | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 | 0.0 |

Table 2. Scheme in [2], without and with the proposed method

|  | JCTVC-F474 [2] |  |  |  |  |  | JCTVC-F474 [2]+ Proposed method |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HE |  |  | LC |  |  | HE |  |  | LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| RA | 0.6 | 0.6 | 0.6 | 0.4 | 0.4 | 0.4 | 0.5 | 0.5 | 0.4 | 0.2 | 0.3 | 0.3 |
| LB | 0.8 | 0.6 | 0.4 | 0.4 | 0.4 | 0.7 | 0.7 | 0.4 | 0.4 | 0.3 | 0.5 | 0.5 |

# Conclusion

This proposal presents a scheme in which the merge index of a previous neighboring PU is used to determine whether the motion information of the candidate located inside that previous neighboring PU shall be used for a current PU. The proposed solution improves coding performance with little complexity overhead. It also works when parsing issue is addressed. We recommend the proposed changes to be adopted into HM.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC 5th Meeting: Geneva, March, 2011.

- Guillaume Laroche, Patrice Onno, Tangi Poirier, Christophe Gisquet (Canon), “CE9: Description of experiments ROB01 and ROB02”, JCTVC-F474, JCT-VC 6th Meeting: Torino, July, 2011

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Draft Text

##### Derivation process for spatial merging candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are (with N being replaced by A, B, C, or D and with X being replaced by 0 or 1)

- the availability flags availableFlagN of the neighbouring prediction units,

- the reference indices refIdxLXN of the neighbouring prediction units,

- the prediction list utilization flags predFlagLXN of the neighbouring prediction units,

- the motion vectors mvLXN of the neighbouring prediction units.

Figure 8-3 – Spatial neighbours that can be used as merging candidates (informative) illustrates the position of the spatial neighbours A, B, C and D relative to the current prediction unit.

For the derivation of availableFlagN, with N being A, B, C or D and ( xN, yN ) being ( xP – 1,  yP ), ( xP,  yP – 1 ), ( xP + nPSW,  yP – 1 ) or ( xP – 1,  yP+nPSH ) the following applies.

–	If one of the following conditions is true, the availableFlagN is set equal to 0, both components mvLXN are set equal to 0, refIdxLXN and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.

  - The prediction unit covering luma location ( xN, yN ) is not available or PredMode is MODE_INTRA.

- PartMode of the current prediction unit is PART_2NxN and PartIdx is equal to 1 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 0 ) have merge_idx equals to 0, 1, or 4.

- PartMode of the current prediction unit is PART_Nx2N and PartIdx is equal to 1 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 0 ) have merge_idx equals to 0, 1, or 3.

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 )  have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xN, yN ]

- PartMode of the current prediction unit is PART_NxN and PartIdx is equal to 3 and the prediction units covering luma location ( xP,  yP – 1 ) ( PartIdx = 1 ) and luma location ( xP – 1,  yP – 1 ) ( PartIdx = 0 ) have identical motion parameters:

  - mvLX[ xP,  yP – 1 ]  = =  mvLX[ xP – 1,  yP – 1 ]

  - refIdxLX[ xP,  yP – 1 ]  = =  refIdxLX[ xP – 1,  yP – 1 ]

  - predFlagLX[ xP,  yP – 1 ]  = =  predFlagLX[ xP – 1,  yP – 1 ]

and the prediction units covering luma location ( xP – 1,  yP ) ( PartIdx = 2 ) and luma location ( xN, yN ) (Cand. N) have identical motion parameters:

  - mvLX[ xP – 1,  yP ]  = =  mvLX[ xN, yN ]

  - refIdxLX[ xP – 1,  yP ]  = =  refIdxLX[ xN, yN ]

  - predFlagLX[ xP – 1,  yP ]  = =  predFlagLX[ xN, yN ]

–	Otherwise, availableFlagN is set equal to 1 and the variables mvLX[ xN, yN ], refIdxLX[ xN, yN ] and predFlagLX[ xN, yN ] of the prediction unit covering luma location ( xN, yN ) are assigned respectively to mvLXN, refIdxLXN and predFlagLXN.