| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F304 WG11 Number: m20725 |
| --- | --- |

| Title: | CE8 Subtest 4: Cross-check of TI’s proposal (JCTVC-F234) on ALF decode with reduced vertical filter size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for TI’s proposal to CE8 Subtest 4 (JCTVC-F234), on ALF decode with reduced vertical filter size. Rate-PSNR numbers match exactly to their results. Decoding time reductions of 3% ~ 4% are reported. Studies of the software are also presented.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | CE8.4 results by TI ( JCTVC-F234[1] ) |  |  | Cross-check results by Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| BD-rate | 0.1 | 0.2 | 0.3 | 0.1 | 0.2 | 0.3 |
| Enc. Time | 109% | 99% | 101% | 98% | 100% | 99% |
| Dec. time | 108% | 93% | 99% | 96% | 97% | 96% |
| Vert. taps | 5 |  |  | 5 |  |  |
| Max# of coeff. | 17 |  |  | 17 |  |  |

# Findings and recommendations

- The code changes are consistent with the 3 shapes in "E060 using Nx5 Set 1", i.e. clipped-diamond 9Hx5V, clipped-diamond 7Hx5V, and diamond 5x5.

- The reduced vertical-size to 5 lines is achieved by removing top/bottom elements from the filter shapes in HM3.0 (clipped-diamond 9Hx7V, diamond 7x7, and diamond 5x5).

- The worst-case number of coefficients is 17 for the clipped-diamond 9Hx5V.

# References

- “CE8, Subset 4, Tool 3: ALF decode with reduced vertical filter size (JCTVC-E060)”, JCTVC-F234, Torino, Italy, Jul. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).