| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F306 |
| --- | --- |

| Title: | CE9: Crosscheck report of CE9-UNI01 (JCTVC-F380) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Information] |  |  |
| Author(s) or Contact(s): | Yunfei Zheng 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | 1-858-651-9568 zhengy@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution verified the implementation and simulation results of CE9-UNI01 which is proposed by Samsung.

# Introduction

In CE9 subtest UNI01, a unified Merge and AMVP candidate checking position is proposed. The proposed method changes both Merge and AMVP candidate checking position and unified them into a new pattern, which is shown in Figure 1.

The candidates will be 1, 2, 2’, 3, 3’, 4. The A and B are not used any more. The maximum candidate number of Merge and AMVP mode are also modified to 4, which is 5 and 3 in HM3.0 respectively. The checking orders in both modes are also modified.

Figure 1 the proposed unified checking position pattern in JCTVC-F380

The method is correctly implemented on HM3.0 based on our brief code review. The simulation results are verified and compared with HM3.0, which matches what was provided.

# Experimental Results:

The test condidtion is defined in CE9 description. The BD bitrate saving and encoder/decoder complexity are summarized in Table 1 and Table 2 respectively.

Table 1. Simulation results - BD bit rate reduction (%) comparing to HM3.0 anchor

|  | HE |  |  | LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RA | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.2 |
| LB | 0.2 | 0.4 | 0.2 | 0.2 | 0.1 | 0.0 |

Table 2. Complexity comparison with HM3.0 anchor

|  | HE |  | LC |  |
| --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder |
| RA | 97% | 98% | 99% | 101% |
| LB | 97% | 99% | 98% | 101% |