| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F307 WG11 Number: m20728 |
| --- | --- |

| Title: | CE8 Subtest 4: Cross-check of MediaTek, Qualcomm and Toshiba’s proposal (JCTVC-F321) on block based ALF and temporal prediction of ALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for MediaTek, Qualcomm and Toshiba’s proposal to CE8 Subtest 1 (JCTVC-F321), on block based ALF and temporal prediction of ALF coefficients. Rate-PSNR numbers match exactly to the proponent’s results. Some decoding time increases in RA and LD cases are confirmed. Studies of the software are also presented.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | CE8.1 results by the proponent ( JCTVC-F321 [1] ) |  |  | Cross-check results by Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| BD-rate | 0.0 | -0.3 | -0.3 | 0.0 | -0.3 | -0.3 |
| Enc. Time | 100% | 101% | 100% | 100% | 101% | 101% |
| Dec. time | 100% | 105% | 101% | 102% | 109% | 105% |

# Findings in software study

- There are two modes for temporal prediction of ALF coefficients: It can be “direct prediction” with no differences sent, or a normal prediction with coefficients differences in the bitstream rather than the coefficients themselves.

- The decision of whether to use temporal prediction can be made on a filter-by-filter basis.

# References

- “CE8.1: Block based Adaptive Loop Filter by MediaTek, Qualcomm and Toshiba”, JCTVC-F321, Torino, Italy, Jul. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).