| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F312 WG11 Number: m20733 |
| --- | --- |

| Title: | CE8 Subset 2: Cross-Verification of SONY’s adaptive loop filter (JCTVC-F285) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC-F285 for CE8 Subset2.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-F285 Case2 (combination of ALF and Parametric ALF (PALF) with lambda modification), which is a contribution to CE8 Subset2. Verification is performed under the CE8 test conditions that refer to JCTVC-E700.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | 0.0 | -0.6 | -1.0 | -2.5 | -0.6 | -1.0 |  |
|  | Class B | -0.1 | -0.5 | -1.0 | -1.3 | -1.3 | -1.5 |  |
|  | Class C | -0.3 | -0.2 | -0.6 | -1.4 | -2.5 | -3.4 |  |
|  | Class D | -0.2 | 1.0 | 1.0 | -0.7 | -1.8 | -2.1 |  |
|  | Class E | 0.1 | -0.1 | -0.2 | -2.1 | -0.2 | -0.9 |  |
|  | Overall | -0.1 | -0.1 | -0.4 | -1.6 | -1.3 | -1.8 |  |
|  | Enc Time[%] | 96% |  |  | 116% |  |  |  |
|  | Dec Time[%] | 97% |  |  | 121% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.2 | -1.5 | -2.1 | -4.2 | -2.9 | -3.0 |  |
|  | Class B | -0.4 | -1.6 | -1.9 | -2.7 | -1.7 | -1.2 |  |
|  | Class C | -0.5 | -1.3 | -1.2 | -2.6 | -1.2 | -2.0 |  |
|  | Class D | -0.6 | 0.0 | -0.2 | -2.9 | -0.2 | -0.1 |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Overall | -0.4 | -1.1 | -1.4 | -3.1 | -1.5 | -1.6 |  |
|  | Enc Time[%] | 97% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 98% |  |  | 114% |  |  |  |
|  |  | Low delay B HE |  |  | Low delay B LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A |  |  |  |  |  |  |  |
|  | Class B | -0.7 | -2.7 | -3.4 | -3.6 | -3.5 | -3.4 |  |
|  | Class C | -1.0 | -3.0 | -3.7 | -3.6 | -3.0 | -4.5 |  |
|  | Class D | -0.9 | -0.6 | -0.4 | -2.8 | -1.0 | -0.9 |  |
|  | Class E | -1.2 | -2.4 | -4.1 | -5.5 | -6.4 | -6.5 |  |
|  | Overall | -0.9 | -2.2 | -2.8 | -3.7 | -3.3 | -3.6 |  |
|  | Enc Time[%] | 97% |  |  | 106% |  |  |  |
|  | Dec Time[%] | 97% |  |  | 113% |  |  |  |

Table 1. SONY’s combination of ALF and PALF

# Source code review

SONY’s proposal is using the slice level switching of ALF and PALF. For ALF, ALF in HM30 is used. In their PALF, 64 (32 for luma and 32 for chroma) set of trained fixed filter coefficients are used. The  performance of PALF seems to be highly dependent on sequences.