| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F315 WG11 Number: m20737 |
| --- | --- |

| Title: | CE3: Non-uniform tap length filtering for bidirectional prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

A experimental result of non-uniform tap length filters for bidirectional prediction is reported.  This is one of proposals of core experiment 3 on interpolation for MC.  The worst cases of complexity of interpolation process are two dimensional quarter pixel positions of bidirectional prediction.  The shorter tap length filters are introduced in those cases.  As a experimental result, loss coding efficiency is an average of 0.20%.  The complexity analyses of 8/6-tap non-uniform tap length filters show worst case of computational complexity and average of computational complexity and memory bandwidth are reduced from them of 8-tap DCT-IF.

# Introduction

The purpose of CE3 [1] is to improve the performance of inter prediction with enhanced motion compensation filters.  In this contribution, according to common condition, JCTVC-E703 [1], the experimental results of non-uniform tap length filtering for bidirectional prediction are reported

# Non-uniform Tap Length Filtering (NTLF)

An interpolation that has non-uniform tap length filters for computational complexity reduction is tested.  For example, in case of inner fractional position, asymmetrical M-tap (M<N) is used and symmetrical N-tap filter is used and in case of border fractional position, both asymmetrical and symmetrical N-tap filter is used.  Since the number of multiplication of symmetrical filter is smaller than that of asymmetrical filter at the same tap-length generally, the worst number of multiplication is reduced.

When both motion vectors of list 0 and 1 indicate fractional position vertically and horizontally, 6-tap asymmetrical filters as Table 1 are introduced.  That is one of the worst cases of number of operations.

In addition, since we found several problems in HM and WD, we removed all intermediate rounding and clipping of interpolation processing.  As a result, the order of operation of filtering is changeable.

Table 1	 6-tap interpolation filter coefficients for bidirectional prediction

| 6-tap | 1/4 | 2,-9,57,18,-6,2 |
| --- | --- | --- |

Figure 1  Non-uniform tap length for each sub-pixel

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-E700.  Table 2 indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-E700, the interpolation filter of anchor is 8-tap DCT-IF.

Table 2	Average results of luminance BD-rate (%)

|  |  | High efficiency | Low complexity |
| --- | --- | --- | --- |
| Random access | Class A | -0.03 | 0.21 |
|  | Class B | 0.06 | 0.22 |
|  | Class C | 0.25 | 0.47 |
|  | Class D | 0.56 | 0.97 |
|  | Class E | N/A | N/A |
|  | All | 0.20 | 0.45 |
|  | Enc Time[%] | 100% | 100% |
|  | Dec Time[%] | 98% | 97% |
| Low delay | Class A | N/A | N/A |
|  | Class B | -0.23 | -0.06 |
|  | Class C | 0.10 | 0.25 |
|  | Class D | 0.32 | 0.70 |
|  | Class E | -0.28 | -0.35 |
|  | All | -0.02 | 0.15 |
|  | Enc Time[%] | 100% | 100% |
|  | Dec Time[%] | 98% | 97% |

# Complexity analysis

CE3 has conducted complexity analyses for each proposal.  At this time, computational operations and memory bandwidth for each block size and for each single and bidirectional prediction are measured.  This proposal can reduce worst case and average of computational operations of bidirectional prediction and also average of memory bandwidth.

Table 3	Summary of complexity analysis

|  |  |  | Add/shift |  |  | Multiple |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Anchor DCT-IF 8-tap | NTLF 8/6-tap | Ratio (%) | Anchor DCT-IF 8-tap | NTLF 8/6-tap | Ratio (%) |
| Single | Operations | Worst case / pixel | 60.00 | 60.00 | 100% | 48.75 | 48.75 | 100% |
|  |  | Average of operations / pixel | 30.10 | 30.10 | 100% | 24.08 | 24.08 | 100% |
|  | Bandwidth | Worst case / pixel | 7.56 | 7.56 | 100% | 7.56 | 7.56 | 100% |
|  |  | Average of bytes / pixel | 2.48 | 2.48 | 100% | 2.48 | 2.48 | 100% |
| Bi | Operations | Worst case / pixel | 120.00 | 103.00 | 86% | 97.50 | 80.50 | 86% |
|  |  | Average of operations / pixel | 60.21 | 53.80 | 89% | 48.17 | 42.23 | 89% |
|  | Bandwidth | Worst case / pixel | 15.13 | 15.13 | 100% | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 4.95 | 4.35 | 88% | 4.95 | 4.35 | 88% |
| Total | Operations | Worst case / pixel | 120.00 | 103.00 | 86% | 97.50 | 80.50 | 86% |
|  |  | Average of operations / pixel | 45.15 | 41.95 | 93% | 36.12 | 33.15 | 93% |
|  | Bandwidth | Worst case / pixel | 15.13 | 15.13 | 100% | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 3.71 | 3.41 | 92% | 3.71 | 3.41 | 92% |

# Conclusions

In this contribution, the simulation results of non-uniform tap length filtering for bidirectional prediction have been shown.  As experimental results and complexity analyses, the coding losses for anchor are less than 0.20% on average and the total complexities have been reduced.

We would like to recommend that this solution should be adopted on HM.

# References

[1] 	T. Chujoh and E.Alshina, “CE3: Interpolation for MC,” Joint Collaborative Team on Video Coding, JCTVC-E703, Geneva, March 2011.

[2] 	F. Bossen, “Common test conditions and software reference configurations,” Joint Collaborative Team on Video Coding, JCTVC-E700, Geneva, March 2011.

[3] 	T. Chujoh, K. Kano and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” Joint Collaborative Team on Video Coding, JCTVC-C078, Guanzhou, Oct. 2010.

[4] 	T. Chujoh, K. Kano and T. Yamakage, “CE3: Non-uniform tap length filtering,” Joint Collaborative Team on Video Coding, JCTVC-D154, Daegu, January 2011.

[5] 	T. Chujoh, K. Kano and T. Yamakage, “CE3: Non-uniform tap length filtering for biprediction,” Joint Collaborative Team on Video Coding, JCTVC-E134, March 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex

Changes in section 8.4.2.2.2 and 8.4.2.2.3.

Variables shift1, shift2, shift3, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2, and the variable shift3 is set equal to 14 – BitDepthC.

–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	The variable offset2 is set equal to 1 << ( shift2 – 1 ).

a0,0 = ( -A-3,0 + 4*A-2,0 - 10*A-1,0 + 57*A0,0 +	 19*A1,0 - 7*A2,0 + 3*A3,0 - A4,0 ) <<6

b0,0 = ( -A-3,0 + 4*A-2,0 - 11*A-1,0 + 40*A0,0 +	 40*A1,0 - 11*A2,0 + 4*A3,0 - A4,0 ) <<6

c0,0 = ( -A-3,0 + 3*A-2,0 - 7*A-1,0 + 19*A0,0 +	 57*A1,0 - 10*A2,0 + 4*A3,0 - A4,0 ) <<6

d0,0 = ( -A0,-3 + 4*A0,-2 - 10*A0,-1 + 57*A0,0 +	 19*A0,1 - 7*A0,2 + 3*A0,3 - A0,4 ) <<6

h0,0 = ( -A0,-3 + 4*A0,-2 - 11*A0,-1 + 40*A0,0 +	 40*A0,1 - 11*A0,2 + 4*A0,3 - A0,4 ) <<6

n0,0 = ( -A0,-3 + 3*A0,-2 - 7*A0,-1 + 19*A0,0 +	 57*A0,1 - 10*A0,2 + 4*A0,3 - A0,4 ) <<6

The samples labeled e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by first calculating intermediate values denoted as d1i,0, h1i,0, n1i,0, h2i,0 and n2i,0 where i = -3..4 by applying the 8-tap or i = -2..3 by applying the 6-tap filter to the nearest integer position samples in vertical direction:

d1i,0 =  -Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4

h1i,0 =  -Ai,-3 + 4*Ai,-2 - 11*Ai,-1 + 40*Ai,0 +	 40*Ai,1 - 11*Ai,2 + 4*Ai,3 - Ai,4

n1i,0 =  -Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 +	 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai,4

d2i,0 =  -Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4

n2i,0 =  -Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 +	 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai,4

–	The final prediction values e0,0, f0,0, g0,0, i0,0, j0,0, k0,0, p0,0, q0,0 and r0,0 shall be derived by applying the 8-tap filter to the intermediate values d1i,0, h1i,0 and n1i,0 where i = -3..4 in horizontal direction:

e0,0 = -d1-3,0 + 4*d1-2,0 - 10*d1-1,0 + 57*d10,0 +	 19*d11,0 - 7*d12,0 + 3*d13,0 - d14,0

f0,0 = -d1-3,0 + 4*d1-2,0 - 11*d1-1,0 + 40*d10,0 +	 40*d11,0 - 11*d12,0 + 4*d13,0 - d14,0

g0,0 = -d1-3,0 + 3*d1-2,0 - 7*d1-1,0 + 19*d10,0 +	 57*d11,0 - 10*d12,0 + 4*d13,0 - d14,0

i0,0 = -h1-3,0 + 4*h1-2,0 - 10*h1-1,0 + 57*h10,0 +	 19*h11,0 - 7*h12,0 + 3*h13,0 - h14,0

j0,0 = -h1-3,0 + 4*h1-2,0 - 11*h1-1,0 + 40*h10,0 +	 40*h11,0 - 11*h12,0 + 4*h13,0 - h14,0

k0,0 = -h1-3,0 + 3*h1-2,0 - 7*h1-1,0 + 19*h10,0 +	 57*h11,0 - 10*h12,0 + 4*h13,0 - h14,0

p0,0 = -n1-3,0 + 4*n1-2,0 - 10*n1-1,0 + 57*n10,0 +	 19*n11,0 - 7*n12,0 + 3*n13,0 - n14,0

q0,0 = -n1-3,0 + 4*n1-2,0 - 11*n1-1,0 + 40*n10,0 +	 40*n11,0 - 11*n12,0 + 4*n13,0 - n14,0

r0,0 = -n1-3,0 + 3*n1-2,0 - 7*n1-1,0 + 19*n10,0 +	 57*n11,0 - 10*n12,0 + 4*n13,0 - n14,0

–	If both predFlagL0 and predFlagL1 are equal to 1 and all xFracL0, yFracL0, xFracL1, and yFracL1 are not zero, e0,0, f0,0, g0,0, i0,0, k0,0, p0,0, q0,0, and r0,0 shall be derived by applying the 8-tap or 6-tap filter and to the intermediate values h1i,0, d2i,0, n2i,0 in horizontal direction:

e0,0 = 2*d2-2,0 - 9*d2-1,0 + 57*d20,0 + 18*d21,0 - 6*d22,0 + 2*d23,0

f0,0 = -d2-3,0 + 4*d2-2,0 - 11*d2-1,0 + 40*d20,0 +	 40*d21,0 - 11*d22,0 + 4*d23,0 – d24,0

g0,0 = 2*d2-2,0 - 6*d2-1,0 + 18*d20,0 + 57*d21,0 - 9*d22,0 + 2*d23,0

i0,0 = -h1-3,0 + 4*h1-2,0 - 10*h1-1,0 + 57*h10,0 +	 19*h11,0 - 7*h12,0 + 3*h13,0 – h14,0

k0,0 = -h1-3,0 + 3*h1-2,0 - 7*h1-1,0 + 19*h10,0 +	 57*h11,0 - 10*h12,0 + 4*h13,0 – h14,0

p0,0 =2*n2-2,0 - 9*n2-1,0 + 57*n20,0 + 18*n21,0 - 6*n22,0 + 2*n23,0

q0,0 = -n2-3,0 + 4*n2-2,0 - 11*n2-1,0 + 40*n20,0 +	 40*n21,0 - 11*n22,0 + 4*n23,0 – n24,0

r0,0 = 2*n2-2,0 - 6*n2-1,0 + 18*n20,0 + 57*n21,0 - 9*n22,0 + 2*n23,0

Table 8-4 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << 12 | d | h | n | a | e | i | p | b | F | j | q | c | g | k | r |

ab0,0 = ( -3*B-1,0 + 60*B0,0 + 8*B1,0 – B2,0 ) <<6		(8-1)

ac0,0 = ( -4*B-1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) <<6		(8-2)

ad0,0 = ( -5*B-1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) <<6		(8-3)

ae0,0 = ( -4*B-1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) <<6		(8-4)

af0,0 = ( -4*B-1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) <<6		(8-5)

ag0,0 = ( -2*B-1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) <<6		(8-6)

ah0,0 = ( -B-1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) <<6		(8-7)

Xb0,0 = -3*Xa-1,0 + 60*Xa0,0 + 8*Xa1,0 – Xa2,0		(8-8)

Xc0,0 = -4*Xa-1,0 + 54*Xa0,0 + 16*Xa1,0 – 2*Xa2,0		(8-9)

Xd0,0 = -5*Xa-1,0 + 46*Xa0,0 + 27*Xa1,0 – 4*Xa2,0		(8-10)

Xe0,0 = -4*Xa-1,0 + 36*Xa0,0 + 36*Xa1,0 – 4*Xa2,0		(8-11)

Xf0,0 = -4*Xa-1,0 + 27*Xa0,0 + 46*Xa1,0 – 5*Xa2,0		(8-12)

Xg0,0 = -2*Xa-1,0 + 16*Xa0,0 + 54*Xa1,0 – 4*Xa2,0		(8-13)

Xh0,0 = -Xa-1,0 + 8*Xa0,0 + 60*Xa1,0 – 3*Xa2,0		(8-14)

Table 8-5 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << 12 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is set equal to 1 << ( shift1 – 1 ) and the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1 are derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL0[ x, y ] + 2048 ) >> 12 )	(8-15)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL1[ x, y ] + 2048 ) >> 12 )	(8-16)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , 
								 predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + 4096 ) >> 13 )	(8-17)