| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F316 WG11 Number: m20738 |
| --- | --- |

| Title: | CE3: Cross-check report of Samsung's tool 2.3 (JCTVC-F247) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-checking results of Samsung's tool 3.2 in CE3 (JCTVC-F247) are shown.  The software that is provided by Samsung is used and run on our platform.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.  Detailed experimental results show attached Excel files.  Samsung’s tool 3.2 in CE3 (JCTVC-F247) has been cross-verified independently.

# Software check

Although the anchor adopts add/shift implementation for filtering, this proposal adopts multiplication implementation for all filtering.  This proposal adopts6-tap filtering; however on the software, values 0 are inserted at both ends of 8-tap coefficients.  This might influence encoding and decoding times.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-E700.  Error: Reference source not found indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-E700, the interpolation filter of anchor is 8-tap DCT-IF.  Used platform is that the OS is Windows 7 64-bit; the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.

Table 1 Results of coding efficiency and Enc/Dec times

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | 0.2 | 0.2 | 0.1 | 0.4 |
| Class B | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class C | 0.4 | 0.4 | 0.3 | 0.7 | 0.3 | 0.3 |
| Class D | 0.9 | 0.7 | 0.6 | 1.5 | 0.7 | 0.8 |
| Class E |  |  |  |  |  |  |
| Overall | 0.3 | 0.2 | 0.3 | 0.5 | 0.3 | 0.4 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | 0.1 | -0.1 | 0.1 | 0.1 |
| Class C | 0.3 | 0.4 | 0.5 | 0.6 | 0.2 | 0.2 |
| Class D | 0.5 | 1.1 | 0.9 | 1.1 | 0.6 | 0.5 |
| Class E | -0.1 | -0.5 | -0.2 | -0.4 | -0.3 | -0.6 |
| Overall | 0.1 | 0.4 | 0.3 | 0.3 | 0.2 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

We have cross-checked Samsung’s tool 3.2 in CE3 (JCTVC-F247) independently.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).