| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F317 WG11 Number: m20739 |
| --- | --- |

| Title: | CE3: Cross-check report for Sony's tool 5 (JCTVC-F101) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

Cross-checking results of Sony's tool 5 in CE3 (JCTVC-F101) are shown.  The software that is provided by Sony is used and run on an independent platform.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.  The results of encoding and decoding time are improved compared with anchor.  The results of complexity analyses are reasonable.  Detailed experimental results show attached Excel files.  Sony’s tool 5 in CE3 (JCTVC-F101) has been cross-verified independently.

# Software check

Although the anchor adopts add/shift implementation for filtering, this proposal adopts multiplication implementation for all filtering.  This might influence encoding and decoding times.

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-E700.  Error: Reference source not found indicates the average results of coding efficiency compared to each anchor.  According to JCTVC-E700, the interpolation filter of anchor is 8-tap DCT-IF.  Used platform is that the OS is Windows 7 64-bit; the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  All local decoded images by encoder match decoded images by decoder perfectly.  All bitrate and PSNR are identical with proponent's results perfectly.

The results of BQSquare on random access anchor are improved mainly.

Table 1 Results of coding efficiency and Enc/Dec times

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.5 | 0.5 | 0.2 | 0.7 |
| Class B | -0.1 | 0.1 | 0.2 | -0.3 | 0.2 | 0.2 |
| Class C | -0.8 | -0.1 | -0.3 | -1.6 | -0.1 | -0.2 |
| Class D | -1.6 | -0.7 | -0.8 | -3.3 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.1 | -0.1 | -1.1 | -0.1 | 0.0 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.6 | 0.1 | 0.2 | 0.0 | -0.3 |
| Class C | -0.1 | 0.3 | 0.2 | -1.0 | 0.1 | 0.0 |
| Class D | -0.1 | 0.0 | 0.7 | -1.3 | 0.2 | -0.1 |
| Class E | 0.6 | 0.9 | 0.3 | 1.7 | -0.5 | 0.4 |
| All | 0.1 | 0.5 | 0.3 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay P |  |  | Low delay P LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.4 | -0.3 | -0.3 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 |
| Class D | 0.0 | -0.1 | -0.3 | 0.3 | 0.0 | -0.7 |
| Class E | -0.2 | 0.0 | -0.6 | -1.3 | -0.8 | -0.6 |
| All | -0.1 | -0.1 | -0.4 | -0.3 | -0.3 | -0.4 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

# Conclusions

We have cross-checked Sony’s tool 5 in CE3 (JCTVC-F101) independently.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).