| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F318r1 WG11 Number: m20740 |
| --- | --- |

| Title: | On the precision of interpolation processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, the precision of interpolation processing is discussed.  Current HEVC Test Mode (HM) adopts 8-tap filter coefficients with 6-bit precision for luminance pixels and 4-tap filter coefficients with 6-bit precision for chrominance pixels and a high accuracy bidirectional prediction method.  As results of investigating Working Draft (WD), HM software and document, it is shown that there are several problems about precision of interpolation processing.  There are two solutions.  One solution is that some rounding and clipping are introduced for 16-bit input and output specification; however since the filter coefficients and tap-length are under consideration in CE, the design might be changed.  The other solution is that all intermediate rounding and clipping are removed.  If there is no intermediate rounding and clipping, the order of operation for interpolation processing is changeable.  This can expand the implementation possibility of encoder and decoder.  In that case, even if 14-bit pixels input, all outputs do not exceed singed 32-bit integer.

# Problems

The following problems are pointed out.

- The results of first step of two-dimensional filtering, equations (8-105), (8-106) and (8-107) might exceed 16 bits when input bit-depth is greater than 9 bits.

- The results of position “j”, equation (8-112), might exceed singed 16-bit integer.  The other results do not exceed signed 16-bit integer.  For example, when BitDepth is equal to 8, shift1 is equal to 0, shift2 is equal to 2 and pixel values are 0 and 255 like checker board as Figure 1, j0,0 is greater than 215-1.  In addition, since the pixel variable of HM software is defined by 16-bit singed short integer, it is possible to cause overflow in case of B-Slice.

- There are some differences between WD and HM software.  Although HM software does not include intermediate rounding in case of single prediction, WD, from (8-99) to (8-104), from (8-108) to (8-116), from (8-119) to (8-132) and from (8-140) to (8-146), includes an intermediate rounding because of integration of high accuracy bidirectional prediction.  Technically, in case of single prediction, the intermediate rounding is not needed.

(2121600+32)>>6 = 33151 > 32767 = 215 -1

Figure 1  the worst case of interpolation process

# Discussion

There are two solutions.  One solution is to introduce rounding to (8-105), (8-106) and (8-107), and clipping to (8-112) for all 16-bit input and output specification.  (In case of chroma, from (8-133) to (8139) should be rounded.)  In annex I, changes of WD are described.  Since filter coefficients and tap-length are under consideration in CE3, the design might be changed.

Therefore, we also would like to recommend the other solution.  That is to remove all intermediate rounding and clipping of interpolation processing at this point.  If there is no intermediate rounding and clipping, the order of operation for interpolation processing is changeable.  This can expand the implementation possibility of encoder and decoder.  In annex II, changes of WD are described.  Attached Excel sheet shows an experimental result.  The coding efficiency is almost the same the encoding and decoding time is a little faster than anchor.  The important point is that even if 14-bit pixels input, all outputs of this change do not exceed singed 32-bit integer as Figure 2.

Vertical		: MaxValue = (214-1)x(40+4)x2 = 1441704

MinValue = (214-1)x(-11-1)x2 = -425958

Horizontal		: MaxValue = 1441704x(40+4)x2-425958x(-11-1)x2 = 137092944 < 231-1

MinValue = -425958x(40+4)x2+1441704 x(-11-1)x2 = -72085200 > -231

Bidirectional	: MaxValue = 137092944+137092944 = 274185888 < 231-1

MinValue = -72085200-72085200 = -144170400 > -231

Figure 2  the worst case when all rounding are removed

If the order of intermediate interpolation processing is changeable, for example, the following two interesting non-normative methods can be introduced.

- Example 1

The order of horizontal filtering and vertical filtering are changeable.

Considering the interpolation processing based on MC block, if the block is rectangle, not square, the number of operation and intermediate pixels is different to the order.

If the block is horizontally long, the order of vertical filtering and horizontal filtering is better.

If the block is vertically long, the order of horizontal filtering and vertical filtering is better.

- Example 2

In the case of bidirectional prediction, the order of temporal averaging and spatial filtering are changeable.

If MVs of both list 0 and 1 indicate same fractional position, firstly two pixel values are added and then those values can be filtered.  As a result, about half operations can be achieved.

When two motion vectors of both list 0 and 1 indicate “e”, it might be the worst case of number of operations if the predictive values are averaged by two independent interpolation images.

However, if the order of operations is changeable, two pixels of list 0 and 1 are added firstly and the result is done.

fq[]=−1,4,−10,57,19,−7,3,−1predictionSamplesL0e+predictionSamplesL1e =i=−34j=−34fqifqjA0,i,j+i=−34j=−34fqifqjA1,i,j =i=−34j=−34fqifqjA0,i,j+A1,i,j				(1)

# Conclusions

Several problems of interpolation processing are reported.  Although there are two solutions, we would like to recommend that all intermediate rounding and clipping of interpolation processing should be removed at this point.  The design of rounding and clipping should be discussed after the design of interpolation processing has been finished.

# References

[1] 	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm and G. J. Sullivan,“WD3: Working Draft 3 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding, JCTVC-E603r7, Geneva, March 2011.

[2]		E.Alshina, J.Chen, A.Alshin, N.Shlyakhov and W.-J.Han, “CE3: Experimental results of DCTIF by Samsung,” Joint Collaborative Team on Video Coding, JCTVC-D344, Daegu, January,2011.

[3]		J.Chen, E.Alshina and W.-J.Han, CE4: Experimental results of DCTIF application for Chroma MC by Samsung, Joint Collaborative Team on Video Coding, JCTVC-D347, Daegu, January,2011.

[4] 	K.Ugur, J.Lainema and A.Hallapuro,“High precision bi-direction averaging,” Joint Collaborative Team on Video Coding, JCTVC-D321,Daegu, January,2011.

[5] 	K.Ugur, J.Lainema,“On clipping in bi-directional averaging,” Joint Collaborative Team on Video Coding, JCTVC-E242, Geneva, March 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex I

Changes in section 8.4.2.2.2 and 8.4.2.2.3.

Variables shift1, shift2, shift3, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to BitDepthY – 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 – BitDepthY.

d1i,0 = ( -Ai,-3 + 4*Ai,-2 - 10*Ai,-1 + 57*Ai,0 + 19*Ai,1 - 7*Ai,2 + 3*Ai,3 - Ai,4 + offset1 ) >> shift1	(8-1)

h1i,0 = ( -Ai,-3 + 4*Ai,-2 - 11*Ai,-1 + 40*Ai,0 +	 40*Ai,1 - 11*Ai,2 + 4*Ai,3 - Ai,4 + offset1 ) >> shift1	(8-2)

n1i,0 = ( -Ai,-3 + 3*Ai,-2 - 7*Ai,-1 + 19*Ai,0 +	 57*Ai,1 - 10*Ai,2 + 4*Ai,3 - Ai,4 + offset1 ) >> shift1	(8-3)

j0,0 = Clip3( -32768, 32767 , 
	( -h1-3,0 + 4*h1-2,0 - 11*h1-1,0 + 40*h10,0 +	 40*h11,0 - 11*h12,0 + 4*h13,0 - h14,0 + offset2 ) >> shift2 )	(8-4)

bai,0 =  (-3*B0,-1 + 60*B0,0 + 8*B0,1 – B0,2 + offset1 ) >> shift1	(8-5)

cai,0 =  (-4*B0,-1 + 54*B0,0 + 16*B0,1 – 2*B0,2 + offset1 ) >> shift1	(8-6)

dai,0 =  (-5*B0,-1 + 46*B0,0 + 27*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-7)

eai,0 =  (-4*B0,-1 + 36*B0,0 + 36*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-8)

fai,0 =  (-4*B0,-1 + 27*B0,0 + 46*B0,1 – 5*B0,2 + offset1 ) >> shift1	(8-9)

gai,0 =  (-2*B0,-1 + 16*B0,0 + 54*B0,1 – 4*B0,2 + offset1 ) >> shift1	(8-10)

hai,0 =  (-B0,-1 + 8*B0,0 + 60*B0,1 – 3*B0,2 + offset1 ) >> shift1	(8-11)

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL0[ x, y ] + 2048 ) >> shift1 )	(8-12)

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL1[ x, y ] + 2048 ) >> shift1 )	(8-13)

# Annex II

Changes in section 8.4.2.2.2 and 8.4.2.2.3.

Variables shift1, shift2, shift3, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to BitDepthC – 8, the variable shift2 is set equal to BitDepthC – 2, and the variable shift3 is set equal to 14 – BitDepthC.

–	If the variable shift1 is equal to 0, the variable offset1 is set equal to 0, otherwise, the variable offset1 is set equal to 1 << ( shift1 – 1 ).

–	The variable offset2 is set equal to 1 << ( shift2 – 1 ).

a0,0 = ( -A-3,0 + 4*A-2,0 - 10*A-1,0 + 57*A0,0 +	 19*A1,0 - 7*A2,0 + 3*A3,0 - A4,0 ) <<6	(8-14)

b0,0 = ( -A-3,0 + 4*A-2,0 - 11*A-1,0 + 40*A0,0 +	 40*A1,0 - 11*A2,0 + 4*A3,0 - A4,0 ) <<6	(8-15)

c0,0 = ( -A-3,0 + 3*A-2,0 - 7*A-1,0 + 19*A0,0 +	 57*A1,0 - 10*A2,0 + 4*A3,0 - A4,0 ) <<6	(8-16)

d0,0 = ( -A0,-3 + 4*A0,-2 - 10*A0,-1 + 57*A0,0 +	 19*A0,1 - 7*A0,2 + 3*A0,3 - A0,4 ) <<6	(8-17)

h0,0 = ( -A0,-3 + 4*A0,-2 - 11*A0,-1 + 40*A0,0 +	 40*A0,1 - 11*A0,2 + 4*A0,3 - A0,4 ) <<6	(8-18)

n0,0 = ( -A0,-3 + 3*A0,-2 - 7*A0,-1 + 19*A0,0 +	 57*A0,1 - 10*A0,2 + 4*A0,3 - A0,4 ) <<6	(8-19)

e0,0 = -d1-3,0 + 4*d1-2,0 - 10*d1-1,0 + 57*d10,0 +	 19*d11,0 - 7*d12,0 + 3*d13,0 - d14,0	(8-20)

f0,0 = -d1-3,0 + 4*d1-2,0 - 11*d1-1,0 + 40*d10,0 +	 40*d11,0 - 11*d12,0 + 4*d13,0 - d14,0	(8-21)

g0,0 = -d1-3,0 + 3*d1-2,0 - 7*d1-1,0 + 19*d10,0 +	 57*d11,0 - 10*d12,0 + 4*d13,0 - d14,0	(8-22)

i0,0 = -h1-3,0 + 4*h1-2,0 - 10*h1-1,0 + 57*h10,0 +	 19*h11,0 - 7*h12,0 + 3*h13,0 - h14,0	(8-23)

j0,0 = -h1-3,0 + 4*h1-2,0 - 11*h1-1,0 + 40*h10,0 +	 40*h11,0 - 11*h12,0 + 4*h13,0 - h14,0	(8-24)

k0,0 = -h1-3,0 + 3*h1-2,0 - 7*h1-1,0 + 19*h10,0 +	 57*h11,0 - 10*h12,0 + 4*h13,0 - h14,0	(8-25)

p0,0 = -n1-3,0 + 4*n1-2,0 - 10*n1-1,0 + 57*n10,0 +	 19*n11,0 - 7*n12,0 + 3*n13,0 - n14,0	(8-26)

q0,0 = -n1-3,0 + 4*n1-2,0 - 11*n1-1,0 + 40*n10,0 +	 40*n11,0 - 11*n12,0 + 4*n13,0 - n14,0	(8-27)

r0,0 = -n1-3,0 + 3*n1-2,0 - 7*n1-1,0 + 19*n10,0 +	 57*n11,0 - 10*n12,0 + 4*n13,0 - n14,0	(8-28)

Table 8-1 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]

| xFracL | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracL | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 |
| predSampleLXL[ xL, yL ] | A << 12 | d | h | n | a | e | i | p | b | f | j | q | c | g | k | r |

ab0,0 = ( -3*B-1,0 + 60*B0,0 + 8*B1,0 – B2,0 ) <<6		(8-29)

ac0,0 = ( -4*B-1,0 + 54*B0,0 + 16*B1,0 – 2*B2,0 ) <<6		(8-30)

ad0,0 = ( -5*B-1,0 + 46*B0,0 + 27*B1,0 – 4*B2,0 ) <<6		(8-31)

ae0,0 = ( -4*B-1,0 + 36*B0,0 + 36*B1,0 – 4*B2,0 ) <<6		(8-32)

af0,0 = ( -4*B-1,0 + 27*B0,0 + 46*B1,0 – 5*B2,0 ) <<6		(8-33)

ag0,0 = ( -2*B-1,0 + 16*B0,0 + 54*B1,0 – 4*B2,0 ) <<6		(8-34)

ah0,0 = ( -B-1,0 + 8*B0,0 + 60*B1,0 – 3*B2,0 ) <<6		(8-35)

Xb0,0 = -3*Xa-1,0 + 60*Xa0,0 + 8*Xa1,0 – Xa2,0		(8-36)

Xc0,0 = -4*Xa-1,0 + 54*Xa0,0 + 16*Xa1,0 – 2*Xa2,0		(8-37)

Xd0,0 = -5*Xa-1,0 + 46*Xa0,0 + 27*Xa1,0 – 4*Xa2,0		(8-38)

Xe0,0 = -4*Xa-1,0 + 36*Xa0,0 + 36*Xa1,0 – 4*Xa2,0		(8-39)

Xf0,0 = -4*Xa-1,0 + 27*Xa0,0 + 46*Xa1,0 – 5*Xa2,0		(8-40)

Xg0,0 = -2*Xa-1,0 + 16*Xa0,0 + 54*Xa1,0 – 4*Xa2,0		(8-41)

Xh0,0 = -Xa-1,0 + 8*Xa0,0 + 60*Xa1,0 – 3*Xa2,0		(8-42)

Table 8-2 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively

| xFracC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| yFracC | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| predSampleLXC[ xC, yC ] | B << 12 | ba | ca | da | ea | fa | ga | ha | aY | bY | cY | dY | eY | fY | gY | hY |

Variables shift1, shift2, offset1 and offset2 are derived as follows.

–	The variable shift1 is set equal to 14 – bitDepth and the variable shift2 is set equal to 15 – bitDepth,

–	The variable offset1 is set equal to 1 << ( shift1 – 1 ) and the variable offset2 is set equal to 1 << ( shift2 – 1 ).

Depending on the value of predFlagL0 and predFlagL1, the prediction samples predSamples[ x, y ] with x = 0..(nPSW)-1 and y = 0..(nPSH)-1 are derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL0[ x, y ] + 2048 ) >> 12 )	(8-43)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , ( predSamplesL1[ x, y ] + 2048 ) >> 12 )	(8-44)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

predSamples[ x, y ] = Clip3( 0, ( 1 << bitDepth ) – 1 , 
								 predSamplesL0[ x, y ] + predSamplesL1[ x, y ] + 4096 ) >> 13 )	(8-45)