| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F320 |
| --- | --- |

| Title: | Subjective Tests on ALF and SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Request for further study |  |  |
| Author(s) or Contact(s): | Osman G. Sezer Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1861 osman@ti.com madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of informal subjective tests conducted on two loop filtering operations in HM-3.0: SAO and ALF. Test videos were presented at their actual frame rates to the viewers. Four configurations were tested using low delay B high efficiency (LB-HE) condition: HM-3.0 Anchor v/s SAO-off+ALF-off, HM-3.0 Anchor v/s SAO-off, HM-3.0 Anchor v/s ALF-off, HM-3.0 Anchor v/s ALF-Luma-off. Overall, the subjective difference between the HM-3.0 Anchor and the test configurations is less than expected. Among these four configurations, ALF-Luma-off gave subjective results closest to the HM-3.0 Anchor. This contribution requests that JCTVC conduct subjective testing of ALF and SAO in a core experiment setting to evaluate the subjective gains provided by the tools and decide on the minimal set of configurations that best provide subjective quality improvements so that the minimal set can be included in the final standard.

# Introduction

Informal subjective test are conducted on two separate learning-based loop filtering techniques in HM-3.0: Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) for LB-HE condition. ALF seeks for a set of filters learned through Wiener-Hopf equations that tries to get the reconstructed frame close to the original one via iterative minimization of rate-distortion cost. SAO, on the other hand, learns a set of offset values for different local geometric features, and adds these values to the reconstructed frame when that feature is observed in the filtering process. These two filtering operations are observed to provide bit savings in HM-3.0, refer to Table 2.

To better understand the subjective gains provided by ALF and SAO when they work together or separately, we have devised several experiments. Due to limited resources, six sequences are selected from HEVC committee streams, for which the loop filters provide some subjective difference at low bitrates (high qps) when a frame-by-frame comparisons is done. The reported subjective tests in this document try to understand how these subjective differences reflect on actual viewer experience once the sequences are view in their original frame rates.

In the experiments, each sequence is encoded with HM-3.0 Anchor and a test configuration. Reconstructed streams of two coding conditions are presented consecutively but in randomized orders to the viewers. In these viewing sessions, the subjects are asked to report the video that seem better or mark neutral if they can not see a difference. Since the first frame of the second video is likely to have higher PSNR values compared to the last frame of the first video, the viewers might be bias towards reporting the second video as the better one. As a remedy, the subjects are asked to compare the subjective quality of same segments in two videos, and make an overall decision.

# Test Conditions

A 46-inch LCD HDTV monitor with an approximate viewing distance of 80 inches is used.  Streams are played with original frame rates. QP values of 32 and 37 are used with six sequences, hence a total of 12 streams are shown in each sessions. The following HEVC committee streams are included in the experiments:

- BasketballDrill_p832x480_50fps_420pl_500fr.yuv

- RaceHorsesC_p832x480_30fps_420pl_300fr.yuv

- vidyo1_p1280x720_60fps_420pl_600fr.yuv

- vidyo3_p1280x720_60fps_420pl_600fr.yuv

- vidyo4_p1280x720_60fps_420pl_600fr.yuv

- BasketballDrive_p1920x1080_50fps_420pl_500fr.yuv

The viewing sessions are done on different days with three viewers for each test configuration. There are four test configurations used in our experiments. Each test configuration is subjectively compared against the HM-3.0 Anchor. From three viewers and 12 sequences, 36 different scores are obtained for our informal subjective tests. The viewers have three options. They can, without knowing, vote for test or anchor configuration, or they can stay neutral. The total number of neutral, test, and anchor decisions are used to compare subjective gains. Basically, higher neutral score means the test configuration looks similar to the HM-3.0 Anchor.

# Results

Table 1. Summary of the subjective test scores conducted on ALF and SAO. Each configuration has a total of 36 observations.

| Test configuration number | Test Settings | Number of votes for Neutral | Number of votes for Anchor | Number of votes for Test |
| --- | --- | --- | --- | --- |
| 1 | 1. ALF-Off, SAO-Off | 23 | 10 | 3 |
| 2 | 2. ALF-On, SAO-Off | 21 | 9 | 6 |
| 3 | 3. ALF-Off, SAO-On | 23 | 9 | 4 |
| 4 | 4. ALF-Luma-Off, SAO-On | 29 | 4 | 3 |

In the first test configuration, SAO and ALF both disabled. This experiment is designed to provide a baseline for the remaining tests. In other words, once SAO or ALF is separately enabled, they can be compared against this baseline test configuration. Any improvement over the baseline therefore can be detected. Note, even in the baseline case where both tools are off, the neutral votes are 63% of the total votes.

Table 1 provides the summary of subjective scores that gathered from our experiments. In the second experiment, SAO is disabled while ALF is enabled. The third experiment is the reverse case, where ALF is disabled and SAO is enabled. For second and third test configurations, although each one has streams that subjectively perform better than the other, the overall scores are observed to be close to the baseline.

For fair comparison between ALF and SAO, another test configuration is added to our experiments. Essentially, since SAO does not have chroma processing, chroma filtering of ALF is enabled in the fourth experiment. By modifying the HM-3.0, the luma filtering in ALF is disabled, while chroma filtering is set to always operate. As a quick fix, the luma filter is replaced with an impulse function (one at the center and rest is zero) in the gnsSolveByChol routine. Compared to baseline experiment, the fourth experiment produced highest neutral and lowest anchor scores.

# Summary

This contribution presented results of informal subjective tests conducted on two loop filtering operations in HM-3.0: SAO and ALF. Test videos were presented at their actual frame rates to the viewers. Four configurations were tested using low delay B high efficiency (LB-HE) condition: HM-3.0 Anchor v/s SAO-off+ALF-off, HM-3.0 Anchor v/s SAO-off, HM-3.0 Anchor v/s ALF-off, HM-3.0 Anchor v/s ALF-Luma-off. Overall, the subjective difference between the HM-3.0 Anchor and the test configurations is less than expected. Among these four configurations, ALF-Luma-off gave subjective results closest to the HM-3.0 Anchor.

This contribution requests that JCTVC conduct subjective testing of ALF and SAO in a core experiment setting to evaluate the subjective gains provided by the tools and decide on the minimal set of configurations that best provide subjective quality improvements so that the minimal set can be included in the final standard.

# Appendix

Table 2 provides BD-rate results of different test configurations compared to the HM-3.0 anchor LB-HE.

Table 2. BD-Rate results compared to HM-3.0 Anchor LB-HE.

| Tests | Y | U | V | Enc(%) | Dec(%) |
| --- | --- | --- | --- | --- | --- |
| 1. ALF-Off, SAO Off | 7.5 | 1 | 1 | 92 | 77 |
| 2. ALF-On, SAO-Off | 1.7 | -0.3 | -0.4 | 98 | 96 |
| 3. ALF-Off, SAO-On | 4.7 | 1.5 | 1.5 | 91 | 78 |
| 4. ALF-Luma-Off, SAO-On | 5.2(*) | 0.4 | 0.3 | 99(*) | 96(*) |

(*) Note high BD-rate and Enc/Dec time values for experiment 4 is due to inefficient implementation of luma ALF disable.