| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F321_r1 WG11 Number: m20743 |
| --- | --- |

| Title: | CE8.1: Block based Adaptive Loop Filter by MediaTek, Qualcomm and Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2 and Qualcomm3 |  |  |

_____________________________

# Abstract

Coding results of Block based Adaptive Loop Filter that adopts 2-dimentional categorization of filtering blocks are presented.  In addition, in response to the discussion at the previous Geneva meeting, temporal prediction methods of ALF filter coefficients are introduced that re-use ALF filter coefficients from the reference frames’.

By the above methods, coding efficiency gain for luma is 0.2% to 0.3% on average for random access and low delay against the HM3.0 16-pass anchor.  Against the HM3.0 1-pass anchor, coding efficiency gain for luma is 0.3% on average for random access and low delay.  Against the HM3.1-dev anchor, coding efficiency gain for luma is 0.2% on average for random access and low delay.  Coding results were confirmed by Samsung for the mandatory case against HM3.0 16-pass encoding.

It is recommended to replace the current 2-dimentional categorization of filtering blocks and add temporal prediction methods of ALF filter coefficients in this contribution.

# Introduction

At the Geneva meeting in Mar. 2011, it was discussed to study 2-dimentional categorization method of blocks for ALF filtering.  As another context, the study for overhead reduction of ALF filter coefficients is encouraged, in particular, temporal prediction.

This contribution improves coding efficiency by improving 2-dimentional categorization method of blocks for ALF filtering and adopting temporal prediction methods of ALF filter coefficients.  In this contribution, two kinds of temporal prediction are introduced; a normal temporal prediction that signals the difference from the predictor and a temporal direct prediction that directly re-uses the coefficients of the predictor.  These may be performed from the reference frames of the decoding frame so that the same level of error resilience is guaranteed for the case of without temporal prediction.  (i.e. if some parts other than ALF coefficients have some error, the reference frame can not be decoded completely, which propagates the succeeding frames that refer to the error frame.)  Furthermore, when the re-used filter has a plural of filter classes, some of them may be turned off (i.e. no filtering for the designated class(es)).

Based on the common test conditions in JCTVC-E700 [3] and relevant CE 8 description in JCTVC-E708 [4], tests are conducted that have two anchors, HM3.0 16-pass and 1-pass.  Coding efficiency is improved 0.2% to 0.3% against HM3.0 16-pass and 0.3% against HM3.0 1-pass for random access and low delay.  In addition, HM3.1-dev base experiments are conducted.  Coding efficiency is improved 0.2% against HM3.1-dev for random access and low delay.

# 2-Dimentional Categorization Method of Blocks for ALF Filtering

Originally in HM3.0, block classification has totally 15 classes (i.e., 3 direction and 5 classes for Sum of Laplacian based activity metric).  In this proposal we proposed to use a bit finer classifications (i.e., 3 direction and 6 classes for Sum of Laplacian based activity metric).  It provides better classification of block which leads to better coding gain.

# Tempral Prediction of ALF Filter Coefficients

There are two methods introduced for temporal prediction of ALF filter coefficients.  The first one is to signal the difference from the predictor, and the second one is the direct temporal prediction that re-uses the predictor.  In both cases, the predictor can be one of the ALF filter coefficients of the reference frame(s) of the decoding frame. In this way, there is the same level of error propagation concern as the case of no temporal prediction of ALF coefficients.  In other words, even if temporal prediction methods are introduced, the level of error resilience will be unchanged.

Since there are several kinds of prediction methods and up to four reference frames for temporal prediction, it is necessary to newly signal (1) prediction method (none, spatial prediction, temporal prediction, or temporal direct prediction), (2) reference index in the case of temporal (direct) prediction and (3) difference coefficients in the case of temporal prediction.

When current filter coefficients are predicted from temporal predictors, temporal predictors are derived as follows. First one reference frame is chosen and reference index is signalled, and filters used for that reference frame is used predictors of current frame’s filters. For each filter (i.e., filter_k) of current frame, multiple variance indexs are assigned (i.e., vindex_k(1),…,vindex_k(L)). Among those variance indexes of each filter, the minimum variance index (i.e., vindex_k(1)) are used to pick a temporal predictor our of multiple filters of the chosen reference frame. In another words, among reference frame’s filters, the filter assigned to variance index vindex_k(1) is used as a temporal predictor of filter_k.

# Experimental Results against HM3.0

Experiments are conducted based on the test conditions JCTVC-E700 [1] and JCTVC-E708 [2] for High Efficiency cases in HM3.0 (16-pass and 1-pass).  The modification points from the HM 3.0 anchor are (1) categorization method of blocks for ALF filtering and (2) introduction of temporal prediction of ALF filter coefficients.  No encoding strategy nor other portion is modified.

Table 1 shows experimental results against HM3.0 16-pass anchor.  Coding efficiency gain for luma is 0.2% to 0.3% for random access and low delay with 1% to 5% decoding time increase.  Results were confirmed by Samsung for random access and low delay B cases.

Table 1  Results against HM3.0 anchor

|  | IO |  |  | RA |  |  | LDB |  |  | LDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | -0.3 | -0.3 | 0.1 |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.1 | -0.1 | -0.2 | 0.0 | 0.4 | -0.1 | -0.1 | -0.4 |
| Class C | 0.0 | 0.0 | 0.0 | -0.4 | -0.1 | -0.1 | -0.3 | -0.4 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class D | 0.0 | 0.0 | 0.0 | -0.5 | -0.5 | -0.3 | -0.4 | -0.5 | -0.1 | -0.4 | -0.6 | -0.6 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | -0.3 | -0.1 | -0.5 | -0.2 | -0.9 | -0.6 |
| Total | 0.0 | 0.0 | 0.0 | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | 0.0 | -0.2 | -0.4 | -0.5 |
| Enc. time | 100% |  |  | 101% |  |  | 100% |  |  | 101% |  |  |
| Dec. time | 100% |  |  | 105% |  |  | 101% |  |  | 102% |  |  |

Table 2 shows experimental results against HM3.0 1-pass anchor.  Coding efficiency gain for luma is 0.3% for random access and low delay with 3% to 4% decoding time increase.

Table 2  Results against HM3.0 1-pass anchor

|  | IO |  |  | RA |  |  | LDB |  |  | LDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | -0.2 | -0.3 | 0.0 |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.2 | -0.1 | -0.2 | -0.4 | -0.1 | -0.4 | -0.4 |
| Class C | 0.0 | 0.0 | 0.0 | -0.4 | -0.2 | -0.3 | -0.3 | -0.1 | -0.2 | -0.3 | -0.2 | -0.3 |
| Class D | 0.0 | 0.0 | 0.0 | -0.5 | -0.4 | -0.4 | -0.6 | -0.5 | -0.2 | -0.5 | -0.3 | 0.1 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | -0.2 | -0.1 | -0.5 | -0.3 | 0.8 | -0.1 |
| Total | 0.0 | 0.0 | 0.0 | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 | -0.3 | -0.3 | -0.1 | -0.2 |
| Enc. time | 100% |  |  | 100% |  |  | 101% |  |  | 101% |  |  |
| Dec. time | 98% |  |  | 103% |  |  | 103% |  |  | 104% |  |  |

# Experimental Results against HM3.1-dev

Experiments are conducted based on the test conditions JCTVC-E700 [1] and JCTVC-E708 [2] for High Efficiency cases in HM3.1-dev.  The modification points from the HM 3.1-dev anchor are (1) categorization method of blocks for ALF filtering and (2) introduction of temporal prediction of ALF filter coefficients.  No encoding strategy nor other portion is modified.

Table 3 shows experimental results against HM3.1-dev anchor.  Coding efficiency gain for luma is 0.2% for random access and low delay with 1% to 2% decoding time increase.

Table 3  Results against HM3.1-dev anchor

|  | IO |  |  | RA |  |  | LDB |  |  | LDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | -0.1 | 0.3 | -0.1 |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | -0.5 | -0.3 | -0.1 | 0.0 | -0.5 |
| Class C | 0.0 | 0.0 | 0.0 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 | -0.1 | -0.3 | 0.0 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.4 | -0.2 | -0.3 | -0.4 | -0.4 | -0.9 | -0.4 | 0.1 | -0.5 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | -0.3 | -0.2 | -0.1 | -0.1 | -0.6 | 0.6 |
| Total | 0.0 | 0.0 | 0.0 | -0.2 | -0.1 | -0.2 | -0.2 | -0.3 | -0.4 | -0.2 | -0.1 | -0.2 |
| Enc. time | 99% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec. time | 99% |  |  | 102% |  |  | 101% |  |  | 102% |  |  |

# Conclusion

In this contribution, the coding efficiency is improved by improving categorization method of blocks for ALF filtering and adopting temporal prediction of ALF filter coefficients.  It is recommended to replace the current 2-dimentional categorization of filtering blocks and add temporal prediction methods of ALF filter coefficients in this contribution.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011.

[2]	T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, F. Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-E708_r3, March 2011.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc. and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

7.3.3.5	Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_direct_flag | 2 | u(1) |
| if(alf_direct_flag) { |  |  |
| ref_idx_direct | 2 | ue(v) |
| for (i=0; i< 16; i++) |  |  |
| direct_filter_flag[i] | 2 | u(1) |
| } |  |  |
| else { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=0; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| alf_temporal_pred_flag | 2 | u(1) |
| if(alf_pred_method_bit1) |  |  |
| ref_idx_pred | 2 | ue(v) |
| if (AlfNumFilters > 1&& !alf_temporal_pred_flag) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| } |  |  |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

7.4.3.5	Adaptive loop filter parameter semantics

alf_direct_flag specifies whether temporal direct prediction of the filter coefficients is applied. If alf_direct_flag is equal to 1, temporal direct prediction is applied and filter information of the reference picture such as filter adaptation method, filter shape, number of filter sets, filter assignment for each variance index is used.

ref_idx_direct specifies the index of the reference picture for adaptive loop filter in the case of P slice or B slice with combined reference list.  The filter applied to the reference picture is used for temporal direct prediction.

direct_filter_flag[ i ] specifies whether luma samples classified to the i-th variance index are filtered.

alf_temporal_pred_flag specifies whether temporal prediction of the filter coefficients is applied.

ref_idx_pred specifies the index of the reference picture for adaptive loop filter in the case of P slice or B slice with combined reference list.  The filter applied to the reference picture is used as the predictor for temporal prediction of the filter coefficients.

alf_pred_method specifies whether the filter coefficients are coded in a spatially predictive way.