| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F325 |
| --- | --- |

| Title: | Modified temporal MV derivation process for merge/skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taichiro Shiodera Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 taichiro.shiodera@toshiba.co.jp akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution proposes modified derivation process of temporal merge/skip mode in B-slice and presents the experimental results. The temporal merge/skip mode is introduced in latest HM test model. This can reduce an overhead of motion information by reusing temporal motion information in current prediction unit. However, when two kinds of motion information used for bi-prediction refer same reference block, there is an issue that the prediction value for bi-prediction is identical to one for uni-prediction. In this contribution, two modifications for derivation process of temporal merge/skip mode in B-slice are introduced in order to improve both complexity and coding efficiency. First modification is to change inter prediction direction from bi-prediction to uni-prediction in order to reduce the computational complexity of redundant motion compensation process. The other one is to change the derivation process of list 1 motion information for temporal merge/skip mode in B-slice, in order to avoid deriving same motion information in current PU. This modification can improve coding efficiency.

Experimental results show that proposed methods can achieve the average BD-rate gain of 0.2% and 0.3% for LB-HE and LB-LC case, respectively. Average decoding times are 98% and 96% for LB-HE and LB-LC case compared with common condition anchor, respectively.

# Introduction

In current common test conditions [1], the coding structures with the Forward B-slices (referred as “FB-slices” in this contribution) are defined and FB-slices can use forward directional bi-prediction with two reference picture list 0 and list 1 instead of the conventional P-frames and the reference picture list 0 (RefPicList0) is equal to same the reference picture list 1 (RefPicList1). In current HM (HM3.0), temporal merge/skip mode[2] which reuses temporal motion information of co-located reference frames (colPic) is introduced. Figure 1 shows a flowchart of current derivation process of motion vector predictors for temporal merge/skip mode in HM3.0. In case of B- and FB-slices, when co-located partition (colPU) was coded using list 0 uni-prediction, list 0 MV predictor of colPU is set to not only list 0 MV predictor but also list 1 MV predictor for current PU.

Therefore, motion information which includes (MV, refIdx) of list 0 is often identical to one of list 1 in FB-slices. In case of merge/skip mode, mvd (MV difference) and refIdx are not signaled. Thus, the reference blocks of list 0 and list 1 are also same position. It means that the predicted values of uni-prediction are identical to ones of bi-prediction, since both lists have same motion information.

In order to improve such redundant representation, two modifications for derivation process of temporal merge/skip mode in B-slice are introduced. First modification is to change inter prediction direction from bi-prediction to uni-prediction in order to reduce the computational complexity of redundant motion compensation process. The other one is to change the derivation process of list 1 motion information for temporal merge/skip mode in B-slice, in order to avoid deriving same motion information in current PU.

Figure 1. Flowchart of derivation process for temporal motion vector predictor in HM3.0 (JCTVC-E481 sec. 8.3[2])

# Proposed method

## Proposal 1: change to uni-prediction from bi-prediction

In proposal 1, if reference blocks indicated by motion information for each list are same, the inter prediction direction mode of current PU is changed to uni-prediction from bi-prediction. The flowchart of modified derivation process of temporal merge mode for proposal 1 is shown in Figure 2. Non-highlighted blocks are same as original HM3.0 process, and yellow-highlighted blocks are newly added in this modification.

Figure 2. Flowchart of derivation process for temporal motion vector predictor for proposal 1.

## Proposal 2: modify MV derivation for list 1 of bi-prediction

In proposal 2, if RefPicListX[ 1 ] is available and POC number of reference frame corresponding to refIdx for each list are same, the co-located picture (colPic) position that contains the co-located partition (colPU) for only list 1 is changed. Figure 3 shows an example of colPic and colPU position for each list in (a) HM3.0 and (b) this proposal 2 when colocated_from_l0_flag is equal to 1. If the above condition is satisfied, colPic for list 1 is set to RefPicListX[ 1 ] instead of RefPicListX[ 0 ]. By introducing this modification, there is possibility that different motion information from current derivation process is set to one for list 1.

The flowchart of modified derivation process of temporal merge/skip mode for proposal 2 is shown in Figure 4. Non-highlighted blocks are same as original HM3.0 process, and blue-highlighted blocks are newly added in this modification. It is noted that only refIdx information is used for decision on whether colPic for list 1 is set to RefPicListX[ 1 ] or RefPicListX[ 0 ]. If this modification is inserted to the bottom of the flowchart, such derivation process is performed twice. It may cause the computational complexity increase for the derivation process. Then by introducing the top of the flowchart, this decision can be performed only once using only refIdx information.

(a) HM3                             (b) proposal 2

Figure 3. Example of colPic and colPU position for each list when colocated_from_l0_flag is equal to 1.

Figure 4. Flowchart of derivation process for temporal motion vector predictor for proposal 2.

# Syntax and decoding process

The syntax and semantics do not change in proposal 1 and 2. The detailed decoding process for proposal 1 and 2 are shown in appendix of this contribution.

# Experimental Results

HM software version 3.0 with the common configuration [1] is used in these experiments. Firstly, Table 1 shows BD-Rate and relative average encoding/decoding time for proposal 1 compared with HM3.0 anchor. The negative value of BD-Rate indicates gain. It observed that proposed 1 reduces the average decoding time of 4%, 6%, 1% and 1% for LB-HE, LB-LC, RA-HE and RA-LC, respectively. The difference in BD-rate is not obvious for any configurations.

It is noted that the bitstreams for proposal 1 are different from ones of current HM3.0, since the change of inter prediction mode for merge/skip mode affects context calculation of CABAC and adaptive inter mode of LCEC etc, while prediction values for uni-prediction are not changed from ones for bi-prediction.

Secondly, Table 2 shows BD-Rate and relative average encoding/decoding time for proposal 2 compared with HM3.0 anchor. It observed that proposed 2 gives the BD-rate gain of 0.2%, 0.4%, 0.0% and 0.0% for LB-HE, LB-LC, RA-HE and RA-LC, respectively. The difference in the average encoding/decoding time is not obvious for any configurations.

In RA cases, BD-Rate gain of proposal 1 and encoding/decoding time reduction of proposal 2 are smaller than LB cases since there is small number of FB-slices less than 1/8 for LB cases.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (proposal 1)

|  | LB-HE |  |  | LB-LC |  |  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class B | -0.1 | 0.1 | 0.1 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.1 | 0.3 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E | 0.0 | -0.2 | -0.5 | 0.0 | -0.4 | 0.0 |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  | 99% |  |  | 99% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (proposal 2)

|  | RA-HE |  |  | RA-LC |  |  | LB-HE |  |  | LB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class B | -0.2 | -0.3 | -0.1 | -0.4 | -0.6 | -0.5 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.3 | -0.2 | -0.2 | -0.3 | -0.3 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.5 | -0.2 | -0.1 | -0.1 | -0.3 | 0.0 | -0.1 | -0.1 | 0.1 | 0.0 | -0.1 |
| Class E | -0.1 | -0.6 | 0.0 | -0.5 | -0.9 | -0.7 |  |  |  |  |  |  |
| All | -0.2 | -0.4 | -0.1 | -0.3 | -0.5 | -0.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |

Finally, the experiment which combines proposal 1 and 2 is conducted. The flowchart of modified derivation process of temporal merge/skip mode for the combination with proposal 1 and proposal 2 is shown in Figure 5. Table 3 shows BD-Rate and relative average encoding/decoding time for the experiment. In LB cases, the proposed method saves 0.2% bits for HE case and 0.3% bits for LC case on average. The proposed method also reduce the average decoding time of 2% for HE case and 4% for LC case. On the other hand, in RA case, the BD-rate and encoding/decoding time are not changed compared with HM3.0 anchor.

Figure 5. Flowchart of derivation process for temporal motion vector predictor for proposal 1 and 2.

Table 3. BD-Rate[%] and relative encoding/decoding time[%] (proposal 3)

|  | LB-HE |  |  | LB-LC |  |  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class B | -0.3 | -0.3 | -0.7 | -0.5 | -0.6 | -0.7 | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.2 | -0.3 | -0.2 | -0.3 | -0.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class E | -0.3 | -0.5 | -0.7 | -0.5 | -0.5 | -0.8 |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.4 | -0.3 | -0.4 | -0.5 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, a modified usage of temporal merge mode is proposed. Experimental results show that proposed methods can achieve the average BD-rate gain of 0.2-0.3 % for low-delay cases. Moreover, proposed methods can reduce decoding time of 2%-3% relative to HM3.0 anchor.

It’s suggested that this modified temporal merge/skip method is introduced in the next version of HM test model.

# References

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.

- B. Bross et. al., “BoG report of CE9: MV Coding and Skip/Merge operation”, JCTVC-E481, Geneva, March, 2011.

- T. Wiegand et. al., “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, Geneva, March, 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## WD text for proposal. 1

8.4.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction for merge mode in subclause 8.4.2.1.6 is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol.

- If slice_type is equal to B, RefPicOrderCnt( currPic , refIdxL0 , L0 ) is equal to RefPicOrderCnt( currPic , refIdxL0 , L0 ) and mvL0Col is equal to mvL1Col, availableFlagL1Col is derived as follows.

availableFlagL1Col = 0	(8-xx)

The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-xx)

predFlagLXCol = availableFlagLXCol	(8-xx)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

(other text is same as 8.4.2.1.1 inWD3)

## WD text for proposal. 2

8.4.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction for merge mode in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-xx)

predFlagLXCol = availableFlagLXCol	(8-xx)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

(other text is same as 8.4.2.1.1 inWD3)

8.4.2.1.x	Derivation process for temporal luma motion vector prediction for merge mode

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

If slice_type is equal to B, the following applies.

  - If collocated_from_l0_flag is equal to 0, the following applies.

- If LX is equal to L1, RefPicList1[ 1 ] is available and RefPicOrderCnt( currPic , refIdxL0 , L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 1 ].

- Otherwise (LX is equal to L0, RefPicList1[ 1 ] is not available or RefPicOrderCnt ( currPic , refIdxL0 , L0 ) is not equal to RefPicOrderCnt( currPic, refIdxL1, L1 )), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

  - Otherwise (collocated_from_l0_flag is equal to 1), the following applies.

- If LX is equal to L1, RefPicList0[ 1 ] is available and RefPicOrderCnt( currPic , refIdxL0 , L0 ) is equal to RefPicOrderCnt( currPic, refIdxL1, L1 ), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 1 ].

- Otherwise (LX is equal to L0, RefPicList0[ 1 ] is not available or RefPicOrderCnt ( currPic , refIdxL0 , L0 ) is not equal to RefPicOrderCnt( currPic, refIdxL1, L1 )), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

- Otherwise (slice_type is equal to P), the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-xx)

yPRb = yP + nPSH	(8-xx)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-xx)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-xx)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

  - If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following applies.

    - The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

  - If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = mvCol	(8-xx)

  - Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-xx)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-xx)

mvLXCol = ClipMv( ( DistScaleFactor * mvCol + 128 ) >> 8 )	(8-xx)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(8-xx)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-xx)