| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F326 |
| --- | --- |

| Title: | AHG18: Explicit Weighted Prediction with simple WP parameter estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This document presents the experimental results for Weighted Prediction (WP) method with simple WP parameter estimation scheme. The WP has been designed to compensate illumination variation in video sequences and adopt in a part of AVC standard. This feature is effective for video splicing process used in video authoring system and can get significant coding gain for fading sequences.

In this document, WP parameter estimation method based on alpha-blending model is introduced in order to derive optimal WP parameters under 1-pass encoding condition. This method uses only spatial image characteristics and its computational cost is negligible. The experimental results in HM software version 3.0 under common test conditions are reported. The WP using 1-pass encoding scheme has a large gain of 17% to 31% on average (up to 61% for Low delay P structure) for the test sequences with linear fade provided in WP AHG. Furthermore, it is reported that the WP does not have a big impact on coding efficiency for the regular test sequences.

# Weighted Prediction in HEVC

At the last Geneva meeting, a potential gain of weighted prediction (WP) method using ideal WP parameters created by fading tool was reported in [1]. By introducing the WP method to HEVC, it shows a large coding gain. However, an encoding algorithm to derive the optimal WP parameters is not provided in that contribution. Then, AHG to study on the WP method or the other illumination compensation (IC) method [2] has been established. The mandates of this AHG are following;

- Develop software for support of WP in HEVC in a same way as in AVC for study toward support of WP capability.

- Discuss and develop analysis software for estimating weighting parameters for explicit WP.

- Discuss and develop common conditions to test WP and illumination compensation (IC) tools in general, including test sequences and encoding configurations.

- Study technical proposals related to WP and IC in general, evaluate their gains, complexity, and interactions with existing HM design tools.

The several WP parameter estimation methods for WP targeted in AVC standard have already been proposed [3][4] and the simplified WP parameter estimation for bi-prediction [3] was proposed at JVT Fairfax meeting. In this contribution, a similar WP parameter estimation method based on alpha-blending model to derive optimal WP parameters under 1-pass encoding condition is introduced and the experimental results are reported.

## Algorithm description for WP

The WP is a method of an adaptive weighting of inter prediction signal to compensate illumination variation between the reference pictures and the current one. It can be applied in uni-prediction (1) or in bi-prediction (2).

predPartC[ x, y ] = ( ( predPartL0C[ x, y ] * w0 + ( wshift >> 1) ) >> wshift ) + o0		(1)

predPartC[ x, y ] = ( ( predPartL0C[ x, y ] * w0  + predPartL1C[ x, y ] * w1 + wshift ) >> 
					( wshift + 1 ) ) + ( ( o0 + o1 + 1 ) >> 1 ) 				(2)

where, w0 is weighting factor and o0 is offset factor. wshift means a shift value of weighting factor for rounding operation.

For bi-prediction case, high accuracy bi-prediction [5] scheme is introduced in latest HM test model. Then, the equation (2) is modified in the following;

predPartC[ x, y ] = ( ( predPartL0C[ x, y ] * w0  + predPartL1C[ x, y ] * w1 + hab_shift ) >> 
					(hab_shift + 1 ) ) + ( ( o0 + o1 + 1 ) >> 1 ) 				(3)

where hab_shift is a value including bit shift used for high accuracy bi-prediction. A rounding operation for bi-prediction is performed once at the last stage in motion compensation.

WP AHG provides a reference WP software based on HM version 3.0 [6]. Our implementation of normative part of WP scheme is same as this reference software except for equation (3).

## Encoder Algorithm description for WP parameter estimation

In order to derive optimal WP parameters under 1-pass encoding condition, WP parameter estimation method based on alpha-blending model is introduced. This method uses only spatial image characteristics, which are robust with respect to motion. We assume a formula for applying fade effect to video sequences based on alpha-blending model;

Y’( t ) = α( t ) * Y ( t ) + ( 1 – α( t ) ) * C,						(4)

where Y ( t ) is the original picture at time t and Y’ ( t ) is the fading picture. C is a tagert pixel value (such as 16 for black fade and 235 for white fade etc.) and α( t ) means alpha-blending parameter whose range is 0 to 1. We assume that correlation between the previous picture and the current picture is high, which Y ( t ) and Y ( t – 1 ) is same as Y.

Y’( t ) = α( t ) * Y ( t ) + ( 1 – α( t ) ) * C,

= α( t ) * Y + ( 1 – α( t ) ) * C,						(5)

Y’( t – 1 ) = α( t – 1 ) * Y ( t – 1 ) + ( 1 – α( t – 1 ) ) * C,

= α( t – 1 ) * Y + ( 1 – α( t – 1 ) ) * C,						(6)

Then we obtain the following relationship between two fading picture at different times.

Y’( t ) = α( t ) / α( t – 1 ) * Y ( t – 1 ) + ( 1 – α( t ) / α( t – 1 ) ) * C,			(7)

where, the weighting factor w0 is α( t ) / α( t – 1 ) and the offset factor o0 is ( 1 – α( t ) / α( t – 1 ) ) * C.

Let Y represent YAC + YDC, the equations (5) and (6) can be changed in the following;

Y’( t ) = α( t ) * YAC + dc( t ),							(8)

Y’( t – 1 ) = α( t – 1 ) * YAC + dc( t – 1 ),						(9)

where, AC component and DC component of the observed pictures Y’( t ) and Y’( t – 1 ) have following relationships;

AC(Y’( t )) = α( t ) * YAC,		DC(Y’( t )) = dc( t )				(10)

AC(Y’( t – 1 )) = α( t – 1 ) * YAC ,	DC(Y’( t – 1 )) = dc( t – 1 ),			(11)

Finally, we can derive following equations;

w0=NAC(Y'(t))NAC(Y'(t−1))=α(t)α(t−1),							(12)

o0=DC(Y'(t))−w0×DC(Y'(t−1))=dc(t)−w0×dc(t−1),			(13)

The proposed method uses only simple spatial image characteristics, such as AC and DC components. These characteristics are calculated from the original picture only once for each picture and the cost of the WP parameter calculation by (12) and (13) is negligible.

Finally, the 1-pass encoding algorithm consists of following steps;

- 1-pass encoding algorithm for HM version 3.0

- Calculate AC/DC values for current original picture and store them in the local memory corresponding to POC number.

- Compute the weighting factor w0 using the calculated AC value and the stored AC value corresponding to POC number based on equation (12).

- Compute the offset factor o0 using the weighting factor w0, the calculated DC value and the stored DC value corresponding to POC number based on equation (13).

- Compute SAD values between current picture and reference picture with/without WP parameters.

- Select whether WP is used or not based on SAD values.

- Encode the current picture with/without WP method.

## Syntax, semantics and decoding process for WP

The detailed specification for WP including syntax, semantics and decoding process, is shown in JCTVC-E041 and JCTVC-F265.

# Experimental Results

## Experimental Results for black-fade and white fade sequences

HM software version 3.0 (HM3.0) with WP method and fading test sequences are provided by WP AHG. The fading sequences are made by fading tool and are about 2 seconds of linear fade-black and fade-white [7]. The detailed information is shown in [8]. The 1-pass encoding algorithm is implemented on top of this software. Common test conditions [9] are used in these experiments. Table 1 and Table 2 shows BD-rate [%] and encoding/decoding time ratio for black-fade sequences and white-fade sequences compared with HM3.0 anchor, which means no WP method. The negative value indicates gain. The tables show that the WP method can get 17% to 31% gain (up to 60% for low delay case in the attached excel sheet) for all coding structures. The gain is promising and the WP is effective for both black and white fade sequences. We can see the reduction of decoding time, which means that the residual is considerably reduced.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] for black-fade sequences

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -17.5 | -20.7 | -21.0 | -18.4 | -14.1 | -13.6 |
| Class B | -19.6 | -19.8 | -19.9 | -19.9 | -8.7 | -18.3 |
| Class C | -14.9 | -14.7 | -15.1 | -16.2 | -11.6 | -13.8 |
| Class D | -15.0 | -13.8 | -13.8 | -16.5 | -10.6 | -13.2 |
| Class E |  |  |  |  |  |  |
| All | -16.9 | -17.4 | -17.6 | -17.9 | -11.1 | -14.9 |
| Enc Time[%] | 121% |  |  | 127% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | LDB-HE |  |  | LDB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -25.8 | -21.5 | -21.3 | -31.6 | -28.7 | -36.7 |
| Class C | -13.4 | -12.3 | -12.4 | -18.0 | -26.1 | -25.2 |
| Class D | -13.2 | -14.6 | -14.0 | -17.5 | -23.6 | -25.3 |
| Class E | -42.7 | -29.0 | -28.7 | -48.1 | -38.5 | -46.0 |
| All | -22.7 | -18.9 | -18.6 | -27.7 | -28.6 | -32.7 |
| Enc Time[%] | 116% |  |  | 124% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |

|  | LDP-HE |  |  | LDP-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -25.4 | -19.4 | -18.1 | -30.1 | -28.3 | -31.2 |
| Class C | -14.3 | -11.8 | -12.7 | -18.7 | -25.5 | -23.8 |
| Class D | -16.4 | -16.7 | -16.4 | -20.0 | -25.0 | -26.8 |
| Class E | -44.2 | -31.1 | -29.0 | -48.4 | -37.8 | -35.8 |
| All | -23.9 | -19.0 | -18.4 | -28.1 | -28.5 | -29.1 |
| Enc Time[%] | 100% |  |  | 108% |  |  |
| Dec Time[%] | 82% |  |  | 81% |  |  |

Table 2. BD-Rate[%] and relative encoding/decoding time[%] for white-fade sequences

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -22.2 | -25.6 | -26.0 | -22.5 | -16.6 | -17.7 |
| Class B | -23.4 | -25.4 | -24.7 | -23.7 | -15.1 | -22.1 |
| Class C | -16.9 | -17.6 | -18.3 | -18.6 | -14.7 | -17.1 |
| Class D | -16.1 | -15.3 | -14.7 | -17.8 | -12.6 | -16.6 |
| Class E |  |  |  |  |  |  |
| All | -19.9 | -21.2 | -21.1 | -20.8 | -14.8 | -18.6 |
| Enc Time[%] | 117% |  |  | 122% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  |

|  | LDB-HE |  |  | LDB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -28.7 | -24.5 | -23.9 | -35.6 | -32.2 | -39.1 |
| Class C | -15.4 | -13.9 | -14.8 | -20.8 | -28.4 | -27.2 |
| Class D | -12.8 | -16.1 | -14.7 | -18.0 | -24.9 | -26.7 |
| Class E | -44.0 | -32.9 | -29.8 | -50.7 | -40.4 | -47.6 |
| All | -24.3 | -21.3 | -20.4 | -30.3 | -31.0 | -34.6 |
| Enc Time[%] | 111% |  |  | 119% |  |  |
| Dec Time[%] | 82% |  |  | 82% |  |  |

|  | LDP-HE |  |  | LDP-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -28.1 | -22.7 | -21.1 | -34.5 | -30.4 | -33.5 |
| Class C | -16.2 | -13.7 | -15.1 | -21.7 | -27.2 | -26.4 |
| Class D | -16.5 | -16.6 | -16.3 | -21.2 | -27.0 | -28.4 |
| Class E | -45.1 | -32.8 | -31.9 | -51.7 | -39.6 | -39.8 |
| All | -25.4 | -20.8 | -20.4 | -31.2 | -30.5 | -31.6 |
| Enc Time[%] | 96% |  |  | 101% |  |  |
| Dec Time[%] | 82% |  |  | 81% |  |  |

## Experimental Results of 1-pass encoding with WP parameters feedback

Assuming a real-time application, 1-pass encoding scheme with WP parameters feedback is tested additionally. In this case, same 1-pass encoding scheme is used on condition to feed back WP parameters at one-frame delay, which means that a current picture cannot use current WP parameters derived from current original picture. One-frame delayed WP parameter is set in the current picture. Table 3 and Table 4 show the experimental results. Low delay coding structures are only tested in these experiments. It is observed that the coding gain is almost similar to the previous experiment.

Table 3. BD-Rate[%] and relative encoding/decoding time[%] for black-fade sequences

|  | LDB-HE |  |  | LDB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -23.7 | -19.5 | -18.6 | -28.5 | -28.0 | -34.6 |
| Class C | -12.5 | -11.2 | -11.4 | -17.0 | -25.7 | -23.7 |
| Class D | -12.2 | -13.2 | -12.6 | -16.0 | -21.9 | -23.6 |
| Class E | -39.5 | -25.7 | -24.4 | -45.1 | -36.0 | -44.6 |
| All | -21.0 | -17.0 | -16.4 | -25.6 | -27.4 | -31.0 |
| Enc Time[%] | 117% |  |  | 125% |  |  |
| Dec Time[%] | 84% |  |  | 83% |  |  |

|  | LDP-HE |  |  | LDP-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -23.2 | -17.1 | -14.9 | -26.9 | -26.9 | -28.9 |
| Class C | -13.3 | -11.5 | -11.5 | -17.5 | -24.6 | -22.7 |
| Class D | -15.2 | -15.1 | -15.1 | -18.5 | -23.8 | -25.0 |
| Class E | -41.2 | -26.9 | -25.7 | -45.1 | -33.9 | -34.2 |
| All | -22.1 | -17.1 | -16.1 | -25.8 | -26.8 | -27.3 |
| Enc Time[%] | 102% |  |  | 109% |  |  |
| Dec Time[%] | 84% |  |  | 82% |  |  |

Table 4. BD-Rate[%] and relative encoding/decoding time[%] for white-fade sequences

|  | LDB-HE |  |  | LDB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -26.2 | -21.3 | -20.5 | -33.0 | -32.3 | -37.0 |
| Class C | -14.5 | -13.2 | -13.4 | -20.0 | -27.9 | -26.3 |
| Class D | -11.9 | -14.7 | -13.3 | -16.8 | -24.5 | -24.9 |
| Class E | -41.1 | -29.8 | -26.8 | -48.3 | -38.6 | -47.1 |
| All | -22.5 | -19.2 | -18.1 | -28.6 | -30.4 | -33.2 |
| Enc Time[%] | 113% |  |  | 121% |  |  |
| Dec Time[%] | 84% |  |  | 83% |  |  |

|  | LDP-HE |  |  | LDP-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -25.7 | -19.8 | -18.1 | -31.6 | -30.2 | -31.6 |
| Class C | -15.4 | -13.0 | -13.2 | -20.7 | -26.9 | -25.2 |
| Class D | -15.3 | -15.5 | -14.8 | -19.9 | -25.9 | -26.4 |
| Class E | -42.3 | -29.1 | -28.0 | -48.9 | -36.6 | -38.2 |
| All | -23.6 | -18.7 | -17.9 | -29.2 | -29.5 | -30.0 |
| Enc Time[%] | 98% |  |  | 104% |  |  |
| Dec Time[%] | 83% |  |  | 82% |  |  |

## Experimental Results for cross-fade sequences

In order to evaluate cross-fade sequences, we randomly select the sequences for each class excluding Class A and create linear cross-fade sequences using the fading tool provided by WP AHG. Table 5 shows the selected combination of cross-fade sequences. Fading period is 50 frames for all classes to evaluate them easily. Table 6 shows the experimental results. The proposed 1-pass encoding scheme can get 2.4% to 4.8% (up to 16%) gain compared to the anchor on average.

Table 5. combinations of cross-fade sequences

| Class | Cross-fade sequences |  |
| --- | --- | --- |
|  | source | destination |
| Class B | BQTerrace | Kimono |
|  | BasketballDrive | ParkScene |
|  | Cactus | BasketballDrive |
|  | Kimono | Cactus |
|  | ParkScene | BQTerrace |
| Class C | BQMall | BasketballDrill |
|  | BasketballDrill | RaceHorses |
|  | PartyScene | BQMall |
|  | RaceHorses | PartyScene |
| Class D | BQSquare | BasketballPass |
|  | BasketballPass | RaceHorses |
|  | BlowingBubbles | BQSquare |
|  | RaceHorses | BlowingBubbles |
| Class E | Vidyo1 | Vidyo3 |
|  | Vidyo3 | Vidyo4 |
|  | Vidyo4 | Vidyo1 |

Table 6. BD-Rate[%] and relative encoding/decoding time[%] for cross-fade sequences

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -5.8 | -5.3 | -5.7 | -5.7 | -1.1 | -4.5 |
| Class C | -2.1 | -1.9 | -1.7 | -2.3 | -2.1 | -1.8 |
| Class D | -3.2 | 0.1 | -2.3 | -3.3 | 1.4 | -1.4 |
| Class E |  |  |  |  |  |  |
| All | -3.8 | -2.6 | -3.4 | -3.9 | -0.6 | -2.7 |
| Enc Time[%] | 106% |  |  | 114% |  |  |
| Dec Time[%] | 88% |  |  | 95% |  |  |

|  | LDB-HE |  |  | LDB-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -4.9 | -3.8 | -3.6 | -7.5 | -7.9 | -10.3 |
| Class C | -0.7 | -1.3 | -0.8 | -1.9 | -5.7 | -4.7 |
| Class D | -0.9 | -0.4 | -2.5 | -2.6 | -6.2 | -8.2 |
| Class E | -2.6 | -1.8 | -1.4 | -4.9 | -7.0 | -8.8 |
| All | -2.4 | -2.0 | -2.2 | -4.4 | -6.8 | -8.1 |
| Enc Time[%] | 103% |  |  | 111% |  |  |
| Dec Time[%] | 84% |  |  | 92% |  |  |

|  | LDP-HE |  |  | LDP-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -4.4 | -3.4 | -3.3 | -7.0 | -6.8 | -8.1 |
| Class C | -1.1 | -1.4 | -1.3 | -2.4 | -6.2 | -4.8 |
| Class D | -2.1 | -1.1 | -4.0 | -4.0 | -7.2 | -10.0 |
| Class E | -3.4 | -2.3 | -2.7 | -5.4 | -5.8 | -7.3 |
| All | -2.8 | -2.1 | -2.9 | -4.8 | -6.6 | -7.6 |
| Enc Time[%] | 101% |  |  | 111% |  |  |
| Dec Time[%] | 83% |  |  | 92% |  |  |

## Summary of Experimental Results

The following tables show the summary of this contribution. Experimental results with ideal WP parameter are inserted in these tables. There is almost no difference between using ideal WP parameters and using 1-pass WP parameter estimation.

Table 7. Summary of Y BD-Rate[%] for black-fade sequences.

|  | RA-HE | RA-LC | LDB-HE | LDB-LC | LDP-HE | LDP-LC |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal WP param | -17.6% | -19.6% | -22.6% | -28.4% | -23.4% | -28.3% |
| 1-pass WP | -16.9% | -17.9% | -22.7% | -27.7% | -23.9% | -28.1% |
| 1-pass WP LD | N/A | N/A | -21.0% | -25.6% | -22.1% | -25.8% |

Table 8. Summary of Y BD-Rate[%] for white-fade sequences.

|  | RA-HE | RA-LC | LDB-HE | LDB-LC | LDP-HE | LDP-LC |
| --- | --- | --- | --- | --- | --- | --- |
| Ideal WP param | -18.4% | -20.9% | -20.7% | -28.2% | -21.9% | -28.2% |
| 1-pass WP | -19.9% | -20.8% | -24.3% | -30.3% | -25.4% | -31.2% |
| 1-pass WP LD | N/A | N/A | -22.5% | -28.6% | -23.6% | -29.2% |

Table 9. Summary of Y BD-Rate[%] for cross-fade sequences.

|  | RA-HE | RA-LC | LDB-HE | LDB-LC | LDP-HE | LDP-LC |
| --- | --- | --- | --- | --- | --- | --- |
| 1-pass WP | -3.8% | -3.9% | -2.4% | -4.4% | -2.8% | -4.8% |

Table 10 shows the experimental results of WP method with 1-pass encoding algorithm for common test sequences. It means that this algorithm does not have a big impact on coding efficiency for the regular test sequences.

Table 10. Summary of Y BD-Rate[%] for common test sequences (no fade).

|  | RA-HE | RA-LC | LDB-HE | LDB-LC | LDP-HE | LDP-LC |
| --- | --- | --- | --- | --- | --- | --- |
| 1-pass WP | 0.9% | 1.2% | 0.9% | 0.9% | 0.6% | 0.6% |

# Conclusion

In this contribution, Weighted Prediction method with 1-pass encoding scheme is proposed. Experimental results show that proposed method can achieve the average BD-rate gain of 17% to 31% (up to 60%) for all cases with a little encoder complexity. WP method is effective for fading sequences using video splicing applications and by introducing it to HEVC test model, it can get a large coding gain compared to current HM test model for such sequences.

It’s suggested that the WP method is introduced in the HEVC test model (HM).

# References

[1]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-E041, Geneva, March 2011.

[2]	A. Fujibayashi, S. Kanumuri and F. Bossen, ”TE12: Performance of Partition Based Illumination Compensation (PBIC),” Joint Collaborative Team on Video Coding, JCTVC-C041, Guangzhou, October 2010.

[3]	H. Aoki and Y. Miyamoto, “An H.264 weighted prediction parameter estimation method for fade effects in video scenes,” Proc. ICIP2008, pp. 2112-2115, October 2008.

[4]	Y. Kikuchi, C. Takeshi, " Interpolation coefficient adaptation in multi-frame interpolative prediction," Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6), JVT-C103, Fairfax, March 2002.

[5]	K. Ugur, J. Lainema, A. Hallapuro, ” High precision bi-directional averaging,” Joint Collaborative Team on Video Coding, JCTVC-D321, Daegu, January 2011.

[6]	ftp://ftp-renn.thmulti.com/

[7]	Jill Boyce, “Changes to Adaptive Reference Picture Weighting,” JVT Document, JVT-E060, Geneva, October 2002.

[8]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-F265, Torino, July 2011.

[9]	F. Bossen ," Common test conditions and software reference configurations ", Joint Collaborative Team on Video Coding, JCTVC-E600, Geneva, March 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).