| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F328 |
| --- | --- |

| Title: | CE6.d: Cross check report of Sharp's proposal (JCTVC- F605) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results described in JCTVC-E315 proposed by SHARP Corporation on Core Experiment 6 subset d of intra prediction improvement. The software based on the HM software version 3.0 contains three kinds of configurations of parallel intra prediction with 2X parallelism for 4x4 blocks. This contribution shows Test 2 case.

The verification task has been done successfully and the results perfectly match those proposed in JCTVC- F605.

# Cross-checking

The software based on HM version 3.0 and the experimental results were provided on June 24 2011 by SHARP. There are three kinds of configurations of parallel intra prediction with 2X parallelism for 4x4 blocks.

- Stripe: Prediction block (0,1) of z-scan in parallel, then block (2,3) in parallel. Block (0,1) use 8x8 block neighbors, and (2,3) use 8x4 block neighbors including the reconstructed (0,1)

- Checkerboard: Predict block (1,2) of z-scan in parallel, then block (0,3) in parallel. Block (1,2) are predicted using 8x8 block neighbors, and block (0,3) use 4x4 up and left block neighbors. Bipred scheme is not used any more.

- No 4x4 pred: All 4x4 blocks have the same mode and they are predicted using 8x8 block neighbors. But each 4x4 is 4x4 transformed.

In this contribution, cross-checking results of Test (2) are reported. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only coding structures. We reviewed the source code and confirmed that the technique integrated in the software matches those described in JCTVC-F605.

Table 1: Cross-checking results for I slice only coding structure

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9 | 0.3 | 0.3 | 0.9 | 0.5 | 0.6 |
| Class B | 1.0 | 0.5 | 0.5 | 1.2 | 0.8 | 0.8 |
| Class C | 1.9 | 1.4 | 1.4 | 2.2 | 1.5 | 1.4 |
| Class D | 2.2 | 1.8 | 1.9 | 2.6 | 2.0 | 1.9 |
| Class E | 1.7 | 0.7 | 1.0 | 2.1 | 0.7 | 0.9 |
| Overall | 1.5 | 0.9 | 1.0 | 1.8 | 1.1 | 1.1 |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 5th Meeting Geneva, JCTVC-E700, March 2011.

[2] 	"Core Experiment 6: Intra Prediction Improvement ", ITU-T and ISO/IEC JCT-VC 5th Meeting, Geneva, JCTVC-R706, March 2011.