| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F329 |
| --- | --- |

| Title: | CE7: Cross check report of Samsung’s proposal (Tool6 : JCTVC-F283) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Jun Yamaguchi 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of Tool 6 described in JCTVC-F283 proposed by Samsung, reflecting discussions on the JCT-VC reflector on Core Experiment 7 of additional transforms. The verification task has been done successfully and the results perfectly match those provided by proponent. No mismatch between encoder and decoder is observed.

# Cross-checking

JCTVC-E125 was adopted in HM version 3.0 (HM3.0). A modified version of the 4x4 DST from JCTVC-E125 is tested in this contribution. The DST is scaled to match the scaling of the JCTVC-E243 DCT, and some coefficients are modified to reduce the number of arithmetic operations. The DST in current HM3.0 is integrated in the following 4x4 DST matrix;

DST4x4=2955748474740−7484−29−745555−8474−29,

In order to reduce the number of multiplications, the following equation in the modified 4x4 DST matrix of JCTVC-F283 is used;

DST4x4=2856748474740−7484−28−745656−8474−28.

The software based on HM3.0 and the experimental results were provided on May 4 2011 by Samsung. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only and random access coding structures. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by Samsung. Table 1 and Table 2 show the cross-checking results. We reviewed the source code and confirmed that the technique integrated in the software matches JCTVC-F283 contributed by Samsung completely.

Table 1: Cross-checking results for I slice only coding structure

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Cross-checking results for random access coding structure

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | -0.4 | 0.1 | -0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.2 | 0.0 | -0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Furthermore, in order to see if there is unusual behavior, additional experiments based on CE10 condition [3] are checked. In this condition, BD-Rate results for the following QP sets: 1, 5, 9, 13 (low QPs) and 36, 42, 47, 51 (high QPs) on 2 sec for I-slice only coding structure are reported. Table 3 and 4 show the results of this experiment. These results show the BD-rate gain of the modified 4x4 DST matrix does not have big impact compared with current 4x4 DST in HM3.0.

Table 3: Cross-checking results on low QPs for I slice only coding structure

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.3 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class D | 0.5 | 0.1 | 0.1 | 0.2 | 0.0 | 0.0 |
| Class E | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 |
| Overall | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4: Cross-checking results on high QPs for I slice only coding structure

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class C | -0.1 | 0.2 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -4.3 | 0.2 | 0.0 | 0.3 | -0.2 |
| Class E | 0.0 | -0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | -0.8 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 5th Meeting Geneva, JCTVC-E700, March 2011.

[2] 	"Core Experiment 7: Additional Transforms ", ITU-T and ISO/IEC JCT-VC 5th Meeting, Geneva, JCTVC-E707, March 2011.

[3] 	"Core Experiment 10: Core Transforms ", ITU-T and ISO/IEC JCT-VC 5th Meeting, Geneva, JCTVC-E710, March 2011.