| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F331 |
| --- | --- |

| Title: | Cross check report of Technicolor's proposal of JCTVC-F265 for Weighted Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results of Technicolor’s proposal for Explicit WP on AHG 18 Weighted Prediction. The software based on of HM software version 3.0 is provided by Technicolor. AHG 18 provides two sets of fading sequences corresponding to black-fade and white fade, which made by the fading tool also provided by AHG18. In this contribution, the experimental results of Explicit WP derived using Technicolor’s WP estimation scheme are reported for these fading sequences.

The verification task has been done successfully and the results perfectly match those reported in JCTVC-F265.

# Cross-checking

At last JCTVC Geneva meeting, AHG to study on the WP method [1] or the other illumination compensation (IC) method [2] was established. The mandates of this AHG are following;

- Develop software for support of WP in HEVC in a same way as in AVC for study toward support of WP capability.

- Discuss and develop analysis software for estimating weighting parameters for explicit WP.

- Discuss and develop common conditions to test WP and illumination compensation (IC) tools in general, including test sequences and encoding configurations.

- Study technical proposals related to WP and IC in general, evaluate their gains, complexity, and interactions with existing HM design tools.

In this contribution, the experimental results of Technicolor’s WP estimation scheme are cross-checked and reported. The software based on HM version 3.0 and the experimental results were provided on June 28 2011 by Technicolor. We reviewed the source code. This WP parameter estimation method is 1-pass encoding scheme using pre-analysis in slice level. This scheme performs a histogram matching between the current slice in original picture and the corresponding reference slice. The corresponding reference slice means the area indicated by global motion vectors, which means motion vector calculated by the average of co-located motion vectors in current slice. The WP parameter is derived by solving least-squares method based on a value of histogram. To determine that the explicit WP is used in current slice or not, the SSE value is calculated using the derived WP parameter and default WP parameter. Finally, the current slice is encoded with/without WP method.

The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under HM common test conditions [3], which define the random access, low delay B and low delay P coding structures on both high efficiency and low complexity conditions completely. Table 1 shows cross-checking results for black-fade sequences, and Table 2 shows cross-checking results for white-fade sequences. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by Technicolor.

Table 1: Cross-checking results for black-fade sequences

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -16.8 | -14.0 | -10.9 | -18.4 | -13.5 | -11.6 |
| Class B | -17.5 | -9.6 | -5.4 | -19.5 | -10.1 | -9.6 |
| Class C | -13.7 | -8.8 | -10.9 | -16.2 | -11.7 | -11.5 |
| Class D | -13.5 | -8.0 | -10.5 | -16.2 | -10.5 | -13.1 |
| Class E |  |  |  |  |  |  |
| Overall | -15.5 | -10.1 | -9.2 | -17.7 | -11.4 | -11.3 |
| Enc Time[%] | 123% |  |  | 128% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -21.7 | -5.7 | -4.5 | -30.2 | -26.5 | -21.8 |
| Class C | -11.0 | -3.9 | -4.9 | -16.8 | -24.1 | -19.6 |
| Class D | -11.2 | -1.1 | -7.7 | -16.0 | -16.9 | -21.3 |
| Class E | -37.1 | -10.9 | -10.4 | -46.6 | -31.5 | -43.5 |
| Overall | -19.3 | -5.1 | -6.5 | -26.4 | -24.4 | -25.2 |
| Enc Time[%] | 120% |  |  | 127% |  |  |
| Dec Time[%] | 92% |  |  | 89% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -20.9 | -2.2 | 5.0 | -28.0 | -23.0 | -17.9 |
| Class C | -11.4 | -3.3 | -4.2 | -16.9 | -22.4 | -17.9 |
| Class D | -13.3 | -2.8 | -9.7 | -17.9 | -19.5 | -22.3 |
| Class E | -36.0 | -7.3 | -3.5 | -45.0 | -31.8 | -40.8 |
| Overall | -19.4 | -3.6 | -2.6 | -25.9 | -23.6 | -23.3 |
| Enc Time[%] | 104% |  |  | 110% |  |  |
| Dec Time[%] | 90% |  |  | 84% |  |  |

Table 1: Cross-checking results for white-fade sequences

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -20.7 | -18.2 | -16.7 | -22.1 | -16.1 | -14.5 |
| Class B | -21.0 | -13.8 | -9.9 | -23.5 | -16.0 | -14.9 |
| Class C | -15.0 | -11.3 | -13.5 | -17.8 | -14.4 | -14.1 |
| Class D | -14.4 | -8.4 | -11.5 | -17.3 | -12.1 | -15.7 |
| Class E |  |  |  |  |  |  |
| Overall | -17.9 | -13.0 | -12.7 | -20.4 | -14.7 | -14.8 |
| Enc Time[%] | 119% |  |  | 124% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -24.4 | -8.9 | -4.7 | -34.9 | -30.2 | -26.9 |
| Class C | -12.3 | -5.4 | -6.8 | -18.8 | -26.7 | -21.3 |
| Class D | -10.6 | -3.7 | -8.4 | -17.2 | -20.6 | -22.9 |
| Class E | -37.7 | -13.3 | -7.2 | -49.0 | -34.1 | -45.1 |
| Overall | -20.4 | -7.5 | -6.6 | -29.1 | -27.6 | -27.9 |
| Enc Time[%] | 116% |  |  | 122% |  |  |
| Dec Time[%] | 91% |  |  | 88% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -23.3 | -3.8 | 4.8 | -32.9 | -26.5 | -21.9 |
| Class C | -12.4 | -4.2 | -5.8 | -18.9 | -24.5 | -19.9 |
| Class D | -13.1 | -2.7 | -8.7 | -19.1 | -21.8 | -24.6 |
| Class E | -36.4 | -8.7 | -6.7 | -47.5 | -33.7 | -41.7 |
| Overall | -20.5 | -4.5 | -3.4 | -28.7 | -26.2 | -25.8 |
| Enc Time[%] | 100% |  |  | 105% |  |  |
| Dec Time[%] | 89% |  |  | 84% |  |  |

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1]	P. Bordes, "Weighted Prediction," Joint Collaborative Team on Video Coding, JCTVC-E041, Geneva, March 2011.

[2]	A. Fujibayashi, S. Kanumuri and F. Bossen, ”TE12: Performance of Partition Based Illumination Compensation (PBIC),” Joint Collaborative Team on Video Coding, JCTVC-C041, Guangzhou, October 2010.

[3]	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 5th Meeting Geneva, JCTVC-E700, March 2011.