| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F332 |
| --- | --- |

| Title: | CE4 Subtest 2: QP prediction from spatially neighboring CUs (test 2.3.b, 2.3.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution presents the description and the results of spatially neighbouring CU QP prediction methods (test 2.3.b, test 2.3.c) for CE4 subtest 2. Test 2.3.b results in 0.4%, 0.4%, 0.7% reduction in delta QP coding bits for AI_HE, RA_HE and LB_HE configurations. Test 2.3.c. results in 0.4%, 1.1%, 1.8% reduction in delta QP coding bits for AI_HE, RA_HE and LB_HE configurations. BD-rate savings on the average is less than 0.1%.

# Introduction

Perceptual quantization or adaptive quantization targets visual quality improvement [3] [4] [5]. Generally it performs finer quantization on flat areas and coarser quantization on busy areas. Spatial correlations among prediction blocks’ QPs are reflected to the QP prediction.

In this proposal QP prediction from neighboring CUs is used. Two methods for QP prediction are described.

# Algorithm description

## CE4 Subtest 2.3.b

We propose to use left top most neighbor (L) CU’s QP as the predictor for coding of the current CU’s QP. If the left neighbor is not available then the top left most (T) neighboring CU’s QP is used as the predictor, as show in Figure 1.

Figure 1: CU level QP assignment

Hence delta QP calculations becomes

dQPC0 = QPC0 –  QPA5

dQPC1 = QPC1 –  QPC0

dQPC5 = QPC5 –  QPA12

dQPA0 = QPA0 –  QPB8

…………

If left CU is not available then the top CU’s QP is used as a predictor. If the neighboring predictor CU is a skipped CU, then the QP value of the predictor is the QP value of its predictor QP. Basically skip CU’s QP is assumed to of its predictor QP. Top QP predictor gets used at the beginning of each LCU row.

## CE4 Subtest 2.3.c

We propose to use immediate neighbors within a quadtree split CU block for QP prediction as shown in Figure 2.

Figure 2: Quadtree QP prediction direction

Arrows in Figure 2 shows the prediction direction for QPs. In this case dQPs are computed as follows

dQP1 = QP1 – QP0

dQP2 = QP2 – QP0

dQP3= QP3 – QP2

dQP0 computation depends on where this quadtree lies in the hierarchical quadtree structure. It can be based on the QP of the top or left neighboring CU depending on its position.

Across quadtrees, higher level quadtree’s prediction pattern is followed. The left and top neighbor definition follows that of method 1.

Across LCUs rules set in method 1 above are followed.

Figure 2 illustrates the QP prediction at higher CU levels and across quadtrees.

In this case delta QPs are computed as follows

dQPC0 = QPC0 –  QPA5

dQPC1 = QPC1 –  QPC0

dQPC5 = QPC5 –  QPC0

dQPC6 = QPC6 –  QPC5

…………

# Experiments

## Setup

The proposed QP prediction algorithm has been implemented onto the software for CE4 anchor software Error: Reference source not found. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest 2 test conditions that are based on the JCTVC common test condition defined in [1]. The simulation results are from a 64-bit Linux cluster.

## Results

Table 1 shows summary of experimental results for CE4 subtest 2.3.b. This method provides slight improvement over the CE4 Subtest 2 anchor. Table 2Error: Reference source not found summarizes the experimental results for CE4 subtest 2.3.c.  This method performs on average better than subtest 2.3.b.

Table 1: CE4 subtest 2.3.b results

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | -0.1 | -0.2% | 0.0 | 0.0 | 0.0 | -0.2% |
| Class B | 0.0 | 0.0 | 0.0 | -0.3% | 0.0 | 0.0 | 0.0 | -0.3% |
| Class C | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.0 | 0.0 | -0.4% |
| Class D | 0.0 | 0.0 | 0.0 | -0.7% | 0.0 | 0.0 | 0.0 | -0.7% |
| Class E | 0.0 | 0.0 | 0.0 | -0.5% | 0.0 | 0.0 | 0.0 | -0.5% |
| All | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.0 | 0.0 | -0.4% |
| Enc Time[%] | 103% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | 0.0 | -0.4% | 0.0 | 0.1 | 0.2 | -0.4% |
| Class B | 0.0 | 0.1 | 0.0 | -0.3% | 0.0 | 0.0 | 0.0 | -0.5% |
| Class C | 0.0 | 0.0 | 0.0 | -0.5% | 0.0 | 0.0 | 0.1 | -0.7% |
| Class D | 0.0 | 0.0 | -0.1 | -0.5% | -0.1 | 0.0 | 0.0 | -0.7% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.0 | 0.0 | -0.6% |
| Enc Time[%] | 103% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | -0.4% | 0.0 | 0.1 | -0.2 | -0.5% |
| Class C | -0.1 | 0.1 | 0.0 | -0.7% | -0.1 | 0.1 | -0.2 | -0.8% |
| Class D | 0.0 | 0.0 | 0.2 | -0.5% | -0.1 | 0.0 | -0.4 | -0.8% |
| Class E | -0.1 | -0.1 | -0.1 | -1.4% | -0.1 | -0.4 | -0.1 | -1.5% |
| All | 0.0 | 0.0 | 0.0 | -0.7% | -0.1 | 0.0 | -0.2 | -0.8% |
| Enc Time[%] | 102% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |

Table 2: CE4 subtest 2.3.c results.

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.0 | 0.8% | 0.0 | 0.0 | 0.0 | 1.0% |
| Class B | 0.0 | 0.0 | 0.0 | 0.4% | 0.0 | 0.0 | 0.0 | 0.0% |
| Class C | -0.1 | -0.1 | -0.1 | -1.5% | -0.1 | -0.1 | -0.1 | -1.7% |
| Class D | -0.1 | -0.1 | 0.0 | -1.6% | -0.1 | -0.1 | -0.1 | -1.9% |
| Class E | 0.0 | 0.0 | 0.0 | -0.5% | 0.0 | 0.0 | 0.0 | -0.7% |
| All | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.0 | 0.0 | -0.6% |
| Enc Time[%] | 101% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.2 | 0.1 | -0.3% | 0.0 | 0.0 | 0.1 | 0.1% |
| Class B | 0.0 | -0.2 | 0.0 | -0.6% | 0.0 | -0.1 | 0.0 | -0.9% |
| Class C | 0.0 | 0.0 | 0.0 | -2.0% | -0.1 | -0.1 | -0.1 | -2.5% |
| Class D | 0.0 | 0.0 | 0.3 | -1.5% | -0.1 | 0.0 | -0.1 | -2.0% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | -1.1% | 0.0 | 0.0 | 0.0 | -1.3% |
| Enc Time[%] | 100% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | -1.0% | -0.1 | 0.0 | 0.0 | -1.0% |
| Class C | -0.1 | 0.0 | 0.0 | -2.3% | -0.2 | -0.1 | -0.1 | -2.7% |
| Class D | -0.1 | -0.3 | -0.2 | -1.7% | -0.2 | -0.3 | -0.1 | -2.2% |
| Class E | -0.1 | -1.2 | 0.6 | -2.7% | -0.2 | -0.4 | 0.0 | -2.8% |
| All | -0.1 | -0.3 | 0.0 | -1.8% | -0.1 | -0.2 | -0.1 | -2.0% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 99% |  |  |  |

# Conclusions

This contribution has presented the detailed description and the results of spatially neighbouring CU QP prediction methods (test 2.3.b, test 2.3.c) for CE4 subtest 2. Test 2.3.b results in 0.4%, 0.4%, 0.7% reduction in delta QP coding bits for AI_HE, RA_HE and LB_HE configurations. Test 2.3.c. results in 0.4%, 1.1%, 1.8% reduction in delta QP coding bits for AI_HE, RA_HE and LB_HE configurations. BD-rate savings on the average is less than 0.1%.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, March 2011.

- K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- M. Coban, et al., “CU-Level QP Prediction,” JCT-VC document JCTVC-E391, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- M. Kobayashi and M. Shima, “Sub-LCU level delta QP signaling,” JCT-VC document JCTVC-E198, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- K. Sato, et al., “On LBS and Quantization,” JCT-VC document JCTVC-D308, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

# Subtest 2.3.b

The modified or added portions are highlighted with yellow color.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY for a quantization group of coding units where the quantization group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantization group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantization group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantization group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order above quantization group  in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.