| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22July, 2011 | Document: JCTVC-F335 |
| --- | --- |

| Title: | Tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arild Fuldseth Michael Horowitz Shilin Xu Andrew Segall Minhua Zhou | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com +1 778.331.3432 x323 michael@ebriskvideo.com +1 360 817 8487 asegall@sharplabs.com +1 214.480.3816 zhou@ti.com |
| Source: | Cisco Systems, eBrisk Video, Sharp, and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution further refines a coding technique called Tiles proposed in JCTVC-E408 that partitions a picture into rectangular segments. Tiles comprise vertical and horizontal boundaries that partition a picture into columns and rows respectively. Column and row boundaries break prediction mechanisms (e.g., intra prediction and motion vector prediction) in the same way as slice boundaries unless indicated otherwise. Intersecting column and row boundaries delineate rectangular regions called tiles each containing an integer number of LCUs. LCUs are processed in raster scan order within tiles and tiles are processed in raster scan within a picture. Results from three experiments are presented comparing coding efficiency for Tiles against an identically configured encoder using slices with LCUs processed in raster scan-order within a picture. Of particular interest, results related to high-level parallel processing, show that Tiles provide 1.7%, 3.1%, and 4.8% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor.

# Introduction

Partitioning a picture into slices as part of the encoding process is known to negatively impact coding efficiency particularly when the slices are designed to be independently decodable. Many applications and implementations, however, require the partitioning of a picture. Two important examples, network maximum transmission unit (MTU) size matching and high-level parallel processing, are described in detail in [1] and [2]. A third important example is described in [3]. Tiles proposed in this contribution provide picture-partitioning functionality while minimizing the coding efficiency loss resulting from those partitions.

Tiles include vertical and horizontal boundaries that partition a picture into columns and rows respectively. These boundaries break coding dependencies (e.g., dependencies associated with intra prediction, motion vector prediction and parsing) in the same way as slice boundaries unless otherwise indicated. The rectangular regions resulting from the intersecting column and row boundaries are called tiles (hence the name for the technique as a whole). Each tile contains an integer number of LCUs. LCUs are processed in raster scan order within each tile and the tiles themselves are processed in raster scan order within the picture (see Figures 1 and 2). Slice boundaries are introduced by the encoder and need not be coincident with tile boundaries.  For example, a tile may contain more than one slice and slices may contain more than one tile. When a slice contains LCUs in more than one tile, the tiles containing the LCUs shall be contiguous. Tiles T and T+1 are said to be contiguous if the first LCU in T+1 immediately follows, in transmission order, the last LCU in T. Further, Tiles allow the column and row boundaries to be specified with and without uniform spacing.

This contribution is organized as follows. Section 2 contains a description of the Tiles syntax, semantics, and decoding process. Experiment descriptions and results are in Sections 3 and 4 respectively. The conclusion is contained in Section 5 and references in Section 6.

Figure 1. An example of Tiles partitioning using three columns and three rows.

Figure 2. An example of Tiles partitioning using three columns and zero rows.

# Tiles for the HM

## Tiles related software modifications

To implement Tiles, the following decoder modifications were made to the HM 3.0 code base. Analogous changes were made to the encoder source code.

- modification of the LCU decoding order from raster scan within a picture to raster scan within a tile and raster scan of tiles within a picture,

- modification of CU availability to recognize that a CU in a different tile is not available (unless indicated otherwise),

- addition of the syntax and semantics to decode column widths, row heights, and compute column and row boundary locations including an option to indicate uniform spacing, and the

- addition of  a syntax element indicating  whether row and tile boundaries break prediction mechanisms or not.

## Tiles related definitions

tile: An integer number of treeblocks co-occurring in one column and one row, ordered consecutively in the raster scan within the tile. For the primary coded picture, the division of each picture into tiles is a partitioning. Tiles are ordered consecutively in the raster scan within the picture. Although a slice contains treeblocks that are consecutive in the raster scan within a tile, these treeblocks are not necessarily consecutive in the raster scan within the picture.

column: An integer number of treeblocks. Columns are delineated from one another by vertical boundaries that extend from the top boundary to the bottom boundary of the picture and are ordered consecutively from left to right in the picture. For the primary coded picture, the division of each picture into columns is a partitioning.

row: An integer number of treeblocks. Rows are delineated from one another by horizontal boundaries that extend from the left boundary to the right boundary of the picture and are ordered consecutively from top to bottom in the picture. For the primary coded picture, the division of each picture into rows is a partitioning.

treeblock scan order: A specific sequential ordering of treeblocks starting from the top-left treeblock in the picture. The treeblock scan order runs in the raster scan within each tile beginning with the top-left tile and continuing in the tile raster scan of the picture.

slice: An integer number of treeblocks ordered consecutively in treeblock scan order. For the primary coded picture, the division of each picture into slices is a partitioning. The treeblock addresses are derived from the first treeblock address in a slice (as represented in the slice header).

treeblock address: A treeblock address is the index of a treeblock in treeblock scan order of the picture starting with zero for the top-left treeblock in a picture.

## Tiles related syntax

Tiles related syntax is located in both the Sequence and Picture Parameter Sets as described in the following two sub-sections. When present, parameters in the Picture Parameter Set take precedence over those in the Sequence Parameter Set.

### Sequence Parameter Set

Syntax added to the Sequence Parameter Set of the reference software to support Tiles is highlighted in yellow in Table 1.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| bit_depth_luma_increment | ue(v) |
| bit_depth_chroma_increment | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i<num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| interpolation_filter_flag | u(1) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if (num_tile_columns_minus1 != 0 ¦¦ num_tile_rows_minus1 != 0) { |  |
| tile_boundary_independence_idc | u(1) |
| uniform_spacing_idc | u(1) |
| if (uniform_spacing_idc != 1) { |  |
| for (i=0; i<num_tile_columns_minus1 ; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i <num_tile_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 1. Tiles related syntax in the sequence parameter set highlighted in yellow.

### Picture Parameter Set

Syntax added to the Picture Parameter Set of the reference software to support Tiles is highlighted in yellow in Table 2.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26/* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| tile_info_present_flag | u(1) |
| if (tile_info_present_flag == 1) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if (num_tile_columns_minus1 != 0 ¦¦ num_tile_rows_minus1 != 0) { |  |
| tile_boundary_independence_idc | u(1) |
| uniform_spacing_idc | u(1) |
| if (uniform_spacing_idc != 1) { |  |
| for (i=0; i<num_tile_columns_minus1 ; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i <num_tile_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2. Tiles related syntax in the picture parameter set highlighted in yellow.

## Tiles related semantics

The semantics associated with syntax elements introduced in section 2.3 are presented in this sub-section.

tile_info_present_flag equal to 1 specifies that the syntax elements tile_boundary_independence_idc, num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_idc, column_width[i], and row_height[i] may be present in the picture parameter set.  tile_info_present_flag equal to 0 specifies that syntax elements tile_boundary_independence_idc, num_tile_columns_minus1, num_tile_rows_minus1, uniform_spacing_idc, column_width[i], and row_height[i] are not present in the picture parameter set.

tile_boundary_independence_idc equal to 1 indicates that all neighbouring treeblocks not contained within the same tile as the treeblock being decoded are marked as not available. Otherwise, tile_boundary_independence_idc equal to 0 indicates that availability of treeblocks is not affected by tile boundaries. Further, tile_boundary_independence_idc equal to 1 indicates that the initialisation process for the entropy decoder is invoked when decoding the first treeblock in a tile.

num_tile_columns_minus1  plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

uniform_spacing_idc equal to 1 indicates that column boundaries and likewise row boundaries are distributed uniformly across the picture. The value of the variable columnWidth[i], specifying the width of the i-th column in units of treeblocks, is derived as

columnWidth[i] = ((i+1) * FrameWidthInCU) / (num_tile_columns_minus1+1) – (i * FrameWidthInCU) / (num_tile_columns_minus1+1).

The value of the variable rowHeight[i], specifying the height of the i-th row in units of treeblocks, is derived as

rowHeight[i]  = ((i+1) * FrameHeightInCU) / (num_tile_rows_minus1+1) – (i * FrameHeightInCU) / (num_tile_rows_minus1+1).

uniform_spacing_idc equal to 0 indicates that the value of the variable columnWidth[i], specifying the width of the i-th column in units of treeblocks, equals column_width[i] and the value of the variable rowHeight[i], specifying the height of the i-th row in units of treeblocks, equals row_height[i].

The value of the variable columnBoundary[i], specifying the location of the left column boundary of the i-th column in units of treeblocks, is derived as follows.

columnBoundary[0] = 0;

if (num_tile_columns_minus1 > 0) {

columnBoundary[1] = columnWidth[0];

for (i=1; i<num_tile_columns_minus1 ; i++) {

columnBoundary[i+1] = columnBoundary[i] + columnWidth[i];

}

}

The value of the variable rowBoundary[i], specifying the location of the top row boundary of the i-th row in units of treeblocks, is derived as follows.

rowBoundary[0] = 0;

if (num_tile_rows_minus1> 0) {

rowBoundary[1] = rowHeight[0];

for (i=1; i<num_tile_rows_minus1; i++) {

rowBoundary[i+1] = rowBoundary[i] + rowHeight[i];

}

}

column_width[i] specifies the width of the i-th column in units of treeblocks. The value of column_width[i] shall be limited such that columnBoundary[i+1] is less than or equal to FrameWidthInCU for i in the range of 0 to num_tile_columns_minus1 – 1, inclusive, and num_tile_columns_minus1 greater than 0.

row_height[i] specifies the height of the i-th row in units of treeblocks. The value of row_height[i] shall be limited such that rowBoundary [i+1] is less than or equal to FrameHeightInCU for i in the range of 0 to num_tile_rows_minus1 – 1, inclusive, and num_tile_rows_minus1 greater than 0.

## Tiles related decoding process

Tiles related decoding processes are described in this section.

### Tile initial QP

The initial QPY quantisation parameter for the tile is derived as

QPY = 26 + pic_init_qp_minus26 + slice_qp_delta.

The values of pic_init_qp_minus26 and slice_qp_delta equal those transmitted in the slice header of the slice containing the first treeblock of the tile. The value of QPY is to be used by all coding units in the tile until modified by the value of cu_qp_delta or until a slice boundary is encountered.

### Entropy coding reset

When tile_boundary_independence_idc is equal to 1, the initialisation process for the entropy decoder is invoked when decoding the first treeblock in a tile. Following the decoding of the last treeblock in the previous tile, the following ordered steps are performed:

First, the entropy decoding process is terminated.

Second, the decoder advances to the first bit of the next byte aligned byte.

After the order steps, the decoder initialises the entropy decoder.  When entropy_coding_mode is equal to 1, the decoder initialisation process corresponds to 9.3.1.1 and 9.3.1.2 with cabac_init_idc set equal to the value transmitted in the slice header of the slice containing the first treeblock of the tile.  Otherwise, the decoder initialisation process described in subclause 9.2.2 is invoked.

# Description of experiments

Experiments exploring the effects of tiles on coding efficiency for MTU-size matching, high-level parallel processing and on-chip memory constrained motion estimation applications are described in this section.

## Common conditions

The reference software HM 3.0 version 823 was used to produce results for the reference in the experiment.  In addition, the encoder and decoder source code from the reference codebase was modified to introduce Tiles functionality. The common conditions and encoding configurations described in [4] were used except where indicated otherwise.

## Experiment 1: MTU size matching

In this experiment, MTU size matching is considered. The HM 3.0 version 823 [5] using the common conditions served as reference. The reference was compared against HM 3.0 version 823 to which Tiles functionality was added [6], also using the common conditions.  In both cases, a maximum number of bytes per slice were specified in the encoder configuration file to 1500 to be consistent with the MTU size of an Ethernet v2 network. For Tiles, column boundaries and no row boundaries were introduced (i.e., num_tile_rows_minus1 = 0).  The encoder introduced slices as required to meet the 1500 byte per slice constraint. The column widths were selected according to simulation type (e.g., Intra), sequence class, and QP value to achieve approximately square slice shapes.  The column widths used for the Tiles test case are shown in Table 3.

|  | Intra | Intra LoCo | Random Access | Random Access Loco | Low Delay | Low Delay LoCo | Low Delay P | Low Delay P Loco |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A |  |  |  |  |  |  |  |  |
| QP 22 | 40 | 40 | 5 | 4 | --- | --- | --- | --- |
| QP 27 | 40 | 40 | 7 | 6 | --- | --- | --- | --- |
| QP 32 | 4 | 4 | 10 | 8 | --- | --- | --- | --- |
| QP 37 | 4 | 4 | 10 | 10 | --- | --- | --- | --- |
| Class B |  |  |  |  |  |  |  |  |
| QP 22 | 30 | 30 | 6 | 6 | 4 | 6 | 4 | 6 |
| QP 27 | 5 | 5 | 6 | 6 | 8 | 8 | 8 | 8 |
| QP 32 | 6 | 6 | 8 | 8 | 10 | 10 | 10 | 10 |
| QP 37 | 8 | 8 | 10 | 10 | 15 | 15 | 15 | 15 |
| Class C |  |  |  |  |  |  |  |  |
| QP 22 | 13 | 13 | 4 | 3 | 3 | 3 | 3 | 3 |
| QP 27 | 13 | 13 | 7 | 4 | 4 | 4 | 4 | 4 |
| QP 32 | 3 | 3 | 7 | 7 | 7 | 7 | 7 | 7 |
| QP 37 | 4 | 4 | 13 | 13 | 13 | 13 | 13 | 13 |
| Class D |  |  |  |  |  |  |  |  |
| QP 22 | 7 | 7 | 4 | 4 | 4 | 4 | 4 | 4 |
| QP 27 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 |
| QP 32 | 2 | 2 | 7 | 7 | 7 | 7 | 7 | 7 |
| QP 37 | 4 | 4 | 7 | 7 | 7 | 7 | 7 | 7 |
| Class E |  |  |  |  |  |  |  |  |
| QP 22 | 4 | 4 | --- | --- | 7 | 7 | 7 | 7 |
| QP 27 | 5 | 5 | --- | --- | 20 | 20 | 20 | 20 |
| QP 32 | 5 | 5 | --- | --- | 20 | 20 | 20 | 20 |
| QP 37 | 7 | 7 | --- | --- | 20 | 20 | 20 | 20 |

Table 3. Column widths in units of 64x64 LCUs used for experiment 1.

## Experiment 2: High-level parallel processing

In this experiment, high-level parallel processing is considered without MTU size matching. The HM 3.0 version 823 software using the common conditions served as reference. The reference was compared against HM 3.0 version 823 to which Tiles functionality was added, also using the common conditions.  In the Tiles case, a fixed number of LCUs were configured for each tile (except where picture boundaries prevented it) and one slice per picture was used.  The tile sizes for each sequence class were chosen so as to contain roughly the same number of pixels as one picture in a Class D sequence. In the reference case, LCUs were assigned to each slice in raster scan order within the picture and with an integer number of LCU rows per slice. The slice and tile partitioning for each class is shown in Table 4 for the reference case and the test case respectively.

|  | Reference |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Number of slices | Slice sizes (in units of LCUs) | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
|  |  |  | horizontally | vertically | horizontally | vertically |
| A | 25 | 40 | 5 | 5 | 8 | 5 |
| B | 17 | 30 | 6 | 3 | 5 | 6 |
| C | 4 | 26 | 2 | 2 | 7 | 4 |
| D | 1 | 28 | 1 | 1 | 7 | 4 |
| E | 12 | 20 | 4 | 3 | 5 | 4 |

Table 4. Slice and tile partitioning for experiment 2.

## Experiment 3: Motion estimation with an on-chip memory constraint

Experiment 3 explores the effect of using uniformly spaced tiles to improve motion estimation under the constraint of limited on-chip memory size. Specifically, partitioning a picture into uniformly-sized tiles (i.e., uniform_spacing_idc equal to 1) and processing LCUs in raster-scan order within tiles allows for an increased motion estimation search range compared with processing LCUs in raster scan order within a picture when the amount of memory used for motion search is constrained. The HM 3.0 version 823 served as reference. The reference was compared against HM 3.0 version 823 to which Tiles functionality was added.  In both cases, the software was modified to simulate an on-chip memory constraint. In the experiment, a picture was divided into N x 1 tiles where N differs among the sequence classes as shown in Figure 3.  In both the reference and the test cases, the memory size for motion search was constrained to be equal, but the search ranges differ as a result of the difference in LCU scan order. One slice per picture was used in the reference and all test cases.

Figure 3. Growing search window is applied to each tile. SRx and SRy represent the search range relative to the current LCU row of tiles.

In motion estimation with uniformly spaced tiles, a growing window search is assumed, motion vector candidates are eliminated for merge, skip mode, and for the initial motion vector predictor selection of regular motion search when they are outside the growing search window (see Figure 3). The refinement search range for regular motion search is also adjusted on the fly for each PU based on the growing search window size and the PU location.

### Test sequences

Two sets of test sequences were selected, one with high vertical motion, the other with high horizontal motion. The test sequences are described in Table 5. The test sequences in the common test conditions were not used for this experiment because they lack a high-degree of motion.

| Sequence name | Picture resolution | Frame-rate | Number of frames | Motion characteristics |
| --- | --- | --- | --- | --- |
| FV_seq1 | 1920 x 816 | 24 | 60 | High vertical |
| FV_seq2 | 1920 x 816 | 24 | 60 | High vertical |
| FV_seq3 | 1920 x 816 | 24 | 59 | High vertical |
| FH_seq1 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq2 | 1920 x 1080 | 24 | 58 | High horizontal |
| FH_seq3 | 1920 x 1080 | 24 | 60 | High horizontal |
| FH_seq4 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq5 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq6 | 1920 x 1080 | 24 | 30 | High horizontal |

Table 5. Test sequences used in experiment 3.

### Test conditions

Except for the test sequences, the common testing conditions defined in [4] were used. A single slice without restricted search range was used as the anchor. The simulation was carried out for tile number N {1, 2 and 4}. The simulation platform used in this experiment was LSF equipped with Intel Xeon CPU X5570 @ 2.93GHz with a 64 bit Linux operating system.

In the simulation, the on-chip memory budget available for motion estimation was set to 2 MBytes. Assuming four reference pictures in each direction, and the motion estimation performed in parallel for all the reference pictures and both directions (L0/L1), the growing window search ranges used in the simulation are listed in Table 6. The horizontal search range is fixed to 256 samples. Note that the search ranges in Table 6 are different from the refinement search ranges specified in [4]. The search ranges defined in Table 6 specify the search window around the current LCU row in a tile as illustrated in Figure 3.

| Tile number (N) | Horizontal search range (SRx) | Vertical search range (SRy) | Relative ME memory bandwidth |
| --- | --- | --- | --- |
| 1 (anchor) | unlimited | unlimited |  |
| 1 | unlimited | 30 | 1 |
| 2 | 256 | 67 | 1.27 |
| 4 | 256 | 148 | 1.80 |

Table 6. Growing window search ranges and relative ME memory bandwidth requirements used for simulations with tile numbers (N = 1, 2, 4).

# Results

This section contains a summary of the results for the three experiments described in the previous section. Detailed results may be found in the accompanying spreadsheets.

## Experiment 1

The simulation results for MTU size matching are provided in Table 7. The results are similar to those presented in [7] in the context of HM 2.0. Experiments in [8] show that LCU size significantly affects coding efficiency in the context of MTU size matching and that the coding gains due to Tiles increases with decreasing LCU size. Except for the LD simulations, the results were generated using different computers so the time measurements reported by the encoder and decoder cannot be compared.

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.4 | 0.2 | 0.0 | -0.4 | 0.0 |
| Class B | -0.7 | -1.0 | -0.7 | -0.7 | -1.3 | -0.9 |
| Class C | -0.5 | -0.6 | -0.7 | -0.6 | -0.9 | -0.8 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 |
| Class E | -2.3 | -3.0 | -3.2 | -2.5 | -4.8 | -3.9 |
| Overall | -0.7 | -1.0 | -0.8 | -0.7 | -1.3 | -1.0 |
| Enc Time[%] | 72% |  |  | 68% |  |  |
| Dec Time[%] | 58% |  |  | 57% |  |  |

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -1.0 | 0.2 | -0.4 | -0.7 | -0.5 |
| Class B | -0.6 | -1.0 | -0.6 | -0.7 | -1.2 | -0.9 |
| Class C | -0.3 | -0.2 | -0.3 | -0.3 | -0.4 | -0.4 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | -0.3 | -0.6 | -0.2 | -0.4 | -0.6 | -0.5 |
| Enc Time[%] | 84% |  |  | 80% |  |  |
| Dec Time[%] | 69% |  |  | 63% |  |  |

|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.2 | -0.5 | -0.9 | -0.9 |
| Class C | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 | -0.3 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 | 0.0 |
| Overall | -0.1 | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  |

|  | Low delay (P) HE |  |  | Low delay (P) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.8 | -0.7 | -0.5 | -0.8 | -0.8 |
| Class C | -0.3 | -0.2 | -0.5 | -0.3 | -0.2 | -0.4 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | -0.2 | -0.1 | 0.0 | -0.2 | -0.2 |
| Overall | -0.2 | -0.3 | -0.4 | -0.2 | -0.3 | -0.4 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 95% |  |  |

Table 7. MTU size matching results comparing a tile partitioned test case with a slice-partitioned anchor.

## Experiment 2

The simulation results for high-level parallel processing are provided in Table 8. The results, similar to those presented in [7] in the context of HM 2.0, show that tiles provide significant coding gains compared with partitioning a picture with slices containing LCUs in raster-scan order. Tiles provide gains compared with slices having LCUs in the raster scan within the picture because there are fewer boundaries to break prediction mechanisms. On top of this gain, tiles have the added benefit that they have no header and hence no header bit overhead. The reduced bit overhead is particularly significant for the lower bit rate Random Access and Low Delay cases (e.g., Class E sequences).

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.5 | -4.5 | -3.2 | -1.3 | -5.0 | -3.1 |
| Class B | -2.2 | -2.7 | -1.9 | -2.2 | -3.4 | -2.8 |
| Class C | -0.8 | -0.9 | -0.9 | -0.9 | -1.1 | -1.2 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -4.8 | -5.9 | -5.6 | -4.9 | -8.3 | -7.1 |
| Overall | -1.7 | -2.6 | -2.1 | -1.7 | -3.3 | -2.6 |
| Enc Time[%] | 101 % |  |  | 99 % |  |  |
| Dec Time[%] | 104 % |  |  | 102 % |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -4.3 | -8.4 | -6.3 | -4.4 | -8.7 | -6.5 |
| Class B | -5.4 | -6.3 | -5.2 | -5.5 | -6.5 | -5.7 |
| Class C | -2.1 | -2.6 | -2.4 | -2.1 | -2.4 | -2.5 |
| Class D | 0.0 | -0.3 | -0.3 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | -3.1 | -4.5 | -3.7 | -3.1 | -4.5 | -3.8 |
| Enc Time[%] | 101 % |  |  | 101 % |  |  |
| Dec Time[%] | 102 % |  |  | 102 % |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.7 | -5.4 | -5.3 | -4.7 | -5.3 | -4.8 |
| Class C | -1.8 | -2.2 | -2.1 | -1.7 | -2.1 | -2.0 |
| Class D | 0.0 | -0.3 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class E | -15.3 | -16.4 | -16.2 | -15.0 | -15.3 | -15.8 |
| Overall | -4.8 | -5.4 | -5.3 | -4.7 | -5.0 | -5.0 |
| Enc Time[%] | 101 % |  |  | 101 % |  |  |
| Dec Time[%] | 102 % |  |  | 101 % |  |  |

Table 8. BD-rate results for parallel encoding when using tiles (test case) instead of slices (anchor).

## Experiment 3

The simulation results for motion estimation with an on-chip memory size constraint are provided in Tables 9, 10, 11. As shown in Table 9, there is a significant quality loss for Class FV with high vertical motion (15.1% and 15.3% in Random Access and Random Access LoCo respectively, 3.7% in Low Delay and 4.6% in Low Delay LoCo) for single tile coding under the on-chip memory size/search range constraints. The loss can be almost completely mitigated when a picture is divided into four tiles during encoding (see Table 11). Compared to single tile coding with constrained on-chip memory, the proposed Tiles scheme with four uniformly spaced tiles provided an average of 14.6%, 15.0%, and 3.6% and 4.5% Y BD-rate advantage for Random Access, Random Access LoCo, Low Delay, and Low Delay LoCo, respectively.

|  | Random access HE |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 15.1 | 20.8 | 20.6 | 15.3 | 18.3 | 18.4 |
| Class FH | 0.7 | 1.1 | 1.1 | 0.0 | 0.0 | -0.3 |
| All | 5.5 | 7.7 | 7.6 | 5.1 | 6.1 | 5.9 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 108% |  |  | 101% |  |  |

|  | Low delay HE |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 3.7 | 4.4 | 3.2 | 4.6 | 2.3 | 5.1 |
| Class FH | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| All | 1.2 | 1.5 | 1.1 | 1.6 | 0.9 | 1.8 |
| Enc Time[%] | 96% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 112% |  |  |

Table 9. Results for one tile (N = 1) vs. a single slice, no restricted search range anchor.

|  | Random access HE |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 4.6 | 6.9 | 7.0 | 4.2 | 5.5 | 5.3 |
| Class FH | 0.2 | 0.2 | 0.7 | -0.3 | -0.1 | -0.2 |
| All | 1.7 | 2.5 | 2.8 | 1.2 | 1.7 | 1.6 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |

|  | Low delay HE |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.3 | -0.4 | 1.7 | 0.5 | -1.5 | 1.5 |
| Class FH | 0.0 | 0.2 | 0.4 | 0.0 | 0.3 | -0.5 |
| All | 0.1 | 0.0 | 0.8 | 0.1 | -0.3 | 0.2 |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 10. Results for two tiles (N = 2) vs. a single slice, no restricted search range anchor.

|  | Random access HE |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.5 | 0.9 | 0.9 | 0.3 | 0.4 | 0.4 |
| Class FH | 0.3 | 0.3 | 0.7 | 0.0 | 0.0 | 0.3 |
| All | 0.3 | 0.5 | 0.8 | 0.1 | 0.1 | 0.3 |
| Enc Time[%] | 96% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |

|  | Low delay HE |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.1 | 0.8 | 0.6 | 0.1 | -0.2 | 1.2 |
| Class FH | 0.1 | 0.3 | -0.1 | 0.0 | 0.0 | 0.1 |
| All | 0.1 | 0.5 | 0.1 | 0.0 | -0.1 | 0.4 |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 100% |  |  |

Table 11. Results for four tiles (N = 4) vs. a single slice, no restricted search range anchor.

# Conclusion and recommendation

Tiles has been shown to reduce loss in coding efficiency relative to LCU’s processed in raster scan-orderwithin a picture in for three important use cases where picture partitioning is required.  Results for MTU size matching show Tiles to provide 0.7%, 0.3%, and 0.2% coding gains for Intra, Random Access, and Low Delay (B and P) respectively relative to a multi-slice anchor. Results for high-level parallel processing, show that Tiles provide 1.7%, 3.1%, and 4.8% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor. Finally, for memory constrained motion estimation using four tiles provided an average of approximately 14.8%, and 4.2% Y BD-rate advantage for the Random Access and Low Delay casesrespectively compared against a memory constrained multi-slice anchor in the context of sequences with a high-degree of vertical motion. Further the losses when using four tiles relative to a non-motion constrained reference are very small.

In addition, Tiles add no meaningful computational complexity. Further, Tiles is complementary with fine granularity slices [9], [10], entropy slices [11], wavefront coding [12], and low delay coding [13] techniques that have been proposed in earlier meetings and the combination of tools might prove advantageous for some applications. Moreover, Tiles can be extended to support decoder parallelization, as proposed in [14].  Experiment 1 was cross-verified by Ericsson and experiment 2 was cross-verified by Qualcomm. Tiles is endorsed by Cisco Systems, eBrisk Video, Sharp and Texas Instruments. We recommend that Tiles be adopted to the HM.

# References

[1] M. Horowitz and S. Xu, “Generalized slices,” JCT-VC Document JCTVC-D378, Daegu, January 2011.

[2] A. Fuldseth, “Replacing slices with tiles for high level parallelism,” JCT-VC Document JCTVC-D227, Daegu, January 2011.

[3] M. Zhou, “Sub-picture based raster scanning coding order for HEVC UHD video coding”, JCTVC-B062, Geneva, July, 2010.

[4]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, March 2011.

[5] JCT-VC, “Test Model HM 3.0”, http://hevc.kw.bbc.co.uk/svn/jctvc-a124/branches/HM-3.0-dev/

[6] “Tiles software based on HM 3.0”, https://s3.amazonaws.com/ebrisk_ftp/tiles+related+files/Jul+2011+meeting+tiles+software/released+version/HM3.0-Tiles_v0.zip

[7] A. Fuldseth, M. Horowitz, S. Xu, and M. Zhou, “Tiles,” JCT-VC Document JCTVC-E408 r1, Geneva, March 2011.

[8] M. Horowitz, S. Xu, E. Ryu and Y. Ye, “The effect of LCU size on coding efficiency in the context of MTU size matching”, JCTVC-Fxxx, Torino, July 2011.

[9] R. Sjöberg, “Fine Granularity slices”, JCTVC-C154, Guangzhou, October 2010.

[10] Q. Shen, Q. Xie, H. Yu, “Fine granularity slice partition”, JCTVC-D312, Daegu, January 2011.

[11] K. Misra, A. Segall, “Lightweight slicing for entropy coding”, JCTVC-D070, Daegu, January 2011.

[12] K. Misra, A. Segall, “New results for periodic inits for wavefront coding functionality”, JCTVC-E409, Geneva, March 2011.

[13] K. Kazui, J. Koyama, A. Nakagawa, “Benefit of the new syntax and semantics for very low delay coding in HEVC”, JCTVC-D054, Daegu, January 2011.

[14] K. Misra, A. Segall, “Tiles for Parallel Decoding”, JCTVC-E412, Geneva, March 2011.

# Patent rights declaration(s)

Cisco Systems, Inc., eBrisk Video, Inc., Sharp, and Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).