| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F336 |
| --- | --- |

| Title: | CE6.b.5 Report: Harmonization of SDIP and MDIS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Report |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.) , Changcai Lai (Hisilicon Technologies Co. Ltd.), Nam Ling (Santa Clara University), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), | Tel: Email: | +1-408-554-4794 g1li@scu.edu lzliu@huawei.com laichangcai@huawei.com nling@scu.edu zhengjianhua@huawei.com pzhang@huawei.com |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd. |  |  |

_____________________________

# Abstract

The harmonization of SDIP (Short Distance Intra Prediction) and MDIS (Mode Dependent Intra Smoothing) using a look-up-table mapping method is proposed and the simulation results are reported.

# Introduction

The MDIS (Mode Dependent Intra Smoothing) used in HM-3.0 is applied for square PUs in Intra prediction of HEVC. Since SDIP (Short Distance Intra Prediction) introduces non-square PUs for Intra prediction, a solution of harmonization is necessary for using MDIS in SDIP. In this contribution, a mapping method is proposed to reuse the MDIS tool for non-square PUs without significant modifications on the code.

# Harmonization of SDIP and MDIS

The MDIS tool used in current design of HEVC Test Model (HM) selects intra smoothing filter in a look up table (LUT) according to the square PU size and the intra prediction mode. In SDIP, PUs can be rectangular. A method is proposed to select intra-smoothing filters for non-square PUs using the same LUT as the original one in MDIS, using a mapping method.

## MDIS filter-table currently used in HM-3.0

The MDIS filter table used in the current version of HEVC test model (HM-3.0) is shown in Table-1.

|  | Directions | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smoothing Filter Switches | 2x2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 4x4 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8x8 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16x16 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 32x32 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | 64x64 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 128x128 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table-1 MDIS filter table used in HM3.0

## Mapping non-square PUs to square PUs in MDIS filter-table

The current MDIS filter table is only for square Prediction Units (PU). In Short Distance Intra Prediction (SDIP), a PU can be rectangular. In order to utilize the current MDSI filter table, a proper mapping method is necessary for using the look-up-table on non-square PUs.

Multiple mapping methods have been tested prior to this Core Experiment, and the one providing the best result was selected and tested in this CE.6.b.5.

The selected approach is width mapping with filtering only when non-square PU’s width is greater than 4. It selects filters in MDIS LUT according to the width of the current non-square PU, map it to the table entry for square PUs with the same width; when the non-square PU’s width is less than or equal to 4, no intra filtering is applied.

The diagram in figure-1 shows the workflow of the proposed mapping method. And the general mapping table is shown as table-2.

Figure-1 Diagram of the Width mapping method

| Rectangular PUs | Square PUs |
| --- | --- |
| 1x16 | No Filtering |
| 2x8 | No Filtering |
| 4x16 | No Filtering |
| 8x2, 8x32 | 8x8 |
| 16x1, 16x4 | 16x16 |
| 32x8 | 32x32 |
| N/A | 64x64 |

Table-2 Width Mapping Table

# Simulation Result

In the simulation, comparisons were made for 3 cases:

- SDIP=0, MDIS=0, SDIP-MDIS=0

The RD data of this test case is compared with HM3.0 anchor data to see the performance of existing MDIS in HM3.0.

- SDIP=1, MDIS=0, SDIP-MDIS=0

The RD data of this test case is compared with SDIP anchor data to see the performance of existing MDIS in the environment of SDIP software.

- SDIP=1, MDIS=1, SDIP-MDIS=1

The RD data of this test case is compared with SDIP anchor data to see how much gain can be obtained by the harmonization of the two tools.

In the provided code, the compiler flags for SDIP, MDIS, and SDIP-MDIS are HHMTU_SDIP, QC_MDIS, and HHMTU_SDIP_MDIS, respectively.

The R-D performance comparisons are shown in the following tables:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.41 | 0.42 | 0.60 | 0.66 | 0.48 | 0.79 |
| Class B | 0.22 | 0.27 | 0.25 | 0.35 | 0.23 | 0.23 |
| Class C | -0.12 | -0.03 | 0.00 | -0.32 | -0.16 | -0.13 |
| Class D | 0.06 | 0.18 | 0.22 | 0.13 | 0.09 | 0.15 |
| Class E | 0.32 | 0.52 | 0.56 | 0.62 | 0.51 | 0.48 |
| Overall | 0.17 | 0.26 | 0.31 | 0.28 | 0.22 | 0.29 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table-3 R-D performance of HM-3.0 Anchor vs. Case-1 (SDIP=0, MDIS=0, SDIP-MDIS=0)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.37 | 0.22 | 0.51 | 0.53 | 0.25 | 0.47 |
| Class B | 0.20 | 0.27 | 0.23 | 0.29 | 0.21 | 0.25 |
| Class C | -0.04 | 0.09 | 0.11 | -0.06 | 0.10 | 0.04 |
| Class D | 0.06 | 0.19 | 0.12 | 0.12 | 0.10 | 0.08 |
| Class E | 0.36 | 0.42 | 0.30 | 0.59 | 0.39 | 0.38 |
| Overall | 0.18 | 0.23 | 0.25 | 0.28 | 0.20 | 0.24 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

Table-4 R-D performance of HM-3.0-SDIP Anchor vs. Case-2 (SDIP=1, MDIS=0, SDIP-MDIS=0)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.04 | -0.19 | -0.14 | -0.12 | -0.04 | -0.12 |
| Class B | 0.00 | 0.04 | 0.04 | -0.02 | 0.07 | 0.09 |
| Class C | 0.04 | 0.06 | 0.04 | 0.08 | 0.02 | 0.04 |
| Class D | -0.01 | 0.03 | -0.05 | -0.01 | 0.01 | -0.01 |
| Class E | 0.03 | -0.07 | 0.00 | 0.02 | 0.13 | 0.10 |
| Overall | 0.00 | -0.02 | -0.02 | -0.01 | 0.03 | 0.02 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table-5 R-D performance of HM-3.0-SDIP Anchor vs. Case-3 (SDIP=1, MDIS=1, SDIP-MDIS=1)

# Conclusion

The simulation results show that MDIS can contribute similar amount of coding gain for HM-3.0 and HM-3.0 with SDIP. The harmonization of MDIS with SDIP does not bring prominent coding gain, but it has no negative impact on the performance while making the Intra smoothing process consistent for both square PUs and non-square PUs.

# Patent rights declaration(s)

Santa Clara University and Hisilicon Technologies Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).