| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F337 |
| --- | --- |

| Title: | CE1: Results of partition size based selection for motion vector compression (A.12) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Masayoshi Nishitani Motoharu Ueda Kazumi Arakage Hideki Takehara JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jk-holdings.com nishitani.masayoshi@jk-holdings.com ueda.motoharu@jk-holdings.com someya.kazumi@jk-holdings.com takehara.hideki@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

The CE1 was established to evaluate the performance of motion data storage reduction methods at 5th JCT-VC meeting. Partition size based selection for motion vector compression was proposed and evaluated under HM2.0 common test conditions at the meeting. This contribution reports the results of partition size based selection method under HM3.0 common test conditions.

The simulation results report that the proposed technique provides no coding gain for both random access settings and low delay settings under HM3.0 common test conditions.

# Introduction

The CE1 [1] was established to evaluate the performance of motion data storage reduction methods at 5th JCT-VC meeting. Partition size based selection for motion vector compression [2] was proposed and evaluated under HM2.0 common test conditions at the meeting. This contribution reports the results of partition size based selection method under HM3.0 common test conditions [3].

# Algorithm Description

## Proposed Algorithm

Figure 1 illustrates the motion vector reduction process on the proposed algorithm. In the Figure 1, the dotted line shows the division of 4x4 block in which the motion vector is recorded to the memory, and the solid line shows the division of prediction partition by CU splitting or PU partitioning.

The candidates to be selected are center four 4x4 blocks in the 16x16 block. The size of the prediction partition in which each center 4x4 block is included is compared in z-scanning order (i.e. 0,1,2,3). When the partition size is compared between the two 4x4 blocks, the larger block is selected.

Figure 1 Motion vector reduction process on the proposed algorithm

## Implementation

### Software

This proposed technique is implemented into HM3.0 revision828 software (CE1/CE9 anchor).

### Syntax structure

The syntax of this proposal is not changed from HM3.0.

# Experiments

## Test conditions

The common test conditions are used.

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 and Table 3 present the simulation results for random access and low delay, respectively.

Table 2 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.1 | 0.2 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Simulation results for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.3 | 0.0 | 0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.3 |
| Class D | -0.1 | -0.2 | 0.4 | 0.1 | -0.2 | 0.1 |
| Class E | 0.0 | -0.2 | -0.4 | 0.0 | 0.2 | 0.5 |
| Overall | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusions

The simulation results report that the proposed technique provides no coding gain for both random access settings and low delay settings under common test conditions.

Therefore, we suggest that the proposed technique be studied for further improvement.

# References

[1] “Description of Core Experiment CE1: Motion Data Storage Reduction”

JCTVC-E701, Geneva, March 2011

[2] “Partition size based selection for motion vector compression”

JCTVC-E096, Geneva, March 2011

[3] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).