| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F338 |
| --- | --- |

| Title: | CE9: Results of temporal MVP restriction for small blocks (SP08, SP10, SP11) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideki Takehara Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5729 takehara.hideki@jk-holdings.com fukushima.shigeru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

The CE9 was established to evaluate the performance of motion vector coding and skip/merge operations at 5th JCT-VC meeting. Temporal MVP restriction for small blocks was proposed and evaluated under HM2.0 common conditions at the meeting. This contribution reports the results of temporal MVP restriction for small blocks under HM3.0 common test conditions.

The simulation results report that temporal MVP restriction for less than 8x8 provides almost no coding loss for both random access settings and low delay settings under HM3.0 common test conditions.

# Introduction

The CE9 [1] was established to evaluate the performance of motion vector coding and skip/merge operations at 5th JCT-VC meeting. Temporal MVP restriction for small blocks [2] [3] was proposed and evaluated under HM2.0 common conditions at the meeting. This contribution reports the results of temporal MVP restriction for small blocks under HM3.0 common test conditions [4].

# Algorithm Description

## Proposed Algorithm

Temporal MVPs for both merge and AMVP mode are restricted for small blocks.

## Implementation

### Software

This proposed technique is implemented into HM3.0 revision828 software (CE1/CE9 anchor).

### Syntax structure

The syntax of this proposal is not changed from HM3.0.

# Experiments

## Test conditions

The common test conditions are used.

SP07-09: Application of limited use of temporal MVP in HM3.0 AMVP and Merge process with the modification B by changing the PU size threshold ranging among 4x4(SP07), 8x4,4x8(SP08), and 8x8(SP09).

It is noted that the specification conceptually includes what should be tested with JCTVC-E097.

SP10-12: Temporal MVP which is restricted by PU size is allowed to be referred in motion estimation (ME). It is noted in JCTVC-E118.

Table 1 presents availability of temporal MVPs in SP07-SP12.

Table 1 Availability of temporal MVPs

|  | 4x4 | 8x4/4x8 | 8x8 | 16x8/8x16 | 16x16 | 32x16/16x32 | 32x32 | 64x32/32x64 | 64x64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SP07 | × | ○ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP08 | × | × | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP09 | × | × | × | ○ | ○ | ○ | ○ | ○ | ○ |
| SP10 | △ | ○ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP11 | △ | △ | ○ | ○ | ○ | ○ | ○ | ○ | ○ |
| SP12 | △ | △ | △ | ○ | ○ | ○ | ○ | ○ | ○ |

○ means temporal MVPs are used both in candidate lists and in ME. △ means temporal MVPs are used only in ME. × means temporal MVPs are NOT used in candidate lists or ME.

This contribution reports the results of SP08, SP10, and SP11.

## Experimental environment

The experimental environment used is presented in Table 2 and Table 3.

Table 2 Experimental environment for encoding

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Table 3 Experimental environment for decoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 4 and Table 5 present the simulation results of SP08 for random access and low delay, respectively.

Table 6 and Table 7 present the simulation results of SP010 for random access and low delay, respectively.

Table 8 and Table 9 present the simulation results of SP011 for random access and low delay, respectively.

Table 4 Simulation results for random access for SP08

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.3 | 0.1 | 0.0 | -0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table 5 Simulation results for low delay for SP08

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class C | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.0 | 0.3 | 0.2 | 0.1 | -0.1 |
| Class E | 0.0 | 0.7 | 0.2 | -0.1 | 0.0 | 0.3 |
| Overall | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

Table 6 Simulation results for random access for SP10

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 7 Simulation results for low delay for SP10

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.3 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | 0.1 | 0.3 | 0.0 | -0.2 | -0.1 |
| Class E | -0.1 | 0.0 | -0.3 | 0.0 | -0.1 | 0.5 |
| Overall | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 107% |  |  | 109% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 8 Simulation results for random access for SP11

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.0 | 0.2 | 0.0 | 0.1 | -0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 9 Simulation results for low delay for SP11

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | -0.1 | -0.3 | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.2 | 0.6 | 0.2 | 0.1 | -0.1 |
| Class E | 0.2 | 0.2 | 0.1 | 0.1 | -0.1 | 0.4 |
| Overall | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 |
| Enc Time[%] | 106% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

The simulation results report the following.

SP08 provides no coding loss for random access settings and 0.1% coding loss for low delay settings with 3% encoder runtime saving and 2% decoder runtime increase. (See the attached SP08 excel sheet)

SP10 provides no coding loss for both random access settings and low delay settings with 7% encoder runtime increase and 2% no decoder runtime increase. (See the attached SP10 excel sheet)

SP11 provides no coding loss for random access settings and 0.1% coding loss for low delay settings with 6% encoder runtime increase and no decoder runtime increase. (See the attached SP11 excel sheet)

# Conclusions

The simulation results report that temporal MVP restriction for less than 8x8 blocks is relevant to reduce the complexity without coding loss.

# References

[1] “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”

JCTVC-E709, Geneva, March 2011

[2] “Improvement to AMVP/Merge process”

JCTVC-E064, Geneva, March 2011

[3] “Temporal predictor restriction harmonized with motion vector compression”

JCTVC-E097, Geneva, March 2011

[4] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).