| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F339 |
| --- | --- |

| Title: | Intra prediction mode coding based on direction difference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@jk-holdings.com fukushima.shigeru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

MPM for Intra prediction mode is applied in the HM3.0 and it is useful to reduce the entropy of intra prediction mode. The technique is based on a uniform probabilistic model.

It is estimated that the current intra prediction mode correlates with the combination of intra prediction modes of the decoded prediction units. And it is estimated that the suitable probabilistic model is switched by each prediction unit.

In this proposed method, MPM sets are switched by using direction difference of two intra prediction modes in adjacent blocks.

The simulation results reports that the proposed technique provides average 0.2% BD-rate gain for intra high efficiency settings and average 0.1% gain for intra low complexity settings.

# Introduction

Intra sample prediction in the HM3.0 [1] is useful to reduce the energy of residual signal. Since a lot of intra prediction modes are used for intra sample prediction, it is important to reduce the intra mode information. In the 5th JCT-VC meeting, some techniques [2] [3] which aim to reduce intra mode information were proposed. JCTVC-E201 [2] is the technique which uses decoded information to select better probabilistic model.

This proposal focuses direction difference of intra prediction modes of decoded prediction units as the decoded information. Plural MPM sets in which the number of candidates is different are defined and switched adaptively by direction difference between left mode and above mode.

This proposal aims to reduce the entropy of intra prediction mode by using direction difference of two intra prediction modes in adjacent blocks.

# Algorithm Description

## Proposed Algorithm

When intra prediction directions of left mode and of above mode are closely, it is asserted that these prediction units have similar frequency characteristic. In this case, it is deduced that it is likely that the current prediction unit has also the frequency characteristic and the distribution of intra prediction mode of current prediction unit concentrates in reference modes nearby. Otherwise, it is asserted that these prediction units have differing frequency characteristic. In this case, the distribution of intra prediction mode of current prediction unit is dispersed than in case that reference modes are closely.

Plural MPM sets are defined and one of the MPM sets is adaptively selected using direction difference between leftMode and aboveMode. This process is applied only for 4x4, 8x8, 16x16 and 32x32 prediction units. The process for 64x64 prediction units that have only 3 modes is not changed from HM3.0.

Brief decoding process for intra prediction mode consists of following ordered steps.

- Calculate direction difference between leftMode and aboveMode(DiffDir).

- Determine the MPM set by DiffDir.

- Decode prev_intra_pred_flag

- If prev_intra_pred_flag is equal to 1, decode mpm_idx using selected MPM set.

- Otherwise (prev_intra_pred_flag is equal to 0), decode rem_intra_pred_mode. The coding trees of the rem_intra_pred_mode are switched by the number of MPM candidates because the increase of the number of MPM candidates causes the decrease of the number of the modes needed to be decoded as remainder.

Figure 1 presents the flowchart of the corresponding process. The detail of each step is described in following sections.

Figure 1 Flowchart of intra prediction mode decoding process

### DiffDir calculation process

Table 1 presents DirIdx17 used for calculation of DiffDir in 4x4 prediction units.

Table 1 Specification of DirIdx17

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DirIdx17 | 11 | 3 | 16 | 7 | 9 | 13 | 15 | 5 | 1 | 0 | 8 | 10 | 12 | 14 | 6 | 4 | 2 |

Table 2 presents DirIdx17 used for calculation of DiffDir in 8x8, 16x16 and 32x32 prediction units.

Table 2 Specification of DirIdx34

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DirIdx34 | 24 | 8 | 33 | 16 | 20 | 28 | 32 | 12 | 4 | 0 | 18 | 22 | 26 | 30 | 14 | 10 | 6 |
| IntraPredMode | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| DirIdx34 | 2 | 17 | 19 | 21 | 23 | 25 | 27 | 29 | 31 | 15 | 13 | 11 | 9 | 7 | 5 | 3 | 1 |

DiffDir calculation process consists of following ordered steps:

- If both of leftMode and aboveMode are DC, DiffDir = -2

- Else if either leftMode or aboveMode are DC, DiffDir = -1

- Else if current prediction unit is 4x4

DiffDir = ABS (DirIdx17[leftMode] – DirIdx17[aboveMode])

if DiffDir > 8, DiffDir = 16 – DiffDir

- Otherwise (current prediction unit is 8x8 or 16x16 or 32x32)

DiffDir = ABS (DirIdx34[leftMode] – DirIdx34[aboveMode])

if DiffDir > 16, DiffDir = 33– DiffDir

The direction difference is assumed to be cyclic (it is assumed that mode 6 and mode 9 are neighboring each other).

### MPM set determination process

Table 3 presents the MPM sets for decoding intra prediction mode depending on DiffDir. The value in each cell presents the number of MPM candidates. Nothing is changed from HM3.0 for MPM sets which DiffDir is less than 2.

Table 3 Specification of MPM sets

| DiffDir | 4x4 PU | 8x8, 16x16, 32x32 PU |
| --- | --- | --- |
| -2 | 1(no change from HM3.0) | 1(no change from HM3.0) |
| -1 | 2(no change from HM3.0) | 2(no change from HM3.0) |
| 0 | 1(no change from HM3.0) | 1(no change from HM3.0) |
| 1 | 2(no change from HM3.0) | 2(no change from HM3.0) |
| 2 | 3 | 3 |
| Greater or equal 3 | 5 | 7 |

The modes which are added as MPM candidates are selected from the neighboring modes of reference modes or DC mode. The neighboring modes are derived from following tables.

Table 4 specifies the neighboring modes of the intra prediction mode for 4x4 prediction units.

Table 4 Specification of the neighboring mode for 4x4 prediction unit

| IntraPredictionMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NeighborLeft17 | 11 | 16 | 2 | 14 | 10 | 12 | 13 | 15 | 9 | 6 | 3 | 4 | 0 | 5 | 7 | 1 | 8 |
| NeighborRight17 | 12 | 15 | 2 | 10 | 11 | 13 | 9 | 14 | 16 | 8 | 4 | 0 | 5 | 6 | 3 | 7 | 1 |

Table 5 presents the neighboring modes of the intra prediction mode for 8x8, 16x16 and 32x32 prediction units.

Table 5 Specification of the neighboring mode for 8x8, 16x16 and 32x32 prediction unit

| IntraPredictionMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NeighborLeft34 | 21 | 30 | 2 | 26 | 19 | 23 | 25 | 28 | 32 | 6 | 18 | 20 | 22 | 24 | 27 | 29 | 31 |
| NeighborRight34 | 22 | 29 | 2 | 18 | 20 | 24 | 9 | 27 | 31 | 33 | 19 | 21 | 23 | 25 | 26 | 28 | 30 |
| IntraPredictionMode | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| NeighborLeft34 | 33 | 3 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 14 | 7 | 15 | 1 | 16 | 8 | 17 | 9 |
| NeighborRight34 | 32 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 6 | 3 | 14 | 7 | 15 | 1 | 16 | 8 | 17 |

NeighborLeft is set to be NeighborLeft17 if the prediction unit is 4x4 and is set to be NeighborLeft34 if the prediction unit is 8x8, 16x16 or 32x32. NeighborRight is set to be NeighborRight17 if the prediction unit is 4x4 and is set to be NeighborRight34 if the prediction unit is 8x8, 16x16 or 32x32.

Derivation process of LeftHandMode and RightHandMode is described as follows.

If	the size of prediction unit is 4x4

If (DirIdx17[leftMode] – DirIdx17[aboveMode]) < 0

&& (DirIdx17[leftMode] – DirIdx17[aboveMode]) > -8)

LeftHandMode = aboveMode

RightHandMode = leftMode

Else

LeftHandMode = leftMode

RightHandMode = aboveMode

Else 	the size of prediction unit is 8x8, 16x16 or 32x32

If (DirIdx34[leftMode] – DirIdx34[aboveMode]) < 0

&& (DirIdx34[leftMode] – DirIdx34[aboveMode]) > -16)

LeftHandMode = aboveMode

RightHandMode = leftMode

Else

LeftHandMode = leftMode

RightHandMode = aboveMode

Table 6 presents the MPM set that have 3 candidates (MPM3). MPM3 is selected if DiffDir is equal to 2. pinchedMode is the mode which direction is neighboring to both of the direction of leftMode and the direction of aboveMode.

pinchedMode is derived as NeighborRight[LeftHandMode] ( or NeighborLeft[RightHandMode] ).

Table 6 MPM having 3 candidates

| MPM index | MPM |
| --- | --- |
| 0 | min(leftMode, aboveMode, pinchedMode) |
| 1 | median(leftMode, aboveMode, pinchedMode) |
| 2 | max(leftMode, aboveMode, pinchedMode) |

Table 7 presents the MPM set that have 5 candidates (MPM5). MPM5 is selected if DiffDir is equal or greater than 3 and the size of prediction unit is 4x4. MPM candidates are set by Table 7 and sorted with the value of intra prediction mode.

Table 7 MPM having 5 candidates

| MPM candidates | MPM |
| --- | --- |
| 0 | min(leftMode, aboveMode) |
| 1 | max(leftMode, aboveMode) |
| 2 | min(NeighborLeft[LeftHandMode], NeighborRight[LeftHandMode]) |
| 3 | min(NeighborLeft[RightHandMode], NeighborRight[RightHandMode]) |
| 4 | 2(DC) |

Table 8 presents the MPM set that have 5 candidates (MPM5). MPM5 is selected if DiffDir is equal or greater than 3 and the size of prediction unit is 8x8, 16x16 or 32x32. MPM candidates are set by Table 8 and sorted with the value of intra prediction mode.

Table 8 MPM having 7 candidates

| MPM candidate | MPM |
| --- | --- |
| 0 | min(leftMode, aboveMode) |
| 1 | max(leftMode, aboveMode) |
| 2 | NeighborLeft[LeftHandMode] |
| 3 | NeighborRight[LeftHandMode] |
| 4 | NeighborLeft[RightHandMode] |
| 5 | NeighborRight[RightHandMode] |
| 6 | 2(DC) |

### MPM index decoding process

Table 9 presents the code mapping of mpm_idx for the MPM sets. MPM1 and MPM2 is same mapping with HM3.0. The code mapping is switched by the number of MPM candidates.

Table 9 Specification of the code mapping for mpm_idx

| MPM index | MPM1 | MPM2 | MPM3 | MPM5 | MPM7 |
| --- | --- | --- | --- | --- | --- |
| 0 | - | 0 | 0 | 00 | 00 |
| 1 | - | 1 | 10 | 01 | 01 |
| 2 | - | - | 11 | 10 | 100 |
| 3 | - | - | - | 110 | 101 |
| 4 | - | - | - | 111 | 110 |
| 5 | - | - | - | - | 1110 |
| 6 | - | - | - | - | 1111 |

### Remainder bits decoding process

#### Remainder bits decoding process in CABAC

When the number of MPM is increase, the number of non-MPM modes is decrease and the fixed-length coding for remainder bits occurs redundancy. Thus remainder bits representation is changed to be variable-length coding.

Table 10 presents the CABAC code mapping of rem_intra_pred_mode for 4x4 prediction units. RemVal is the value of rem_intra_pred_mode in HM3.0.

Table 10 Specification of the code mapping of rem_intra_pred_mode for 4x4

| RemVal | MPM1 | MPM2 | MPM3 | MPM5 |
| --- | --- | --- | --- | --- |
| 0 | 0000 | 000 | 000 | 000 |
| 1 | 0001 | 0010 | 001 | 001 |
| 2 | 0010 | 0011 | 0100 | 010 |
| 3 | 0011 | 0100 | 0101 | 011 |
| 4 | 0100 | 0101 | 0110 | 1000 |
| 5 | 0101 | 0110 | 0111 | 1001 |
| 6 | 0110 | 0111 | 1000 | 1010 |
| 7 | 0111 | 1000 | 1001 | 1011 |
| 8 | 1000 | 1001 | 1010 | 1100 |
| 9 | 1001 | 1010 | 1011 | 1101 |
| 10 | 1010 | 1011 | 1100 | 1110 |
| 11 | 1011 | 1100 | 1101 | 1111 |
| 12 | 1100 | 1101 | 1110 | - |
| 13 | 1101 | 1110 | 1111 | - |
| 14 | 1110 | 1111 | - | - |
| 15 | 1111 | - | - | - |

Table 11 specifies the CABAC code mapping of rem_intra_pred_mode for 8x8, 16x16 or 32x32.

Table 11 Specification of the code mapping of rem_intra_pred_mode for 8x8, 16x16 or 32x32

| RemVal | MPM1 | MPM2 | MPM3 | MPM7 |  | RemVal | MPM1 | MPM2 | MPM3 | MPM7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 00000 | 00000 | 0000 | 0000 |  | 16 | 10000 | 10000 | 10001 | 01100 |
| 1 | 00001 | 00001 | 00010 | 0001 |  | 17 | 10001 | 10001 | 10010 | 01101 |
| 2 | 00010 | 00010 | 00011 | 0010 |  | 18 | 10010 | 10010 | 10011 | 01110 |
| 3 | 00011 | 00011 | 00100 | 0011 |  | 19 | 10011 | 10011 | 10100 | 01111 |
| 4 | 00100 | 00100 | 00101 | 1000 |  | 20 | 10100 | 10100 | 10101 | 11001 |
| 5 | 00101 | 00101 | 00110 | 1001 |  | 21 | 10101 | 10101 | 10110 | 11010 |
| 6 | 00110 | 00110 | 00111 | 10100 |  | 22 | 10110 | 10110 | 10111 | 11011 |
| 7 | 00111 | 00111 | 01000 | 10101 |  | 23 | 10111 | 10111 | 11000 | 11100 |
| 8 | 01000 | 01000 | 01001 | 10110 |  | 24 | 11000 | 11000 | 11001 | 11101 |
| 9 | 01001 | 01001 | 01010 | 10111 |  | 25 | 11001 | 11001 | 11010 | 11110 |
| 10 | 01010 | 01010 | 01011 | 11000 |  | 26 | 11010 | 11010 | 11011 | 11111 |
| 11 | 01011 | 01011 | 01100 | 10100 |  | 27 | 11011 | 11011 | 11100 | - |
| 12 | 01100 | 01100 | 01101 | 10101 |  | 28 | 11100 | 11100 | 11101 | - |
| 13 | 01101 | 01101 | 01110 | 10110 |  | 29 | 11101 | 11101 | 11110 | - |
| 14 | 01110 | 01110 | 01111 | 10111 |  | 30 | 11110 | 11110 | 11111 | - |
| 15 | 01111 | 01111 | 10000 | 11000 |  | 31 | 111110 | 11111 | - | - |
|  |  |  |  |  |  | 32 | 111111 | - |  | - |

Figure 2 Flowchart of rem_intra_pred_mode decoding process (CABAC) presents the flowchart of decoding process of rem_intra_pred_mode in CABAC. In 4x4 prediction units, most significant 3bits string is decoded at first. If the value of read bits is less than the number of MPM candidates-1, rem_intra_pred_mode is set to be the read 3 bits value and the decoding process is done. Otherwise rem_intra_pred_mode is determined by the 3 bits value and additional one bit. The process for 8x8, 16x16 or 32x32 prediction units is similar with the process for 4x4 prediction units.

Figure 2 Flowchart of rem_intra_pred_mode decoding process (CABAC)

#### Remainder bits decoding process in CAVLC

Variable-length coding is applied in CAVLC for rem_intra_pred_mode. Huffman tables for MPM3, MPM5and MPM7 are additionally defined. The code mappings for MPM1 and MPM2 are not changed from HM3.0.

Table 12 presents the length of rem_intra_pred_mode for 4x4 prediction units.

Table 12 the length of rem_intra_pred_mode for 4x4 prediction units

| Remainder value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM1 | 1 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 |
| MPM2 | 1 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | - |
| MPM3 | 1 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | - | - |
| MPM5 | 1 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | - | - | - | - |
| MPM7 | 1 | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | - | - | - | - | - | - |

Table 13 presents the value of rem_intra_pred_mode for 4x4 prediction units.

Table 13 the value of rem_intra_pred_mode for 4x4 prediction units

| Remainder value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM1 | 1 | 7 | 6 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 3 | 2 | 1 | 0 |
| MPM2 | 1 | 0 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | - |
| MPM3 | 1 | 0 | 1 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | - | - |
| MPM5 | 1 | 0 | 1 | 2 | 3 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | - | - | - | - |
| MPM7 | 1 | 0 | 1 | 2 | 3 | 4 | 5 | 12 | 13 | 14 | 15 | - | - | - | - | - | - |

Table 14 presents the length of rem_intra_pred_mode for 8x8, 16x16 or 32x32 prediction units.

Table 14 the length of rem_intra_pred_mode for 8x8, 16x16 or 32x32 prediction units

| Remainder value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM1 | 1 | 4 | 5 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 |
| MPM2 | 1 | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| MPM3 | 1 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| MPM5 | 1 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| MPM7 | 1 | 5 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| Remainder value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| MPM1 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 8 | 8 | 8 | 8 |
| MPM2 | 6 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 7 | 0 |
| MPM3 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | - | - |
| MPM5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | - | - | - | - |
| MPM7 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | - | - | - | - | - | - |

Table 15 presents the value of rem_intra_pred_mode for 8x8, 16x16 or 32x32 prediction units.

Table 15 the value of rem_intra_pred_mode for 8x8, 16x16 or 32x32 prediction units

| Remainder value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPM1 | 1 | 0 | 7 | 6 | 5 | 4 | 3 | 2 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 61 |
| MPM2 | 1 | 0 | 5 | 4 | 3 | 2 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 |
| MPM3 | 1 | 0 | 1 | 2 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| MPM5 | 1 | 0 | 1 | 2 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| MPM7 | 1 | 0 | 1 | 2 | 3 | 4 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| Remainder value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| MPM1 | 60 | 59 | 58 | 57 | 56 | 55 | 54 | 53 | 52 | 51 | 50 | 49 | 48 | 127 | 126 | 125 | 124 |
| MPM2 | 13 | 12 | 63 | 62 | 61 | 60 | 59 | 58 | 57 | 56 | 55 | 54 | 53 | 52 | 51 | 50 | - |
| MPM3 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 60 | 61 | 62 | 63 | - | - |
| MPM5 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | - | - | - | - |
| MPM7 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | - | - | - | - | - | - |

## Implementation

### Software

This proposed technique is implemented into HM3.0 software.

### Syntax structure

The prediction unit syntax of this proposal is presented in Table 16.

Table 16 Prediction unit syntax of the proposed technique

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| ... |  |
| } |  |
| } |  |

# Experiments

## Test conditions

The common test conditions [4] are used.

## Experimental environment

The experimental environment used is presented in Table 17 and Table 18.

Table 17 Experimental environment for encoding

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Table 18 Experimental environment for decoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 19, Table 20 and Error: Reference source not found present the simulation results for intra only, random access and low delay, respectively.

Table 19 Simulation results for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 |
| Class B | -0.2 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class C | -0.2 | -0.1 | -0.1 | -0.1 | 0.1 | 0.0 |
| Class D | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class E | -0.3 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |
| Overall | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 107% |  |  | 113% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table 20 Simulation results for random access

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.2 | -0.4 | 0.0 | -0.2 | 0.2 |
| Class B | -0.1 | 0.0 | 0.2 | 0.0 | 0.1 | 0.2 |
| Class C | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.1 | 0.1 | -0.1 | -0.1 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 21 Simulation results for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.0 | 0.1 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | -0.2 |
| Class D | 0.0 | 0.4 | 0.3 | 0.0 | 0.1 | 0.0 |
| Class E | -0.1 | 0.3 | 0.1 | 0.0 | 0.3 | -0.6 |
| Overall | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | -0.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The simulation results reports that the proposed technique provides average 0.2% BD-rate [5] gain for intra high efficiency settings and average 0.1% gain for intra low complexity settings. (See the attached excel sheet)

The increase of encoder runtime (7% for intra HE setting and 12%) is not caused by the algorithmic complexity but the implementation redundancy. Thus the increase of the runtime can be reduced.

# Conclusions

This contribution reports the proposed technique can bring stable coding gain for intra HE and intra LC settings.

Therefore, we suggest that the proposed technique be adopted to HM.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”,

JCTVC-E603, Geneva, March 2011

[2] “Context Dependent Intra Mode Coding”,

JCTVC-E201, Geneva, March 2011

[3] “Differential Coding of Intra Modes”,

JCTVC-E318, Geneva, March 2011

[4] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

[5] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).