| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F341_r2 |
| --- | --- |

| Title: | Decoder Performance Restrictions due to Merge/MVP Index Parsing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman, Yong Yu 200 Brickstone Sq Andover, MA 01810 USA | Tel: Email: | 978-719-1348 timhellman@broadcom.com yongyu@broadcom.com |
| Source: | Broadcom |  |  |

_____________________________

# Abstract

Previous contributions have asserted that the present method of decoding Merge Index and MVP Index have significant negative impacts on CABAC and CAVLC performance and implementation in high-performance decoder applications.  This contribution quantifies some of this impact.  It suggests that the present method of Merge/MVP Index encoding will severely limit achievable CABAC decoder bit rates and CAVLC decoder performance.  It proposes one solution to this problem, and recommends accepting one of several possible encoding solutions, even if they have a small negative impact on coding efficiency.

# Introduction

A well-known disadvantage of the CABAC coding method is the inability to parallelize its implementation, either for encoding or decoding.  To surmount this difficulty, it’s common for high-performance video decoders to separate CABAC decoding from the rest of the decoder pipeline, as shown in the figure below.  The advantage is that the CABAC decoder can run at the channel rate (relatively constant bit-arrival times), rather than the decoder rate (highly variable bin consumption times).  Even with this optimization, CABAC decoding speed can easily be the limiting factor in determining the maximum decoder bit rate, especially for high-speed transcoding and advanced trick-play modes.  Therefore, an efficient CABAC is critical.

The authors of the H.264/AVC standard were aware of the need for this decoupled implementation, and designed the CABAC syntax and context to enable it.  For example, the ratio of bits to bins is bounded, to enable separate bin buffering [1], and the CABAC context only references decoded CABAC symbols (like MVDs), not constructed decoder syntax (like MVs).  Unfortunately, HEVC’s introduction of the Merge Index (merge_idx) and MVP Index (mvp_idx_l0/1) syntax elements has broken this model, because the size (and presence) of these syntax elements is determined by the number of candidate predictors, which in turn depends on the value of the neighboring and co-located motion vectors.  This contribution lists the ways in which this syntax choice negatively affects the CABAC decoder, provides detail on the extent of CABAC performance impairment, and enumerates some possible solutions.

CAVLC decoder implementations are also negatively affected by Merge/MVP index coding.  The issue for CALVC is the disruption of the bit parsing process, again due to the dependency between the index size and the values of the neighboring MVs.  The effect on CAVLC decoding is also detailed in this contribution.

# CABAC Problem Description

Merge/MVP index coding affects the HEVC syntax in 3 places: skip, merge and inter prediction units.  In all 3 places the index is used to select which of several motion-vector predictions (MVPs) should be used either as the resultant motion vector (in the case of skip & merge), or as the prediction to which a delta is added.  The size of the index depends on the number of available predictions, which is based on spatial neighbor availability, co-located neighbor availability, and elimination of duplicate motion vector predictions.  The latter two criteria present the problem, because both of them require full motion vector construction inside the CABAC decoder.  This has several disadvantages:

- Increased CABAC logic area.  Vector construction logic (including fetching and storing co-located vector) must be replicated in the CABAC decoder.  This is non-trivial, given the complexity of vector construction, including the multipliers needed for vector scaling.

- Replication of co-located vector buffers:  Since the CABAC decoder operates well ahead (often times several frames ahead) of the rest of the pipeline, it has to have its own co-located vector buffers.  This means that the reference picture management also has to be replicated (to manage the reference lists), and the system memory requirements increase as well.

- Loss of picture-level parallelism:  One way that CABAC can be parallelized is by decoding independent sections of the stream in parallel.  This can be done with slices, but in the general case a decoder must handle single-slice pictures as well.  With H.264/AVC, two or more pictures can be CABAC decoded in parallel.  Here, however, the introduction of co-located vectors into CABAC parsing means that pictures now depend on each other, so this acceleration method is no longer possible.

- Loss of error resiliency:  The use of co-located picture information for syntax parsing also has a detrimental effect on error resiliency, also due to picture dependency.  This is detailed in [2].

- CABAC Performance:  The achievable level of CABAC performance is negatively impacted, because motion vector construction must be serialized with CABAC bin decoding.  This is the subject of section 3.

# CABAC Performance Impairment due to Vector Construction

A high-performance H.264/AVC CABAC decoder can produce an output bin for every one to two clock cycles of processing (one clock for probability updates, with an extra clock for state changes).  This is relatively independent of stream content, because the CABAC context updates are easily done in parallel with bin generation.  Thus, the maximum channel rate of the CABAC decoder can be derived as a simple ratio of its clock frequency.

Adding vector construction to CABAC slows the decoder, because the time to do construction can not be overlapped with bin decoding (see figure below).  Moreover, this construction time is constant no matter how few or how many bins are in a prediction unit.

Vector construction has four phases for a dedicated hardware decoder:

- Co-located vector fetch

- Neighbor computation (availability, list reduction and temporal scaling)

- MV selection and array storage (storing neighbor information for subsequent vectors)

- Co-located vector storage

Phases 1 & 4 are done in parallel to the other stages, so they won’t be considered here (though they can become the bottlenecks for very simple LCUs).  Phase #2 can only be performed once the size and type of the prediction units are known (so that the neighbors can be determined).  CABAC parsing has to stop while this is done, so that the size of the Merge/MVP index can be determined.  Phase #3 is performed once the index (and any associated MVDs) are parsed.  It cannot be overlapped with parsing either, since its results are needed before phase #2 can be performed on the next PU in the coding unit.

The worst-case impact occurs when a picture has many skip blocks.  A skip consists of one to five bins (depending on the size of the merge_idx).  At one clock/bin, that amounts to an average of three clocks of parsing time.

The number of cycles for neighbor computation depends on the amount of parallelism in the implementation, but a reasonable assumption is about seven clock cycles to do the selection, list reduction and vector scaling.  MV reconstruction takes a single clock cycle.  The array storage stage takes the constructed MV and stores it in a local array (at 4x4 granularity), so this stage is proportional to the number of 4x4 neighbors of the PU.  For a 16x16 block, this is seven clocks.  Therefore the total time spent in vector construction is 7 + 1 + 7 = 15 clocks.

Thus, the total time to parse a skip block is 15 (construction) + 3 (CABAC parsing) = 18 clocks.  This is over 500% slower than for an equivalent block in H.264/AVC (3 clocks)!  Obviously, the overhead of a standard prediction unit would be smaller, since MVDs and coefficients would need to be parsed, but the fixed time lost to MV construction degrades these coding units as well.  Moreover, the skip case can occur very frequently in common sequences (such as relatively still backgrounds), so this inefficiency is not just a theoretical concern.

# CAVLC Performance Impact

Since it can be parallelized, CAVLC processing does not have to be done in advance.  For this to not be the limited factor in pixel decoding, a worst-case coding unit must be parsed in a fixed amount of time, determined by dividing the number of coding units in the picture by the picture decoding time.  For H.264/AVC, CAVLC parsing time is dominated by coefficient parsing, because the motion vectors can be quickly parsed, and motion vector construction can be done in parallel with coefficient parsing, as shown in the figure below.

Adding variable-sized Merge/MVP index parsing takes away the parallel processing option.  Now the parsing has to stop to allow the vector computation, just as for CABAC.  Here that means that less time is available for coefficient parsing, because of the fixed-time coding-unit decode requirement (see figure below).  CABAC bin parsing is affected in a similar way, but since CAVLC coefficient decoding is already more time consuming than the CABAC equivalent (CAVLC has more symbols to decode), it has a more serious bottleneck.  Therefore any reduction in the available CAVLC coefficient decoding time is going to be very difficult to accommodate.

# Proposed Changes

The fundamental change needed to solve this problem is that the coding of merge_idx and mvp_idx_0/1 must depend only on spatial neighbor availability. Neighbor availability is easy to compute in advance, and eliminates the need to do vector construction at the parsing stage.  Always assuming co-located vector availability is also important, because it removes the need for the CABAC decoder to manage co-located vector information. The crux of this issue is the need to eliminate the fully constructed MVs from the CABAC entropy decoder.

Simply eliminating the vector comparison with no other optimizations produces a compression ratio loss of about 0.9% in the high efficiency random access category.  Submissions [2] and [3] presented methods of reducing this loss, but neither completely solved all the parsing issues.  However, the expanded size of the index symbol gives the opportunity to present other predictor choices to the encoder, as was recognized by [4].  That submission reduces the coding loss significantly, at the cost of some added motion vector construction complexity.  An alternative algorithm is proposed here, which is simpler, though with somewhat larger coding loss than [4].

This proposed algorithm change has two components; one for skip/merge and one for standard MVs:

- Decoding mvp_idx_l0/1 (not skip/merge coding unit):

- The coder first checks the availability of the spatial motion vector predictors. If neither the left nor the above block are available for motion vector prediction (for normal reasons), the MVP for current block will be derived from the collocated block.

- Otherwise, (either spatial neighbor block is available), a flag “collocated_MVP_flag” is signaled in the bit stream. collocated_MVP_flag = 1 indicates that the MVP is derived from the collocated block. collocated_MVP_flag = 0 is used to indicate that the MVP is derived from neighbor blocks.

- If only one neighbor block is available, the MVP of the current block is assigned with the available MVP.  Otherwise, a “prediction_direction_flag” is signaled in the stream to indicate which block the MVP is from (top or above).

- Finally check all the availale MVP candidates. Scanning the MVP candidate list in the order of MVP candidate index, if any MVP is identical to previous MVP, set the second MVP to 0. Note that this step is not necessary in the process of CABAC decoding, since it does not change the binarization process.

- Decoding “merge_idx” (Skip and Merge coding units) only changes the MVP list construction process:

- Determine NumMergeCand. The size of the NumMergeCand syntax element is determined based on the availablity of MVP candidates only. All the available MVPs are used in an initial candidate list.

- The existing duplication MVP candidate removal process is invoked after the MVP candidate list is constructed in the previous step. This process scans the MVP candidate list from left. If an MVP is found to be the same as any previously scanned MVPs. However, instead of removing the second identical MVP, the second identical MVP in the list is set to be zero.

- The last step is to move the duplicated MVPs (that were set to be zero) to the end of the list.

Example:

Assuming the neighboring PU’s of current prediction unit have the availablities and motion vectors as in the following table. The current PU’s “left” and “Corner BL” PUs are not available for prediction, and the “collocated” PU has the same the same motion vector as “above” PU (after the scaling process).

|  | Left | Above | Collocated | Corner | Corner BL |
| --- | --- | --- | --- | --- | --- |
| MVP | n/a | yes | Same as “above” | yes | n/a |

After the MVP list construction process, “merge_idx” is mapped to the neighboring motion vectors with the following table.

| merge_idx | 0 | 1 | 2 |
| --- | --- | --- | --- |
| MVP value | MVP value from “above” PU | MV from “Cornor” PU | (0,0) |

# Experimental Results

The modification proposed in this proposal is based on the latest test model HM3.0. The experiments is conducted follow the common guide line.

Table 2 Summary of bit rate impact from the modified syntax

| Random Access High Efficiency |  |  |  | Random Access Low Complex |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Average | 0.02 | -0.5 |  | Average | 0.02 | -0.8 |
| A | 0.008 | -0.33 |  | A | 0.011 | -0.48 |
| B | 0.016 | -0.66 |  | B | 0.024 | -0.94 |
| C | 0.023 | -0.59 |  | C | 0.030 | -0.76 |
| D | 0.024 | -0.53 |  | D | 0.031 | -0.72 |
| Low Delay High Efficiency |  |  |  | Low Delay Low Complex |  |  |
| Average | 0.04 | -1.2 |  | Average | 0.04 | -1.3 |
| A |  |  |  | A |  |  |
| B | 0.025 | -0.92 |  | B | 0.026 | -1.01 |
| C | 0.037 | -0.93 |  | C | 0.032 | -0.85 |
| D | 0.029 | -0.68 |  | D | 0.032 | -0.71 |
| E | 0.085 | -2.65 |  | E | 0.100 | -3.12 |

# Conclusions

The present Merge/MVP index coding method produces a performance burden on both CABAC and CAVLC decoder implementations, as shown.  This impact may be serious enough to limit the deployment of HEVC decoders in high-bitrate and high-definition applications.  The proposed solution is to make the Merge/MVP index size depend solely on spatial neighbor availability, and to present more prediction choices with the ‘extra’ index encodings that result from identical MVs.  The methods proposed to date, including the one presented here, have shown a small coding loss from this change, but that seems a reasonable tradeoff for the performance gain.  Plus, it’s quite possible that more study of prediction choices will produce a solution that has no coding burden at all.

We propose that one of the submitted solutions be chosen as a baseline, and that further study be done on the optimal method of chosing the MV predictors.

# References

- Frank Bossen, "Bounding the complexity of arithmetic decoding", Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, 3rd Meeting Fairfax, Virginia, USA, 6-10 May, 2002.

- M. Zhou and V. Sze, “A study on HM2.0 bitstream parsing and error resiliency issues”, JCTVC-E0118, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- Y Yu, “Impact of Motion Vector Predictor Index Decoding on Parallelization between CABAC and Other Decoder Components”, JCTVC-E451, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- G Laroche, Christophe Gisquet et al, “Robust Solution for the AMVP parsing issue”, JCTVC-E0219, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).