| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F342_r2 |
| --- | --- |

| Title: | ALF Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | Tim Hellman 200 Brickstone Sq Andover, MA 01810 USA | Tel: Email: | 978-719-1348 timhellman@broadcom.com |
| Source: | Broadcom Corp |  |  |

_____________________________

# Abstract

This submission examines the implementation complexity of the adaptive loop filter (ALF).  It reports that the worst-case ALF complexity, as presently specified, is on the order of 130 operations (multiplies and adds) per luminance pixel, or roughly 16 billion operations per second for an HD video stream.  It compares this complexity to that of the inverse transform and motion compensator, and recommends study of ways to reduce this complexity.

# Introduction

The adaptive loop filter (ALF) is a new tool for HEVC.  It is an in-loop, post-deblocking 2D filter that’s conditionally applied to all pixel components in a picture.  The filter is adaptive in that several sets of coefficients are sent in the slice header, and a variance calculation is used to select the coefficient set to apply to a given block.  3 luma filter shapes are presently defined: 5 tap, 7 tap and 9 tap.

The chroma components are filtered as well, using a rectangular shape with similar extent.  The filter coefficients have diagonal symmetry, i.e. for the 9-tap filter coefficient 0 and 38 are the same, as are 1 and 37, etc.

This submission looks at the number of operations (add, multiply, etc) that it takes to implement this operation in a decoder, in comparison to other decoder stages.  The analysis is biased towards a hardware decoder implementation, but should also largely apply to a SIMD software decoder as well.

# ALF Complexity

The ALF filter has 2 major processing stages: variance calculation (for filter selection) and the filter operation itself.  ALF can be applied on a region or block basis, but the block-based filter is more computationally complex, so it is used here.  Only addition and multiplication operations are counted, since shifts are ‘free’ in a hardware implementation.  Operation counts for software implementations would therefore be slightly larger.

## Variance Complexity

ALF variance is computed for each luminance pixel by the absolute value of a simple difference in the horizontal and vertical direction:

variance_horiz = abs( 2 * pix – pixy-1 – pixy+1)

variance_vert = abs( 2 * pix – pixx-1 – pixx+1)

variance_both = variance_horiz + variance_vert

Each variance direction takes 3 add operations (abs. value is equivalent to an add, and *2 is a shift).

For each 4x4 block in the picture, 3 sums are computed:

sum_vert = sum of all 16 variance_vert in the block

sum_horiz = sum of all 16 variance_horiz in the block

sum_block = for each pixel, sum all of variance_both in a 3x3 block, then sum those 16 block sums

The value of these 3 sums is used to select 1 of 16 sets of filter coefficients.

In an optimized implementation, each variance direction calculation would be performed once per pixel.  Additionally, the sum_block calculation can be done as a single two-dimensional weighted sum of a 6x6 array, rather than a sum of sums.  This sum is separable into 6 horizontal sums, each of which is:  x1 + 2(x2 + x5) + 3(x3 + x4) + x6.  Multiply by 2 is just a shift, and multiply by 3 is a shift and add, so this gives 6 adds per horizontal row.  These 6 horizontal sums are then added together using the same weights, for 6 more operations.  Thus, sum_block takes (6*6) + 6 = 42 add operations to sum up all the variances.

The total number of adds per 16-pixel block is therefore:

Generating variance_horiz and variance_vert:  16 * 3 + 16 * 3 = 96 add operations

Generating variance_both: 16 add operations

Vertical/Horizontal sums:  15 add operations each

Block sum:  42 add operations

Total = 96 + 16 + (15 * 2) + 42 = 184 add operations for 16 pixels, or 11.5 add operations/luma pixel.

## Filter Complexity

Obviously, the 9-tap luma filter is the most computationally complex.  Its value is a simple sum-of-products of the 39 coefficient locations.  However, the coefficients are symmetric, so two pixels share the same coefficient, which cuts the number of required multiplies in half.  Thus, the luma component requires 38 add and 20 multiply operations per luma pixel.

The two chroma components are defined with rectangular filter shapes of up to 9 taps.  Thus it has 81 coefficients, or 80 adds and 41 multiplies per chroma component.  There are two chroma components for each luma, but they are sub-sampled by 4, so the chroma filters end up adding 40 add and 20.5 multiply operations per luma pixel.

## Overall ALF Complexity

The overall complexity of the block-based ALF filter is:

|  | Add Ops/Luma Pixel | Mult Ops/ Luma Pixel |
| --- | --- | --- |
| Variance | 11.5 | 0 |
| Luma Filter | 38 | 20 |
| Chroma Filters | 40 | 20.5 |
| Total | 89.5 | 40.5 |

HD resolution is roughly 2 million pixels, so a 60 frame-per-second HD movie would require 2 * 60 * (89 + 40.5) = 15.5 billion operations per second just to perform this filtering!

The region-based ALF eliminates the variance calculation, since the filter selection is given explicitly for each image region.  As can be seen from the table above, this does not reduce the ALF complexity significantly, since the variance calculation is only 12% of the add operations, and none of the multiplies.

# Complexity Comparison to Transform and Motion Compensation

The two most computationally complex stages in an H.264/AVC decoder are the inverse transform and motion compensation blocks, so it’s useful to use the presently defined HEVC versions of those stages as a complexity metric.

## Transform Complexity

[2] gives a complexity summary for various HEVC transform sizes.  These are reproduced here (partial butterfly implementation):

| Transform size | Mult | Add | Shift |
| --- | --- | --- | --- |
| 4x4 | 48 | 64 | 32 |
| 8x8 | 352 | 448 | 64 |
| 16x16 | 2752 | 3200 | 256 |
| 32x32 | 21888 | 23808 | 2048 |

Clearly, the 32x32 transform is the worst-case in terms of operations per pixel.  A 32x32 luma transform block has a corresponding pair of 16x16 chroma transforms, so the overall complexity is:

|  | Add Ops/Luma Pixel | Mult Ops/Luma Pixel |
| --- | --- | --- |
| Luma Transform | 23.3 | 21.4 |
| Chroma Transform | 6.3 | 5.4 |
| Total | 29.6 | 26.8 |

## Motion Compensation Complexity

Almost all of the complexity of the motion compensator is in the interpolation filtering, so that is analyzed here.  In the worst-case (quarter-pel vertical and horizontal addressing), each luma sample location requires 8 8-tap vertical filter stages and 1 8-tap horizontal filter (9 8-tap filters in total), and each chroma sample requires 4 4-tap vertical filters and 1 4-tap horizontal (5 4-tap filters in total, for ½ the number of luma pixels).  The 8-tap filter requires 6 multiplies and 8 adds, and the 4-tap requires 4 multiplies and 4 adds.  This gives:

|  | Add Ops/Luma Pixel | Mult Ops/Luma Pixel |
| --- | --- | --- |
| Luma Filter | 72 | 54 |
| Chroma Filters | 10 | 10 |
| Total | 82 | 64 |

# Conclusions

The various complexity metrics are summarized here:

|  | Add Ops/Luma Pixel | Mult Ops/Luma Pixel |
| --- | --- | --- |
| ALF | 89.5 | 40.5 |
| Transform | 29.6 | 26.8 |
| Motion Comp | 82 | 64 |

As presently specified, the ALF filter’s complexity is well beyond that of the transform, and on the order of the motion compensator!  This level of complexity seems out of proportion to ALF’s contribution to bit-rate reduction.  Given this, we recommend that the filter implementation be further studied, with the goal of finding a significantly simpler method of producing similar results.

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, B. Bross, G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 3,” JCTVC-E603, Geneva, March, 2011.

- A. fuldseth, G.  Bjontegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation”, JCTVC-E243, Geneva, March, 2011.

# Patent rights declaration(s)

Broadcom Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).