| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F343 |
| --- | --- |

| Title: | Evaluation of SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ehsan Maani, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ehsan.maani@am.sony.com ali.tabatabai@am.sony.com |
|  | Teruhiko Suzuki 1-11-1 Osaki, Shinagawa-ku, Tokyo 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc. / Sony Corp. |  |  |

_____________________________

# Abstract

This document provides investigation summary, both in terms of coding performance and the HW complexity, for support of 1x16(16x1) and 8x2(2x8) partitions in SDIP. It is shown that removing 1x16 and 16x1 TU partitions, results in significant HW complexity reduction with negligible coding performance loss i.e., 0.1% for AI_HE and 0.0% for AI_LC. Partial results for coding performance as well as HW complexity are also provided for 8x2(2x8) partitions.

# Problem statement

In HM3.0 SDIP branch[1] introduces non-square partitions for CU. For example, in a 16x16 CU, a rectangular partition 4x16/16x4 can be further split into four 1x16/16x1 line partitions when a one-level RQT is applied. The four line partitions share the same intra prediction mode and the signaling of intra prediction mode will follow the way of 4x16/16x4. Branches are accordingly set up to support the RQT of SDIP in the coding path.

In case of HW implementation of intra coding he prediction by block size of 16 pixels (i.e. 4x4, 16x1, 1x16, 8x2 and 2x8) is in general a challenging because of the reconstruction loop critical path i.e., prediction->transform->reconstruction->prediction…In WD3 4x4 prediction block is already supported.  Support for 16x1, 1x16, 8x2 and 2x8 blocks are new due to SDIP and they create additional critical path.

In Table 1 an estimation of HW complexity in terms of gate sizes is shown:

|  | 16x1 and 1x16 | 8x2 and 2x8 | Other size in WD3 |
| --- | --- | --- | --- |
| Gate size | + 36 kgate | + 11 kgate | 13 kgate |

Table 1: Additional HW complexity by SDIP

Based on the above Table HW complexity of 16x1 and 1x16 is about three times larger than other sizes in WD3. With 16x1/1x16 support, this means that the total gate size will be four times larger than that of WD3. Similarly, the gate size of 8x2 and 2x8 is also large and almost similar to the existing prediction size. Because these additional tools are n the critical path of intra prediction, they introduce significant additional cost. Coding efficiency gains by these tools need to be significant in order to be considered for as intra prediction tools.

# Coding performance Evaluation

The coding performance of supporting 1x16(16x1) and 8x2 (2x8) partitions from SDIP are evaluated.

# Experiment setting and results

## Software implementation

The software implementation is based on SDIP branch “hm-3.0-dev-sdip” in the SVN server with revision number r866.

## Coding conditions

The conditions are the same as specified in JCTVC-E700 [2] for low complexity settings.

## Test set

- The same set as specified in JCTVC-E700 is used.

### 16x1(1x16) Results

Anchor is with 16x1(1x16) support and test is with no 16x1(1x16) support

Positive means loss

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.1 | -0.1 | -0.2 | -0.2 |
| Class B | 0.1 | 0.1 | 0.2 | 0.0 | -0.2 | -0.1 |
| Class C | 0.2 | 0.3 | 0.4 | 0.1 | 0.0 | 0.1 |
| Class D | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |
| Class E | 0.1 | 0.2 | 0.2 | -0.1 | -0.1 | -0.2 |
| Overall | 0.1 | 0.2 | 0.2 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

### 8x2(2x8) Results for 30 Frames Only

Anchor is with 8x2(2x8) support and test is with no 8x2(2x8) support

Positive means loss

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.16 | 0.05 | -0.16 | 0.16 | 0.14 | 0.11 |
| Class B | 0.38 | 0.00 | -0.04 | 0.36 | 0.11 | 0.07 |
| Class C | 1.25 | 0.46 | 0.49 | 1.20 | 0.60 | 0.52 |
| Class D | 1.30 | 0.72 | 0.97 | 1.26 | 0.61 | 0.75 |
| Class E | 0.42 | -0.43 | -0.08 | 0.28 | -0.17 | -0.05 |
| Overall | 0.70 | 0.18 | 0.24 | 0.65 | 0.28 | 0.28 |

### 8x2(2x8) Results

Anchor is with 8x2(2x8) support and test is with no 8x2(2x8) support

Positive means loss

|  | All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.0 | 0.0 | 0.2 | 0.1 | 0.1 |
| Class B | 0.4 | 0.0 | -0.1 | 0.4 | 0.2 | 0.1 |
| Class C | 1.1 | 0.4 | 0.4 | 1.1 | 0.6 | 0.5 |
| Class D | 1.2 | 0.6 | 0.7 | 1.2 | 0.6 | 0.7 |
| Class E | 0.4 | -0.1 | -0.1 | 0.3 | 0.0 | 0.0 |
| Overall | 0.7 | 0.2 | 0.2 | 0.6 | 0.3 | 0.3 |
| Enc Time[%] | 90% |  |  | 89% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# Conclusion

This proposal shows that by removing 16x1(1x16) line partition support in SDIP complexity is reduced with minor degradation in coding efficiency.

# Reference

- “CE6.b1 Report on Short Distance Intra Prediction Method”, JCTVC-E278, March. 2011

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March. 2011

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).