| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F344-r1 |
| --- | --- |

| Title: | CE3: Cross-check report on CE3 Subtest 4.1 and 4.4 (JCTVC-F100) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com hao@bu.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of CE3 Subtest 4.1 and 4.4 described in JCTVC-F100. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

- Software analysis

The following modifications are confirmed through the analysis of software provided by SONY:

- Switch filter coefficients according to the sample position in an MC reference block:

  - If a sample position to be interpolated locates on around MC reference block boundary, its sample value is generated by using modified versions of the HM DCTIF filter.

  - Otherwise, a sample value is generated by using the HM DCTIF filter.

- The modified DCTIF filter is designed so as not to refer to outside the specified full-pel samples.

The software supports different sets of modified DCTIF filters. The CE3 subtest 4.1 is to test one set whose memory access to reference samples is the same as that by using 6-tap filters for both horizontal and vertical interpolations, as depicted in Figure 1. The CE3 subtest 4.4 is on top of the CE3 subtest 4.1 and uses the HM DCTIF for sub-pel motion estimation.

Figure 1	Example of MC reference samples for 4x4 MC block.

- Results

Software provided by Sony was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encoding and decoding executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| Windows Vista 64-bit Professional Core i7 2.93GHz and Memory 12GB | CE3 subtest 4.1. Up to 4 processes for encoding and decoding run at the same time. |
| Windows 7 64-bit Professional Core i7 2.80GHz and Memory 12GB | CE3 subtest 4.4. Up to 4 processes for encoding and decoding run at the same time. |

Simulation was conducted according to the CE3 common test coding conditions based on JCTVC-E700. Attached Excel sheets, JCTVC-F344-NEC-HM-3.0-Anchor-vs-Sony4.1.memc.xls and JCTVC-F344-NEC-HM-3.0-Anchor-vs-Sony4.4.mc.xls, contain the detailed results. It was confirmed that BD-PSNR/Rate values obtained are the same as those of provided by proponent.

# Conclusion

It is confirmed that the CE3 Subtest 4.1 and 4.4 results reported by proponent are accurate.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).