| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F346 |
| --- | --- |

| Title: | CE4 Subtest 2.3: Cross-check of Qualcomm’s proposal(JCTVC-F332) on delta QP prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Jia Joonyoung Park | Tel: Email: | jie.jia@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This report presents LG’s cross-check results for Tool 2.3.b and Tool 2.3.c of CE 4 on QP prediction by Qualcomm. It is verified that the BD-rate values and number of bits for cu_qp_delta over all configurations are consistent with those provided by Qualcomm, while some of running times differ slightly. Source code was also studied to verify implementation of Qualcomm’s proposal.

# Verification on SW implementation

The source code for the subtest 2.3.b and 2.3.c[1][2] has been studied, and it appears to work according to the proponents’ technique description.

# Configurations for simulation

Two separate tests were carried out on QP prediction from neighboring CUs, corresponding to tool 2.3.b and tool 2.3.c[1], respectively.

Tool 2.3.b) use left top neighboring CU’s QP as the predictor for coding of current CU’s QP.

Enabled by “#define REFERENCE_QP_TYPE                 2”

Tool 2.3.c) use immediate neighbors within a quadtree split CU block for QP prediction.

Enabled by “#define REFERENCE_QP_TYPE                 3”

Table 1 illustrates configurations for running the simulations.

Table 1 Configurations for simulation

|  | REFERENCE_QP_TYPE | MaxCUDQPDepth |
| --- | --- | --- |
| Anchor | 1 | 3 |
| Subtest 2.3.b | 2 | 3 |
| Subtest 2.3.c | 3 | 3 |

# Verification on simulation

Experimental verification was conducted under the test conditions defined in [2], and the simulation environment is illustrated in Table 2.

Table 2 Simulation environment

| Platform | Windows XP 64 bit |
| --- | --- |
| Compiler | MS Visual Studio with x64 |
| CPU | i7 875K 2.93Hz |
| RAM | 16G |

Results of cross verification on tool 2.3.c are summarized and reported in Table 3 and Table 4. For detailed results of tool 2.3.b and 2.3.c, please refer to enclosed excel sheets.

Table 3 CE4 Tool 2.3.c QP prediction from immediate neighbors within a quad-tree split CU (HE)

|  | cross checked |  |  |  | Qualcomm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra HE |  |  |  | All Intra HE |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.0 | 0.8% | 0.0 | 0.0 | 0.0 | 0.8% |
| Class B | 0.0 | 0.0 | 0.0 | 0.4% | 0.0 | 0.0 | 0.0 | 0.4% |
| Class C | -0.1 | -0.1 | -0.1 | -1.5% | -0.1 | -0.1 | -0.1 | -1.5% |
| Class D | -0.1 | -0.1 | 0.0 | -1.6% | -0.1 | -0.1 | 0.0 | -1.6% |
| Class E | 0.0 | 0.0 | 0.0 | -0.5% | 0.0 | 0.0 | 0.0 | -0.5% |
| All | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.0 | 0.0 | -0.4% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 98% |  |  |  |
|  | Random Access HE |  |  |  | Random Access HE |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.2 | 0.1 | -0.3% | 0.0 | 0.2 | 0.1 | -0.3% |
| Class B | 0.0 | -0.2 | 0.0 | -0.6% | 0.0 | -0.2 | 0.0 | -0.6% |
| Class C | 0.0 | 0.0 | 0.0 | -2.0% | 0.0 | 0.0 | 0.0 | -2.0% |
| Class D | 0.0 | 0.0 | 0.3 | -1.5% | 0.0 | 0.0 | 0.3 | -1.5% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | -1.1% | 0.0 | 0.0 | 0.1 | -1.1% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B HE |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | -1.0% | 0.0 | -0.1 | -0.1 | -1.0% |
| Class C | -0.1 | 0.0 | 0.0 | -2.3% | -0.1 | 0.0 | 0.0 | -2.3% |
| Class D | -0.1 | -0.3 | -0.2 | -1.7% | -0.1 | -0.3 | -0.2 | -1.7% |
| Class E | -0.1 | -1.2 | 0.6 | -2.7% | -0.1 | -1.2 | 0.6 | -2.7% |
| All | -0.1 | -0.3 | 0.0 | -1.8% | -0.1 | -0.3 | 0.0 | -1.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 97% |  |  |  |

Table 4 CE4 Tool 2.3.c QP prediction from immediate neighbors within a quad-tree split CU (LC)

|  | cross checked |  |  |  | Qualcomm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra LC |  |  |  | All Intra LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.0 | 1.0% | 0.0 | 0.0 | 0.0 | 1.0% |
| Class B | 0.0 | 0.0 | 0.0 | 0.0% | 0.0 | 0.0 | 0.0 | 0.0% |
| Class C | -0.1 | -0.1 | -0.1 | -1.7% | -0.1 | -0.1 | -0.1 | -1.7% |
| Class D | -0.1 | -0.1 | -0.1 | -1.9% | -0.1 | -0.1 | -0.1 | -1.9% |
| Class E | 0.0 | 0.0 | 0.0 | -0.7% | 0.0 | 0.0 | 0.0 | -0.7% |
| All | 0.0 | 0.0 | 0.0 | -0.6% | 0.0 | 0.0 | 0.0 | -0.6% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Random Access LC |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.0 | 0.1 | 0.1% | 0.0 | 0.0 | 0.1 | 0.1% |
| Class B | 0.0 | -0.1 | 0.0 | -0.9% | 0.0 | -0.1 | 0.0 | -0.9% |
| Class C | -0.1 | -0.1 | -0.1 | -2.5% | -0.1 | -0.1 | -0.1 | -2.5% |
| Class D | -0.1 | 0.0 | -0.1 | -2.0% | -0.1 | 0.0 | -0.1 | -2.0% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -1.3% | 0.0 | 0.0 | 0.0 | -1.3% |
| Enc Time[%] | 100% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Low delay B LC |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | 0.0 | -1.0% | -0.1 | 0.0 | 0.0 | -1.0% |
| Class C | -0.2 | -0.1 | -0.1 | -2.7% | -0.2 | -0.1 | -0.1 | -2.7% |
| Class D | -0.2 | -0.3 | -0.1 | -2.2% | -0.2 | -0.3 | -0.1 | -2.2% |
| Class E | -0.2 | -0.4 | 0.0 | -2.8% | -0.2 | -0.4 | 0.0 | -2.8% |
| All | -0.1 | -0.2 | -0.1 | -2.0% | -0.1 | -0.2 | -0.1 | -2.0% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |

# Observations and Conclusions

It is observed that QP prediction from immediate neighbors within a quad-tree split provides relative better performance in comparison with the prediction from neighbors across a quad-tree split. In addition, regarding the QP prediction within a quad-tree split, simulation results report that it generally performs better for inter picture coding than that for intra picture coding.

For CUs located at left or upper boundary of a picture, the two methods on QP prediction are same.

It is verified that the cross checked results are matched with those provided by the proponents in terms of BD-rate and number of bits for cu_qp_delta.

Slight difference on running time is noticed between the cross checked results and those provided by the proponents. A reason is considered to be running simulations over different platforms (workstation/cluster).

# References

- M. Coban, W. Chien, and M. Karczewicz, “CU-Level QP Prediction” JCTVC-E391, Geneva, Switzerland, March 2011.

- K. Sato, M. Budagavi, M. Coban, H. Aoki and X. Li, “Description of Core Experiment 4: Quantization”, JCTVC-E704, Geneva, Switzerland, March 2011.