| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F347 M20770 |
| --- | --- |

| Title: | A method of decoupling motion data reconstruction from entropy decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Vivienne Sze Yasutomo Matsuba Texas Instruments Inc. Toshiyasu Sugio Takahiro Nishi Panasonic Corporation Jianle Chen Tammy Lee Samsung Electronics Co., Ltd Wade Wan Yong Yu Broadcom Corporation | Tel: Email:Tel: Email:Tel: Email Tel: Email: Tel: Email: Tel: Email | +1 214.480.3816 zhou@ti.com +1 214-567-4005 sze@ti.com +1 214-480-3246 matsuba@ti.com +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com +82-31-279-8826 jianle.chen@samsung.com tammy.lee@samsung.com +1-949-926-6387 wwan@broadcom.com yongyu@broadcom.com |
| Source: | Texas Instruments Inc, Panasonic, Samsung, Broadcom |  |  |

_____________________________

# Abstract

# It has been reported that the current HM3.0 design has a parsing throughput issue due to interdependency between motion data reconstruction and entropy decoding. In this contribution, a consolidated method is proposed to address this issue. The proposed method include the following three tools: 1) truncated unary coding for merge/skip and AMVP MVP index with fixed maximum lengths signaled in slice header, 2) simplified CABAC/CAVLC coding for inter_pred_flag and ref_idx_lx, and 3) appending the zero vector to the merge/skip and AMVP MVP lists as an additional candidate.  Compared to HM3.0, the proposed method leads to an average loss of 0.6% in RA-HE, 0.7% in RA-LC, 0.9% in LB-HE and 1.2% in LB-LC, respectively. It is recommended to adopt this method into HM4 to completely decouple motion data reconstruction from entropy decoding, and provide a starting point for further study on this subject.

# Introduction

The current HM3.0 design is reported to have a parsing throughput issue due to interdependency between the motion data reconstruction and entropy decoding. Several contributions [3][4][5][6] have been submitted to provide solutions to this issue.  This contribution proposes a consolidated method which includes the following three tools:

- truncated unary coding for merge/skip and AMVP MVP index with fixed maximum lengths signaled in the slice header;

- simplified CABAC/CAVLC coding for inter_pred_flag and ref_idx_lx;

- appending the zero vector to merge/skip and AMVP MVP lists as an additional candidate.

# Algorithm Description

In this section a brief description of the proposed tools is provided.

## Truncated unary MVP index coding

The merge/skip MVP index (merge_idx) and AMVP MVP index (mvp_idx) are coded with truncated unary code with fixed maximum length signaled in the slice header. Furthermore, the merge_idx context selection was modified to only depend on binIdx. The total number of contexts used for merge_idx (i.e. 4) remains the same.

The truncated unary code tables for merge_idx and mvp_idx are provided in Table 1 and Table 2. Two slice-level syntax elements, namely max_active_merge_cands and max_active_amvp_cands, are used to signal which code tables are used for merge_idx and mvp_idx coding in the entire slice. For example, if max_active_merge_cands  = 3 and max_active_amvp_cands = 2, then the highlighted code tables in Table 1 and Table 2 are used.

Note that merge_idx or mvp_idx is not present in the bitstream when the maximum table size of the truncated unary code (max_active_merge_cands  or max_active_amvp_cands) used for a slice is set to one.

| Merge/skip MVP index (merge_idx) | max_active_merge_cands |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| 0 | N/A | 1 | 1 | 1 | 1 |
| 1 |  | 0 | 01 | 01 | 01 |
| 2 |  |  | 00 | 001 | 001 |
| 3 |  |  |  | 000 | 0001 |
| 4 |  |  |  |  | 0000 |

Table 1. truncated unary tables for CABAC binarization or variable length coding of merge_idx

| AMVP MVP index (mvp_idx) | max_active_amvp_cands |  |  |
| --- | --- | --- | --- |
|  | 1 | 2 | 3 |
| 0 | N/A | 1 | 1 |
| 1 |  | 0 | 01 |
| 2 |  |  | 00 |

Table 2, truncated unary tables for CABAC binarization or variable length coding of mvp_idx

## Simplified CABAC/CAVLC coding for inter_pred_flag and ref_idx_lx

To avoid the parsing problem highlighted above, the following two adaptive switching methods are disabled in this proposal.

- Adaptive context selection method for inter_pred_flag and ref_idx_lx (CABAC)
 It is changed such that ctxIdx is fixed to 0

- Adaptive combined coding method for inter_pred_flag and ref_idx_lx (CAVLC)
 It is changed such that use_combined_inter_pred_ref is fixed to true in B slice

Upper methods refer neighbor inter_pred_flag and ref_idx_lx. In Merge, motion vector, inter_pred_flag and ref_idx_lx are copied from Merge candidate. When an error occurs, such as packet loss, the temporal motion predictor represents an incorrect value.  In this situation, the removal process of duplicated candidates will remove some candidates that were not intended to be removed. As the result, not only the motion vector, but also inter_pred_flag and ref_idx_lx, which is selected by decode Merge index, will be calculated incorrectly. When these incorrect values are used to parse any syntax, this also causes a parsing problem.

## Appending zero vector to merge/skip and AMVP list

To improve the coding efficiency, the merge/skip MVP list construction is modified as follows after the HM3.0 merge/skip list derivation process:

- If the number of valid MVP candidates is greater than max_active_merge_cands, the lowest priority MVP candidates are discarded and the final list size is set equal to max_active_merge_cands. A low-priority MVP candidate has higher MVP index in the list. (Case A in Figure 1)

- If the number of valid MVP candidate is less than max_active_merge_cands, a zero vector is appended to the final list (case B in Figure 1). For B-slices, the zero vector is a bi-directional zero vector with ref_idx equal to zero for list0 and list 1.

Final list

Final list

MVP candidates

n

y

Valid MVP candidates

≥ max_active_merge_cands

HM3.0 Merge/skip MVP list construction

Discard low-priority MVP candidates to reduce the MVP list size to  max_active_merge_cands

Add zero MVP candidate to the MVP list if list is not empty

2

3

1

4

5×

1

2

Final MVP list

Valid MVP candidates = 4

max_active_merge_cands = 3

(Case A)

(case a)

MVP candidates

2

3

1

4

5

1

2

Final MVP list

Valid MVP candidates = 2

max_active_merge_cands = 3

(Case B)

(case b)

Zero MVP candidate

0

0

Figure 1. Diagram of modified merge/skip MVP list construction

The AMVP list construction process is modified in a similar fashion as follows:

- After HM3.0 AMVP list construction, a zero vector is appended to the end of list if it does not already exist in the list.

- If size of the AMVP list size is larger than max_active_amvp_cands, low-priority MVP candidates are discarded, and the final list size is set equal to max_active_amvp_cands. A low-priority MVP candidate has higher MVP index in the list.

# Test Settings and Conditions

The simulations of this document have used HM3.0 software, the common test conditions and reference configurations specified in [1] are followed. .The simulation platform at Texas Instruments is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of various frequencies. The computing platform at Panasonic is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The simulation platform at Samsung is LSF equipped with Xeon CPU of same frequencies and 64 bits Linux. The following table summarizes the tested settings and how cross-check is arranged among participants.

| Test | Tester 1 | Tester2 |
| --- | --- | --- |
| max_active_merge_cands = 5 max_active_amvp_cands = 3 | TI | Panasonic |
| max_active_merge_cands = 3 max_active_amvp_cands = 2 | TI | Samsung |

Table 3.  Summary of test settings

# Experimental results

The simulation results are summarized in Table 4 and Table 5. As shown in Table 4, for max_active_merge_cands = 3 and max_active_amvp_cands = 2 the proposed algorithm has a loss of 0.6% in RA-HE, 0.7% in RA-LC, 0.9% in LB-HE and 1.2% in LB-LC, respectively. Truncated unary table sizes have an impact on coding efficiency. As shown in Table 4 and Table 5,  setting of max_active_merge_cands = 3 and max_active_amvp_cands   =  2 provides an average improvement of  0.1% in RA-HE, 0.2% in RA-LC, 0.2% in LB-HE and 0.4% in LB-LC over the setting of  max_active_merge_cands = 5 and max_active_amvp_cands   =  3.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.5 | 0.6 | 0.6 | 0.6 | 0.6 |
| Class B | 0.7 | 0.4 | 0.3 | 0.8 | 0.7 | 0.7 |
| Class C | 0.7 | 0.7 | 0.8 | 0.7 | 0.7 | 0.8 |
| Class D | 0.6 | 0.7 | 0.7 | 0.6 | 0.5 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | 0.6 | 0.6 | 0.6 | 0.7 | 0.7 | 0.7 |
| Enc Time[%] | 108% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.4 | 0.3 | 0.9 | 0.6 | 0.6 |
| Class C | 0.7 | 0.5 | 0.3 | 0.7 | 0.5 | 0.4 |
| Class D | 0.6 | 0.4 | 0.7 | 0.7 | 0.5 | 0.7 |
| Class E | 2.2 | 2.7 | 1.9 | 2.7 | 2.7 | 2.9 |
| All | 0.9 | 0.9 | 0.7 | 1.2 | 0.9 | 1.0 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 97% |  |  | 103% |  |  |

Table 4.  Bit-rate increase of proposed algorithm relative to HM3.0 anchor for max_active_merge_cands = 3 and max_active_amvp_cands = 2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.5 | 0.4 | 0.7 | 0.8 | 0.7 |
| Class B | 0.7 | 0.4 | 0.4 | 1.0 | 0.8 | 0.8 |
| Class C | 0.8 | 0.8 | 0.8 | 1.0 | 0.9 | 0.9 |
| Class D | 0.8 | 0.9 | 0.8 | 0.9 | 0.8 | 0.8 |
| Class E |  |  |  |  |  |  |
| Overall | 0.7 | 0.6 | 0.6 | 0.9 | 0.8 | 0.8 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.8 | 0.6 | 0.2 | 1.2 | 1.0 | 1.0 |
| Class C | 0.8 | 0.7 | 0.7 | 1.0 | 0.8 | 0.8 |
| Class D | 0.8 | 0.8 | 1.1 | 1.1 | 1.0 | 0.9 |
| Class E | 2.4 | 2.2 | 1.5 | 3.4 | 3.0 | 3.4 |
| Overall | 1.1 | 1.0 | 0.8 | 1.6 | 1.3 | 1.4 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 5.  Bit-rate increase of proposed algorithm relative to HM3.0 anchor for max_active_merge_cands = 5 and max_active_amvp_cands = 3

# Conclusion and recommendations

A consolidated method is proposed to address the reported parsing throughput issues of the current HM design. The proposed method introduces a minimum amount of changes to the current design but completely remove the interdependency between motion data reconstruction and entropy decoding. It is therefore recommended to adopt this method into HM4 as a starting point for further study of this subject.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.

[3] M. Zhou, V. Sze, “A study on HM2.0 bitstream parsing and error resiliency issue”. JCT-VC Document, JCTVC-F068, Torino, IT, July 2011.

[4] T. Sugio, T. Nishi, “Parsing Robustness for Merge/AMVP”, JCT-VC Document, JCTVC-470, Torino, IT, July 2011.

[5] J. Chen, T. Lee, “MVP index parsing with fixed number of candidates”, JCT-VC Document, JCTVC-F402, Torino, IT, July 2011.

[6] T. Hellman, Y. Yu, “Decoder Performance Restrictions due to Merge/MVP Index Parsing”, JCT-VC Document, JCTVC-F341, Torino, IT, July 2011.

# Patent rights declaration(s)

Texas Instruments, Inc. , Panasonic Corporation , Samsung Electronics Co., Ltd  and Broadcom may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD texts

Related sections

### 7. 3. 3 Slice header syntax

### 7.4.3 Slice header semantics

### 7.3.7 Prediction unit syntax

7.4.7 Prediction unit semantics

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

##### 8.4.2.1.4 Derivation process for luma motion vector prediction

9.3.1.1 Initialisation process for context variables

9.3.3.1 Derivation process for ctxIdx

9.3.3.1.1 Assignment process of ctxIdxInc using neighbouring syntax elements

9.3.3.1.1.1 Derivation process of ctxIdxInc using left and above syntax elements

# Appendix

## Working Draft Text

### 3. 3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if (slice_type ! = I) { |  |
| max_active_merge_cands | u(3) |
| max_active_amvp_cands | u(2) |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

### 7.4.3 Slice header semantics

Add :

max_active_merge_cands specifies maximum code length of truncated unary table size used for merge_idx coding of the slice. The value of max_active_merge_cands is in the range of 1 to 5, inclusive.

max_active_amvp_cands specifies maximum code length of truncated unary table size used for mvp_idx coding of the slice. The value of max_active_amvp_cands is in the range of 1 to 3, inclusive.

### Syntax

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| If ( max_active_merge_cands > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && max_active_merge_cands > 1) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| If ( max_active_amvp_cands > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| If ( max_active_amvp_cands > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Decoding Process

### 7.4.7 Prediction unit semantics

combined_inter_pred_ref_idx specifies the values of inter_pred_flag and ref_idx_lX with X being 0, 1 or c. When it is not present, it shall be inferred to be equal to 0. [Ed. (WJ): needs to be improved]

The variables NumPredRefLC, NumPredRefL0 and NumPredRefL1 are specified as

NumPredRefLC = Min( 4, num_ref_idx_lc_active_minus1 + 1 )	(7-1)
NumPredRefL0 = Min( 2, num_ref_idx_l0_active_minus1 + 1 )	(7-2)
NumPredRefL1 = Min( 2, num_ref_idx_l1_active_minus1 + 1 )	(7-3)

The variable MaxPredRef is specified as

- If num_ref_idx_lc_active_minus1 is greater than 0, the following applies.

MaxPredRef = NumPredRefLC + NumPredRefL0 * NumPredRefL1	(7-4)

- Otherwise (combined list is not used), the following applies.

MaxPredRef = NumPredRefL0 + NumPredRefL0 * NumPredRefL1	(7-5)

The function use_combined_inter_pred_ref( x, y ) is specified as follows: [Ed. (WJ): move to syntax function section?]

use_combined_inter_pred_ref ( x, y ) = 	check_inter_pred_ref_neighbour( x-1, y ) &&

check_inter_pred_ref_neighbour( x, y-1 ) &&

check_inter_pred_ref_neighbour( x-1, y-1 )

The function check_inter_pred_ref_neighbour( x, y ) is specified as follows: [Ed. (WJ): move to syntax function section?]

if x or y is outside current slice, check_inter_pred_ref_neighbour( x, y ) = true

else if ( num_ref_idx_lc_active_minus1 <= 0 && ref_idx_l0[ x ][ y ] < 2 && ref_idx_l1[ x ][ y ] < 2 )

check_inter_pred_ref_neighbour( x, y ) = true

else if ( num_ref_idx_lc_active_minus1 > 0 && inter_pred_flag[ x ][ y ] != PredLC &&

ref_idx_l0[ x ][ y ] < 2 && ref_idx_l1[ x ][ y ] < 2 )

check_inter_pred_ref_neighbour( x, y ) = true

else if ( num_ref_idx_lc_active_minus1 > 0 && inter_pred_flag[ x ][ y ] == PredLC && ref_idx_lc[ x ][ y ] < 4 )

check_inter_pred_ref_neighbour( x, y ) = true

else

check_inter_pred_ref_neighbour( x, y ) = false

#### 9.3.1.1 Initialisation process for context variables

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 2 | 3..5 | 6..8 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0...2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_type | Error: Reference source not found | 0 | 1..4 | 5..9 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | merge_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Table 9- |  |  | 0..2 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9- |  | 0..5 0, 4 | 6..11 6, 10 |
|  | mvd_l0[ ][ ][ 0 ] | Error: Reference source not found |  | 0..6 | 14..20 |
|  | mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found |  |  | 14..20 |
|  | mvd_l0[ ][ ][ 1 ] | Error: Reference source not found |  | 7..13 | 21..27 |
|  | mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found |  |  | 21..27 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 | Error: Reference source not found |  | 0..1 | 2..3 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0..3 | 4..7 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_cb | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_cr | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| residual_coding() | last_significant_coeff_x | Error: Reference source not found | 0..40 | 41..81 | 82..122 |
|  | last_significant_coeff_y | Error: Reference source not found | 0..40 | 41..81 | 82..122 |
|  | significant_coeff_flag | needs update | Error: Reference source not found | Error: Reference source not found | Error: Reference source not found |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..79 | 80..159 | 160..239 |

Table 9-28 – Values of variable m and n for inter_pred_flag ctxIdx

| Initialisation variables | inter_pred_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | -2 | -5 | -9 |
| n | 58 | 70 | 85 |

Table 9-29 – Values of variable m and n for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialisation variables | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| m | -6 | -10 | -8 | -17 | 1 | 0 | -9 | -9 | -9 | -12 | -18 | 0 |
| n | 59 | 75 | 75 | 96 | 59 | 64 | 55 | 71 | 76 | 86 | 55 | 64 |

#### 9.3.3.1 Derivation process for ctxIdx

| Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Error: Reference source not found | 0 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause ) | na | na | na | na |
| split_coding_unit_flag | Error: Reference source not found | 0 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 6 | 0,1,2 (subclause ) | na | na | na | na |
| skip_flag | Error: Reference source not found | 0 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause ) | na | na | na | na |
| cu_qp_delta | Error: Reference source not found | 0 | 0 | 2 | 3 | 3 | 3 |
|  |  | 4 | 0 | 2 | 3 | 3 | 3 |
|  |  | 8 | 0 | 2 | 3 | 3 | 3 |
| pred_type | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | 1 | 2 | 3 | na |
|  |  | 5 | 0 | 1 | 2 | 3 | 4 |
| prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | 1 | 0 | 0 | 0 | 0 | 0 |
|  |  | 2 | 0 | 0 | 0 | 0 | 0 |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0,1,2 (subclause ) | 3 | 3 | 3 | na |
|  |  | 4 | 0,1,2 (subclause ) | 3 | 3 | 3 | na |
|  |  | 8 | 0,1,2 (subclause ) | 3 | 3 | 3 | na |
| merge_flag | Error: Reference source not found | 0 | 0,1,2 (subclause ) | na | na | na | na |
|  |  | 3 | 0,1,2 (subclause ) | na | na | na | na |
| merge_idx | Error: Reference source not found | 0 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
|  |  | 4 | 0,1,2,3 (subclause Error: Reference source not found) | 0,1,2,3 (subclause Error: Reference source not found) | 0,2,3 (subclause Error: Reference source not found) | 3 | na |
| inter_pred_flag | Table 9- | 0 | 0,1,2 (subclause ) | na | na | na | na |
| ref_idx_l0 | Table 9- | 0 | 0,1,2,3 (subclause ) | 4 | 4 | 4 | 4 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Table 9- | 6 | 0,1,2,3 (subclause ) | 4 | 4 | 4 | 4 |
| mvd_l0[ ][ ][ 0 ] | Error: Reference source not found | 0 | 0,1,2 (subclause ) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ] | Error: Reference source not found | 7 | 0,1,2 (subclause ) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found | 15 | 0,1,2 (subclause ) | 3 | 4 | 5 | 6 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found | 21 | 0,1,2 (subclause ) | 3 | 4 | 5 | 6 |
| mvp_idx_l0 | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | 1 |
| mvp_idx_l0, mvp_idx_l1, mvp_idx_lc | Error: Reference source not found | 2 | 0 | 1 | 1 | 1 | 1 |
| no_residual_data_flag | Error: Reference source not found | 0 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause ) | na | na | na | na |
| split_transform_flag | Error: Reference source not found | 0 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 4 | cuDepth + trafoDepth | na | na | na | na |
|  |  | 8 | cuDepth + trafoDepth | na | na | na | na |
| cbf_luma | Error: Reference source not found | 0 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause ) | na | na | na | na |
| cbf_cb ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause ) | na | na | na | na |
| cbf_cb ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |
| cbf_cr ( PredMode = = MODE_INTRA ) | Error: Reference source not found | 0 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 4 | 0,1,2,3 (subclause ) | na | na | na | na |
|  |  | 8 | 0,1,2,3 (subclause ) | na | na | na | na |
| cbf_cr ( PredMode != MODE_INTRA ) | Error: Reference source not found | 0 | trafoDepth | na | na | na | na |
|  |  | 4 | trafoDepth | na | na | na | na |
|  |  | 8 | trafoDepth | na | na | na | na |

##### 9.3.3.1.1 Assignment process of ctxIdxInc using neighbouring syntax elements

Subclause 9.3.3.1.1.1 specifies the derivation process of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_l0, mvd_l1, mvd_lc, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr.

###### 9.3.3.1.1.1 Derivation process of ctxIdxInc using left and above syntax elements

Input to this process is the luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture.

Output of this process is ctxIdxInc.

Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit with xL = xP − MinPuSize and yL = yP, the variable availableL is derived as follows.

- If the prediction unit covering location ( xL, yL ) is available, availableL is set equal to 1.

- Otherwise (the prediction unit covering location ( xL, yL ) is not available), availableL is set equal to 0.

Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit with xA = xP and yA = yP − MinPuSize, the variable availableA is derived as follows. If the prediction unit covering location ( xA, yA ) is available, availableA is set equal to 1.

- Otherwise (the prediction unit covering location ( xA, yA ) is not available), availableA is set equal to 0

The assignment of ctxIdxInc for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_lc, mvd_l0, mvd_l1, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr is specified in Table 9-50– Specifcation of ctxIdxInc using left and above syntax elements.

Table 9-50– Specifcation of ctxIdxInc using left and above syntax elements

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| alf_cu_flag | alf_cu_flag[ xL ][ yL ] | alf_cu_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| split_coding_unit_flag | cuDepth[ xL ][ yL ] > cuDepth[ xP ][ yP ] | cuDepth[ xA ][ yA ] > cuDepth[ xP ][ yP ] | ( condL && availableL ) + ( condA && availableA ) |
| skip_flag | skip_flag[ xL ][ yL ] | skip_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| merge_flag | merge_flag[ xL ][ yL ] | merge_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_chroma_pred_mode | IntraPredMode[ xL ][ yL ] = = 4 | IntraPredMode [ xA ][ yA ] = = 4 | ( condL && availableL ) + ( condA && availableA ) |
| inter_pred_flag | inter_pred_flag[ xL ][ yL ] | inter_pred_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) |
| ref_idx_lc | ref_idx_lc[ xL ][ yL ] > 0 | ref_idx_lc[ xA ][ yA ] > 0 | ( condL && availableL ) + ( condA && availableA ) << 1 |
| ref_idx_l0 | ref_idx_l0[ xL ][ yL ] > 0 | ref_idx_l0[ xA ][ yA ] > 0 | ( condL && availableL ) + ( condA && availableA ) << 1 |
| ref_idx_l1 | ref_idx_l1[ xL ][ yL ] > 0 | ref_idx_l1[ xA ][ yA ] > 0 | ( condL && availableL ) + ( condA && availableA ) << 1 |
| mvd_lc | mvd_lc[ xL ][ yL ] > 16 | mvd_lc[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |
| mvd_l0 | mvd_l0[ xL ][ yL ] > 16 | mvd_l0[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |
| mvd_l1 | mvd_l1[ xL ][ yL ] > 16 | mvd_l1[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |
| no_residual_data_flag | no_residual_data_flag[ xL ][ yL ] | no_residual_data_flag[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) << 1 |
| cbf_luma | cbf_luma[ xL ][ yL ] | cbf_luma[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) << 1 |
| cbf_cb | cbf_cb[ xL ][ yL ] | cbf_cb[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) << 1 |
| cbf_cr | cbf_cr[ xL ][ yL ] | cbf_cr[ xA ][ yA ] | ( condL && availableL ) + ( condA && availableA ) << 1 |

##### 8.4.2.1.1 Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(8-6)

predFlagLXCol = availableFlagLXCol	(8-7)

- The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

  - A, if availableFlagA is equal to 1

  - B, if availableFlagB is equal to 1

  - Col, if availableFlagCol is equal to 1

  - C, if availableFlagC is equal to 1

  - D, if availableFlagD is equal to 1

  - Z,  zero merging candidate (always available)

- If several merging candidates (excluding Z merging candidate) have the motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand max_active_merge_cands within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-8)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-9)

refIdxLX = refIdxLXN	(8-10)

predFlagLX = predFlagLXN	(8-11)

- If all availability flags availableFlagN (with N being replaced by A, B, Col, C, or D) are equal to 0, mergeIdx is set equal to 0 and The derivation process of  zero merging candidate mvLXZ, refIdxLXZ and predFlagLXZ (with X being replaced by 0 or 1) is are inferred derivated  as follows.

If slice_type is equal to P, the following applies.

mvLXZ[ 0 ] = 0	(8-12)

mvLXZ[ 1 ] = 0	(8-13)

refIdxL0Z = 0		  (8-14)

refIdxL1Z = -1	(8-15)

predFlagL0Z = 1	(8-16)

prefFlagL1Z = 0	(8-17)

Otherwise ( slice_type is equal to B ), the following applies.

mvLXZ[ 0 ] = 0	(8-18)

mvLXZ[ 1 ] = 0	(8-19)

refIdxL0Z = 0		(8-20)

refIdxL1Z = 0		(8-21)

predFlagL0Z = 1	(8-22)

prefFlagL1Z = 1	(8-23)

##### 8.4.2.1.4 Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor list, mvpListLX, is constructed of which elements are given as specified order:

- mvLXCol, if availableFlagLXCol is equal to 1

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXZ, zero vector, always available

- If several motion vectors have the same value, the motion vectors are removed from the list except the motion vector which has the smallest order in the mvpListLX.

- If the number of elements NumMVPCand( LX ) within the mvpListLX max_active_amvp_cands is equal to 1, mvpIdx is set equal to 0, otherwise, mvpIdx is set equal to mvp_idx_lX[ xP, yP ].

- The motion vector of mvListLX[ mvpIdx ] is assigned to mvpLX.