| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F352_r1 |
| --- | --- |

| Title: | CE10: Scaled orthogonal integer transforms supporting recursive factorization structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Joshi, Y. Reznik, J. Sole, M. Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled orthogonal integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM 3.0. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM 3.0. A multiplication-free implementation for the proposed transforms is also provided. The luma BD-rate performance of the proposed transforms is on average (over 6 configurations) +0.03% for normal QP range, +0.05% for high QP range and +0.25% for low QP range when compared to HM 3.0 transforms.

- Introduction

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled orthogonal integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM 3.0. A multiplication-free implementation for the proposed transforms is also provided. Compared to our previous contribution [1], we have made some changes to the way the inputs to the 1st and 2nd stages of the inverse transforms are clipped. The amount of storage needed to store the dequantization matrices has also been reduced.

# Transform specification

Figure 1 shows the 16-point transform that was proposed in [2]. The factors on the right hand side are normalization factors (not including). Thus, the transform is a scaled transform. In order for the transform to be orthogonal, the butterfly factors have to satisfy:

Z4=A42+B42, Z8=A82+B82=C82+D82, and

Z16=A162+B162=C162+D162=E162+F162=G162+H162.

Similar relationship holds for butterfly factors in the odd part of 32-point transform, that is:

Z32=A322+B322=C322+D322 and so on.

Figure 1: Proposed 16-point transform

It should be noted that the upper (even) part of the 16-point transform already contains the specification of the 8-point transforms. The choice of butterfly factors for the 8, 16 and 32-point transforms is shown in Tables 1-3.

| A4 | B4 | A8 | B8 | C8 | D8 |
| --- | --- | --- | --- | --- | --- |
| 17/64 | 41/64 | 111/128 | 22/128 | 94/128 | 63/128 |

| A16 | B16 | C16 | D16 | E16 | F16 | G16 | H16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 232/256 | 29/256 | 224/256 | 67/256 | 203/256 | 116/256 | 181/256 | 148/256 |

| A32 | C32 | E32 | G32 | I32 | K32 | M32 | O32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1013/1024 | 1003/1024 | 982/1024 | 958/1024 | 922/1024 | 859/1024 | 827/1024 | 757/1024 |
| B32 | D32 | F32 | H32 | J32 | L32 | N32 | P32 |
| 34/1024 | 146/1024 | 251/1024 | 331/1024 | 421/1024 | 538/1024 | 586/1024 | 674/1024 |

Table 1-3: Butterfly factors for the proposed 16 and 32 point transforms

Tables 1-3 specify the butterfly factors for a 32-point transform. Tables 1-2 specify the butterfly factors for a 16-point transform. Table 1 specifies the butterfly factors for an 8-point transform.

# Multiplierless implementation

The proposed transforms can be implemented using only shifts and additions. This is accomplished by replacing a pair of multiplies in a butterfly by shifts and additions. Table 4 specifies a multiplierless design for the proposed transforms.

| Pair of factors | 1st factor (A) | 2nd factor (B) | Algorithm: y = A*x; z = B*x; | Complexity |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Additions | Shifts |
| A4, B4 | 17 | 41 | x1 = x << 4; x2 = x << 3; x3 = x1 + x2; y = x + x1; z = x3 + x4; | 3 | 2 |
| A8, B8 | 111 | 22 | x1 = x << 3; x2 = x1 - x; x3 = x2 << 4; x4 = x2 << 1; y = x3 - x; z = x1 + x4; | 3 | 3 |
| C8, D8 | 94 | 63 | x1 = x << 5; x2 = x - x1; z = x1 - x2; y = z - x2; | 3 | 1 |
| A16, B16 | 232 | 29 | x1 = x << 2; x2 = x1 - x; x3 = x1 << 3; z = x3 - x2; y = z << 3; | 2 | 3 |
| C16, D16 | 224 | 67 | x1 = x << 5; x2 = x + x1; x3 = x2 << 1; z = x + x3; x5 = x1 << 3; y = x5 - x1; | 3 | 3 |
| E16, F16 | 203 | 116 | x2 = x + (x<<1); x3 = x << 5; x4 = x3 - x2; x5 = x4 << 3; z = x4 << 2; y = x5 - x4; | 3 | 4 |
| G16, H16 | 181 | 148 | x1 = x << 5; x2 = x << 2; x3 = x + x1; x4 = x2 + x3; z = x4 << 2; y = x3 + z; | 3 | 3 |
| A32, B32 | 1013 | 34 | x1 = x << 3; x2 = x << 1; x3 = x1 + x2; x4 = x << 10; x5 = x4 - x; x6 = x1 << 2; z = x2 + x6; y = x5 - x3; | 4 | 4 |
| C32, D32 | 1003 | 146 | x2 = x+(x<<6); x4 = x2+(x<<3); x6 = x+(x<<2); x8 = (x<<10)-x; x9 = x6<<2; y = x8-x9; z = x4<<1; | 5 | 6 |
| E32, F32 | 982 | 251 | x1 = x << 2; x2 = x << 4; x3 = x + x1; x4 = x2 << 4; z = x4 - x3; x6 = x2 - x4; x7 = z - x6; y = x7 << 1; | 4 | 4 |
| G32, H32 | 958 | 331 | x2 = x+(x<<5); x4 = (x<<9)-x2; x6 = x2+(x5<<2); z = x+(x6<<1); y = x4<<1; | 4 | 5 |
| I32, J32 | 922 | 421 | x2 = (x<<3)-x; x4 = (x2<<1)-x; x6 = x4+(x2<<6); x8 = (x2<<5)-x2; z = (x8<<1)-x4; y = x6<<1; | 5 | 6 |
| K32, L32 | 859 | 538 | x2 = x+(x<<5); x4 = (x<<8)-x2; y = (x4<<2)-x2; x7 = x+(x2<<3); x9 = x7+(x<<2); z = x9<<1); | 5 | 6 |
| M32, N32 | 827 | 586 | x2 = x+(x<<2); x4 = x2+(x<<8); x6 = x4+(x<<5); x8 = (x2<<6)-x2; y = x8+(x<<9); z = x6<<1; | 5 | 6 |
| O32, P32 | 757 | 674 | x2 = (x<<2)-x; x4 = x+(x2<<8); y = x4-(x2<<2); x7 = x+(x2<<7); x9 = x7-(x2<<4); z = x9<<1; | 5 | 6 |

Table 4: Multiplierless implementation of the proposed transforms

- Complexity measurement

## Arithmetic operations

Table 5 compares the complexity of the proposed transforms with the existing transforms in HM 3.0 in terms of multiplications and additions for 8, 16 and 32 point transforms. Here we have not counted the additions needed for adding the offset if bit-shifting is performed before or after a transform stage. Table 6 specifies the complexity of the proposed multiplierless transforms in terms of shifts and additions

|  |  | HM 3.0 | Proposed transform |
| --- | --- | --- | --- |
| 8-point | Additions | 448 | 416 |
|  | Multiplications | 352 | 192 |
| 16-point | Additions | 3200 | 2304 |
|  | Multiplications | 2752 | 1152 |
| 32-point | Additions | 23808 | 11904 |
|  | Multiplications | 21888 | 5888 |

Table 5: Complexity comparison of the proposed transforms with HM 2.0 transforms

| Transform | Complexity of factorization (for 1-D) | Complexity of multiplications (for 1-D) | Complexity of full multiplierless transform |
| --- | --- | --- | --- |
| 8-point | 26 adds + 12 mults (6 pairs) | 2 * (9 adds + 6 shifts) | 44 adds + 12 shifts |
| 16-point | 72 adds + 36 mults (18 pairs) | 2 * (26 adds + 23 shifts) | 124 adds + 46 shifts |
| 32-point | 186 adds + 92 mults (46 pairs) | 2 * (81 adds + 78 shifts) | 348 adds + 156 shifts |

Table 6: Transform complexity estimates in terms of shifts and additions.

## Dynamic range analysis

The bitdepth expansion due to each stage of the proposed 8, 16 and 32-point transforms is, in the worst case, 3, 4 and 5 bits, respectively. This is true both for forward transforms and inverse transforms.

The software framework provided by Cisco/TI was used to generate the bitdepths at various stages of the 2D inverse transform when a random matrix was used as an input to the 2D forward transform. This was repeated a large number of times. Table 7 shows the bitdepths at various stages of the 2D inverse transform.

To reduce the bitdepths at the inputs of inverse transform stages, we applied the method described in JCTVC-D071 and JCTVC-E411 in a modified form to the output of the dequantization stage and the output of the 1st inverse transform stage. The output of the dequantization stage was shifted to the right by B1 bits after adding a rounding offset. Then the resulting values were clipped to 18 bits. In our simulations, for all transform sizes, B1 was equal to 1. The output of the 1st inverse transform stage was shifted to the right by B2 bits after adding a rounding offset. Then the resulting values were clipped to 18 bits. In our simulations, B2 was set equal to 1, 3, and 3, for 8, 16, and 32 point transforms, respectively.

|  | IBDI ON |  |  |  | IBDI OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4×4 | 8×8 | 16×16 | 32×32 | 4×4 | 8×8 | 16×16 | 32×32 |
| Output of inverse quantization | 12 | 13 | 14 | 15 | 12 | 13 | 14 | 15 |
| Input to the1st stage of inverse transform | 16 | 18 | 18 | 18 | 16 | 18 | 18 | 18 |
| Input to the 2nd stage of inverse transform | 15 | 17 | 18 | 18 | 15 | 17 | 18 | 18 |
| Output of the 2nd stage of inverse transform | 11 | 12 | 12 | 12 | 9 | 9 | 9 | 9 |

Table 7: Bitdepths at various stages of inverse transform

We also tested a scenario for restricting the transpose buffer to 16 bits. In this case, B2 is increased by 2 bits to 3, 5, and 5, for 8, 16, and 32 point transforms, respectively. The resulting output is clipped to 16 bits and then left shifted by 2 bits.

## Quantization matrix storage

The Q_BITS and DQ_BITS for various transform sizes was adjusted so that the quantization matrix for block size of 32×32 contained quantization matrices for block sizes of 16×16 and 8×8. DQ_BITS was also adjusted so that the dequantization matrix elements would fit within a byte. Thus, the following additional storage is needed for dequantization matrices:

- A 12×12×6 matrix (864 bytes).

- LUTs of sizes 32, 16, and 8 (56 bytes).

Thus, the total additional storage on the decoder side is 920 bytes.

## IDCT pruning

## Number of sequential operations

The number of cascaded multipliers is 1 for 8-point transform and 2 for 16 and 32-point transforms. The number of sequential operations in the longest path is shown in Table 8 for each transform size.

| Transform size | Number of sequential operations in the longest path |
| --- | --- |
| 8 | 1 multiply and 4 additions |
| 16 | 2 multiplies and 5 additions |
| 32 | 2 multiplies and 7 additions |

Table 8: The number of sequential operations in the longest path for each transform size

# Results

The proposed transforms were implemented in HM3.0. Simulation results are presented for the common test conditions for normal QP range, low QP range and high QP range. Table 9 shows the results for normal QP range. On average, for 6 test configurations, the BD-rate is +0.03%. Table 10 shows the results for low QP range. On average, for 6 test configurations, the BD-rate is +0.25%. Table 11 shows the results for high QP range. On average, for 6 test configurations, the BD-rate is +0.05%.

Additional results for restricting the transpose buffer to 16 bits are presented in tables 12, 13, and 14 for normal QP range, low QP range, and high QP range, respectively. For class D sequences in RA-HE and LB-HE configuration somewhat erratic BD-rate results are observed for chroma component U.

Table 9: BD-rate results for normal QP range

Table 10: BD-rate results for low QP range

Table 11: BD-rate results for high QP range

Table 12: BD-rate results for normal QP range (16 bit transpose buffer)

Table 13: BD-rate results for low QP range (16 bit transpose buffer)

Table 14: BD-rate results for high QP range (16 bit transpose buffer)

# Conclusions

In this contribution, 8, 16, and 32 point transforms are proposed. These transforms are scaled orthogonal integer transforms supporting a recursive factorization structure. The 4-point transform is unchanged from HM 3.0. The 8, 16 and 32-point transforms use 192, 1152, and 5888 multiplications, respectively, compared to 352, 2752, and 21888 multiplications for the corresponding transforms in HM 3.0. A multiplication-free implementation for the proposed transforms is also provided. The luma BD-rate performance of the proposed transforms is on average (over 6 configurations), +0.03% for normal QP range, +0.05% for high QP range and +0.25% for low QP range when compared to HM 3.0 transforms. Additional results for restricting the transpose buffer size to 16 bits are also presented. The luma results are similar to the 18 bit case. Some erratic BD-rate behavior is observed for chroma component U for class D sequences in RA-HE and LB-HE configurations.

- References

- R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz, "CE10: Scaled orthogonal integer transforms supporting recursive factorization structure", JCTVC-E370, Geneva, CH, March 2011.

# Text for Working Draft

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).