| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F353 |
| --- | --- |

| Title: | CE1: Results of experiments B.1 and B.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jinho Lee Jin Soo Choi ETRI, 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr hykim5@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1 experiments B.1 and B.7 proposed in JCTVC-E142. The experiment results are summarized as follows. In experiment B.1, clipping the temporal motion vector component to 10 bits reportedly shows no bit rate increase for HE-RA, LC-RA, HE-LD, and LC-LD, and the required memory size for storing the temporal motion vector is reduced to 70.90%. In experiment B.7, clipping the temporal motion vector component to 8 bits reportedly shows 0.3%, 0.4%, 0.0%, and 0.0% bit rate increase for HE-RA, LC-RA, HE-LD, and LC-LD, respectively, and the required memory size for storing the temporal motion vector is reduced to 56.72%.

# Experiment results

The evaluation of experiments B.1 and B.7 proposed in JCTVC-E142 [1] was performed using the CE1 [2] software based on HM 3.0 software under common test conditions [3]. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. The experiments results are summarized in the following subsections and the detailed results can be found in the attached Excel files.

## Experiment B.1: 10 bits per temporal motion vector component

### Algorithm description

This method uses a dynamic range restriction of temporal motion vector in order to reduce the memory size and the memory access bandwidth. The proposed method simply clips the value of each motion vector component according to a predefined fixed bit-width before saving it in the memory. For this experiment, 10 bits per temporal motion vector component is evaluated.

Each motion vector component is clipped to the bit width of 10 bits by the following equations:

| clippedMV_X = min( 1<<9, max( -1<<9, MV_X ) ). | (1) |
| --- | --- |
| clippedMV_Y = min( 1<<9, max( -1<<9, MV_Y ) ). | (2) |

MV_X and MV_Y represent the value of motion vector component x and y, respectively. After clipping of each motion vector component, the clipped values of motion vector component, clippedMV_X and clippedMV_Y, are stored in the memory with fixed bit width. Then, the clipped motion vectors are used as a temporal motion vector predictor.

### Macro definitions

The following macro definitions are used to experiment B.1.

#define CANON_COMPRESS_ONLY_REF_FRAMES    1    ///< Don't compress I frames and non-reference frames

#define CANON_MV_SCALING_NEAREST_REF	 0    ///< MV Scaling to the nearest reference frame from L0

#define CANON_MV_SCALING_2_LISTS             0    ///< MV Scaling to nearest reference frame from L0 and L1

#define CANON_MV_SHIFTING			 0    ///< MV Shifting

#define CANON_CLIPPING                        10    ///< Bits per components 8, 10, 12, 16

### Results

Table 1 shows the summary result of experiment B.1 using the bit width of 10 bits. No coding loss is observed for four test conditions on average. Since this method simply clips the value of temporal motion vector component before it is stored in the memory, additional complexity impact on software runtime in encoding and decoding is negligible as shown in Table 1.

Table 1. Summary result of experiment B.1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.3 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | -0.2 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Experiment B.7: 8 bits per temporal motion vector component

### Algorithm description

This method uses a dynamic range restriction of temporal motion vector in order to reduce the memory size and the memory access bandwidth. The proposed method simply clips the value of each motion vector component according to a predefined fixed bit-width before saving it in the memory. For this experiment, 8 bits per temporal motion vector component is evaluated.

Each motion vector component is clipped to the bit width of 8 bits by the following equations:

| clippedMV_X = min( 1<<7, max( -1<<7, MV_X ) ). | (1) |
| --- | --- |
| clippedMV_Y = min( 1<<7, max( -1<<7, MV_Y ) ). | (2) |

MV_X and MV_Y represent the value of motion vector component x and y, respectively. After clipping of each motion vector component, the clipped values of motion vector component, clippedMV_X and clippedMV_Y, are stored in the memory with fixed bit width. Then, the clipped motion vectors are used as a temporal motion vector predictor.

### Macro definitions

The following macro definitions are used to experiment B.7.

#define CANON_COMPRESS_ONLY_REF_FRAMES    1    ///< Don't compress I frames and non-reference frames

#define CANON_MV_SCALING_NEAREST_REF	 0    ///< MV Scaling to the nearest reference frame from L0

#define CANON_MV_SCALING_2_LISTS             0    ///< MV Scaling to nearest reference frame from L0 and L1

#define CANON_MV_SHIFTING			 0    ///< MV Shifting

#define CANON_CLIPPING                          8    ///< Bits per components 8, 10, 12, 16

### Results

Table 2 shows the summary result of experiment B.7 using the bit width of 8 bits. About 0.2% of coding loss is observed for four test conditions on average. Experiment B.7 shows 0.3%, 0.4%, 0.0%, and 0.0% bit rate increase for HE-RA, LC-RA, HE-LD, and LC-LD, respectively. Since this method simply clips the value of temporal motion vector component before it is stored in the memory, additional complexity impact on software runtime in encoding and decoding is negligible as shown in Table 2.

Table 2. Summary result of experiment B.7

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.9 | 0.6 | 0.8 | 1.0 | 0.6 | 0.8 |
| Class B | 0.2 | 0.1 | 0.1 | 0.3 | 0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.3 | 0.2 | 0.3 | 0.4 | 0.2 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 | 0.1 | 0.0 | 0.3 |
| Class C | 0.0 | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.3 | 0.0 | 0.0 | -0.2 | -0.2 |
| Class E | 0.0 | 0.2 | 0.1 | -0.1 | -0.3 | 0.4 |
| Overall | 0.0 | 0.2 | 0.0 | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Compression factor of temporal motion vector data

Table 3 shows the required memory size for storing the temporal motion vector data and the memory size reduction by using the test B.1 and B.7.

Table 3. Required memory size (Mbits/frame) and memory reduction (%)

|  | Temporal motion vector storage |  |  |
| --- | --- | --- | --- |
|  | HM 3.0 | B.1 | B.7 |
| Class A | 3.71 | 2.56 | 2.05 |
| Class B | 1.81 | 1.30 | 1.04 |
| Class C | 0.31 | 0.25 | 0.20 |
| Class D | 0.07 | 0.06 | 0.05 |
| Class E | 0.78 | 0.58 | 0.46 |
| Average | 1.34 | 0.95 | 0.76 |
| Memory size ratio | 100% | 70.90% | 56.72% |
| Memory size reduction | 1x | 1.41x | 1.76x |

# Modified syntax, semantics, and decoding process

## Method 1: signaling the bit width of temporal motion vector in SPS

### Proposed syntax

The bit width information to restrict the dynamic range of motion vector component can be signalled in sequence parameter set as described in Table 4.

Table 4. Proposed syntax design in sequence parameter set

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| ..... |  |  |
| bit_width_temporal_motion_vector_minus8 | 0 | se(v) |
| ..... |  |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

### Proposed semantics

And associated semantics is as follows:

bit_width_temporal_motion_vector_minus8 specifies the bit width of the temporal motion vector component. When bit_width_temporal_motion_vector_minus8 is not present, it shall be inferred to be equal to 0. The bit width of temporal motion vector component is specified as follows:

TMVBitWidth = bit_width_temporal_motion_vector_minus8 + 8

### Proposed decoding process

- 8.4.2.1.6	Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable colPu and its position ( xPCol, yPCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(2-1)

yPRb = yP + nPSH	(2-2)

- The variable colPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the colPic.

- If colPu is coded in an intra prediction mode or colPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(2-3)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(2-4)

  - The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic.

- ( xPCol, yPCol ) is set equal to the top-left luma sample of the colPu relative to the top-left luma sample of the colPic.

The variables mvLXCol and availableFlagLXCol are derived as follows.

- If colPu is coded in an intra prediction mode or colPu is unavailable, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (colPu is not coded in an intra prediction mode and colPu is available), the variables mvCol and refIdxCol are derived as follows,

- If PredFlagL0[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL1[ xPCol ][ yPCol ] and RefIdxL1[ xPCol ][ yPCol ], respectively.

- Otherwise (PredFlagL0[ xPCol ][ yPCol ] is equal to 1), the following applies.

  - If PredFlagL1[ xPCol ][ yPCol ] is equal to 0, the motion vector mvCol and the reference index refIdxCol are set equal to MvL0[ xPCol ][ yPCol ] and RefIdxL0[ xPCol ][ yPCol ], respectively.

  - Otherwise (PredFlagL1[ xPCol ][ yPCol ] is equal to 1), the following applies.

- The following asignments are made with X being 0 or 1.

      - RefIdxColLX = RefIdxLX[ xPCol ][ yPCol ]

      - If PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( colPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( colPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxColLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

- If one of the following conditions is true, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL1[ xPCol ][ yPCol ], RefIdxColL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

- Otherwise, the motion vector mvCol, the reference index refIdxCol and ListCol are set equal to MvL0[ xPCol ][ yPCol ], RefIdxColL0 and L0, respectively.

and the variable availableFlagLXCol is set equal to 1 and the following applies.

- If PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXCol = Clip3( -1<<( TMVBitWidth -1), 1<<( TMVBitWidth -1)-1, mvCol) 	(2-5)

- Otherwise, mvLXCol is derived as scaled version of the motion vector mvCol as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(2-6)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(2-7)

mvLXCol’ = Clip3( -1<<( TMVBitWidth -1), 1<<( TMVBitWidth -1)-1, mvCol)	(8-82)

mvLXCol = ClipMv( ( DistScaleFactor * mvLXCol’ + 128 ) >> 8 )	(2-83)

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, ListCol ) )	(2-84)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(2-85)

## Method 2: defining the bit width of temporal motion vector according to level

### Proposed level limits

The maximum bit width of temporal motion vector component, MaxTMVBitWidth, is defined in level limits. TMVBitWidth in section 3.1.3 is set equal to MaxTMVBitWidth. Note that the following level number and values of MaxTMVBitWidth in Table 5 should be changed according to the future level definition.

Table 5. Example of level limits

| Level number | Max Temporal MV component bit depth MaxTMVBitWidth |
| --- | --- |
| 1 | 8 |
| 1b | 8 |
| 1.1 | 8 |
| 1.2 | 8 |
| 1.3 | 8 |
| 2 | 8 |
| 2.1 | 8 |
| 2.2 | 8 |
| 3 | 8 |
| 3.1 | 10 |
| 3.2 | 10 |
| 4 | 10 |
| 4.1 | 10 |
| 4.2 | 10 |
| 5 | 10 |
| 5.1 | 10 |

### Proposed decoding process

The process is same as in section 3.1.3.

## Method 3: defining one single bit width of temporal motion vector in both encoder and decoder

### Proposed decoding process

The process is same as in section 3.1.3 except defining TMVBitWidth set equal to 8 or 10.

# Additional result of combined experiments B.1 and B.7

When the method 2, defining the bit width of temporal motion vector according to level as in Table 5, is used, the coding efficiency of the method can be summarized as Table 6. Specifically, to set the fixed bit width of temporal motion vector according to level, level number and max frame size (MaxFS) in AVC level is used as shown in Table 7. The max frame size of Class A, B, C, D, E are 16000 MBs, 8160 MBs, 1560 MBs, 390 MBs, 3600MBs, respectively. Thus, Table 6 shows the results of combination of 8 bits per temporal motion vector component for Class C and D, and 10 bits per temporal motion vector component for Class A, B, and E. The combined experiments of B.1 and B.7 shows 0.1%, 0.1%, 0.0%, and 0.0% bit rate increase for HE-RA, LC-RA, HE-LD, and LC-LD, respectively.

Table 6. Summary result of combined experiments B.1 and B.7

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.3 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class D | 0.1 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | -0.2 | -0.1 |
| Class C | 0.0 | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.3 | 0.0 | 0.0 | -0.2 | -0.2 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Overall | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 7. Maximum temporal motion vector component bit depth according to frame size

| Sequence Class | Level number | Max frame size MaxFS (MBs) | Max Temporal MV component bit depth MaxTMVBitWidth |
| --- | --- | --- | --- |
| Class D | 1.1 | 396 | 8 |
| Class C | 2.2 | 1 620 | 8 |
| Class E | 3.1 | 3 600 | 10 |
| Class B | 4 | 8 192 | 10 |
| Class A | 5 | 22 080 | 10 |

# Conclusions

This contribution reports the results of CE1 experiments B.1 and B.7. In experiment B.1, no bit rate increase are shown for HE-RA, LC-RA, HE-LD, and LC-LD, and 70.90% of memory size is required to store the temporal motion vector. In experiment B.7, 0.3%, 0.4%, 0.0%, and 0.0% bit rate increase are shown for HE-RA, LC-RA, HE-LD, and LC-LD, respectively, and 56.72% of memory size is required to store the temporal motion vector. We recommend to define the fixed bit width of temporal motion vector component according to level as in the method 2.

# References

[1]	S.-C. Lim, H. Y. Kim, J. Kim, J. Lee, and J. S. Choi, “Dynamic range restriction of temporal motion vector,” JCTVC-E142, Mar. 2011.

[2]	J. Jung, P. Onno, and Y.-W. Huang, “Description of Core Experiment CE1: Motion Data Storage Reduction,” JCTVC-E701, Mar. 2011.

[3]	Frank Bossen, "Common test conditions and software reference configurations," JCTVC-E700, Mar. 2011.

# Patent rights declaration(s)

ETRI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).