| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F358 |
| --- | --- |

| Title: | Mode dependent filtering for intra predicted sample |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Sung-Chang Lim, Hui Yong Kim, Jin Soo Choi. Realistic Media Research Team, ETRI 161 Gajeong-dong, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents Mode-Dependent Filtering (MDF) for intra predicted samples. The proposed method applies mode-dependent filters on DC, planar, and angular predicted samples to improve the prediction continuity in the block boundary region. It is reported that the average BD-rate gain of the proposed method is 0.4% and 0.3% in Intra, High-Efficiency (HE) and Low-Complexity (LC), respectively.

# Introduction

LUT-based adaptive filtering on intra predicted samples was proposed in JCTVC-E069 [1] and adopted to HM3.0 [2]. DC predicted samples are filtered using three types of filter according to the block size as described in Figure 1. In details, Filter #A, Filter #B, and Filter #C are used for block size 4x4, 8x8, and 16x16, respectively.

Figure 1 : Filter types for DC predicted samples

# Proposed methods

The contribution proposes single filter type for DC predicted samples and filters for planar and angular predicted samples. At last, the mode-dependent filtering for intra predicted samples is proposed.

## Proposal 1: Single filter for DC predicted samples

As the filter for DC predicted samples, HM3.0 uses three types of filter according to the block size. In this contribution, single filter described in Figure 2 is proposed rather than three types of filter .

Figure 2 : Single filter for DC predicted samples

## Proposal 2: Filtering for planar predicted samples

The filtering process of DC predicted samples improves the prediction continuity in the boundary region. Planar prediction samples could have discontinuity in the boundary region because the samples are predicted by interpolation of four samples. Therefore, we applied same filter described in Figure 2 to the planar predicted samples.

## Proposal 3: Filtering for angular predicted samples

Angular prediction uses different neighboring samples according to the prediction modes. For examples, 'Vertical_right' modes in Figure 3 use above or above_right neighboring samples, and 'Horizontal_below' modes use left or left_below neighboring samples. So, for the 'Vertical_right' modes, the discontinuity could exist at the left block boundary. In the same reason, for the ‘Horizontal_below’ modes, the discontinuity could exist at the above block boundary.

Figure 3 : Angular prediction modes

To improve the prediction continuity in the boundary region, filtering process for angular predicted samples is proposed. Figure 4 shows filtering methods for 'Vertical_right' and 'Horizontal_below' modes. For the 'Vertical_right' modes, one left column samples of the predicted block are filtered using diagonal 2-tap filter as described in Figure 4 (a). For the 'Horizontal_below' modes, one above row samples of the predicted block are filtered using diagonal 2-tap filter as described in Figure 4 (b).

(a) Vertical_right                         (b) Horizontal_below

Figure 4 : Filter types for 'Vertical_right' and 'Horizontal_below' modes.

## Proposal 4: Mode-dependent filtering for intra predicted samples

Mode-Dependent Filtering (MDF) for intra predicted samples is proposed by combining the proposal 1, 2, and 3. Table 1 represents specification of MDF for intra predicted samples, and intraPostFilterType is represented below:

intraPostFilterType 1: DC, planar prediction modes

intraPostFilterType 2: Vertical_right modes

intraPostFilterType 3: Horizontal_below modes

Table 1 : Specification of MDF

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPostFilterType | 0 | 0 | 1 | 0 | 0 | 2 | 2 | 0 | 3 | 3 | 0 | 0 | 2 | 2 | 0 | 0 | 3 | 3 |
| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| intraPostFilterType | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 1 | - |

# Experimental results

The proposed method was implemented based on HM3.2rc1. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM. The proposals are cross-checked by Mitsubishi (JCTVC-F649).

The proposals are applied to 4x4~16x16 block sizes. Table 2,3,4, and 5 show the performance of proposal 1, 2, 3, and 4, respectively. Table 6 shows the performance of each sequence when the proposal 4 is applied.

Table 2 : Performance of proposal 1 (Single filter for DC predicted samples)

Table 3 : Performance of proposal 2 (Filtering for planar predicted samples)

Table 4 : Performance of proposal 3 (Filtering for angular predicted samples)

Table 5 : Performance of proposal 4 (Mode-dependent filtering for intra predicted samples)

Table 6 : Performance of each sequence for the proposal 4

# Conclusion

The mode-dependent filtering for intra predicted samples gives additive and constant gain for all sequences without complexity increase. We suggest mode-dependent filtering (proposal 4) to be adopted into the HM.

# Reference

- JCT-VC, “CE6.f: LUT-based adaptive filtering on intra prediction samples”, JCTVC-E069, JCT-VC Meeting, Geneva, Mar 2011.

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 3”, JCTVC-E603, JCT-VC Meeting, Geneva, Mar 2011.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex - Decoding process (Based on WD3_d8)

8.3.3.1.5 	Specification of Intra_DC prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction block size.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 2.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived as the following ordered steps:

- A variable DCVal predSamples[ x, y ] is derived as:

DCVal predSamples[ x, y ] = x'=0nS−1p[x', −1]+y'=0nS−1p[−1, y']+nS>>(k+1), with x, y = 0..nS-1	(8-1)
where k=log2(nS)

- Depending on the prediction block size nS, the following applies.

- If DCPredFilterFlag is equal to 1 and nS is less than 32, the following applies.

- If nS is equal to 4, the prediction samples predSamples[ x, y ] are derived as

predSamples[ 0, 0 ] = ( 3*p[ -1, 0 ] + 2*DCVal + 3*p[ 0, -1 ] + 4 ) >> 3	(8-2)
predSamples[ x, 0 ] = ( 3*p[ x, -1 ] + 5*DCVal + 4 ) >> 3, with x = 1..nS-1	(8-3)
predSamples[ 0, y ] = ( 3*p[ -1, y ] + 5*DCVal + 4 ) >> 3, with y = 1..nS-1	(8-4)
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1	(8-5)

- Otherwise, if nS is equal to 8, the prediction samples predSamples[ x, y ] are derived as

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 2*DCVal + 1*p[ 0, -1 ] + 2 ) >> 2	(8-6)
predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 3*DCVal + 2 ) >> 2, with x = 1..nS-1	(8-7)
predSamples[ 0, y ] = ( 1*p[ -1, y ] + 3*DCVal + 2 ) >> 2, with y = 1..nS-1	(8-8)
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1	(8-9)

- Otherwise, if nS is equal to 16, the prediction samples predSamples[ x, y ] are derived as

predSamples[ 0, 0 ] = ( 1*p[ -1, 0 ] + 6*DCVal + 1*p[ 0, -1 ] + 4 ) >> 3	(8-10)
predSamples[ x, 0 ] = ( 1*p[ x, -1 ] + 7*DCVal + 4 ) >> 3, with x = 1..nS-1	(8-11)
predSamples[ 0, y ] = ( 1*p[ -1, y ] + 7*DCVal + 4 ) >> 3, with y = 1..nS-1	(8-12)
predSamples[ x, y ] = DCVal, with x, y = 1..nS-1	(8-13)

- Otherwise, the prediction samples predSamples[ x, y ] are derived as

predSamples[ x, y ] = DCVal, with x, y = 0..nS-1

8.3.3.1.6

...

8.3.3.1.7

...

8.3.3.1.8

...

8.3.3.1.9	Filtering process of prediction samples

Inputs to this process are:

–	a variable IntraPredMode specifying the luma intra prediction mode

–	a variable nS specifying the prediction size,

–	neighboring samples p[ x, y ], with x, y = -1..nS,

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

Output of this process is:

–	filtered predicted samples predSamplesF[ x, y ], with x, y =0..nS-1.

Table 8-x – Specification of intraPostFilterType[ IntraPredMode ]

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPostFilterType | 0 | 0 | 1 | 0 | 0 | 2 | 2 | 0 | 3 | 3 | 0 | 0 | 2 | 2 | 0 | 0 | 3 | 3 |
| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| intraPostFilterType | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 3 | 1 | - |

If nS is less than 32, filtered sample array predSamplesF [ x, y ] with x, y = 0..nS-1, are derived as follows:

–	If intraPostFilterType [ IntraPredMode ] is equal to 1, the prediction samples predSamplesF[ x, y ] are derived as

predSamplesF[ 0, 0 ] = ( p[ -1, 0 ] + 2*predSamples[ 0, 0 ] + p[ 0, -1 ] + 2 ) >> 2		(8- x)

predSamplesF[ x, 0 ] = ( p[ x, -1 ] + 3*predSamples[ x, 0 ] + 2 ) >> 2 for x = 1..nS-1		(8- x)

predSamplesF[ 0, y ] = ( p[ -1, y ] + 3*predSamples[ 0, y ] + 2 ) >> 2 for y = 1..nS-1		(8- x)

predSamplesF[ x, y ] = predSamples[ x, y ] for x,y = 1..nS-1					(8- x)

–	Otherwise, if intraPostFilterType [ IntraPredMode ] is equal to 2, the prediction samples predSamplesF[ x, y ] are derived as

predSamplesF[ 0, y ] = ( p[ -1, y+1 ] + 3*predSamples[ 0, y ] + 2 ) >> 2 for y = 0..nS-1	(8- x)

predSamplesF[ x, y ] = predSamples[ x, y ] for x=1..nS-1, y = 0..nS-1			(8- x)

–	Otherwise, if intraPostFilterType [ IntraPredMode ] is equal to 3, the prediction samples predSamplesF[ x, y ] are derived as

predSamplesF[ x, 0 ] = ( p[ x+1, -1 ] + 3*predSamples[ x, 0 ] + 2 ) >> 2 for x = 0..nS-1	(8- x)

predSamplesF[ x, y ] = predSamples[ x, y ] for x=0..nS-1, y = 1..nS-1			(8- x)

–	Ohterwise, the prediction samples predSamplesF[ x, y ] are derived as

predSamplesF[ x, y ] = predSamples[ x, y ] for x,y = 0..nS-1					(8- x)